3GPP TSG-RAN WG3 Meeting #121bis	R3-235232

Xiamen, China, 9th – 13th October 2023

Agenda item:	 10.2.3

Source:		Lenovo

Title:		(TP for SON BLCR to TS38.423 and TS36.423) Discussion on SN RACH enhancements

Document for:		Discussion and Decision

1	Introduction

RAN3#117bis meeting [1] had agreements on SN RA report as below:

RAN3 has supported EN-DC, (NG)EN-DC, and NR-DC scenarios in Rel-17. No further work will be triggered in RAN3.

If RAN2 decides to support SN RA Report for EN-DC and (NG)EN-DC, UE should report the PScell identity outside the RACH report to help the network forward the report to the correct node.

RAN3#118 meeting [2] got more agreements:

SN should indicate the potential availability of RA report to the MN, MN can fetch the RA report and transfer it to SN.

Define a new class-2 message (e.g., RACH INDICATION) over Xn so that the S-NG-RAN can inform M-NG-RAN that one or more RACH reports are available at the UE.

The new Xn message should be non-UE associated.

RAN3#119bis meeting [3] achieved further agreements:

The Random Access Indication IE in the RACH INDICATION message is not needed.

The criticality of the RACH indication list IE in the RACH INDICATION message is “reject”.

RAN3#120 meeting [4] made further agreements:

Not consider the NG and S1 forwarding of RA report.

Include the “PSCell list” optionally in addition to the “RACH Report Container” in ACCESS AND MOBILITY INDICATION over Xn.

RAN3#121 meeting [5] further agreed:

It can be left to gNB implementation whether and how to forward the SN RA Reports in case there is no X2/Xn connectivity between the node retrieving the RA Report from the UE and the node serving the PSCells indicated by UE in the RA Report.

In this paper, we would further discuss enhancements for SN RA report.

2	Discussion

SN RA report is supported in (NG)EN-DC and NR-DC scenario. In R18, it has agreed to define a new class-2 message (i.e., RACH INDICATION) over X2/Xn so that the SN can inform MN that one or more RA reports are available at the UE. In this way, SN where RACH procedure occurs can indicate the potential availability of RA report to the MN, then, MN can fetch SN RA report from the UE using UE information Request/Response procedure, i.e. a flag for RA report request is included in the UE information Request message. Then NR formatted RA-ReportList is reported by the UE information Response message including unique PSCell identities (i.e. if a PSCell occurs more than once in NR RA-ReportList, it is recorded only once in the list of PSCell identities) outside the NR RA report container.

There is one case that inter-MN HO is triggered after RACH INDICATION message from SN to source MN is sent and before SN RA report is requested in Uu interface. How to enable target MN to understand there is one or more SN RA report stored at the UE to trigger retrieval of SN RA report needs to be discussed. Figure 1 shows two options:

- Option 1: Source MN indicates target MN that there is one or more SN RA report stored at the UE, e.g. an random access indication concerning existence of one or more SN RA report at the UE is included in the HO request message.

- Option 2: SN indicates target MN that there is one or more SN RA report stored at the UE, e.g. an random access indication concerning existence of one or more SN RA report at the UE is included in the SgNB addition request Acknowledge message or S-Node addition request Acknowledge message.

Figure 1: SN RA report retrieval

There is another case that inter-MN HO is triggered before RACH INDICATION message from SN to source MN is sent. Similarly, to enable target MN to understand there is one or more SN RA report stored at the UE to trigger retrieval of SN RA report, two potential options are shown in Figure 2:

- Option 1: Source MN indicates target MN that there is one or more SN RA report stored at the UE o, e.g. the source MN initiates the procedure by sending the RACH INDICATION message to target MN. The RACH INDICATION message contains information concerning one or more performed random access procedures or existence of one or more SN RA report at the UE, for example, gNB UE XnAP ID or eNB UE X2AP ID allocated at the target MN.

- Option 2: SN indicates target MN that there is one or more SN RA report stored at the UE, e.g. an random access indication concerning existence of one or more SN RA report at the UE is included in the SgNB addition request Acknowledge message or S-Node addition request Acknowledge message.

Figure 2: SN RA report retrieval

In general, for above two cases, option 2 is a common and easy solution, i.e. in case that inter-MN HO is triggered after RACH INDICATION message from SN to source MN is sent and before SN RA report is requested in Uu interface, or inter-MN HO is triggered before RACH INDICATION message from SN to source MN is sent, SN indicates target MN that there are occurrences of successful random access procedures in the SN, e.g. an random access indication concerning existence of one or more SN RA report at the UE is included in the SgNB addition request Acknowledge message or S-Node addition request Acknowledge message.

Proposal 1: In case that inter-MN HO is triggered after RACH INDICATION message from SN to source MN is sent and before SN RA report is requested in Uu interface, or inter-MN HO is triggered before RACH INDICATION message from SN to source MN is sent, SN indicates target MN that there are occurrences of successful random access procedures in the SN, e.g. an random access indication concerning existence of one or more SN RA report at the UE is included in the SgNB Addition Request Acknowledge message or S-Node Addition Request Acknowledge message.

3	Conclusion

In this contribution, enhancements for SN RA report are further discussed. We have the following proposals:

Proposal 1: In case that inter-MN HO is triggered after RACH INDICATION message from SN to source MN is sent and before SN RA report is requested in Uu interface, or inter-MN HO is triggered before RACH INDICATION message from SN to source MN is sent, SN indicates target MN that there are occurrences of successful random access procedures in the SN, e.g. an random access indication concerning existence of one or more SN RA report at the UE is included in the SgNB Addition Request Acknowledge message or S-Node Addition Request Acknowledge message.

Reference

[1] Chairman notes for RAN3#117bis meeting

[2] Chairman notes for RAN3#118 meeting

[3] Chairman notes for RAN3#119bis meeting

[4] Chairman notes for RAN3#120 meeting

[5] Chairman notes for RAN3#121 meeting

Annex 1: TP for TS38.423

9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PDU Session Resources Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >PDU Session Resources Admitted To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE nor the PDU Session Resource Setup Response Info – MN terminated IE is present in a PDU Session Resources Admitted to be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Setup Response Info – SN terminated | O |  | 9.2.1.6 |  | – |  |
| >>PDU Session Resource Setup Response Info – MN terminated | O |  | 9.2.1.8 |  | – |  |
| PDU Session Resources Not Admitted List | O |  |  |  | YES | ignore |
| >PDU Session Resources Not Admitted List – SN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 |  | – |  |
| >PDU Session Resources Not Admitted List – MN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message or the CG-CandidateList message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | reject |
| Admitted Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Location Information at S-NODE | O |  | Target Cell Global ID 9.2.3.25 | Contains information to support localisation of the UE | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Available fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 is enabled. | YES | ignore |
| Direct Forwarding Path Availability | O |  | ENUMERATED (direct path available, …) | Indicates direct forwarding path is available between the target S-NG-RAN node and source NG-RAN node for intra-system handover or between the target S-NG-RAN node and the source SN. | YES | ignore |
| SCG Activation Status | O |  | 9.2.3.155 |  | YES | ignore |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >Candidate PSCell List |  | 1 |  |  | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to PSCell change; T-SN provides it in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| Random access Indication | O |  | ENUMARATED (true, …) | Indicates occurrences of successful random access procedures. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

SRB-ID,

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

HashedUEIdentityIndexValue,

RaReportIndicationList,

SuccessfulPSCellChangeReportInformation,

CPACConfiguration,

RandomaccessIndication

FROM XnAP-IEs

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ManagementBasedMDTPLMNModificationList,

id-F1-terminatingIAB-donorIndicator,

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

id-HashedUEIdentityIndexValue,

id-RaReportIndicationList,

id-SuccessfulPSCellChangeReportInformation,

id-CPACConfiguration,

id-RandomaccessIndication,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells

FROM XnAP-Constants;

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SNodeAdditionRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequestAcknowledge-IEs}},

...

}

SNodeAdditionRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionAdmittedAddedAddReqAck	CRITICALITY ignore		TYPE PDUSessionAdmittedAddedAddReqAck		PRESENCE mandatory}|

{ ID id-PDUSessionNotAdmittedAddReqAck		CRITICALITY ignore		TYPE PDUSessionNotAdmittedAddReqAck		PRESENCE optional }|

{ ID id-SN-to-MN-Container					CRITICALITY reject		TYPE OCTET STRING								PRESENCE mandatory}|

{ ID id-admittedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes								PRESENCE optional }|

{ ID id-RRCConfigIndication					CRITICALITY reject		TYPE RRCConfigIndication						PRESENCE optional }|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore		TYPE CriticalityDiagnostics					PRESENCE optional }|

{ ID id-LocationInformationSN				CRITICALITY ignore		TYPE Target-CGI									PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo		PRESENCE optional }|

{ ID id-AvailableFastMCGRecoveryViaSRB3		CRITICALITY ignore		TYPE AvailableFastMCGRecoveryViaSRB3		PRESENCE optional }|

{ ID id-DirectForwardingPathAvailability	CRITICALITY ignore		TYPE DirectForwardingPathAvailability		PRESENCE optional }|

{ ID id-SCGActivationStatus					CRITICALITY ignore		TYPE SCGActivationStatus						PRESENCE optional	}|

{ ID id-CPAInformationAck					CRITICALITY ignore		TYPE CPAInformationAck							PRESENCE optional }|

{ ID id-SNMobilityInformation				CRITICALITY ignore		TYPE SNMobilityInformation					PRESENCE optional }|

{ ID id-RandomaccessIndication				CRITICALITY ignore		TYPE RandomaccessIndication					PRESENCE optional },

...

}

PDUSessionAdmittedAddedAddReqAck ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionAdmittedAddedAddReqAck-Item

PDUSessionAdmittedAddedAddReqAck-Item ::= SEQUENCE {

pduSessionId							PDUSession-ID,

sn-terminated			PDUSessionResourceSetupResponseInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupResponseInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE

-- nor the PDU Session Resource Setup Response Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.1.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionAdmittedAddedAddReqAck-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionAdmittedAddedAddReqAck-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionNotAdmittedAddReqAck ::= SEQUENCE {

pduSessionResourcesNotAdmitted-SNterminated		PDUSessionResourcesNotAdmitted-List OPTIONAL,

pduSessionResourcesNotAdmitted-MNterminated		PDUSessionResourcesNotAdmitted-List OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {PDUSessionNotAdmittedAddReqAck-ExtIEs} }	OPTIONAL,

...

}

PDUSessionNotAdmittedAddReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AvailableFastMCGRecoveryViaSRB3 ::= ENUMERATED {true, ...}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ChannelOccupancyTimePercentageUL																	ProtocolIE-ID ::= xx1

id-EnergyDetectionThresholdUL																		ProtocolIE-ID ::= xx2

id-SuccessfulPSCellChangeReportInformation															ProtocolIE-ID ::= xx3

id-PSCellListContainer																				ProtocolIE-ID ::= xx4

id-RadioResourceStatusNR-U																			ProtocolIE-ID ::= xx5

id-CPACConfiguration																				ProtocolIE-ID ::= xxx

id-RandomaccessIndication																				ProtocolIE-ID ::= xyz

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

Annex 2: TP for TS36.423

#### 9.1.4.2	SGNB ADDITION REQUEST ACKNOWLEDGE

This message is sent by the en-gNB to confirm the MeNB about the SgNB addition preparation.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| E-RABs Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Added Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>S1 DL GTP Tunnel Endpoint at the SgNB | M |  | GTP Tunnel Endpoint 9.2.1 | en-gNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – |  |
| >>>>SgNB UL GTP Tunnel Endpoint at PDCP | C-ifMCGpresent |  | GTP Tunnel Endpoint 9.2.1 | en-gNB endpoint of the X2-U transport bearer at PDCP. For delivery of UL PDCP PDUs. | – |  |
| >>>>RLC Mode | C-ifMCGpresent |  | RLC Mode 9.2.119 | Indicates the RLC mode. | – |  |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the transport bearer used for forwarding of DL PDUs | – |  |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the transport bearer used for forwarding of UL PDUs | – |  |
| >>>>Requested MCG E-RAB Level QoS Parameters | C-ifMCGandSCGpresent_GBRpresent |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the MCG. | – |  |
| >>>>UL Configuration | C-ifMCGandSCGpresent |  | 9.2.118 | Information about UL usage in the MeNB. | – |  |
| >>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the UL. | YES | ignore |
| >>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the DL. | YES | ignore |
| >>>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>>>Security Result | O |  | 9.2.182 |  | YES | ignore |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>SgNB DL GTP Tunnel Endpoint at SCG | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>Secondary SgNB DL GTP Tunnel Endpoint at SCG | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs in case of PDCP duplication | – |  |
| >>>>LCID | O |  | 9.2.138 | LCID for the primary path in case of PDCP duplication | YES | ignore |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| SgNB to MeNB Container | M |  | OCTET STRING | Includes the CG-Config message or the CG-CandidateList message, as defined in TS 38.331[31]. | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| Admitted split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |
| SgNB Resource Coordination Information | O |  | 9.2.117 | Information used to coordinate resources utilisation between en-gNB and MeNB. | YES | ignore |
| RRC config indication | O |  | 9.2.132 | Indicates the type of RRC configuration used at the en-gNB. | YES | reject |
| Location Information at SgNB | O |  | 9.2.142 | Contains information to support localisation of the UE | YES | ignore |
| Available fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 is enabled. | YES | ignore |
| Direct Forwarding Path Availability | O |  | ENUMERATED (direct path available, …) | Indicates direct forwarding path is available between the target en-gNB and source NG-RAN node for SA to EN-DC handover. | YES | ignore |
| SCG Activation Status | O |  | 9.2.178 |  | YES | ignore |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >Candidate PSCell List |  | 1 |  |  | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.111 |  | – |  |
| Random access Indication | O |  | ENUMARATED (true, …) | Indicates occurrences of successful random access procedures. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for X2AP.

--

-- **************************************************************

X2AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

RaReportIndicationList,

RandomaccessIndication

FROM X2AP-IEs

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-RaReportIndicationList,

id-RandomaccessIndication,

maxCellineNB,

maxnoofBearers,

maxnoofPDCP-SN,

maxFailedMeasObjects,

maxnoofCellIDforMDT,

maxnoofTAforMDT,

maxCellinengNB,

maxnoofCellIDforQMC,

maxnoofTAforQMC,

maxnoofPLMNforQMC,

maxnoofProtectedResourcePatterns,

maxnoNRcellsSpectrumSharingWithE-UTRA,

maxnoofNrCellBands,

maxnoofSSBAreas

FROM X2AP-Constants;

-- **************************************************************

--

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- SENB ADDITION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SeNBAdditionRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBAdditionRequestAcknowledge-IEs}},

...

}

SeNBAdditionRequestAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-E-RABs-Admitted-ToBeAdded-List		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-List	PRESENCE mandatory}|

{ ID id-E-RABs-NotAdmitted-List				CRITICALITY ignore	TYPE E-RAB-List							PRESENCE optional}|

{ ID id-SeNBtoMeNBContainer					CRITICALITY reject	TYPE SeNBtoMeNBContainer				PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional}|

{ ID id-GW-TransportLayerAddress			CRITICALITY ignore	TYPE TransportLayerAddress				PRESENCE optional}|

{ ID id-SIPTO-L-GW-TransportLayerAddress	CRITICALITY ignore	TYPE TransportLayerAddress				PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension				PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension				PRESENCE optional}|

{ ID id-Tunnel-Information-for-BBF			CRITICALITY ignore	TYPE TunnelInformation					PRESENCE optional}|

{ ID id-RandomaccessIndication				CRITICALITY ignore		TYPE RandomaccessIndication					PRESENCE optional },

...

}

E-RABs-Admitted-ToBeAdded-List ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeAdded-ItemIEs} }

E-RABs-Admitted-ToBeAdded-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeAdded-Item	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-Item 		PRESENCE mandatory}

}

E-RABs-Admitted-ToBeAdded-Item ::= CHOICE {

sCG-Bearer		E-RABs-Admitted-ToBeAdded-Item-SCG-Bearer,

split-Bearer	E-RABs-Admitted-ToBeAdded-Item-Split-Bearer,

...

}

E-RABs-Admitted-ToBeAdded-Item-SCG-Bearer ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

s1-DL-GTPtunnelEndpoint					GTPtunnelEndpoint,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																	OPTIONAL,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-Item-SCG-BearerExtIEs} }	OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-Item-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-SourceDLForwardingIPAddress				CRITICALITY ignore	EXTENSION TransportLayerAddress		PRESENCE optional},

...

}

E-RABs-Admitted-ToBeAdded-Item-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

seNB-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-Item-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-Item-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-SourceDLForwardingIPAddress			CRITICALITY ignore	EXTENSION TransportLayerAddress		PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

X2AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-RaReportIndicationList													ProtocolIE-ID ::= 999 -- to be allocated

id-PSCellListContainer													ProtocolIE-ID ::= 999 -- to be allocated

id-RandomaccessIndication																				ProtocolIE-ID ::= xyz

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>