3GPP TSG-RAN WG3 Meeting #121bis	R3-235233

Xiamen, China, 9th – 13th October 2023

Agenda item:	 10.2.5

Source:		Lenovo

Title:		(TP for SON BLCR to TS38.423) Discussion on MRO for NR-U

Document for:		Discussion and Decision

1	Introduction

In RAN3#117-e meeting [1], some agreements for MRO for NR-U were achieved:

- Add to RLF report indications concerning Measured RSSI and HOF due to consistent LBT failure.

- Send an LS to RAN2 requesting:

to support latest Measured RSSI and Indication of HOF due to consistent LBT failure in RLF report

to evaluate addition in RLF report of: Energy Detection Threshold, LBT configuration parameter lbt-FailureRecoveryConfig, Channel Occupancy in UL, Time duration for LBT during SpCell change

to support “Indication of consistent LBT failure” in RA report

- Keep existing failure type definition and detection to indicate RLF or HOF or PSCell change failure due to consistent LBT failure.

In RAN3#118 meeting [2], RAN3 replied RAN2’s LS in [3] on possibility on LBT-FailureRecoveryConfig.

RAN3#119 bis-e meeting further agreed [4]:

- Enhancements of RLF reports and RA reports are beneficial to separate mobility related errors from the LBT-related ones.

- RLF Report and RA report can be enhanced to include information concerning the LBT failures in RA procedures, the granularity and implementation details needs to be further discussed based on progress in RAN2.

In this paper, we would further discuss the details of MRO for NR-U.

2	Discussion

After RAN3#121 meeting, there is one FFS to be discussed

For HO execution, FFS whether the source node can deduce from UE reports – excluding UL LBT failures information - whether the target node suffered from DL LBT issues during the HO execution.

There a high possibility that before or during RACH procedure towards the target cell, uplink LBT in the UE side is successful, but downlink LBT fails in the target node, e.g. the target cell fails to send response e.g. RAR/MSG4/MSG B during the RACH procedure in the case that unlicensed channel resources in target cell are unavailable. When T304 expires, from UE point of view, it does not know DL LBT fails in the target PCell when it stores the RLF report. Based on RAN2’s agreements achieved until now, we can find that currently LBT related information (e.g. information of multiple RA procedures related to consistent LBT failures, latest measured RSSI of NR-U channel of one frequency if measRSSI-ReportConfig is configured for the corresponding frequency, or all the BWPs (including the first one) in which the UE experienced the consistent UL LBT failure prior to the successful completion of the RA) included in the RLF report mainly focused on UL LBT failure at UE side, but UL LBT related information can’t indicate DL LBT issue happened in target node during HO execution phase. Therefore, the source node can’t know the target node suffered from DL LBT issues during HO execution based on RLF report reported by the UE.

Observation 1: Based on RLF report reported by the UE, the source node can’t know the target node suffered from DL LBT issues during HO execution.

Since there is no DL LBT related information in the RLF report, the source node may execute failure cause analysis and modification for handover configuration as legacy e.g. update HO trigger threshold. However, actually the failure may be caused by setting inappropriate LBT configuration rather than handover configuration, in such a case, modifying handover configuration at network side is not essential and needs to be avoided. So, it is necessary for source node to distinguish DL LBT issue at target node from radio link quality issue or UL LBT issue.

As above analyzed, the source node can’t distinguish it based on current RLF report. We should further discuss whether to enable a UE-based solution or a NW-based solution to inform the source gNB of a handover about DL LBT issues occurring at the target gNB during a handover execution. As summarized in [5], there are two options on the table:

- Option 1: (UE-based solution) the UE adds in RLF report an indication of DL LBT failure causing absence of SSB transmissions during handover.

- Option 2: (NW-based solution) the target gNB sends to source gNB an indication of DL LBT failure during handover execution.

For Option1, it should be decided in RAN2, since it is a UE based solution. On the other hand, we don’t think absent SSB transmissions during handover execution can exactly mean the existence of DL LBT issues at network side, for example, there is a possibility that the UE can’t detect the DL SSB due to beam quality is quite poor.

In Option 2, the target node can indicate the source node that DL LBT failure occurred in the target cell, e.g. when the target node fails to respond the UE during the RACH procedure due to unlicensed channel resources in target cell are unavailble. Currently, HANDOVER REPORT message is transferred from target node to source node for MRO for a RLF case rather than a HOF case, i.e. HANDOVER REPORT message can’t be reused to indicate the source node that HOF happens due to DL LBT failure in the target cell. Therefore, it seems better to introduce a new Xn message from target node to source node to indicate DL LBT failure. In such a way, the source node may make proper failure analysis after receiving the DL LBT failure indication from the target node and the RLF report from the receiving node. Additionally, target node may optimize LBT configuration when DL LBT failure occurred, or when the source node informs the target node to do optimization for LBT configuration if the source node decides there is a LBT issue in the target node. In Option 2, even the source gNB has to collect RLF report from UE and indication of DL LBT failure from target node at different time for MRO analysis, it is NW implementation to do the correlation, there is no any specific issue. For Option 2, there is no need to involve other WGs, and the spec impact to XnAP is small.

Since it is a RAN3 raised issue, from RAN3’s perspective, the NW-based solution i.e. Option 2 is simpler and more straightforward without sending an LS to RAN2 and wating for RAN2’s further discussion and reply. In general, Option 2 is preferred than Option 1.

Proposal 1: In case that HOF happens due to DL LBT failure at target node, the target node can send an indication concerning DL LBT failure to the source node for MRO analysis e.g. via a new introduced Xn message.

Furthermore, RAN2#123 meeting agreed that:

Agreements:

1	Introduce a new field that counts the number of preamble transmissions blocked by LBT for the last BWP selected for the RA procedure. FFS how to solve the issue of no preamble transmission attempts transmitted in a selected beam due to LBT blockage.

2	All the BWPs (including the first one) in which the UE experienced the consistent UL LBT failure, prior to the successful completion of the RA, are included in the RA-Report.

3	UE log the RA-InformationCommon in the RLF-Report when the RLF cause is lbtFailure and the UE was performing random access at the moment of RLF.

4	The UE logs the following information in the SHR:

a.	The ra-InformationCommon including the new Rel.18 information (i.e. the number of UL LBT failures during HO, the info on the multiple BWPs in which consistent UL LBT failures was triggered), if T304 triggering conditions is fulfilled.

b.	FFS: The RSSI measurements of the frequencies associated to the source/target/neighbouring cells, if the measRSSI-ReportConfig is configured for those frequencies.

5	BWPs information included in the RA-Report can be included, within the list of attempted BWP(s), in chronological order of BWP selection.

We can observe that RAN2#123 meeting also agreed that nothing should be logged related to detected power/ED information. For other information to be reported by the UE e.g. lbt-FailureRecoveryConfig, or BWP information for all the BWPs in which the UE detected the consistent UL LBT failure, it is a left FFS issue in RAN2, now RAN2 has a on-going email discussion to discuss it, from RAN3 point of view, to avoid overlapped work, we can wait for RAN2 progress.

Proposal 2: Wait for RAN2 progress on the information that reported by the UE in the RLF report.

3	Conclusion

In this contribution, MRO for NR-U is discussed. We have the following observation and proposals:

Observation 1: Based on RLF report reported by the UE, the source node can’t know the target node suffered from DL LBT issues during HO execution.

Proposal 1: In case that HOF happens due to DL LBT failure at target node, the target node can send an indication concerning DL LBT failure to the source node for MRO analysis e.g. via a new introduced Xn message.

Proposal 2: Wait for RAN2 progress on the information that reported by the UE in the RLF report.

Reference

[1] Chairman notes for RAN3#117-e meeting

[2] Chairman notes for RAN3#118 meeting

[3] R3-226809, Response LS on Possibility on LBT-FailureRecoveryConfig, Nokia

[4] Chairman notes for RAN3#119bis-e meeting

[5] R3-231872, SoD on NR-U: CB # SONMDT5_NRU, Ericsson

Annex: TP for TS38.423

## 8.1	Elementary procedures

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| RACH Indication | RACH INDICATION |
| LBT Failure | LBT FAILURE |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.x	LBT Failure

#### 8.4.x.1	General

The purpose of the LBT failure procedure is to inform that DL LBT failure happens in NG-RAN node2.

#### 8.4.x.2	Successful Operation

Figure 8.4.x.2: LBT Failure procedure

NG-RAN node2 initiates the procedure by sending the LBT FAILURE message to NG-RAN node1, when NG-RAN node2 detects that one or more DL LBT failures occurred in NG-RAN node2.

Editor’s note: The procedure text can be updated further based on the agreements.

#### 8.4.x.3	Unsuccessful Operation

Not applicable.

#### 8.4.x.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.3.x	LBT FAILURE

This message is sent by the NG-RAN node2 to the NG-RAN node1 to indicate that one or more DL LBT failures occurred in NG-RAN node2.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node1. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

NotificationControlIndication,

RANPaging,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextConfirm,

RetrieveUEContextFailure,

XnUAddressIndication,

SecondaryRATDataUsageReport,

SNodeAdditionRequest,

SNodeAdditionRequestAcknowledge,

SNodeAdditionRequestReject,

SNodeReconfigurationComplete,

SNodeModificationRequest,

SNodeModificationRequestAcknowledge,

SNodeModificationRequestReject,

SNodeModificationRequired,

SNodeModificationConfirm,

SNodeModificationRefuse,

SNodeReleaseRequest,

SNodeReleaseRequestAcknowledge,

SNodeReleaseReject,

SNodeReleaseRequired,

SNodeReleaseConfirm,

SNodeCounterCheckRequest,

SNodeChangeRequired,

SNodeChangeConfirm,

SNodeChangeRefuse,

RRCTransfer,

XnRemovalRequest,

XnRemovalResponse,

XnRemovalFailure,

XnSetupRequest,

XnSetupResponse,

XnSetupFailure,

NGRANNodeConfigurationUpdate,

NGRANNodeConfigurationUpdateAcknowledge,

NGRANNodeConfigurationUpdateFailure,

E-UTRA-NR-CellResourceCoordinationRequest,

E-UTRA-NR-CellResourceCoordinationResponse,

ActivityNotification,

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ResetRequest,

ResetResponse,

ErrorIndication,

PrivateMessage,

DeactivateTrace,

TraceStart,

HandoverSuccess,

ConditionalHandoverCancel,

EarlyStatusTransfer,

FailureIndication,

HandoverReport,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

MobilityChangeRequest,

MobilityChangeAcknowledge,

MobilityChangeFailure,

AccessAndMobilityIndication,

CellTrafficTrace,

RANMulticastGroupPaging,

ScgFailureInformationReport,

ScgFailureTransfer,

F1CTrafficTransfer,

IABTransportMigrationManagementRequest,

IABTransportMigrationManagementResponse,

IABTransportMigrationManagementReject,

IABTransportMigrationModificationRequest,

IABTransportMigrationModificationResponse,

IABResourceCoordinationRequest,

IABResourceCoordinationResponse,

CPCCancel,

PartialUEContextTransfer,

PartialUEContextTransferAcknowledge,

PartialUEContextTransferFailure,

RachIndication,

LbtFailure

FROM XnAP-PDU-Contents

id-handoverPreparation,

id-sNStatusTransfer,

id-handoverCancel,

id-notificationControl,

id-retrieveUEContext,

id-rANPaging,

id-xnUAddressIndication,

id-uEContextRelease,

id-secondaryRATDataUsageReport,

id-sNGRANnodeAdditionPreparation,

id-sNGRANnodeReconfigurationCompletion,

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-mNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeCounterCheck,

id-sNGRANnodeChange,

id-activityNotification,

id-rRCTransfer,

id-xnRemoval,

id-xnSetup,

id-nGRANnodeConfigurationUpdate,

id-e-UTRA-NR-CellResourceCoordination,

id-cellActivation,

id-reset,

id-errorIndication,

id-privateMessage,

id-deactivateTrace,

id-traceStart,

id-handoverSuccess,

id-conditionalHandoverCancel,

id-earlyStatusTransfer,

id-failureIndication,

id-handoverReport,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-mobilitySettingsChange,

id-accessAndMobilityIndication,

id-cellTrafficTrace,

id-RANMulticastGroupPaging,

id-scgFailureInformationReport,

id-scgFailureTransfer,

id-f1CTrafficTransfer,

id-iABTransportMigrationManagement,

id-iABTransportMigrationModification,

id-iABResourceCoordination,

id-retrieveUEContextConfirm,

id-cPCCancel,

id-partialUEContextTransfer,

id-rachIndication,

id-lbtFailure

FROM XnAP-Constants;

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURES XNAP-ELEMENTARY-PROCEDURE ::= {

XNAP-ELEMENTARY-PROCEDURES-CLASS-1			|

XNAP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-1 XNAP-ELEMENTARY-PROCEDURE ::= {

handoverPreparation										|

retrieveUEContext										|

sNGRANnodeAdditionPreparation							|

mNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

sNGRANnodeinitiatedSNGRANnodeModificationPreparation	|

mNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeinitiatedSNGRANnodeRelease					|

sNGRANnodeChange										|

xnRemoval												|

xnSetup													|

nGRANnodeConfigurationUpdate							|

e-UTRA-NR-CellResourceCoordination						|

cellActivation											|

reset													|

resourceStatusReportingInitiation						|

mobilitySettingsChange									|

iABTransportMigrationManagement							|

iABTransportMigrationModification						|

iABResourceCoordination									|

partialUEContextTransfer								,

...

}

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport 			|

deactivateTrace							|

traceStart								|

handoverSuccess							|

conditionalHandoverCancel				|

earlyStatusTransfer						|

failureIndication						|

handoverReport							|

resourceStatusReporting					|

accessAndMobilityIndication				|

cellTrafficTrace							|

rANMulticastGroupPaging					|

scgFailureInformationReport				|

scgFailureTransfer						|

f1CTrafficTransfer						|

retrieveUEContextConfirm				|

cPCCancel								|

rachIndication							|

lbtFailure								,

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- LBT FAILURE

--

-- **************************************************************

LbtFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{LbtFailure-IEs}},

...

}

LbtFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory },

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-lbtFailure										ProcedureCode ::= xyz

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>