3GPP TSG-RAN WG3 #121bis	R3-235242

Xiamen, China, 9th – 13th Octorber 2023

Agenda Item:	12.2.2.4

Source:	Lenovo

Title: 	        (TP for TS37.480 TS38.470) Discussion on E1 F1 interface impact

Document for:	Discussion and Approval

# Introduction

In this paper, we further discuss the impact to E1 and F1 interfaces.

# Discussion

## 2.1	F1 interface impacts

One impact discussed in the last meeting and aim to be supported in Rel18 is the exchange of measured EC over F1 interface from gNB DU to gNB CU.

Work on the measured EC transmission from gNB-DU to gNB-CU over F1 in R18. Whether reusing the current F1AP procedures or defining new procedures needs to be further discussed.

With respect to whether a new F1AP procedure is needed to serve the purpose, we believe so in order to keep the same design principle as for Xn interface, i.e., modularized design to support AIML related information exchange over dedicated procedure. In addition, considering there will be potentially more information exchanged over F1 interface in future releases for other use cases (e.g., the AI for air interface use cases), it is also future proof to introduce AIML dedicated procedures.

RAN3 introduces new AIML dedicated procedures (e.g., DATA COLLECTION REQUEST/RESPONSE/UPDATE) over F1 interface for measured EC transmission from DU to CU.

## 2.2	E1 interface impacts

It has been discussed during the last meeting that whether EC measurement and collection is applicable between gNB CU-CP and gNB CU-UP. In our understanding, it depends on the deployment if CU-CP and CU-UP are deployed on different hardware. If so, EC of CU-UP can be easily measured, otherwise it could be tricky. But we still consider it possible to measure using smart software in virtualized deployment.

RAN3 considers it possible to measure EC from CU-UP and transmit to CU-CP depending on the real deployment.

Another aspect raised by companies is the UE traffic measurement and prediction exchange over E1 interface. We understand not much time left in this release. But considering many other controversial issues are not pursued in Rel18, there seems to be spare time discussing this, i.e., the UE traffic measurement and prediction exchange over E1 interface.

UE traffic metric takes the data volume for a UE as the starting point.

First of all, there is no doubt that the UE traffic (e.g., data volume) is measured by CU-UP. RAN3 needs to discuss which node will perform the AI inference and generate the predicted UE data volume, i.e., by CU-CP or CU-UP. Both are possible with different implications in our view:

- Option 1: CU-CP could predict UE data volume based on measured data volume report received from CU-UP

- Option 2: CU-UP could predict UE data volume based on its own measurement and send the predicted UE data volume to CU-CP upon request

Option 1 does not require any AI capability for CU-UP and allows CU-CP to predict the future UE data volume using information collected from other sources other than CU-UP, e.g., traffic characteristics from UE or core network. On the other hand, Option 1 would require CU-UP to provide the measured data volume to CU-CP as demanded for the sake of AI inference. Note that current Data Usage reporting procedure over E1 interface follows instruction from OAM. Thus, to support option 1 enhancement is needed for CU-UP to provide measured data volume as demanded by CU-CP, e.g., upon request from CU-CP.

Comparatively, option 2 would require CU-UP to perform AI inference, and CU-UP has better knowledge of actual UE data volume than CU-CP, which can be used for the prediction. To support option 2 enhancement is needed for CU-UP to send the predicted UE data volume to CU-CP upon request.

If time allows, RAN3 discusses if UE data volume can be predicted at CU-CP or CU-UP, or both.

If Proposal 2 or Proposal 3 can be agreed, it seems some request/response procedure (similar as the resource status request/response procedure) is necessary for the CU-UP to provide the measurement or prediction as CU-CP requested. Procedures similar as the new Class 1 and Class 2 procedures over Xn interface can be supported over E1 interface.

RAN3 introduces new AIML dedicated procedures (e.g., DATA COLLECTION REQUEST/RESPONSE/UPDATE) over E1 interface to transmit measurement/prediction from CU-UP to CU-CP.

# 3	Conclusion

Based on the discussion above, we propose:

Proposal 1	RAN3 introduces new AIML dedicated procedures (e.g., DATA COLLECTION REQUEST/RESPONSE/UPDATE) over F1 interface for measured EC transmission from DU to CU.

Proposal 2	RAN3 considers it possible to measure EC from CU-UP and transmit to CU-CP depending on the real deployment.

Proposal 3	If time allows, RAN3 discusses if UE data volume can be predicted at CU-CP or CU-UP, or both.

Proposal 4	RAN3 introduces new AIML dedicated procedures (e.g., DATA COLLECTION REQUEST/RESPONSE/UPDATE) over E1 interface to transmit measurement/prediction from CU-UP to CU-CP.

# TP for 37.480

**********************************************	    Change Starts    ******************************************************

## 5.1.x Data collection function

This function is used for the CU-CP to collect data (e.g., UE traffic load)  from CU-UP for network optimization. The network optimization could be based on AI/ML techniques.

## 6.x Data collection procedure

-	Data Collection Initiation procedure

-	Data Collection Reporting procedure

**********************************************	    Change Ends    ******************************************************

# TP for 38.470

**********************************************	    Change Starts    ******************************************************

## 5.2.x Data collection function

This function is used for the CU to collect data (e.g., Energy Cost) from DU for network optimization. The network optimization could be based on AI/ML techniques.

## 6.1.x Data collection procedure

-	Data Collection Initiation procedure

-	Data Collection Reporting procedure

**********************************************	    Change Ends    ******************************************************