3GPP TSG-RAN WG3 Meeting #121bis	R3-235259

Xiamen, CN, 09-13 Oct, 2023

Agenda Item:	10.2.2

Source:	Huawei

Title:	(TPs for SON BLCRs for TS 38.300 and TS 38.423) MRO on CPAC

Document for:	other

# 1	Introduction

This topic was discussed in RAN3#121 and the following agreements and open issues were listed:

- The MN performs the initial analysis when SCGFailureInformation is received from the UE e.g. whether it is CPA or CPC, if CPC whether it is MN initiated or SN initiated.

- For CPA or MN initiated CPC, there are two options on the table

- For SN initiated CPC,  there are two options on the table

The two different options are further elaborated in the SoD (R3-234540).

For CPA or MN initiated CPC,  there are two options:

Option 1:

  - if the suitable PSCell is one of the candidate PSCells provided by the MN, but not one of the candidate PSCells selected by the (candidate) target SN, MN informs the (candidate) target SN that wrong candidate target PSCell was selected by the (candidate) target SN via SCG Failure Information Report message.

The following information are included in the message from the MN to the (candidate) target SN:

    - Suitable PSCell ID

    - FFS: Indication whether a candidate PSCell is accepted by the (candidate) target (The candidate PSCell list recommended by the initiating node is already agreed).

  - Otherwise, the MN makes the optimization

Option 2:

  - MN always send the SCG Failure Information Report message to the last serving SN.

  - The last serving SN feedback if the problem is not brought by itself.

  - the MN makes the optimization

For SN initiated CPC,  there are two options:

Option 1:

  - The MN sends SCG failure information report message to source SN, and source SN performs root cause analysis

  - if the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the (candidate) target SN, the source SN sends message to the MN, then the MN sends message to the (candidate) target SN.

  - Otherwise, the source SN makes the optimization

    - SCG FAILURE TRANSFER is used from the source SN to the MN

    - SCG Failure Information Report message is used from the MN to the (candidate) target SN

- Option 2: the same as the option 2 for MN initiated CPC.

# 2	Discussion

According to Rel-17 MRO for PSCell change failure, upon reception of SCGFailureInformation, the MN may need to verify first that whether an intra-SN PSCell change has been initiated in the last serving SN via SCG Failure Information procedure just in case MRO for SN-initiated intra-SN PSCell change is required, and stores the SCGFailureInformation for the time needed to receive possible response from the last serving SN. This also applies to Rel-18 MRO for CPAC too when the MN receives the SCGFailureInformation from the UE. After last serving SN repsonds to the MN that it is not the node causing the failure, then the MN further decides the analysis strategy. It is noted that there is a lack of verification procedure initiated by the MN to the last serving SN in option 1. Despite in option 2 a verification procedure is initiated by the MN in the first place, MN always makes the optimization upon reception of feedback from last serving SN, ragrdless whether it is MN’s fault to cause the failure, i.e the wrong PSCell selection at the candidate SN in MN-initiated case, or the SN-initiated case.

Observation 1: It is noted that there is a lack of verification procedure initiated by the MN to the last serving SN in option 1. Despite in option 2 a verification procedure is initiated by the MN in the first place, whether MN always makes the optimization is questionable.

After assuring that it is not the intra-SN PSCell change causing the failure, RAN3 further studies the two options concerning whether MN or source SN further perfoms root cause analysis. We address this issue from the following aspects: the capability of root cause analysis, the most likely node causing the failure.

- The capability of root cause analysis

Whether MN supports root cause analysis depends on different cases, i.e., MN-initiated or SN initiated cases, in EN-DC or NR-DC. For MN-initiated cases in EN-DC and NR-DC, MN could anyway obtain the SCG measurement results at the time of SCG failure from SCGFailureInformation and at the time of recommending candidate PSCells to candidate SNs. However, in case of SN-initiated CPC in EN-DC, MN could not obatian neither of the SCG measurement results from those two moments, since LTE MN could not decode the SCG measurement result configured by SN NR. So in case of SN-initiated CPC, it is a safer solution for source SN as initiating node to perform root cause analysis.

Observation 2: it is a safer solution for the initiating node to perform root cause analysis in term of capability of root cause analysis.

- The most likely node causing the SCG failure

The possible approach to resolve the SCG failure related to CPAC, could be optimize the SN mobility configuration performed by the initiating node, optimize the RA configuration perfomed by target SN, and optimize the SCG link RLM timers performed by source SN, according to the specific failuretype indicated by UE. Amongst those, the initiatin node is the most likely node respondible for SCG failure and performing corresponding SN mobility optimization, e.g., controlling UE to execute PSCell change timely, the proper target PSCell. It is efficient for the initiating node to perform root cause analysis.

Observation 3: it is efficient for the initiating node as the most likely node causing the SCG failure to perform root cause analysis.

- It is feasible and efficient to let the initiating node perfom root cause analysis.

So in option 2 the following procedure is wrongfully designed: Once MN receives the feedback from the last serving SN, the MN makes optimization. In our understanding, it is better to combine the proposed option 1 and option 2, and reformulate the analysis mechanism as follows:

For CPA or MN initiated CPC, the analysis could be performed accordingly as below:

- MN always send the SCG Failure Information Report message to the last serving SN.

- The last serving SN feedback if the problem is not brought by itself.

- if the suitable PSCell is one of the candidate PSCells provided by the MN, but not one of the candidate PSCells selected by the (candidate) target SN, MN informs the (candidate) target SN that wrong candidate target PSCell was selected by the (candidate) target SN. The following information are included in the message from the MN to the (candidate) target SN:

- Otherwise, the MN makes the optimization

For SN initiated CPC, the analysis could be performed accordingly as below:

- MN always send the SCG Failure Information Report message to the last serving SN.

- The last serving SN feedback if the problem is not brought by itself.

- The MN sends SCG failure information report message to source SN, and source SN performs root cause analysis

if the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the (candidate) target SN, the source SN sends message to the MN, then the MN sends message to the (candidate) target SN.

- Otherwise, the source SN makes the optimization

Reformulate and agree that the solution use the initial step of option 2 (verify whether last serving caused the failure) followed by the steps for option 1

- After root cause analysis, if it is SN-initiated CPC, the source SN needs to notify MN of forwarding SCGFailureInformation to corresponding candidat SN. It is benefical to enhance the SCG Failure Transfer procedure with the indication of suitable PSCell ID and whether the wrong PSCell selection at the candidate SN or not to assist the MN of further SCGFailureInformation forwarding.

- When the MN forwards the SCGFailureInformation to the corresponding SN via SCG Failure Information Report procedure, the candidate SN could make the optimizations based on the existing information of SCGFailureInformation. No enxtra enhancement is required for SCG Failure Information Report procedure.

- Observation 4: No enxtra enhancement is required for SCG Failure Information Report procedure.

- It is benefical to enhance the SCG Failure Transfer procedure with the indication of suitable PSCell ID and whether the wrong PSCell selection at the candidate SN or not to assist the MN of further SCGFailureInformation forwarding.

# 3	Conclusion

This document observes:

Observation 1: It is noted that there is a lack of verification procedure initiated by the MN to the last serving SN in option 1. Despite in option 2 a verification procedure is initiated by the MN in the first place, whether MN always makes the optimization is questionable.

Observation 2: it is a safer solution for the initiating node to perform root cause analysis in term of capability of root cause analysis.

- Observation 3: it is efficient for the initiating node as the most likely node causing the SCG failure to perform root cause analysis. Observation 4: No enxtra enhancement is required for SCG Failure Information Report procedure.

This document proposes:

- It is feasible and efficient to let the initiating node perfom root cause analysis.

- Reformulate and agree that the solution use the initial step of option 2 (verify whether last serving caused the failure) followed by the steps for option 1

- It is benefical to enhance the SCG Failure Transfer procedure with the indication of suitable PSCell ID and whether the wrong PSCell selection at the candidate SN or not to assist the MN of further SCGFailureInformation forwarding.

# Annex1	Text Proposal to TS 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 15.5.2.6	PSCell change failure

For analysis of PSCell change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN which is responsible for the PSCell change failures. Otherwise, the MN transfers the SCG Failure Information to the last serving SN, which may respond using the SCG Failure Transfer procedure to inform the MN it is not responsible for the SCG failure. If needed, the MN transfer the SCG Failure Information to the source SN or to the candidate SN which the suitable PSCell belongs to.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex1	Text Proposal  to TS 38.423

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.2.30	SCG FAILURE TRANSFER

This message is sent by the S-NG-RAN node to the M-NG-RAN node to indicate that the root cause of the SCG failure may have occurred in the other nodes.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node. | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node. | YES | ignore |
| Suitable PSCell ID | O |  | NR CGI 9.2.2.7 | Indicates a suitable PSCell recommended by the node initiating CPAC but not prepared by the candidate SN, which the suitable PSCell belongs to. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- SCG FAILURE TRANSFER

--

-- **************************************************************

ScgFailureTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ScgFailureTransfer-IEs}},

...

}

ScgFailureTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory }|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory }|

{ ID id-SuitablePSCellCGI						CRITICALITY ignore		TYPE NR-CGI											PRESENCE optional },

...

}