3GPP TSG-RAN3 Meeting #121bis                                                              R3-235280

# Xiamen, China, Oct 9th – 13th, 2023

Agenda Item:	25.2.1

Source: 	Qualcomm Incorporated

Title: 		XR Enhancements for PDU Set Handling

Document for:	Discussion/Decision

# Introduction

Following highlighted XR WID objectives and related proposals will be discussed in this contribution.

Specify the enhancements related to capacity:

-	Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

-	Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2);

-	Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2);

-	Delay reporting of buffered data in uplink (RAN2);

-	Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

Specify the enhancements for XR Awareness:

-	Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2);

-	Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2);

-	Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3);

-	Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

# XR Enhancements

## PDU Set QoS handling:

During RAN3#121, it was agreed that

Add the PDU Set QoS Parameters in the QoS Flow Level QoS Parameters IE over NGAP, XnAP, F1AP and E1AP.

The PDU Set QoS parameters includes:

- PDU Set Delay Budget (PSDB).

- PDU Set Error Rate (PSER).

- PDU Set Integrated Handling Information (PSIHI).

PDU Set Information as below needs to be transferred over NG-U, Xn-U and F1-U:

- PDU Set Sequence Number.

- Indication of End PDU of the PDU Set.

- PDU Sequence Number within a PDU Set.

- PDU Set Size in bytes.

- PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

During PDU Session Resource setup/Modification procedure, SMF provides PDU Set QoS Parameters IE within QoS Flow Level QoS Parameters. gNB has to provide PDU Set QoS handling indication to SMF, so that SMF can inform UPF to provide PDU Set Information to CU-UP via GTP-U header information.

Thus, we propose

Upon receiving PDU Set QoS parameters from 5GC/SMF, gNB indicates to SMF whether “PDU Set QoS handling” is successfully setup or not in PDU Session Resource Setup Response Transfer and PDU Session Resource Modify Response Transfer IEs.

Upon receiving PDU Set QoS parameters within QoS Flow level QoS parameters from SMF, CU-CP provides same parameters to CU-UP via E1-AP Bearer Context Setup/Modification messages and to DU via F1-AP UE Context Setup/Modification procedures. Both CU-UP and DU have to provide PDU Set Support Indication to CU-CP, so that CU-CP knows whether PDU Set QoS parameters handling is supported by CU-UP and DU.

Upon receiving PDU Set QoS parameters from CU-CP, DU indicates to CU-CP whether PDU Set QoS handling is supported or not in F1AP-UE Context Setup/Modification Response messages.

Upon receiving PDU Set QoS parameters from CU-CP, CU-UP indicates to CU-CP whether PDU Set QoS handling is supported or not in E1AP-Bearer Context Setup/Modification Response messages.

RAN3#121 discussed following issue without any conclusion.

Continue to discuss the solution based on the possible cases as below:

Case1: NG-RAN node supports PDU set QoS parameters, and the corresponding QoS flow is setup successfully

Case2: NG-RAN node supports PDU set QoS parameters, but the corresponding QoS flow is setup as legacy QoS flow

Case3: NG-RAN node does not support PDU set QoS parameters, the corresponding QoS flow is setup as legacy QoS flow

Based on PDU Set QoS parameters received from SMF, if the gNB supports PDU set QoS parameters handling then it will setup resources for PDU set QoS handling in user plane , which is case 1 mentioned above.

For case 2, even if RAN supports PDU Set QoS handling but due to resource limitations RAN may not be able to setup PDU Set QoS parameters but instead RAN may decide to fallback to some other corresponding legacy 5QI. But in such situations, both RAN and 5GC provided PDU Set QoS Policies in PCC may be different. In such cases, we think RAN can reject PDU Set QoS Parameters and inform SMF about which legacy 5QI based QoS Flow can be setup. This will help SMF to make appropriate QoS policy decision based on PCC configured Operator Policy. This requires discussion in SA2 as well. Thus, we propose to send an LS to SA2 for case.

For case 2: “NG-RAN node supports PDU set QoS parameters, but the corresponding QoS flow is setup as legacy QoS flow”, RAN alone can’t make its independent decision and RAN has to inform 5GC. Send LS to SA2 to describe the scenario.

Due to congestion or other gNB internal issues if gNB can’t handle PDU Set QoS parameters anymore and when gNB is able handle PDU Set QoS parameters after sometime then gNB has to inform SMF though PDU Session Resource Notify procedure. This will ensure that both SMF and gNBs will remain in sync about PDU Set handling and SMF can indicate to UPF whether to continue to provide PDU Set Information in N3 GTP-U headers.

gNB indicates to SMF whether PDU Set QoS handling can’t be supported anymore or when PDU Set QoS handling can be supported again using NGAP PDU SESSION RESOURCE NOTIFY message.

## Unidirectional handling of PDU Set QoS parameters:

TS 23.501 [1] clause 5.7.7.2 states that "a QoS Flow is associated with only one PDU Set Delay Budget. The value of the PDU Set Delay Budget is the same in UL and DL." Equivalent text on PSER is included in clause 5.7.7.3.

It appears that this text was copied by SA2 from PDU QoS without any thought process. As UE support for detecting PDU Sets is optional, it may not always be possible to support UL PDU Set QoS handling. At the same time, it should still be possible to support PDU Set QoS in downlink direction even if a UE (or a UE application) does not support UL PDU Set QoS. This would however require the network to be able to only signal PSDB and PSER for downlink to NG-RAN so that RAN does not unnecessarily reserve resources for the uplink.

Furthermore, it is in general not reasonable to assume that PSDB and PSER would necessarily be the same in uplink and downlink for all XR applications (e.g., for virtual reality applications).

Similarly the PDU Set Integrated Handling Information may be different in UL and DL direction given that the application requirements in terms of whether all PDUs of a PDU Set are needed or not may vary in UL and DL direction.

Observation 1 : Even if UE does XR Application does not support UL PDU Set QoS handling, it should be possible for RAN to handle DL PDU Set QoS parameters received from 5GC.

Observation 2 : PDU Set QoS requirements may not be same for DL and UL.

Therefore, we proposes that SMF to signal PSDB, PSER and PSIHI separately for uplink and downlink and may also decide to only provide uplink or downlink values for these parameters.

RAN3 agrees that SMF signal PSDB, PSER and PSIHI separately for uplink and downlink and 5GC may also decide to only provide uplink or downlink values for these parameters.

## PDU Set QoS handling in non-Homogeneous deployment:

It is possible that PDU Set handling may not be supported by all gNBs in non-homogeneous deployment. During handover from source gNB not supporting PDU Set handling to another target gNB supporting PDU set handling, source gNB is not expected to provide PDU Set QoS parameters to target gNB. If target gNB is capable of supporting PDU Set QoS parameters then similar to MBS Support Indication IE, target gNB can indicate PDU Set support Indication to SMF in Path Switch Request Transfer IE within NGAP Path Switch Request message. Based on this indication, SMF may provide PDU Set QoS parameters to target gNB either in PATH SWITCH RESPONSE Message or by using PDU SESSION RESOURCE MODIFICATION Procedure after handover completion.

Observation 3 : In MBS work item, RAN3 introduced “MBS Support Indication” IE in Path Switch Request Transfer IE , Handover Request Acknowledge Transfer IE and PDU Session Resource Modify Response Transfer IE

Based on this, we propose

In Non-homogeneous deployment of PDU Set handling, during handover procedure, target gNB supporting PDU set handling always provides new “PDU Set support indication IE” to SMF in Path Switch Request Transfer IE of PATH SWITCH REQUEST Message.

In NGAP signalling messages, add “PDU Set support indication IE” in Handover Request Acknowledge Transfer IE and PDU Session Resource Modify Response Transfer IE.

SMF may provide PDU Set QoS parameters to target gNB in PATH SWITCH RESPONSE Message or using PDU SESSION RESOURCE MODIFICATION Procedure after handover completion.

# Summary

Based on above discussion, we have the following observations and proposals for XR PDU Set handling enhancements.

Proposal 1.	Upon receiving PDU Set QoS parameters from 5GC/SMF, gNB indicates to SMF whether “PDU Set QoS handling” is successfully setup or not in PDU Session Resource Setup Response Transfer and PDU Session Resource Modify Response Transfer IEs.

Proposal 2.	Upon receiving PDU Set QoS parameters from CU-CP, DU indicates to CU-CP whether PDU Set QoS handling is supported or not in F1AP-UE Context Setup/Modification Response messages.

Proposal 3.	Upon receiving PDU Set QoS parameters from CU-CP, CU-UP indicates to CU-CP whether PDU Set QoS handling is supported or not in E1AP-Bearer Context Setup/Modification Response messages.

Proposal 4.	For case 2: “NG-RAN node supports PDU set QoS parameters, but the corresponding QoS flow is setup as legacy QoS flow”, RAN alone can’t make its independent decision and RAN has to inform 5GC. Send LS to SA2 to describe the scenario.

Proposal 5.	gNB indicates to SMF whether PDU Set QoS handling can’t be supported anymore or when PDU Set QoS handling can be supported again using NGAP PDU SESSION RESOURCE NOTIFY message.

Observation 1 : Even if UE does XR Application does not support UL PDU Set QoS handling, it should be possible for RAN to handle DL PDU Set QoS parameters received from 5GC.

Observation 2 : PDU Set QoS requirements may not be same for DL and UL.

Proposal 6.	RAN3 agrees that SMF signal PSDB, PSER and PSIHI separately for uplink and downlink and 5GC may also decide to only provide uplink or downlink values for these parameters.

Observation 3 : In MBS work item, RAN3 introduced “MBS Support Indication” IE in Path Switch Request Transfer IE , Handover Request Acknowledge Transfer IE and PDU Session Resource Modify Response Transfer IE

Proposal 7.	In Non-homogeneous deployment of PDU Set handling, during handover procedure, target gNB supporting PDU set handling always provides new “PDU Set support indication IE” to SMF in Path Switch Request Transfer IE of PATH SWITCH REQUEST Message.

Proposal 8.	In NGAP signalling messages, add “PDU Set support indication IE” in Handover Request Acknowledge Transfer IE and PDU Session Resource Modify Response Transfer IE.

Proposal 9.	SMF may provide PDU Set QoS parameters to target gNB in PATH SWITCH RESPONSE Message or using PDU SESSION RESOURCE MODIFICATION Procedure after handover completion.

# References

[1] 23.501: System architecture for the 5G System

[2] S2-2309860: PDU Set QoS handling: Race conditions at mobility from supporting to non-supporting gNB