3GPP TSG-RAN WG3 Meeting 121bis     	R3-235294

Xiamen, China, Oct 9-13, 2023

Agenda Item:	16.4

Source:	NEC

Title:	Discussion on mode 1 RA for U2N remote UE under multi-path relay

Document for:	Decision

1	Introduction

During RAN3 #121 meeting, for U2N multi-path relay, it has been agreed that:

WA: RAN3 agrees to limit mode 1 resource allocation scheme for U2N remote UE under only in intra-DU scenario under multi-path relay. Check with SA2 (possible via an LS) to get feedback related to the handling of the UE location information in the multi-path operation. Discuss which node to decide the DL RRC messages sent over split SRB is transmitted over the direct path or the indirect path. Whether to reuse the existing SpCell ID for path addition in the F1AP signaling Setup the tunnels for different paths, focus on stage-3 details How to support and capture Scenario 2 in the BL CRs, e.g., stage-3 details Continue working on path addition/release

So in this contribution, we will further work on the following open issues.

2	Discussion

Firstly, during last RAN3 meeting, the WA of mode 1 RA scheme has been agreed. To us, during the discussion, the impact of this WA and potential RAN1/2  impact has been clearly filtered out. So it is natural to turn this WA into agreement.

Turn the following WA into agreement

- WA: RAN3 agrees to limit mode 1 resource allocation scheme for U2N remote UE under only in intra-DU scenario under multi-path relay.

Also, during last meeting’s discussion, once the WA turned into agreement, some RAN2 spec impact would be caused, as well as the related discussion of this issue was raised from RAN2, so they also ask official feedback from RAN3 if we can get any consensus. So it is suggested to adopt the following reply LS to RAN2 for their incoming LS[1], which is captured in the annex section.

RAN3 should inform the agreement about mode 1 RA to U2N remote UE in the reply LS towards RAN2

Another issue is about how to inform a gNB-DU to release a specific path with both direct and indirect path being used in the same gNB-DU. According to the discussion, three alternatives were given out:

- Alt1: The path is identified by an ENUMERATED IE with value “direct” and “indirect”

- Alt2: The direct path is identified by PCell ID, and indirect path is identified by Relay UE ID

- The path is identified by a path ID

- Legacy signalling of path switch and Pcell change can be reused without explicit spec impact

From our understanding, for indirect path release, there is path switch procedure in rel-17, so legacy signalling can be reused without relay UE ID. Also for direct path release, legacy PCell change signalling from direct to indirect path switch can also be reused.

Use legacy signalling for path release to inform a gNB-DU without relay UE ID or PCell ID indication.

3	Conclusion and Proposals

In this contribution, we have discussed the remaining issues of U2N multi-path relay, and the following observations and proposals are given out:

Proposal 1	Turn the following WA into agreement

-	WA: RAN3 agrees to limit mode 1 resource allocation scheme for U2N remote UE under only in intra-DU scenario under multi-path relay.

Proposal 2	RAN3 should inform the agreement about mode 1 RA to U2N remote UE in the reply LS towards RAN2

Proposal 3	Use legacy signalling for path release to inform a gNB-DU without relay UE ID or PCell ID indication.

4	References

[1]	R2-2306781_LS to RAN3 on mode 1 scheduling in inter-DU multi-path case	NEC

5	Annex

3GPP TSG-RAN WG3 Meeting 121bis R3-230xxx Toulouse, France, Aug 21-25, 2023 Title: [draft]Reply LS to RAN2 on mode 1 scheduling in inter-DU multi-path Response to: R2-2306781_LS to RAN3 on mode 1 scheduling in inter-DU multi-path case Release: Rel 18 Work Item: NR_SL_relay_enh-Core Source: NEC [to be TSG RAN WG3] To: TSG RAN WG2 Cc: Contact person: Boyuan Zhang zhang_boyuan@nec.cn Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: 1 Overall description RAN3 thanks RAN2 for their LS on the agreement of mode 1 scheduling for U2N Remote UE in inter-DU multi-path case. During RAN3 #121 meeting, RAN3 has considered RAN2’s agreement and further agree that mode 1 scheduling for U2N Remote UE should only be supported in intra-DU multi-path case. 2 Actions To RAN2 ACTION: RAN3 kindly asks RAN2 to take above information into account for their future work. 3 Dates of next RAN3 meetings Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/ RAN3#122 13-17 Nov 2023 Chicago, USA