3GPP TSG-RAN WG3#121bis	　　　　　　　　　　　　　　　　　R3-235296

Xiamen, China, 9th – 13th Oct 2023

Agenda Item:  11.3

Source:       NEC

Title:		Remaining issues on support of NR-DC

Document for:	Discussion

Introduction

In the RAN3#121 meeting, we achieved some progress that:

For s-based QoE configuration received by MN

- MN sends the QoE configuration via SRB1

- QoE reports can be sent via SRB4 or SRB5

For management based QoE, there is no need for SN to explicitly indicate to the MN whether it wants to configure the UE directly via SRB3 or via a container and SRB1.

WA: The transparent reporting for RVQoE over RRC is not supported.

WA: RRC Transfer procedure can be considered to transfer the RVQoE report between MN and SN subjects to validation of whether the transparent reporting for RVQoE over RRC is precluded or not.

Define two different reporting leg indications for QoE and RVQoE.

For a UE in NR-DC, each legacy QoE configuration can have only one corresponding RVQoE configuration when needed.

This contribution discusses the problems left in NR-DC QoE.

Discussion

Discussion on if the QoE/RVQoE report can be transparently sent in SRB4

In the RAN3#120 meeting, we got an agreement that:

Management-based QoE configuration is received directly by the SN from the OAM， the SN can explicitly indicate to the MN whether it is going to receive the QoE/RVQoE reports via the SRB5 or SRB4，whether it is transparently via SRB4 needs further discussion.

In the RAN3#121 meeting, "Is there a need for the UE to report QoE/RVQoE transparently via SRB4?" is discussed, and two working assumptions are achieved:

WA: The transparent reporting for RVQoE over RRC is not supported.

WA: RRC Transfer procedure can be considered to transfer the RVQoE report between MN and SN subjects to validation of whether the transparent reporting for RVQoE over RRC is precluded or not.

For each QoE/RVQoE report (i.e., MeasReportAppLayer IE in 38.331), an RRC ID (i.e., measConfigAppLayerId) is included to identify the different QoE/RVQoE configurations and an RRC ID are only correlated to one QoE reference for a UE. Recalling the agreements, "The MN is responsible for RRC ID allocation for m-based sessions configured by the MN or SN and notifies the allocated RRC ID(s) to the SN." The MN handles the assignment of the RRC IDs, and the MN shall inform the assignment results to the SN.

RVQoE report analysis:

For RVQoE, if the SN receives a management-based QoE configuration directly from the OAM, the SN sends a request to MN indicating it wants to receive the QoE/RVQoE reports via SRB4. The MN accepts this request and assigns the RRC ID to the SN in the response message. Then, the SN can correlate the RRC ID to the QoE reference for a UE. Then, the UE sends the RVQoE RRC container via SRB4, and the MN forwards this RRC container of the RVQoE report to the SN by RRC Transfer message. So, the transparent report for RVQoE via SRB4 is workable. However, the RVQoE transparent report needs RAN2 support since RAN2 has to design the RVQoE report RRC container from UE to MN.

Of course, the non-transparent method of RVQoE reporting via SRB4 is also available. The MN reads the QoE/RVQoE reports and finds that the RRC ID is related to the SN. Then, the MN forwards this report to the SN by RRC Transfer message, and this procedure has no impact on RAN2.

Observation 1: The transparent report for RVQoE via SRB4 is workable but has a RAN2 impact, while the non-transparent report for RVQoE via SRB4 is workable and has no impact.

Observation 2: The MN can deliver the RVQoE report to SN via RRC Transfer message regardless of whether this report is in an RRC container.

Proposal 1: Turning the "WA: The transparent reporting for RVQoE over RRC is not supported" as an agreement.

Proposal 2: Turning the "WA: RRC Transfer procedure can be considered to transfer the RVQoE report between MN and SN subjects to validation of whether the transparent reporting for RVQoE over RRC is precluded or not" as an agreement.

QoE report analysis:

MCE collects the QoE reports, and it does not matter whether the MN or SN transfers QoE reports to MCE.

If the SN receives a management-based QoE configuration directly from the OAM, the SN is confirmed to configure the QoE and report the QoE measurement via SRB4. The UE can send the QoE report transparently via SRB4 to the MN, and the MN forwards it to the SN. The SN knows the MCE IP address and delivers it to MCE. However, it has RAN2 impacts, like the RVQoE report discussed above.

The UE can send the QoE report directly to the MN via SRB4, and the MN can also send it to the MCE once the MN knows the MCE IP address. QMC Initial Coordination Request IE can contain the MAC IP address from the SN to the MN for QMC initialization, so the MN also knows the MAC IP address.

Observation 3: Both QoE reports transparently via SRB4 and non-transparently via SRB4 are workable, but the transparent method has RAN2 impacts.

Proposal 3: If the SN receives a management-based QoE configuration directly from the OAM, the QoE report can be delivered to the MN directly, and the MN will forward this QoE report to the MCE directly.

Discussion on whether the first RVQoE report shall be delivered to the RVQoE configuration node

There are FFS left from the last meeting:

If the MN configures RVQoE, the first RVQoE report shall be delivered to the MN via SRB4.

If the SN configures RVQoE, the first RVQoE report shall be delivered to the SN.

First of all, we list related history agreements below:

The MN can receive RVQoE reports directly from the UE.

The SN can receive RVQoE reports directly from the UE.

UE can send RVQoE report to the MN, the MN then forward the RVQoE report to the SN if needed, and vice versa

To determine which node(s) provide the bearers carrying an application session, a node can configure RVQoE measurements at a UE in NR-DC:

- For the first RVQoE configuration, it is blindly configured by MN or SN.

- From the PDU session ID and QFI in the first RVQoE report this node determines which node(s) provide the bearer(s) associated to the corresponding application session.

- After the node determines which node(s) carry the session including bearer type change, the RVQoE configuration may be modified

MN or SN blindly configures the first RVQoE configuration. The PDU session ID and QFI of the first RVQoE report can help to determine which node provides the bearer associated with the corresponding application session. The MN has hosting PDCP and SDAP entities, which determine the bearer nodes based on the PDU session ID and QFI. So, only the MN can determine which node(s) carry the application session.

Observation 4: Only MN can use the PDU session ID and QFI of the first RVQoE report to determine which node(s) carry the application session.

Proposal 4: For both MN configures RVQoE and the SN configures RVQoE, the first RVQoE report is suggested to be sent to the MN.

The Coordination of the RVQoE-configuring node receives the RVQoE report and determines that the non-RVQoE-configuring node provides the bearer(s) for the application session.

This discussion comes from the following agreements:

When the RVQoE-configuring node receives an RVQoE report and determines that the non-RVQoE-configuring node provides the bearer(s) for the application session, the RVQoE-configuring node indicates that to the non-RVQoE-configuring node. FFS whether the indication is explicit or implicit.

The RVQoE-configuring node configures the RVQoE configuration, and the first RVQoE measurement is reported to the RVQoE-configuring node. Then, the RVQoE-configuring node checks the PDU session ID and QFI from the first RVQoE report and determines that the non-RVQoE-configuring node provides the bearer(s) associated with the corresponding application session. How do we indicate to the non-RVQoE-configuring node that the following RVQoE reports corresponding to the RRC ID/PDU session ID should be reported to the non-RVQoE-configuring node instead of the RVQoE-configuring node? In the last meeting, two approaches were posted: the implicit indication via forwarding the RVQoE report and the PDU session ID is used as an explicit indication.

If the RVQoE-configuring node implicit indications to the non-RVQoE-configuring node via forwarding the RVQoE report, the non-RVQoE-configuring node can know it is an implicit indication because the non-RVQoE-configuring node did not receive the RVQoE reports related to the PDU session ID. However, the RVQoE report contains a list of appLayerBufferLevelList IE and playoutDelayForMediaStartup IE, which is not helpful for implicit indication and introduces extra overhead.

When the RVQoE-configuring node sends an explicit indication to the non-RVQoE-configuring node, it is more straightforward for the non-RVQoE-configuring node to know it should take responsibility for the RVQoE measurement configuration and reporting. The RVQoE measurement configuration is also suggested to be sent with the explicit indication because the non-RVQoE-configuring node needs to know the configuration and modify it further. A new explicit indication or a session indication is enough in this case. The explicit indication is a more straightforward approach and has less signaling overhead.

Proposal 5: When the RVQoE-configuring node receives an RVQoE report and determines that the non-RVQoE-configuring node provides the bearer(s) for the application session, the RVQoE-configuring node indicates that to the non-RVQoE-configuring node. The RVQoE-configuring node sends an explicit IE with RVQoE measurement configuration to the non-RVQoE-configuring.

The discussion on LS from RAN2

Furthermore, RAN2 discussed the details of QoE reporting in NR-DC scenarios and reached the following agreement:

1: As working assumption, for encapsulated QoE report associated with the non-receiving RAN node, use option 1 (i.e., MeasurementReportAppLayer message) to send to the receiving RAN node. This can be revisited if RAN3 decisions warrant something different for RVQoE.

With respect to this agreement, RAN2 would like to ask RAN3 the following question:

Q3: Does RAN3 see any issue with the above working assumption, e.g., whether RAN3 sees any problem if RAN visible QoE report associated with the non-receiving RAN node is reported to the receiving RAN node via MeasurementReportAppLayer message?

The QoE measurement report may be configured by one node but received by the peer node so that the MeasurementReportAppLayer message will be transmitted to the peer node. Moreover, RAN2 thinks the peer node will decode this message.

MeasurementReportAppLayer contains RRC ID, QoE measurement report container, and RVQoE measurement report (not in a container). The RRC ID helps the receiving node to determine which node is related to this QoE report. QoE measurement report is transparent to the receiving node, and no issues are detected. RVQoE measurement is also supported, which is already analyzed in section 2.1. Overall, the above working assumption is acceptable.

Observation 5: The above working assumption is acceptable.

Conclusion and Proposal

We summarize our proposals below.

Observation 1: The transparent report for RVQoE via SRB4 is workable but has a RAN2 impact, while the non-transparent report for RVQoE via SRB4 is workable and has no impact.

Observation 2: The MN can deliver the RVQoE report to SN via RRC Transfer message regardless of whether this report is in an RRC container.

Observation 3: Both QoE reports transparently via SRB4 and non-transparently via SRB4 are workable, but the transparent method has RAN2 impacts.

Observation 4: Only MN can use the PDU session ID and QFI of the first RVQoE report to determine which node(s) carry the application session.

Observation 5: The above working assumption is acceptable.

Proposal 1: Turning the "WA: The transparent reporting for RVQoE over RRC is not supported" as an agreement.

Proposal 2: Turning the "WA: RRC Transfer procedure can be considered to transfer the RVQoE report between MN and SN subjects to validation of whether the transparent reporting for RVQoE over RRC is precluded or not" as an agreement.

Proposal 3: If the SN receives a management-based QoE configuration directly from the OAM, the QoE report can be delivered to the MN directly, and the MN will forward this QoE report to the MCE directly.

Proposal 4: For both MN configures RVQoE and the SN configures RVQoE, the first RVQoE report is suggested to be sent to the MN.

References