3GPP TSG-RAN WG3 Meeting #121bis	R3-235322

Xiamen, China, 9th – 13th October 2023

Agenda Item:	10.2.2

Source:	ZTE

Title:	(TPs for SON BL CR 36.300, 38.300, 38.413) MRO for inter-system handover for voice fallback

Document for:	Discussions & Approval

1	Introduction

In RAN3#121 meeting, the MRO for inter-system handove for voice fallback has been discussed, and the agreement has been reached as below.

Convert the WA into agreement “Define a new handover report type in the Inter-system HO Report over S1 and NG”.

However, some details on the IE design should be further studied. In this contribution, we provide the discussion on the details of the new handover report type and make the stage 2 clean-up on the name of the voice fallback indication.

2	Discussion

2.1   Stage 3 details on the new handover report type

In last meeting, the “Inter-system Mobility Failure for Voice Fallback” has been defined as one the handover report type in the Inter-system HO Report over NG[1]. While the details have not been completed.

According to the agreed use cases for inter-system handover for voice fallback as given below, after the RLF due to Voice Fallback, the UE shall attempt to re-connect a E-UTRAN cell. While, after the HOF due to Voice Fallback, the UE shall attempt to re-connect a E-UTRAN cell or a NG-RAN cell.

Consider Case 1-2 for MRO enhancements for inter-system inter-RAT handover for voice fallback: - Case 1: after failure (HOF/RLF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell. - Case 2: after failure (HOF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, none suitable E-UTRAN cell can be selected, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in NR.

Furthermore, after the HOF due to Voice Fallback, the NR RLF Report shall be generated and it could be stored in the UE for 48 hours. When the UE moves back to NR, the NR RLF Report could be sent to the source NR cell by using the existing Xn or NG procedures. And the source NG-RAN node is able to decode the NR RLF Report. Therefore, there is no RAN3 impact for the HOF case.

While, after the RLF due to Voice Fallback, the LTE RLF Report is generated and it cannot be decoded directly by the NG-RAN node after the UE moves back to NR. Therefore, the LTE RLF Report should be sent to the new E-UTRAN cell for decoding. Then, the RLF related information should be sent from the E-UTRAN cell to the source NG-RAN cell. Thus, the S1 and NG interfaces should be enhanced to support the inter-system handover for voice fallback.

The above analysis is the intention of introducing the new handover report type over NG and S1. And the new handover report type “Inter-system Mobility Failure for Voice Fallback” was introduced only for the RLF case due to Voice Fallback.

Observation: The new inter-system report type “Inter-system Mobility Failure for Voice Fallback” was introduced only for the RLF case due to Voice Fallback

Furthermore, the name “Re-connected Cell ID” is appropriate, and the re-connected cell should be an E-UTRAN cell to follow the description of the agreed Case 1. Meanwhile, the LTE RLF Report should be optional, since it is not essential for the source NG-RAN node.

Proposal 1: In the Inter-system Mobility Failure for Voice Fallback, the re-connect cell should be an E-UTRAN cell and the UE RLF Report should be optional.

2.2   Stage 2 clean-up on the name of voice fallback indication

In RAN2, the name of voice fallback indication in the LTE RLF report and NR RLF report has been defined as “voiceFallbackHO”[3][4], which is given as below.

LTE Running CR for MRO:

4> if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from NR and if the UE supports Radio Link Failure Report for Inter-RAT MRO NR: 5> include the previousNR-PCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3; 5> set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3. 5> if voiceFallbackIndication is included in the MobilityFromNRCommand: 6> set voiceFallbackHO to true;

NR Running CR for MRO:

1> else if the failure is detected due to Mobility from NR failure as described in 5.4.3.5, set the fields in VarRLF-report as follows: 2> set the connectionFailureType to hof; 2> if last MobilityFromNRCommand concerned a failed inter-RAT handover from NR to E-UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO EUTRA (NR to EUTRA): 3> set the eutraFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover; 2> include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last MobilityFromNRCommand message was received; 2> set the timeConnFailure to the elapsed time since the initialization of the handover associated to the last MobilityFromNRCommand message; 2> if voiceFallbackIndication is included in the last MobilityFromNRCommand: 3> include the voiceFallbackHO Editor´s note: FFS whether to differentiate emergency voice call fallback failure.

Therefore, the stage 2 description of Inter-system Mobility Failure for Voice Fallback in SON BL CR 36.300 and 38.300 should be changed accordingly.

Proposal 2: Align the name of voice fallback indication in the stage 2 description of Inter-system Mobility Failure for Voice Fallback with RAN2.

3	Conclusion

Observation: The new inter-system report type “Inter-system Mobility Failure for Voice Fallback” was introduced only for the RLF case due to Voice Fallback

Proposal 1: In the Inter-system Mobility Failure for Voice Fallback, the re-connect cell should be an E-UTRAN cell and the UE RLF Report should be optional.

Proposal 2: Align the name of voice fallback indication in the stage 2 description of Inter-system Mobility Failure for Voice Fallback with RAN2.

The corresponding TPs for SON BL CR 36.300, 38.300 and 38.413 on MRO for voice fallback are given in the Annex.

References

- R3-234625, Introduce a new handover report type for inter-RAT voice fallback, CATT, Nokia, Nokia Shanghai Bell, ZTE, Samsung

- R2-2308458, LTE Running CR for Rel-18 SON MRO, Ericsson

- R2-2308459, Running CR for Rel-18 SON MRO, Ericsson

Annex

Text Proposal for SON BL CR 36.300 on MRO for voice fallback

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

22.4.2.2a	Connection failure due to inter-RAT mobility

One of the functions of Mobility Robustness Optimisation is to detect connection failures that occurred due to Too Early or Too Late inter-RAT handovers or Inter-system Mobility Failure for Voice Fallback. These problems are defined as follows:

-	[Too Late Inter-RAT Handover] An RLF occurs after the UE has stayed in an E-UTRAN cell for a long period of time; the UE attempts to re-connect to a UTRAN cell.

-	[Too Early Inter-RAT Handover] An RLF occurs shortly after a successful handover from a UTRAN cell to a target cell in E-UTRAN; the UE attempts to re-connect to the source cell or to another UTRAN cell.

-	[Inter-system Mobility Failure for Voice Fallback] An RLF occurs shortly after a successful handover triggered due to Voice Fallback, or a failure occurs during an handover triggered due to Voice Fallback, from a cell belonging to an NG-RAN node to a cell belonging to an E-UTRAN node; the UE attempts to re-connect to a cell belonging to an E-UTRAN node, or an NG-RAN node.

The UE makes the RLF Report available to an eNB, when RLF happens in E-UTRAN and the UE re-connects to an eNB cell. Availability of the RLF Report at the RRC connection setup or at a handover to E-UTRAN cell is the indication that the UE suffered a connection failure and that the RLF Report from this failure was not yet delivered to the network.

The eNB receiving the RLF Report from the UE may forward the report to the eNB that served the UE before the reported connection failure using the RLF INDICATION message over X2 or by means of the eNB configuration transfer procedure and MME configuration transfer procedure over S1. If present in the RLF Report, the radio measurements may be used to identify lack of coverage as the potential cause of the failure. This information may be used to exclude those events from the MRO evaluation and redirect them as input to other algorithms.

Detection mechanisms for Too Late Inter-RAT Handover, Too Early Inter-RAT Handover and Inter-system Mobility Failure for Voice Fallback are carried out through the following:

-	[Too Late Inter-RAT Handover]
The connection failure occurs while being connected to an LTE cell, and there is no recent handover for the UE prior to the connection failure i.e., the UE reported timer is absent or larger than the configured threshold, e.g., Tstore_UE_cntxt, and the first cell where the UE attempts to re-connect is a UTRAN cell.

-	[Too Early Inter-RAT Handover]
The connection failure occurs while being connected to an LTE cell, and there is a recent inter-RAT handover for the UE prior to the connection failure i.e., the UE reported timer is smaller than the configured threshold, e.g., Tstore_UE_cntxt, and the first cell where the UE attempts to re-connect and the cell that served the UE at the last handover initialisation are both UTRAN cells.

-	[Inter-system Mobility Failure for Voice Fallback]
The connection failure occurs while being connected to an LTE cell, and there is a recent Inter-system handover due to voice fallback from NR prior to the connection failure i.e., the UE reported timer is smaller than the configured threshold, e.g., Tstore_UE_cntxt. The RLF Report from the UE includes a voiceFallbackHO, as defined in TS 36.331 [16].

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure.

In case the failure is a Too Early Inter-RAT Handover, the eNB receiving the RLF INDICATION message may inform the UTRAN node by means of the eNB Direct Information Transfer procedure over S1. The information contains:

-	Type of detected handover problem (Too Early Inter-RAT Handover);

-	UE RLF Report Container: the RLF Report received from the UE, as specified in TS 36.331 [16];

-	Mobility Information (optionally, if provided in the last Handover Resource Allocation procedure from the UTRAN node);

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

Text Proposal for SON BL CR 38.300 on MRO for voice fallback

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

15.5.2.2.3	Connection failure due to inter-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occurred due to Too Early or Too Late inter-system handovers or inter-system Mobility Failure for Voice Fallback. These problems are defined as follows:

-	Inter-system/ Too Late Handover: an RLF occurs after the UE has stayed in a cell belonging to an NG-RAN node for a long period of time; the UE attempts to re-connect to a cell belonging to an E-UTRAN node.

-	Inter-system/ Too Early Handover: an RLF occurs shortly after a successful handover from a cell belonging to an E-UTRAN node to a target cell belonging to an NG-RAN node; the UE attempts to re-connect to the source cell or to another cell belonging to an E-UTRAN node.

-	Inter-system Mobility Failure for Voice Fallback: an RLF occurs shortly after a successful handover triggered due to Voice Fallback, or a failure occurs during an handover triggered due to Voice Fallback, from a cell belonging to an NG-RAN node to a cell belonging to an E-UTRAN node; the UE attempts to re-connect to a cell belonging to an E-UTRAN node, or an NG-RAN node.

Detection mechanism

A failure indication may be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

In case the last serving node is an E-UTRAN node, the detection mechanism proceed as defined in TS 36.300 [2].

In case the last serving node is an NG-RAN node, the detection mechanisms for Too Late Inter-system Handover, Too Early Inter-system Handover, and Inter-system Mobility Failure for Voice Fallback are carried out through the following:

-	Too Late Inter-system Handover: the connection failure occurs while being connected to a NG-RAN node, and there is no recent handover for the UE prior to the connection failure i.e., the UE reported timer is absent or larger than the configured threshold, e.g., Tstore_UE_cntxt, and the first node where the UE attempts to re-connect is a E-UTRAN node.

-	Too Early Inter-system Handover: the connection failure occurs while being connected to a NG-RAN node, and there is a recent inter-system handover for the UE prior to the connection failure i.e., the UE reported timer is smaller than the configured threshold, e.g., Tstore_UE_cntxt, and the first cell where the UE attempts to re-connect and the node that served the UE at the last handover initialisation are both E-UTRAN node.

-	Inter-system Mobility Failure for Voice Fallback: in case the connection failure occurs during an inter-system handover for voice fall back from NR, the RLF Report from the UE includes a voiceFallbackHO, as defined in TS 38.331 [12].

The “UE reported timer” above indicates the time elapsed since the last handover initialisation until connection failure. The UE may make the RLF Report available to an NG-RAN node. The NG-RAN node may forward the information using the FAILURE INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG to the node that served the UE before the reported connection failure.

In case the failure is a Too Early Inter-system Handover, the NG-RAN node receiving the failure indication may inform the E-UTRAN node by means of the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

Text Proposal for SON BL CR 38.413 on MRO for voice fallback

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.40	Inter-system HO Report

This IE contains the inter-system HO report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Handover Report Type | M |  |  |  |  |  |
| >Too early Inter-system HO |  |  |  |  | - |  |
| >>Source Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the source cell for the HO. |  |  |
| >>Failure Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the target cell for the HO. |  |  |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |  |  |
| >Inter-system Unnecessary HO |  |  |  |  | - |  |
| >>Source Cell CGI | M |  | NG-RAN CGI 9.3.1.73 | Source NR cell in NG-RAN |  |  |
| >>Target Cell CGI | M |  | E-UTRA CGI 9.3.1.9 | Target cell in E-UTRAN |  |  |
| >>Early IRAT HO | M |  | ENUMERATED (true, false, ...) | Is set to “true” if the measurement period expired due to an inter-RAT handover towards NR executed within the configured measurement duration and otherwise set to “false” |  |  |
| >>Candidate Cell List |  | 1 |  |  |  |  |
| >>>Candidate Cell Item |  | 1..<maxnoofCandidateCells> |  |  |  |  |
| >>>>CHOICE Candidate Cell Type | M |  |  |  |  |  |
| >>>>>Candidate CGI |  |  |  |  |  |  |
| >>>>>>Candidate Cell ID | M |  | NR CGI 9.3.1.7 | This IE contains an NR CGI. |  |  |
| >>>>>Candidate PCI |  |  |  |  |  |  |
| >>>>>>Candidate PCI | M |  | INTEGER (0..1007, …) | This IE includes the NR Physical Cell Identifier of detected cells not included in the Candidate Cell List IE and for which an NR CGI could not be derived. |  |  |
| >>>>>>Candidate NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) | RF Reference Frequency as defined in TS 38.104 [39], section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. |  |  |
| >Inter-system Mobility Failure for Voice Fallback |  |  |  |  | YES | ignore |
| >>Source Cell ID | M |  | NG-RAN CGI 9.3.1.73 | CGI of the source cell for HO |  |  |
| >>Failure Cell ID | M |  | E-UTRA CGI 9.3.1.9 | CGI of the failure cell for HO |  |  |
| >>Re-connected Cell ID | O |  | E-UTRA CGI 9.3.1.9 | CGI of the re-connected cell after HO |  |  |
| >>UE RLF Report Container | O |  | 9.3.3.41 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateCells | Maximum no. of candidate cells. Value is 32 |
| maxNRARFCN | Maximum value of NRARFCNs. Value is 3279165. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

InterSystemHandoverReportType ::= CHOICE {

tooearlyIntersystemHO				TooearlyIntersystemHO,

intersystemUnnecessaryHO			IntersystemUnnecessaryHO,

choice-Extensions		ProtocolIE-SingleContainer { { InterSystemHandoverReportType-ExtIEs} }

}

InterSystemHandoverReportType-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-intersystemMobilityFailureforVoiceFallback  CRITICALITY ignore	TYPE  IntersystemMobilityFailureforVoiceFallback 	PRESENCE mandatory },

...

}

IntersystemMobilityFailureforVoiceFallback ::= SEQUENCE {

sourcecellID			NGRAN-CGI,

targetcellID			EUTRA-CGI,

reconnectedCellID	EUTRA-CGI                      OPTIONAL,

uERLFReportContainer	UERLFReportContainer		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { IntersystemMobilityFailureforVoiceFallback-ExtIEs} }			OPTIONAL,

...

}

IntersystemMobilityFailureforVoiceFallback-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>