3GPP TSG-RAN WG2 #121bis	R3-235347

9th – 13th Oct 2023 Xiamen, China

Agenda item:		11.3

Source:				CATT

Title:	Discussion on Support for QoE in NR-DC

Document for:		Discussion and Decision

1		Introduction

In RAN3#121 meeting, the NR QoE supporting in NR-DC was discussed. The agreements and open issues are captured in chair Note[1] as below:

Reuse legacy procedures for the MN-SN coordination in NR-DC to support QoE.

For s-based QoE configuration received by MN

- MN sends the QoE configuration via SRB1

- QoE reports can be sent via SRB4 or SRB5

For management based QoE, there is no need for SN to explicitly indicate to the MN whether it wants to configure the UE directly via SRB3 or via a container and SRB1.

Modify the previous agreement as follows:

In case the SN is interested in configuring a UE with an m-based QoE measurement configuration, the MN can decide and notify the SN whether:

- The MN sends the configuration information to the UE, or

- The SN should send the configuration to the UE

WA: The transparent reporting for RVQoE over RRC is not supported.

P6: If the MN configures RVQoE, the first RVQoE report shall be delivered to the MN via SRB4.

P7: If the SN configures RVQoE, the first RVQoE report shall be delivered to the SN.

WA: RRC Transfer procedure can be considered to transfer the RVQoE report between MN and SN subjects to validation of whether the transparent reporting for RVQoE over RRC is precluded or not.

Define two different reporting leg indications for QoE and RVQoE.

For a UE in NR-DC, each legacy QoE configuration can have only one corresponding RVQoE configuration when needed.

This contribution will further discuss the supporting on the QoE in NR-DC. And provide the view on the open issues .

2		Discussion

2.1 Legacy QoE in NR-DC

2.1.1 Reporting and configuring leg indication

We agreed that the leg switching command can be sent to the UE by the node that configured that specific QoE configuration. Both the MN node and SN node may encounter the overload status, to avoid introducing delay and reduce the overhead in interface, we should allow both MN and SN can send the indication of switching reporting leg based on its overload status. The SN may send the command via SRB3 or SRB1

Proposal 1: Both MN and SN node can indicate the UE to switch the reporting leg based on its overload status

We agreed in last meeting: Define two different reporting leg indications for QoE and RVQoE. In R17, the RVQoE will not be paused when the overload happened in the node. Also the RVQoE report is mostly configured by the receiving node. So the RVQoE reporting will not be changed for the RAN node overload.

Proposal 2: The RVQoE report leg will not be changed for the reason of node overload.

2.1.2 SN coordination with MN about M-based QMC

The SN may receive the below parameter of m-based QoE configuration from OAM as specified in TS 28.405 [2]: Service Type, area Scope, QoE Collection Entity Address, QoE Reference, MDT Alignment Information, available RAN visible QoE Metrics and QMC Configuration File. In the agreed BLCR, we already include QoE Reference, Measurement Collection Entity IP Address and Measurement Configuration Application Layer ID in the request from SN to MN. Except these parameters, other parameters received from OAM may be sent the MN if the MN sends the configuration to UE.

Proposal 3: The SN may send more parameters to MN including Service Type, area Scope, MDT Alignment Information (if support), available RAN visible QoE Metrics, QMC Configuration File

2.1.3 Support for alignment of QoE measurements and radio related measurement

In NR-DC, the immediate MDT can be configured in the MN and SN. But the logged MDT can be configured only in MN. The logged MDT aligns with the QoE in idled and inactive state should be discussed in the work item of the idle QoE and the NR-DC will not impact the alignment. For the QoE in connected state align with the immediate MDT in MN and SN, aligned immediate MDT in MN and SN may be the same MDT or different MDT. To simple the specification, we should restrict the alignments to only supporting the same MDT. How to trigger the MDT configured should be enhanced base on the current specification. The QoE start indication should send to SN for the SN MDT configuration.

Proposal 4: aligned immediate MDT in MN and SN should be the same MDT

Proposal 5: The QoE start indication should be sent to SN for the SN MDT configuration initial

2.2 RVQoE in NR-DC

2.2.1 RVQoE Reporting

In last meeting, we made one WA: The transparent reporting for RVQoE over RRC is not supported. Also RAN2 has one WA: As working assumption, for encapsulated QoE report associated with the non-receiving RAN node, use option 1 (i.e.MeasurementReportAppLayer message) to send to the receiving RAN node. This can be revisited if RAN3 decisions warrant something different for RVQoE. And they send the LS [3] to RAN3 whether see any issue with the above working assumption. With these WA from RAN3 and RAN2, we can conclude that the transparent reporting for RVQoE over RRC is not supported and send response to RAN2.

Proposal 6: Turn WA to agreement: The transparent reporting for RVQoE over RRC is not supported

We also discussed the how the first RVQoE report is delivered from UE. Try to have the below two proposals, but no conclusion was made.

P6: If the MN configures RVQoE, the first RVQoE report shall be delivered to the MN via SRB4.

P7: If the SN configures RVQoE, the first RVQoE report shall be delivered to the SN.

If we agree these proposals, if the receiving node is not the bearer provided, the report should be forwarded to another node. It may introduce more delay and complexity. As we know the RV QoE is more time sensitive. So we may let the RVQoE report directly sending to bearer provided node based on the below analysis.

We already agree both RVQoE-configuring node and non-RVQoE-configuring node can receive the report. So the UE need not to differentiate the initial node when the UE send the first RV QoE report.

The UE APP layer will send the RV QoE report with PDU session ID plus QoS flow ID to AS layer and then the AS layer send the report to the node which the QoS flow is located.

The report receiving node may compare the report metrics with its configured RVQoE. There are three possibilities:

- If the reporting metrics is aligned with its configuration, this node is RVQoE-configuring node. It may store and use this report as normal.

- If the reporting metrics is not aligned with its configuration, this node is non-RVQoE-configuring node. It may discard this report and indicate RVQoE-configuring node to release the RVQoE configuration if it does not want it.

- If the reporting metrics is not aligned with its configuration, this node is non-RVQoE-configuring node. It also may store the report if it is interested it, and indicate RVQoE-configuring node to release the RVQoE configuration and configure the RVQoE by itself as above Proposal 2.

Proposal 7: UE may directly send the RV-QoE report to service bearer provided node according QoS flow information

Proposal 8: if non-RVQoE-configuring receive report, the node can discard the report if it doesn’t want it, and indicate RVQoE-configuring node to release the configured RVQoE

2.2.2 RVQoE Configuring

In last RAN3 meeting, we also have agreement: For a UE in NR-DC, each legacy QoE configuration can have only one corresponding RVQoE configuration when needed. So we need support RVQoE configuring coordination between MN and SN before sending to UE.

After the RVQoE of SN and MN wanted configuration coordinated, we may let the legacy QoE configured node to generate the final configuration and send to UE.

Proposal 9: The legacy QoE configured node generates one final RVQoE configuration covers both MN and SN and sends to UE

During the coordination, But the value of the configured parameters may be different from the two nodes. The configuration generated node need to do the alignment before sending the configuration. Regarding how to align, it may up to implementation

Proposal 10: up to implementation, the MN and SN may align the value of parameters if both MN and SN configure same RV-QoE parameter

3		Conclusion

In the present contribution we make the following observations and proposal:

Proposal 1: Both MN and SN node can indicate the UE to switch the reporting leg based on its overload status

Proposal 2: The RVQoE report leg will not be changed for the reason of node overload.

Proposal 3: The SN may send more parameters to MN including Service Type, area Scope, MDT Alignment Information (if support), available RAN visible QoE Metrics, QMC Configuration File

Proposal 4: aligned immediate MDT in MN and SN should be the same MDT

Proposal 5: The QoE start indication should be sent to SN for the SN MDT configuration initial

Proposal 6: Turn WA to agreement: The transparent reporting for RVQoE over RRC is not supported

Proposal 7: UE may directly send the RV-QoE report to service bearer provided node according QoS flow information

Proposal 8: if non-RVQoE-configuring receive report, the node can discard the report if it doesn’t want it, and indicate RVQoE-configuring node to release the configured RVQoE

Proposal 9: The legacy QoE configured node generates one final RVQoE configuration covers both MN and SN and sends to UE

Proposal 10: up to implementation, the MN and SN may align the value of parameters if both MN and SN configure same RV-QoE parameter

4		Reference

- RAN3_121_agenda_20230825_EOM

- TS28.405 Quality of Experience (QoE) measurement collection; Control and configuration

- R3-235011 LS on QoE in RRC IDLE/INACTIVE and NR-DC scenarios