3GPP TSG-RAN WG3 Meeting #121-bis	R3-235376

Xiamen, China, 9 - 13 October 2023

Agenda Item:	14.4

Source:	ZTE

Title:	(TP to TS 38.423 and 37.340) Discussion on support of subsequent CPAC

Document for:	Discussions & Approval

- Introduction

- We made the following progress at the RAN3#120 meeting.

RAN3 follows RAN2 and uses the same name “subsequent CPAC” or “S-CPAC” for the selective activation. The information of other candidate PSCells in other candidate target SNs should be provided in the SN Addition Request message/SN Modification Request message from the MN to the candidate target SNs, if RAN2 agrees. After CPC execution from the source SN to other SN, the MN should inform the source SN of the CPC execution and start late data forwarding. - Case 1: if the source SN is configured as a candidate SN for subsequent CPAC, the source SN needs to be informed. FFS on which procedure to inform. - Case 2: if the source SN is not configured as a candidate SN for subsequent CPAC, the source SN is released. The SN reconfiguration complete message is sent to the selected SN after CPC execution. How to transfer the data forwarding addresses to the selected SN to start early data forwarding? How to support reference configuration? How to support execution conditions? New indicators in S-NODE MODIFICATION REQUEST message, UE CONTEXT SETUP/MODIFICAION REQUEST message and BEARER CONTEXT SETUP REQUEST message?

This paper further discusses how to support the subsequent CPAC feature.

# 2	Discussion

## 2.1	Notification to source SN and selected SN

At the last meeting, we agreed that the source SN needs to be informed if the source SN is configured as a candidate SN for subsequent CPAC after CPC execution from the source SN to other SN. It is still FFS which procedure to inform.

There are three candidate solutions as below:

1) Xn-U address indication procedure

2) MN initiated SN release procedure

3) MN initiated SN modification preparation procedure

Observation 1: There are three candidate procedures to inform the source SN after CPC execution from the source SN to other SN, including Xn-U address indication procedure, MN initiated SN release procedure and MN initiated SN Modification Preparation procedure.

After CPC execution, the MN needs to trigger the Xn-U address indication procedure to inform the source SN of the address of the selected candidate PSCell and start late data forwarding.

Observation 2: After CPC execution, the MN needs to trigger the Xn-U address indication procedure to inform the source SN of the address of the selected candidate PSCell and start late data forwarding.

Both the MN initiated SN release procedure and MN initiated SN modification preparation procedure are class 1 procedure, but no response is required from the source SN because the purpose is just to inform the source SN of the CPC execution.

Observation 3: Both the MN initiated SN release procedure and MN initiated SN modification preparation procedure are class 1 procedure, but no response is required from the source SN because the purpose is just to inform the source SN of the CPC execution.

Therefore, from the perspective of signalling efficiency, it would be more efficient to use the Xn-U address indication procedure to inform the source SN of the CPC execution.

Proposal 1: If the source SN is configured as a candidate SN for subsequent CPAC after CPC execution, the MN should use the Xn-U address indication message to inform the source SN of the CPC execution and start late data forwarding.

At the last meeting, three options are listed to transfer the data forwarding addresses to the selected SN to start early data forwarding when it needs.

1) single enhanced Xn-U Address Indication procedure which carries the data forwarding addresses of all other candidate SNs

2) multiple Xn-U Address Indication procedures, and each procedure carries data forwarding addresses of one candidate SN

3) single SN RRC Reconfiguration Complete procedure which carries the data forwarding addresses of all other candidate SNs

In legacy procedure, the SN status transfer procedure is triggered after the Xn-U address indication procedure, thus using the Xn-U address indication is more suitable to transfer the data forwarding addresses.

Considering that option 2 has less impact on the spec, we prefer to choose option 2. In addition, a new indicator is also needed to be introduced in the Xn-U address indication message to indicate the CPC execution.

Observation 4: In legacy procedure, the SN status transfer procedure is triggered after the Xn-U address indication procedure, thus using the Xn-U address indication is more suitable to transfer the data forwarding addresses.

Observation 5: Option 2 has less impact on the spec.

Proposal 2: Use multiple Xn-U address indication procedures to transfer the data forwarding addresses of all other candidate SNs to the selected SN to start early data forwarding when it needs.

Proposal 3: Introduce a new indicator for subsequent CPAC in the Xn-U address indication message to notify the source SN of the CPC execution.

## 2.2	New indicators

At the last meeting, it is still FFS whether to introduce a new indicator for subsequent CPAC in the SN modification request message to indicate that the request is for subsequent CPAC. From our point of view, it would apply to the scenario when the MN decides to configure subsequent CPAC before the execution of CPAC.

Observation 6: Introducing a new indicator for subsequent CPAC would be beneficial for the scenario when the MN decides to configure subsequent CPAC before the execution of CPAC.

Proposal 4: Introduce a new indicator for subsequent CPAC in the SN modification request message to indicate that the request is for subsequent CPAC.

At the previous meetings, some companies propose to also introduce new indicators in the UE context setup/modification message and bearer context setup request message. However, from our point of view, the subsequent CPAC feature can still work without introducing new indicators in these messages.

Proposal 5: No need to introduce new indicators in UE context setup message, UE context modification message and bearer context setup request message.

## 2.3	Data forwarding

We have agreed to reuse the Xn-U Address Indication message and the Early Status Transfer message to support early data forwarding for SCG Selective Activation. It is still FFS on whether these message needs enhancements and when to use them. From our point of view, the early data forwarding for subsequent CPAC should reuse the principle for CPAC. Namely, the source SN forwards the DL user data to the candidate SN via the MN. To achieve this, after receiving the RRCReconfigurationComplete message from the UE, the MN should send the Xn-U Address Indication message to the source SN to provide the data forwarding addresses, the source SN should send the Early Status Transfer message to the MN and the MN should send the Early Status Transfer message to the target SN to start early data forwarding.

Proposal 6: After receiving the RRCReconfigurationComplete message from the UE, the MN should send the Xn-U Address Indication message to the source SN to provide the data forwarding addresses, the source SN should send the Early Status Transfer message to the MN and the MN should send the Early Status Transfer message to the target SN to start early data forwarding.

Some companies proposed to reuse Early Status Transfer message to indicate the candidate SN to discard the already forwarded data for subsequent CPAC after CPC execution. From our point of view, the discarding of the forwarded data can be up to implementation when the candidate SN receives the Early Status Transfer message when the second round of early data forwarding is started during subsequent CPC.

Proposal 7: It is up to implementation to discard of the forwarded data for the first round of early data forwarding when the candidate SN receives the Early Status Transfer message when the second round of early data forwarding is started during subsequent CPC.

## 2.4	Reference configuration

As for how to indicate whether the associated the SCG configuration is a delta with respect to the reference SCG configuration in the SN addition request acknowledge message, some companies propose to reuse the existing RRC config indication IE. The existing RRC config indication IE is a per node indication other than a per cell indication, therefore it can not support a per cell indication as specified in the RAN2 agreement.

- R2 understands that A target SN may include an indication in SN Addition Request Ack for each candidate target PSCell, denoting whether the associated SCG configuration is a delta with respect to the reference SCG configuration.

Observation 7: The existing RRC config indication IE is a per node indication other than a per cell indication, therefore it can not support a per cell indication as specified in the RAN2 agreement.

This IE can be reused but needs to be added as a sub IE of the Conditional PSCell Addition Information Acknowledge IE in the SN addition request acknowledge message to achieve a per cell indication.

Proposal 8: Add the RRC config indication IE as a sub IE of the Conditional PSCell Addition Information Acknowledge IE in the SN addition request acknowledge message to indicate whether the associated SCG configuration is a delta with respect to the reference SCG configuration. This can be revisited based on RAN2 progress.

According to R2-2309831 Post meeting discussion on stage 2 signalling open issues, RAN2 companies still have different understandings on the following agreement:

Assume for now that there is only one reference configuration.

- Understanding 1: There is only one reference configuration for MN/SN initiated inter-SN SCPAC and SN-initiated intra-SN SCPAC. If co-existence is supported, MN is responsible to generate the reference configuration and provide the reference configuration to UE in MN format in order to guarantee a valid configuration for all candidate cells.

- Understanding 2: There is only one reference configuration for each scenario, i.e., one reference configuration for MN/SN initiated inter-SN SCPAC and one reference configuration for SN-initiated intra-SN SCPAC. In this solution, SN can generate the reference configuration for SN-initiated intra-SN SCPAC and MN does not need to be involved.

Since the understanding of this agreement will impact RAN3 signalling design for intra-SN subsequent CPAC. RAN3 should wait for RAN2 progress before discussing the reference configuration signalling design for SN-initiated intra-SN subsequent CPAC.

Observation 8: RAN2 companies still have different understandings on whether there is only one reference configuration for MN/SN initiated inter-SN SCPAC and SN-initiated intra-SN SCPAC or there is only one reference configuration for each scenario, i.e., one reference configuration for MN/SN initiated inter-SN SCPAC and one reference configuration for SN-initiated intra-SN SCPAC.

Proposal 9: RAN3 waits for RAN2 progress before discussing the reference configuration signalling design for SN-initiated intra-SN subsequent CPAC.

According to the endorsed RAN2 running CR R2-2309336, the reference configuration for CPAC is put into the CG-ConfigInfo message, which is included in the existing M-NG-RAN node to S-NG-RAN node Container IE. Therefore, the transfer of reference configuration has no RAN3 impact.

Observation 9: According to the endorsed RAN2 running CR R2-2309336, the reference configuration for CPAC is put into the CG-ConfigInfo message, which is included in the existing M-NG-RAN node to S-NG-RAN node Container IE. Therefore, the transfer of reference configuration has no RAN3 impact.

For MN/SN initiated inter-SN subsequent CPAC, the candidate SN or the source SN needs to generate the SCG part of the reference configuration. Therefore, the MN needs to request the source SN or the candidate SN to generate the SCG part of the reference configuration.

Observation 10: For MN/SN initiated inter-SN subsequent CPAC, the candidate SN or the source SN needs to generate the SCG part of the reference configuration.

Proposal 10: For MN/SN initiated inter-SN subsequent CPAC, the MN requests the source SN or the candidate SN to generate the SCG part of the reference configuration.

## 2.5	Coexistence of subsequent CPAC and legacy CPAC

According to R2-2309831 Post meeting discussion on stage 2 signalling open issues, coexistence of subsequent CPAC and legacy CPAC is agreed by majority RAN2 companies. If the coexistence is supported, RAN3 needs to discuss the potential impact.

Taking the SN addition procedure as an example, there are three candidate options to support the coexistence of subsequent CPAC and legacy CPAC.

- Keep the Subsequent CPAC Request IE as a sub IE of the Conditional PSCell Addition Information Request IE in the SN addition request message. Add a new indicator as a sub IE of the Conditional PSCell Addition Information Acknowledge IE in the SN addition request acknowledge message to indicate whether a candidate PSCell is configured with legacy CPAC or subsequent CPAC.

- Move the Subsequent CPAC Request IE to the same level as the Conditional PSCell Addition Information Request IE in the SN addition request message. Add a new response IE in the same level of the Conditional PSCell Addition Information Acknowledge IE in the SN addition request acknowledge message.

- Leave it to implementation, i.e. the MN sends two separate SN addition request messages, one message is used to configure legacy CPAC and the other one is used to configure subsequent CPAC.

- Option 1 and option 2 has more impact on the spec. In addition, option 2 requires a dynamic allocation of the PSCell numbers for legacy CPAC and subsequent CPAC. Therefore, we prefer option 3.

Proposal 11: Leave it to implementation to support the coexistence of subsequent CPAC and legacy CPAC, i.e. the MN sends two separate SN addition request messages, one is used to configure legacy CPAC and the other one is used to configure subsequent CPAC.

# 3	Conclusion

Based on above analysis, we provide the following observations and proposals.

Observation 1: There are three candidate procedures to inform the source SN after CPC execution from the source SN to other SN, including Xn-U address indication procedure, MN initiated SN release procedure and MN initiated SN Modification Preparation procedure.

Observation 2: After CPC execution, the MN needs to trigger the Xn-U address indication procedure to inform the source SN of the address of the selected candidate PSCell and start late data forwarding.

Observation 3: Both the MN initiated SN release procedure and MN initiated SN modification preparation procedure are class 1 procedure, but no response is required from the source SN because the purpose is just to inform the source SN of the CPC execution.

Proposal 1: If the source SN is configured as a candidate SN for subsequent CPAC after CPC execution, the MN should use the Xn-U address indication message to inform the source SN of the CPC execution and start late data forwarding.

Observation 4: In legacy procedure, the SN status transfer procedure is triggered after the Xn-U address indication procedure, thus using the Xn-U address indication is more suitable to transfer the data forwarding addresses.

Observation 5: Option 2 has less impact on the spec.

Proposal 2: Use multiple Xn-U address indication procedures to transfer the data forwarding addresses of all other candidate SNs to the selected SN to start early data forwarding when it needs.

Proposal 3: Introduce a new indicator for subsequent CPAC in the Xn-U address indication message to notify the source SN of the CPC execution.

Observation 6: Introducing a new indicator for subsequent CPAC would be beneficial for the scenario when the MN decides to configure subsequent CPAC before the execution of CPAC.

Proposal 4: Introduce a new indicator for subsequent CPAC in the SN modification request message to indicate that the request is for subsequent CPAC.

Proposal 5: No need to introduce new indicators in UE context setup message, UE context modification message and bearer context setup request message.

Proposal 6: After receiving the RRCReconfigurationComplete message from the UE, the MN should send the Xn-U Address Indication message to the source SN to provide the data forwarding addresses, the source SN should send the Early Status Transfer message to the MN and the MN should send the Early Status Transfer message to the target SN to start early data forwarding.

Proposal 7: It is up to implementation to discard of the forwarded data for the first round of early data forwarding when the candidate SN receives the Early Status Transfer message when the second round of early data forwarding is started during subsequent CPC.

Observation 7: The existing RRC config indication IE is a per node indication other than a per cell indication, therefore it can not support a per cell indication as specified in the RAN2 agreement.

Proposal 8: Add the RRC config indication IE as a sub IE of the Conditional PSCell Addition Information Acknowledge IE in the SN addition request acknowledge message to indicate whether the associated SCG configuration is a delta with respect to the reference SCG configuration. This can be revisited based on RAN2 progress.

Observation 8: RAN2 companies still have different understandings on whether there is only one reference configuration for MN/SN initiated inter-SN SCPAC and SN-initiated intra-SN SCPAC or there is only one reference configuration for each scenario, i.e., one reference configuration for MN/SN initiated inter-SN SCPAC and one reference configuration for SN-initiated intra-SN SCPAC.

Proposal 9: RAN3 waits for RAN2 progress before discussing the reference configuration signalling design for SN-initiated intra-SN subsequent CPAC.

Observation 9: According to the endorsed RAN2 running CR R2-2309336, the reference configuration for CPAC is put into the CG-ConfigInfo message, which is included in the existing M-NG-RAN node to S-NG-RAN node Container IE. Therefore, the transfer of reference configuration has no RAN3 impact.

Observation 10: For MN/SN initiated inter-SN subsequent CPAC, the candidate SN or the source SN needs to generate the SCG part of the reference configuration.

Proposal 10: For MN/SN initiated inter-SN subsequent CPAC, the MN requests the source SN or the candidate SN to generate the SCG part of the reference configuration.

Proposal 11: Leave it to implementation to support the coexistence of subsequent CPAC and legacy CPAC, i.e. the MN sends two separate SN addition request messages, one is used to configure legacy CPAC and the other one is used to configure subsequent CPAC.

4	Text Proposal for BL CR TS 38.423

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.11	XN-U ADDRESS INDICATION

This message is either sent by the new NG-RAN node to transfer data forwarding information to the old NG-RAN node, or by the M-NG-RAN node to provide either data forwarding or Xn-U bearer address related information for SN terminated bearers to the S-NG-RAN node.

Direction: new NG-RAN node  old NG-RAN node, M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node | YES | ignore |
| Xn-U Address Information per PDU Session Resources List |  | 1 |  | This IE is ignored if the CHO DC Indicator IE is included and set to "coordination-only" or if the CPC Data Forwarding indicator IE is included and set to "coordination-only". | YES | reject |
| >Xn-U Address Information per PDU Session Resources Item |  | 1..<maxnoofPDUSessions> |  |  | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>Data Forwarding Info from target NG-RAN node | O |  | Data Forwarding Info from target NG-RAN node 9.2.1.16 |  | – |  |
| >>Secondary Data Forwarding Info from target NG-RAN node List | O |  | 9.2.1.31 | This IE would be present only when the target M-NG-RAN node decide to split a PDU session between MN and SN | YES | ignore |
| >>PDU Session Resource Setup Complete Info – SN terminated | O |  | 9.2.1.30 |  | – |  |
| >>DRB IDs taken into use | O |  | DRB List 9.2.1.29 | Indicating the DRB IDs taken into use by the target NG-RAN node, as specified in TS 37.340 [8]. | YES | reject |
| >>Data Forwarding Info from target E-UTRAN node | O |  | 9.2.1.35 |  | YES | ignore |
| CHO MR-DC Indicator | O |  | ENUMERATED (true, ..., coordination-only) | Indicating that the XN-U ADDRESS INDICATION message is for Conditional Handover, as specified in TS 37.340 [8]. | YES | reject |
| CHO MR-DC Early Data Forwarding Indicator | O |  | ENUMERATED (stop, ...) |  | YES | ignore |
| CPC Data Forwarding indicator | O |  | ENUMERATED (triggered, early data transmission stop, ..., coordination-only) | Indicating that the XN-U ADDRESS INDICATION message is for a Conditional PSCell Change. | YES | reject |
| CPC Execution Indicator | O |  | ENUMERATED (true, ...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSsessions | Maximum no. of PDU sessions. Value is 256 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDCP Change Indication | O |  | 9.2.3.74 |  | YES | ignore |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| SCG Configuration Query | O |  | 9.2.3.27 |  | YES | ignore |
| UE Context Information |  | 0..1 |  |  | YES | reject |
| >UE Security Capabilities | O |  | 9.2.3.49 |  | – |  |
| >S-NG-RAN node Security Key | O |  | 9.2.3.51 |  | – |  |
| >S-NG-RAN node UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate 9.2.3.17 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >Lower Layer presence status change | O |  | 9.2.3.60 |  | – |  |
| >PDU Session Resources To Be Added List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. | – |  |
| >>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| >>>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.2.3.82 | Expected UE Activity Behaviour for the PDU Session. | YES | ignore |
| >PDU Session Resources To Be Modified List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Modification Info – SN terminated IE nor the PDU Session Resource Modification Info – MN terminated IE is present in a PDU Session Resources To Be Modified Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. | – |  |
| >>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>>PDU Session Resource Modification Info – SN terminated | O |  | 9.2.1.9 |  | – |  |
| >>>PDU Session Resource Modification Info – MN terminated | O |  | 9.2.1.11 |  | – |  |
| >>>S-NSSAI | O |  | 9.2.3.21 |  | YES | reject |
| >>>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.2.3.82 | Expected UE Activity Behaviour for the PDU Session. | YES | ignore |
| >PDU Session Resources To Be Released List | O |  | PDU session List with Cause 9.2.1.26 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2. of TS 38.331 [10]. | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | ignore |
| Requested Split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested to be released. | YES | ignore |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Additional DRB IDs | O |  | DRB List 9.2.1.29 | Indicates additional list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| Requested Fast MCG recovery via SRB3 Release | O |  | ENUMERATED (true, ...) | Indicates that resources for fast MCG recovery via SRB3 are requested to be released. | YES | ignore |
| SN triggered | O |  | ENUMERATED (TRUE ...) |  | YES | ignore |
| Target Node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | Indicates the target node ID of the handover procedure decided by the M-NG-RAN node. | YES | ignore |
| PSCell History Information Retrieve | O |  | ENUMERATED (query, ...) | Indicates that the SN UE history information is requested. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| CHO Information SN Modification | O |  |  |  | YES | ignore |
| >Conditional Reconfiguration | M |  | ENUMERATED (intra-MN-CHO, ...) |  | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |
| Conditional PSCell Addition Information Modification Request | O |  |  | This IE may be sent to the target SN. | YES | ignore |
| >Maximum Number of PSCells To Prepare | O |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. |  |  |
| >Subsequent CPAC Information Request | O |  | 9.2.3.xxx | This information is used for subsequent CPAC. |  |  |
| Conditional PSCell Change Information Update | O |  |  | This IE may be sent to the source SN. | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>Candidate PSCell List |  | 1 |  |  | – |  |
| >>>>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | ignore |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.2.3.169 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

5	Text Proposal for BL CR TS 38.423

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

MN initiated conditional SN Change

The Conditional Secondary Node Change procedure is initiated by the MN for inter-SN CPC or inter-SN subsequent CPAC configuration and inter-SN CPC or inter-SN subsequent CPAC execution.

Figure 10.5.2-3: Conditional SN change procedure - MN initiated

Editor’s note: FFS whether to have a separate signaling flow for subsequent CPAC procedure, depending on further progress from RAN2 and RAN3.

Figure 10.5.2-3 shows an example signalling flow for the conditional SN Change initiated by the MN:

1/2.	The MN initiates the conditional SN change by requesting the candidate SN(s) to allocate resources for the UE by means of the SN Addition procedure, indicating that the request is for CPAC or subsequent CPAC. The MN also provides the candidate cells recommended by MN via the latest measurement results for the candidate SN(s) to choose and configure the SCG cell(s), provides the upper limit for the number of PSCells that can be prepared by the candidate SN. If subsequent CPAC is requested, the MN may also provide a reference SCG configuration for the candidate SN to generate the candidate PSCell configuration. Within the list of cells as indicated within the measurement results indicated by the MN, the candidate SN decides the list of PSCell(s) to prepare (considering the maximum number indicated by the MN) and, for each prepared PSCell, the candidate SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRCReconfiguration** message contained in the SN Addition Request Acknowledge message with the prepared PSCell ID(s). If data forwarding is needed, the candidate SN provides data forwarding addresses to the MN. The candidate SN includes the indication of the full or delta RRC configuration. If subsequent CPAC has been requested, the candidate SN may include an indication of that the provided SCG radio resource configuration is a complete or delta RRC configuration with respect to the reference SCG configuration. The candidate SN can either accept or reject each of the candidate cells listed within the measurement results indicated by the MN, i.e. it cannot configure any alternative candidates.

Editor’s note: FFS which node initially generates the reference configuration in subsequent CPAC.

Editor’s note: FFS whether the reference SCG configuration is optionally provided to the candidate SN(s).

Editor’s note: FFS which node(s) and how/when to generate execution conditions for subsequent CPC, e.g. when the determination node decides the candidate PSCells for initial CPC, or after the determination node knows all candidate PSCells prepared by other candidate SNs. FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC.

NOTE 4:	The MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration and to allow provision of data forwarding related information before step 1.

2a.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message to the candidate SN(s).

3.	The MN sends to the UE an RRCReconfiguration message including the CPC configuration or the subsequent CPAC configuration, i.e. a list of RRCReconfiguration* messages and associated execution conditions, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 2 and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, e.g., to configure the required conditional measurements. In subsequent CPAC, the RRCReconfiguration message may also include a reference SCG configuration.

Editor’s note: FFS if the reference configuration is optional in subsequent CPAC. FFS whether MCG configuration is included in the reference configuration.

Editor’s note: FFS whether the MCG configuration associated with the SCG configuration of a candidate PSCell is included in subsequent CPAC configuration.

4.	The UE applies the RRCReconfiguration message received in step 3, stores the CPC configuration or the subsequent CPAC configuration and replies to the MN with an RRCReconfigurationComplete message. In case the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message, it performs the reconfiguration failure procedure.

4a.	Upon receiving the MN RRCReconfigurationComplete message from the UE, the MN informs the source SN that the CPC or the subsequent CPAC has been configured via Xn-U Address Indication procedure, the source SN, if applicable, together with the Early Status Transfer procedure, starts early data forwarding. The PDCP SDU forwarding may take place during early data forwarding.

NOTE 4a:	Separate Xn-U Address Indication procedures may be invoked to provide different forwarding addresses of the prepared candidate target SNs. In this case, it is up to the MN and the source SN implementations to make sure that the EARLY STATUS TRANSFER message(s) from the source SN, if any, is forwarded to the right target destination. The Xn-U Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some SN-terminated bearers if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional SN change procedures.

NOTE 4b:	For the early transmission of MN terminated split/SCG bearers, the MN forwads the PDCP PDU to the candidate SN(s).

5.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an NR RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell. In subsequent CPAC, the UE keeps configured candidate PSCell configurations and evaluates the execution conditions of other candidate PSCells for subsequent CPAC.

Editor’s note: FFS whether to support the coexistence of legacy CPA/CPC and subsequent CPAC, i.e. there are some candidates for subsequent CPAC but others for legacy CPA/CPC.

6a-6c.	If the source SN was not configured as a candidate SN, the MN triggers the MN initiated SN Release procedure to inform the source SN to stop providing user data to the UE, and if applicable, triggers the Xn-U Address Indication procedure to inform the source SN the address of the SN of the selected candidate PSCell, to start late data forwarding. If the source SN was configured as a candidate SN, the MN triggers the Xn-U Address Indication procedure to inform the source SN of the CPC execution and stop providing user data to the UE, and inform the source SN the address of the SN of the selected candidate PSCell, to start late data forwarding.

7a-7c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SN Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

Editor’s note: FFS. It’s up to RAN3 how to notify the selected target SN in subsequent CPAC.

Editor’s note: FFS. It’s up to RAN3 whether/how to inform other candidate SN(s) in subsequent CPAC.

8.	The UE synchronizes to the PSCell indicated in the RRCReconfiguration* message applied in step 5.

9a-9b.	If PDCP termination point is changed for bearers using RLC AM, the source SN sends the message, which the MN sends then to the SN of the selected candidate PSCell, if needed.

10.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the early data forwarding address in step 4a.

11.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 5:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

12-16.	If applicable, a PDU Session path update procedure is triggered by the MN.

17.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

NOTE X:	In subsequent CPAC, if the execution condition of one candidate PSCell is satisfied, the UE executes steps 5-16, e.g. based on the configuration provided in step 3.

SN initiated conditional SN Change

The SN initiated conditional SN change procedure is used for inter-SN CPC or inter-SN subsequent CPAC configuration and inter-SN CPC or inter-SN subsequent CPAC execution.

The SN initiated conditional SN change procedure may also be initiated by the source SN, to modify the existing SN initiated inter-SN CPC or inter-SN subsequent CPAC configuration, or to trigger the release of the candidate SN by cancellation of all the prepared PSCells at the candidate SN and releasing the CPC or subsequent CPAC related UE context at the candidate SN.

NOTE 5a0:	To modify or release an existing intra-SN CPC or intra-SN subsequent CPAC configuration, the source SN triggers an SN initiated Conditional SN Modification (with or without SRB3) without MN involvement, as specified in 10.3.

Figure 10.5.2-4: Conditional SN change procedure - SN initiated

Editor’s note: FFS whether to have a separate signaling flow for subsequent CPAC procedure, depending on further progress from RAN2 and RAN3.

Figure 10.5.2-4 shows an example signalling flow for the conditional SN Change initiated by the SN:

1.	The source SN initiates the conditional SN change procedure by sending the SN Change Required message, which contains a CPC initiation indication or a subsequent CPAC initiation indication. The message also contains candidate node ID(s) and may include the SCG configuration (to support delta configuration), and contains the measurements results which may include cells that are not CPC candidates. The message also includes a list of proposed PSCell candidates recommended by the source SN, including execution conditions, the upper limit for the number of PSCells that can be prepared by each candidate SN, and may also include the SCG measurement configurations for CPC (e.g. measurement ID(s) to be used for CPC).

2/3.	The MN requests each candidate SN(s) to allocate resources for the UE by means of the SN Addition procedure(s), indicating the request is for CPAC or subsequent CPAC, and the measurements results which may include cells that are not CPC candidates received from the source SN to the candidate SN, and indicating a list of proposed PSCell candidates received from the source SN, but not including execution conditions. If subsequent CPAC is requested, the MN may provide a reference SCG configuration for the candidate SN to generate the candidate PSCell configuration. Within the list of PSCells suggested by the source SN, the candidate SN decides the list of PSCell(s) to prepare (considering the maximum number indicated by the MN) and, for each prepared PSCell, the candidate SN decides SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRCReconfiguration** message contained in the SgNB Addition Request Acknowledge message. If data forwarding is needed, the candidate SN provides data forwarding addresses to the MN. The candidate SN includes the indication of full or delta RRC configuration, and the list of prepared PSCell IDs to the MN. The candidate SN can either accept or reject each of the candidate cells suggested by the source SN, i.e., it cannot configure any alternative candidates. If subsequent CPAC has been requested, the candidate SN may include an indication of that the SCG radio resource configuration is a complete or delta RRC configuration with respect to the reference SCG configuration. Besides, the candidate SN generates execution conditions for subsequent CPC.

Editor’s note: FFS which node initially generates the reference configuration in subsequent CPAC.

Editor’s note: FFS how/when to generate execution conditions for subsequent CPC, e.g. when the candidate SN decides the candidate PSCells for initial CPC, or after the candidate SN knows all candidate PSCells prepared by other candidate SNs.

3a.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message to the candidate SN(s).

4/5.	The MN may indicate the candidate PSCells accepted by each candidate SN to the source SN via SN Modification Request message before it configures the UE, e.g., when not all candidate PSCells were accepted by the candidate SN(s). If the MN does not send such indication, step 4 and 5 are skipped. If requested, the source SN sends an SN Modification Request Acknowledge message and if needed, provides an updated measurement configurations and/or the execution conditions to the MN.

6.	The MN sends to the UE an RRCReconfiguration message including the CPC configuration or the subsequent CPAC configuration, i.e. a list of RRCReconfiguration* messages and associated execution conditions, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3 and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN, e.g., to configure the required conditional measurements. In subsequent CPAC, the RRCReconfiguration message may also include a reference SCG configuration.

Editor’s note: FFS if the reference configuration is optional in subsequent CPAC. FFS whether MCG configuration is included in the reference configuration.

Editor’s note: FFS whether the MCG configuration associated with the SCG configuration of a candidate PSCell is included in subsequent CPAC configuration.

7.	The UE applies the RRCReconfiguration message received in step 6, stores the CPC configuration or the subsequent CPAC configuration and replies to the MN with an RRCReconfigurationComplete message, which can include an NR RRCReconfigurationComplete*** message. In case the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message, it performs the reconfiguration failure procedure.

8.	If an SN RRC response message is included, the MN informs the source SN with the SN RRCReconfigurationComplete*** message via SN Change Confirm message. If step 4 and 5 are skipped, the MN will indicate the candidate PSCells accepted by each candidate SN to the source SN in the SN Change Confirm message.

The MN sends the SN Change Confirm message towards the source SN to indicate that CPC or subsequent CPAC is prepared, and in such case the source SN continues providing user data to the UE. If early data forwarding is applied, the MN informs the source SN the data forwarding addresses as received from the candidate SN(s), the source SN, if applicable, together with the Early Status Transfer procedure, starts early data forwarding. The PDCP SDU forwarding may take place during early data forwarding. In case multiple candidate SNs are prepared, the MN includes a list of Target SN ID and list of data forwarding addresses to the source SN.

NOTE 5a:	The Xn-U Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some PDCP SDUs if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional PSCell change.

NOTE 5b:	For the early transmission of MN terminated split/SCG bearers, the MN forwads the PDCP PDU to the candidate SN(s).

9a-9d.	The source SN may send the SN Modification Required message to trigger an update of CPC execution condition and/or corresponding SCG measurement configuration for CPC. In such case in step 9b, the MN reconfigures the UE and in step 9c the UE responds with RRCReconfigurationComplete, similarly as in steps 6 and 7.

10.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell. In subsequent CPAC, the UE keeps configured candidate PSCell configurations and evaluates the execution conditions of other candidate PSCells for subsequent CPAC.

Editor’s note: FFS whether to support the coexistence of legacy CPA/CPC and subsequent CPAC, i.e. there are some candidates for subsequent CPAC but others for legacy CPA/CPC.

11a-11c.	If the source SN was not configured as a candidate SN, the MN triggers the MN initiated SN Release procedure to inform the source SN to stop providing user data to the UE, and triggers the Xn-U Address Indication procedure to inform the source SN the address of the SN of the selected candidate PSCell and if applicable, starts late data forwarding. If the source SN was configured as a candidate SN, the MN triggers the Xn-U Address Indication procedure to inform the source SN of the CPC execution and stop providing user data to the UE, and inform the source SN the address of the SN of the selected candidate PSCell, to start late data forwarding.

12a-12c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SN Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

Editor’s note: FFS. It’s up to RAN3 how to notify the selected target SN in subsequent CPAC.

Editor’s note: FFS. It’s up to RAN3 whether/how to inform other candidate SN(s) in subsequent CPAC.

13.	The UE synchronizes to the PSCell indicated in the RRCReconfiguration* message applied in step 10.

14.	If PDCP termination point is changed for bearers using RLC AM, the source SN sends the SN Status Transfer message, which the MN sends then to the SN of the selected candidate PSCell, if needed.

15.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the data forwarding address related information from the MN.

16.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 6:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

17-21.	If applicable, a PDU Session path update procedure is triggered by the MN.

22.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

NOTE X:	In subsequent CPAC, if the execution condition of one candidate PSCell is satisfied, the UE executes steps 10-21, e.g. based on the configuration provided in step 6.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>