3GPP TSG-RAN WG3 Meeting #121	R3-235378

Xiamen, China, 9 - 13 October 2023

Agenda Item:	25.2.1

Source:	ZTE

Title:	TP for XR BL CR to TS 38.415

Document for:	Discussions & Approval

1	Introduction

The discussion part can be seen in R3-235377 (TP to TS 38.413) Discussion on support of PDU Set Handling.

This paper provides the corresponding TP for TS 38.415 to support XR.

2	Text Proposal for BL CR TS 38.415

START OF CHANGES

### 5.5.2	Frame format for the PDU Session user plane protocol

#### 5.5.2.1	DL PDU SESSION INFORMATION (PDU Type 0)

This frame format is defined to allow the NG-RAN to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective DL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 |  |  | 5 | 4 | 3 |  |  | 2 |  | 1 |  | 0 |  |  |
| PDU Type (=0) |  |  |  |  |  | QMP |  |  | SNP |  | MSNP |  |  | PSNI | 1 |
| PPP | RQI |  |  | QoS Flow Identifier |  |  |  |  |  |  |  |  |  |  | 1 |
| PPI |  |  |  |  | PSS |  | EPI |  |  | EDBI |  | Spare |  |  | 0 or 1 |
| DL Sending Time Stamp |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL QFI Sequence Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL MBS QFI Sequence Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| EPDU |  | EDB |  |  |  |  |  | PSI |  |  |  |  |  |  | 0 or 1 |
| PSSN |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 2 |
| PSSN |  |  | PSN |  |  |  |  |  |  |  |  |  |  |  |  |
| PSSize |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL PDU SESSION INFORMATION (PDU Type 0) Format

NEXT CHANGE

### 5.5.3	Coding of information elements in frames

#### 5.5.3.1	PDU Type

Description: The PDU Type indicates the structure of the PDU session UP frame. The field takes the value of the PDU Type it identifies; i.e. "0" for PDU Type 0. The PDU type is in bit 4 to bit 7 in the first octet of the frame.

Value range: {0= DL PDU SESSION INFORMATION, 1=UL PDU SESSION INFORMATION, 2-15=reserved for future PDU type extensions}.

Field length: 4 bits.

#### 5.5.3.2	Spare

Description: The spare field is set to "0" by the sender and should not be interpreted by the receiver. This field is reserved for later versions.

Value range: (0–2n-1).

Field Length: n bits.

*// skip unchanged part //*

#### 5.5.3.23	MBS Sequence Number Presence (MSNP)

Description: This parameter indicates the presence of the DL MBS QFI Sequence Number in the DL PDU Session Information frame.

Value range: {0= DL MBS QFI Sequence Number not present, 1= DL MBS QFI Sequence Number present}.

Field length: 1 bit.

#### 5.5.3.24	DL MBS QFI Sequence Number

Description: This parameter indicates the sequence number as assigned by the MB-UPF associated with a given MBS QoS Flow.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.x1	PSNI (PDU Sequence Number Indicator)

Description: This parameter indicates the presence of PDU Set Sequence Number (PSSN) and PDU Sequence Number within a PDU Set (PSN).

Value range: {0= PSSN and PSN not present, 1= PSSN and PSN present}.

Field length: 1 bit.

#### 5.5.3.x2	PSS (PDU Set Presence)

Description: This parameter indicates the presence of PDU Set Importance (PSI) and PDU Set Size (PSSize).

Value range: {0= PSI and PSSize not present, 1= PSI and PSSize present}.

Field length: 1 bit.

#### 5.5.3.x3	EPI (End PDU indicator)

Description: This parameter indicates the presence of End PDU of the PDU Set (EPDU).

Value range: {0= EPDU not present, 1= EPDU present}.

Field length: 1 bit.

#### 5.5.3.x4	EDBI (End Data Burst indicator)

Description: This parameter indicates the presence of End of Data Burst (EDB).

Value range: {0= EDB not present, 1= EDB present}.

Field length: 1 bit.

#### 5.5.3.x5	End PDU of the PDU Set (EPDU)

Description: This parameter indicates whether the current PDU is the last PDU of the PDU set.

Value range: {0= all other PDUs of the PDU Set, 1= last PDU of the PDU set}.

Field length: 1 bit.

#### 5.5.3.x6	End of Data Burst (EDB)

Description: This parameter indicates the end of a Data Burst.

Value range: {0..23-1}.

Field length: 3 bits.

#### 5.5.3.x7	PDU Set Importance (PSI)

Description: This parameter indicates the importance of the current PDU Set compared to other PDU Sets within the same QoS flow. Lower values shall indicate a higher importance. PDU Set with the highest importance PDU Set is indicated by 0 and the lowest importance PDU Set is indicated by 15.

Value range: {0..24-1}.

Field length: 4 bits.

#### 5.5.3.x8	PDU Set Sequence Number (PSSN)

Description: This parameter indicates the sequence number of the PDU Set to which the current PDU belongs acting as an identifier for the PDU Set.

Value range: {0..216-1}.

Field length: 2 octets.

#### 5.5.3.x9	PDU Sequence Number within a PDU Set (PSN)

Description: This parameter indicates the sequence number of the current PDU within the PDU Set. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU set in order of transmission from the sender.

Value range: {0..28-1}.

Field length: 1 octet.

#### 5.5.3.x10	PDU Set Size (PSSize)

Description: This parameter indicates the total size of all PDUs of the PDU Set to which the current PDU belongs.

Value range: {0..224-1}.

Field length: 3 octets.

END OF CHANGES