3GPP TSG-RAN WG3 Meeting #121bis	R3-235396

Xiamen, 9 Oct – 13 Oct 2023

Title: 	Discussion on QoE measurement in NR-DC

Source: 	China Unicom

Agenda item:	11.3

Document Type:	Discussion

Introduction

The Work Item on QoE reporting has been agreed in RAN#96 (RP-221803) with the following objectives:

- Specify to support for QoE in NR-DC, e.g. enable QoE reporting via SN [RAN3, RAN2]. - Specify the QoE configuration, and measurement reporting over MN/SN for NR-DC architecture, and specify the QoE measurement reporting over the other DC leg in order to maintain the reporting continuity. Note 1: The QoE measurements are not performed separately for each leg. - Support RAN-visible QoE and radio related measurement configuration and reporting in NR-DC scenarios. - Specify the QoE measurement continuity in mobility scenarios in NR-DC. - Specify the alignment of QoE measurements (including legacy QoE and RAN visible QoE measurements) and radio related measurement in NR-DC.

This paper further discusses how to configure the QoE measurement and RAN-Visible QoE measurement in NR-DC based on the agreements and FFS of the previous meetings.

Discussion

QMC Initial Coordination for NR-DC

QMC Initial Coordination procedure for NR-DC is used to indicate the information that the S-NG-RAN node needs to provide to the M-NG-RAN node or the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements.

If the QoE configuration is received by the MN, the MN can send the QoE configuration directly to UE, and the MN can generate and send the RAN visible QoE configuration directly to UE, but for the QoE reporting/RAN visible QoE reporting, it is reasonable that the SN can also coordinate to receive them. On the other hand, if the coordination procedure is also used in the SN change procedure, the coordination request message is always sent from the MN to SN, it is also nessesary to coordinate the QoE reporting/RAN visible QoE reporting leg.

Proposal 1: To express MN’s preference for the QoE reporting and RAN visible QoE reporting, the reporting option need to be supported in the coordination request message send from MN to SN.

QMC Modification for NR-DC

QMC Modification procedure is used to indicate the information that the S-NG-RAN node needs to provide to the M-NG-RAN node or the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, for requesting modification of one or more QoE and/or RAN visible QoE measurements.

If the MN intends to change the reporting leg of QoE or RAN visible QoE to SN, it need to send the QMC Modification Request message to indicate SN about the reporting leg to be changed, and the SN can trigger the SRB5 establishment procedure if there is no SRB for this UE. If the SN can’t establish the SRB5, SN can send the QMC Modification Response with the IE QoE Reporting Modification response/RVQoE Reporting Modification response set to reject, then the QoE reporting/RAN visible QoE reporting will still report via MN.

Proposal 2: The reporting leg modification to SN can trigger the SRB5 establishment, if the SN can’t establish the SRB5, the QoE reporting/RAN visible QoE reporting can still report via MN.

In the QMC Modification Response message, RVQoE Reporting Modification response IE can indicate whether the peer node accept to receive the RAN visible QoE reporting, but there is a new case that the peer node doesn’t want to configure the RAN visible QoE at that time, it intends to release the RAN visible QoE configuration, and the code point needs to be extended to support this case.

Proposal 3: RVQoE Reporting Modification response IE in QMC Modification Response message need to be extended to indicate that the node doesn’t intend to configure want to configure the RAN visible QoE at that time.

In the QMC Modification Request message, the RAN Visible QoE Reporting Modification Request IE indicates the non-RVQoE-reporting node that it is this node that provides the bearer(s) for the application session, so that the reporting leg of RAN Visible QoE need to be switched to the non-RVQoE-reporting node. There is a new case that the bearer(s) for the application session is running both in the MN and SN, and RVQoE-reporting node intends to get the RAN Visible QoE Configuration Preference of the non-RVQoE-reporting node, but the reporting leg can be switch or not. Then the RVQoE-reporting node need to send the QMC Modification Request message to indicate the non-RVQoE-reporting node about the session status. If the non-RVQoE-reporting node receives an indication that the application session is running in this node, it can send the RAN Visible QoE Configuration Preference to RVQoE-reporting node, and the reporting leg can remain unswitched.

Proposal 4: An individual session status indication should be sent from the RVQoE-reporting node to non-RVQoE-reporting node independent of the RAN Visible QoE Reporting Modification Request.

There is a case that the RVQoE-configuring node is not the node providing the bearer(s) for the application session. In this case, when the node that provides the bearer(s) intending to reconfigure the RAN visible QoE, it needs to send the RAN Visible QoE Configuration Preference to the RVQoE-configuring node. Actually, if the bearer(s) for the application session is only running on the non-RVQoE-configuring node, it can send the RAN visible QoE configuration to UE directly without the forwarding to the other node.

Proposal 5: The session status indication should indicate whether the bearer(s) for the application session is(are) only established in the current non-RVQoE-configuring node, or established in both RVQoE-configuring node and non-RVQoE-configuring node.

Proposal 6: If the application session is(are) only established in the current non-RVQoE-configuring node, the current non-RVQoE-configuring node can generate and the RAN visible QoE configuration directly to UE when it intends to reconfigure RAN visible QoE.

QoE measurement continuity in NR-DC

For QoE measurement continuity in mobility scenarios in NR-DC, RAN3 have the following agreements in last meeting:

Reuse legacy procedures for the MN-SN coordination in NR-DC to support QoE.

The MN-SN coordination procedure is added in the S-NODE ADDITION procedure, and it can be used for the QoE measurement continuity in NR-DC.

The mobility procedure for NR-DC can be classified to the following cases:

- Intra-RAT MN handover procedure: reusing the messages of normal handover procedure, so there is no RAN3 specification impacts in this case.

- SN change procedure: SN Addition procedure and SN Release procedure will be used.

- Inter-RAT MN handover procedure: reuse the inter-RAT handover procedure.

Figure 10.7.2-1: Inter-MN handover with/without MN initiated SN change procedure

Figure 10.5.2-1: SN change procedure - MN initiated

Figure 10.5.2-2: SN change procedure - SN initiated

Figure 10.7.2-1: Inter-MN handover with/without MN initiated SN change procedure

Figure 10.8.1-1: Master Node to eNB Change procedure

Figure 10.8.1-1: Master Node to eNB Change procedure

Figure 10.9.2-1: ng-eNB/gNB to MN change procedure

After the analysis of the above procedure for mobility in NR-DC, the messages for mobility procedure in NR-DC are similar with the Secondary Node Addition/Secondary Node Release procedure, so it is preferred to include the QMC Coordination messages in the existing procedure, include Inter-MN handover with/without MN initiated SN change procedure, MN initiated SN change procedure, SN initiated SN change procedure and Inter-MN handover with/without MN initiated SN change procedure. The QMC configuration information can be exchanged in these messages which will have the minimal change to the current specification.

Proposal 7: QMC Initial Coordination in the S-NODE ADDITION procedure can be used in the SN change scenarios, to indicate the new SN the QoE configuration and to indicate the new SN to receive the QoE/RVQoE reporting if needed.

In the SN change procedure, there is a case that the target SN want to reconfigure the RAN visible QoE, and it need to send the new RAN visible QoE configuration to the MN, and the MN can send the new RAN visible QoE configuration to UE. The QMC Initial Coordination procedure need to be enhanced, the available RAN visible QoE configuration need to be included in the QMC Initial Coordination request message, and the RAN Visible QoE Configuration Preference need to be included in the QMC Initial Coordination Response message to indicate the new SN’s preference for RAN Visible QoE Configuration.

Proposal 8: Including the available RAN visible QoE configuration to the QMC Initial Coordination request message to indicate the bearers for the application session is provided by the new SN, and the SN can reconfigure the RAN visible QoE configuration.

Proposal 9: Include the RAN Visible QoE Configuration Preference to indicate the new SN’s preference for RAN Visible QoE Configuration.

Conclusion

Proposal 1: To express MN’s preference for the QoE reporting and RAN visible QoE reporting, the reporting option need to be supported in the coordination request message send from MN to SN.

Proposal 2: The reporting leg modification to SN can trigger the SRB5 establishment, if the SN can’t establish the SRB5, the QoE reporting/RAN visible QoE reporting can still report via MN.

Proposal 3: RVQoE Reporting Modification response IE in QMC Modification Response message need to be extended to indicate that the node doesn’t intend to configure want to configure the RAN visible QoE at that time.

Proposal 4: An individual session status indication should be sent from the RVQoE-reporting node to non-RVQoE-reporting node independent of the RAN Visible QoE Reporting Modification Request.

Proposal 5: The session status indication should indicate whether the bearer(s) for the application session is(are) only established in the current non-RVQoE-configuring node, or established in both RVQoE-configuring node and non-RVQoE-configuring node.

Proposal 6: If the application session is(are) only established in the current non-RVQoE-configuring node, the current non-RVQoE-configuring node can generate and the RAN visible QoE configuration directly to UE when it intends to reconfigure RAN visible QoE.

Proposal 7: QMC Initial Coordination in the S-NODE ADDITION procedure can be used in the SN change scenarios, to indicate the new SN the QoE configuration and to indicate the new SN to receive the QoE/RVQoE reporting if needed.

Proposal 8: Including the available RAN visible QoE configuration to the QMC Initial Coordination request message to indicate the bearers for the application session is provided by the new SN, and the SN can reconfigure the RAN visible QoE configuration.

Proposal 9: Include the RAN Visible QoE Configuration Preference to indicate the new SN’s preference for RAN Visible QoE Configuration.