3GPP TSG RAN WG3 121-bis	R3-235399

Xiamen, China, 09 - 13 October, 2023

Title: 	Discussion on QoE measurement in RRC_INACTIVE and RRC_IDLE states

Source: 	China Unicom

Agenda item:	11.2

Document Type:	Discussion

Introduction

The Work Item on QoE reporting has been agreed in RAN#96 (RP-221803) with the following objectives:

- Specify the new service and the existing service defined or to be supported by SA4, combined with high mobility scenarios, e.g., High Speed Trains. - Specify for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service [RAN3, RAN2]. - Specify the mechanism to support the alignment of the existing radio related measurement and QoE reporting.

This paper discusses to support the QoE measurement configuration and reporting in RRC_INACTIVE and RRC_IDLE states for MBS service, as well for the high mobility scenarios.

Discussion

QoE configuration for MBS service

A reply LS on QoE measurement collection for application sessions delivered via MBS broadcast or multicast in R3-233457 as below:

Q1: RAN3 kindly requests SA4 to provide guidance on which of the options RAN3 should pursue in Rel-18, in order to support QMC for application sessions delivered via MBS broadcast or multicast. Answer: As mentioned in previous LS S4-230347, since there are no ongoing Rel-18 study or normative work regarding MBS QoE in SA4, SA4 suggests RAN3 to pursue the Option 2 in Rel-18. The "MBS" is considered as a communication service only, which can be used to deliver the application services, e.g. DASH streaming, VR streaming.

The reply LS states that option 2 would be pursued, since there is no MBS service QoE metric defined in Rel-18. Thus R3 agree to consider MBS as a communication service and introduce the concept of “MBS delivery mode” in Rel-18.

Proposal 1: RAN3 agree to consider MBS as a communication service in Rel-18.

QoE configuration information for MBS in RRC_CONNECTED from RRC_INACTIVE and RRC_IDLE states

An agreement has been made during the previous meetings:

RAN3 shall discuss which of QoE configuration info for MBS BC QoE shall be available in the new gNB.

QoE reference (agreed in 119)

Measurement Collection Entity Information, the detail information can be further discussed (agreed in 119)

Measurement Configuration Application Layer ID (RRC level ID) (agreed in 119bis)

QoE measurement type (signalling based, management based) (agreed in 119bis)

Service Type (agreed in 121)

Awareness of session status (agreed in 121)

Slice scope – Not needed for broadcast(agreed in 121)

WA: Available RVQOE metrics

Area Scope of QMC (area scope)

MDT Alignment Information

With respect to whether to use MCE IP address or MCE ID, considering MCE IP address IE would be consuming for Uu interface, then MCE ID can be considered for UE-based solution similar as the logged MDT case. For CN-based solution, MCE IP address can be directly reused because there is no storage issue for CN although SA2 should be asked for the feasibility.

Proposal 2: MCE IP ID information can be provided to new gNB for UE-based solution, and MCE IP address IE can be provided to new gNB for CN-based solution.

An agreement has been made during in last meeting:

When a UE is in the RRC_CONNECTED state, the area scope checking is done by the RAN, based on the Area Scope of QMC IE, in line with the current network behavior as specified in TS 38.413.

In last meeting, RAN3 decided that RAN check the area scope in RRC_CONNECTED state. When UE comes back from RRC_IDLE to RRC_CONNECTED state, gNB needs to perform area scope checking, and then area scope IE should be available at new gNB.

Proposal 3: Area Scope IE can be provided to new gNB for MBS BC QoE.

Based on the following agreements made in #119 meeting:

For MBS QoE, an M-based QoE configuration shall not overwrite the S-based QoE configuration stored at the UE by the new gNB.

QoE measurement type (s-based or m-based measurement) for MBS broadcast service should be available in the gNB serving the UE after the transition from RRC_IDLE to RRC_CONNECTED.

To avoid overriding issue, the amount of configured QoE measurement should be provided to gNB at the beginning of RRC setup, and then the new gNB can send the new QoE configuration(s) to the UE based on the existing number of QoE measurement configurations already in UE. UE can indicate the total number of the configured QoE measurements.

Proposal 4: UE can include the number of existing QoE configuration when the UE switches from RRC_IDLE to RRC_CONNECTED in RRCSetupComplete message to avoid overriding issue.

The new gNB should be provided with the information demonstrated above, both CN-based and UE-based solution seem feasible to handle this issue. But considering CN-based solution would bring large impacts on Core Network which need to store all the network instance QoE configuration for each UEs, UE-based solution is simpler and it is a straightforward solution to store the gNB’s configuration and report it within the legacy QoE report as the same solution of the logged MDT mechanism.

Proposal 5: RAN3 agrees UE-based solution to handle the MBS broadcast QoE measurements can proceed after the UE switches from RRC_IDLE to RRC_CONNECTED.

RVQoE configuration for MBS service

An agreements made in #121 meeting:

There is no need for UEs in RRC_INACTIVE and RRC_IDLE to perform RVQoE measurement collection.

For UEs in RRC_CONNECTED state, RAN visible QoE measurement collection needs to be supported. Regarding the RVQoE metric for MBS service, RAN3 can reuse the existing RVQoE metric for each service type delivered by broadcast.

Proposal 6: RVQoE measurement is supported in RRC_CONNECTED state.

Proposal 7: RAN3 decide to reuse the existing RVQoE metric for each service type delivered by MBS.

For UEs coming back to RRC_CONNECTED from RRC_IDLE state, the new gNB can configure its own RVQoE measurement based on the available RVQoE metrics for the ongoing QoE measurement configured by old gNBs. Then the new gNB can configure the RVQoE directly and it is no need to wait for the new QoE measurement sent from OAM.

Proposal 8: Turn WA into agreement that available RVQoE metrics need to be provided at new gNB for MBS BC QoE.

Priority information

We have a FFS in previous meeting:

FFS whether to support some selection policies to better report/discard reports in case of limited storage space

And there is a reply LS on the feasibility of introducing assistance information for handling of QoE reporting during RAN overload.

SA5 has the following responses to RAN2's questions: Q1: Can there be multiple types of consumers for receiving the QoE reports (pursuing the role of the MCE)? If yes, what are those potential consumers as supported by the current specifications? A: No, SA5 does not specify the different types of consumers so far. Existing activation and deactivation procedures could be applied to any consumer. Q2: If the answer to the first part of Q1 is “yes”, and if different consumers can have different priorities in receiving the QoE reports, is the OAM able to compare and rank by priority the preferences of different consumers or rank the consumers themselves? Can the OAM make the final decision regarding setting the reporting priorities? Can the OAM coordinate with the consumers and inform the consumers if the intended priority is not met? A: see Q1. From SA5 point of view, even though there are no different types of consumers specified by SA5, it is useful to introduce a priority per QoE configuration as assistance information to the NG-RAN node. Q3: Is there any other issue(s) related to sending such an explicit priority per QoE configuration as assistance information to the NG-RAN node? A: The mechanisms of QMC defined in TS 28.405 does not support such priority per QoE configuration currently. SA5 think it is possible to introduce a priority per QoE configuration for one certain service type or QoE reference in case of the QoE reporting to an NG-RAN node that is in overload.

We now have a positive response from SA5 that introducing a priority per QoE configuration as assistance information is useful in RAN overload. Similar as in RAN overload scenario, the priority information can also be used as the selection policy for UE in case of limited storage when UE stays in RRC_INACTIVE/IDLE for a long time. UE can use the priority information to discard those QoE report with lower priority when the buffer is full.

Proposal 9: Introduce priority information as selection policy for UE in case of limited storage space.

QoE configuration for high mobility scenarios

The following was captured in the previous meeting with respect to high speed scenarios:

RAN3 first focus on supporting the following scenario QMC:

- QoE measurement collection and reporting when the UE is in HSDN cells

For confining the QoE measurements to HSDN cells, RAN3 to choose between the HSDN-wide indication, existing area scope and other possible enhancements if needed.

For supporting QMC in high mobility scenarios, RAN3 to determine the meaning of “high mobility”.

Regarding HSDN cells, OAM is aware of the whole HSDN cells list and it can configure those HSDN cell accordingly. Enlarging the QoE area scope of whole list of HSDN cells seems to be costly for the purpose of the continuity QMC in HSDN cells. Then a HSDN-cell indication can be considered, only if the HSDN cell should performance the HSDN-QoE configuration to its UEs according to this indication sending from the OAM.

Meanwhile considering the handover scenario, the target gNB can check whether it belongs to the HSDN-cell, then to decide whether to release the corresponding HSDN QoE measurement or not. The non-HSDN cell can release the HSDN dedicated QoE measurement while the HSDN cell can proceed the corresponding QMC.

Observation 1: OAM can configure the HSDN dedicated QoE measurement towards the HSDN cells.

Proposal 10: A HSDN-wide indication can be added for HSDN QoE configuration for handover scenarios.

Proposal 11: The non-HSDN cells can release the HSDN-dedicated QoE measurement.

For determining the meaning of “high mobility”, based on the previous agreement, we first focus on the UEs in HSDN cell in high-mobility state, then RAN3 can start with the HSDN cells in high mobility scenarios.

Proposal 12: For supporting QMC in high mobility scenarios, RAN3 decides to support HSDN cell only scenario.

Conclusion

Proposal 1: RAN3 agree to consider MBS as a communication service in Rel-18.

Proposal 2: MCE IP ID information can be provided to new gNB for UE-based solution, and MCE IP address IE can be provided to new gNB for CN-based solution.

Proposal 3: Area Scope IE can be provided to new gNB for MBS BC QoE.

Proposal 4: UE should include the number of existing QoE configuration when the UE switches from RRC_IDLE to RRC_CONNECTED in RRCSetupComplete message to avoid overriding issue.

Proposal 5: RAN3 agrees UE-based solution to handle the MBS broadcast QoE measurements can proceed after the UE switches from RRC_IDLE to RRC_CONNECTED.

Proposal 6: RVQoE measurement is supported in RRC_CONNECTED state.

Proposal 7: RAN3 decide to reuse the existing RVQoE metric for each service type delivered by MBS.

Proposal 8: Turn WA into agreement that available RVQoE metrics need to be provided at new gNB for MBS BC QoE.

Proposal 9: Introduce priority information as selection policy for UE in case of limited storage space.

Observation 1: OAM can configure the HSDN-dedicated QoE measurement towards the HSDN cells.

Proposal 10: A HSDN-wide indication can be added for HSDN QoE configuration for handover scenarios.

Proposal 11: The non-HSDN cells can release the HSDN-dedicated QoE measurement.

Proposal 12: For supporting QMC in high mobility scenarios, RAN3 decides to support HSDN cell only scenario.