3GPP TSG-RAN WG3 #121-bis	R3-235414

Xiamen, China, 9th – 13th October 2023

Source:	CATT

Title:	Discussion on Multicast over Inactive

Agenda Item:	15.3

Document for:	Discussion and decision

- Introduction

This discussion paper discusses four aspects related to the “Multicast over Inactive” mode.

- Discussion

  - Rel-18 mode distinguisher with group paging messages

1) NG group paging

It is captured in Clause 7.2.2.3 “Multicast session leave requested by the network or MBS session release” in TS 23.247 that:

Figure 7.2.2.3-1: MBS Session Release or Multicast session leave requested by the network 1a. For MB-SMF triggered MBS session release, the SMF receives Nmbsmf_MBSSession_ContextStatusNotify (MBS Session ID, multicast session release) from the MB-SMF with MBS Session ID. The SMF checks all joined UEs and perform step 2 to step 9 for each UE. 1b. The SMF decides to remove a UE from the MBS session without MBS session release (e.g. due to UE moving out of MBS service area for local or location dependent MBS service as described in clause 7.2.4). 2. For UEs without activated UP, the SMF may perform the same procedure as defined in step 3-7 in clause 7.2.5.2. Alternatively, for UEs without activated UP, the SMF does not trigger message to the AMF, instead the SMF marks that the UE is to be informed of the MBS Session release. In this case, the SMF initiates PDU Session Modification to inform the UE of the MBS Session release at next UP activation of the associated PDU Session, if needed. 3. For the joined UEs with UP activated, the SMF invokes Namf_Communicate_N1N2MessageTransfer to the AMF. The N1 SM container indicates UE removed from MBS session with appropriate cause (e.g. MBS session release, out of MBS service area, etc.). In N2 SM information, the SMF informs the NG-RAN to remove the UE from the MBS session. If there are associated QoS Flow(s) for individual delivery, the SMF also releases those QoS Flow(s) as specified in TS 23.502 [6] clause 4.3.3.2. 4. The AMF sends N2 Request to the NG-RAN.

The clause name of Clause 7.2.5.2 is “MBS session activation procedure”, and Step 3–7 in Clause 7.2.5.2 includes the procedure of performing CN-initiated group paging:

5. [Conditional] If AMF determines that there are UEs in CM-IDLE state and involved in the multicast MBS Session, the AMF figures out the paging area covering all the registration areas of those UE(s), which need to be paged. The AMF may apply paging differentiation as specified in clause 6.12. The AMF sends a Multicast Group paging request message to the NG-RAN node(s) belonging to this Multicast Paging Area with the involved UE list and TMGI as the identifier to be paged if the related NG-RAN node(s) support MBS. If the NG-RAN node(s) do not support MBS, the AMF sends Paging message(s) to the NG-RAN node(s) per UE as described in step 4b in clause 4.2.3.3 of TS 23.502 [6].

So it can be concluded that, it is up to the SMF’s implementation whether to trigger group paging.

Observation 1: According to TS 23.247, CN group paging is used to move RRC_IDLE UEs into RRC_CONNECTED upon network-triggered multicast session leave.

Observation 2: According to TS 23.247, it is up to the SMF’s implementation whether to initiate the CN group paging procedure upon network-triggered multicast session leave. The only alternative implementation is to wait until UE accesses later. UE individual CN paging is never used upon network-triggered multicast session leave.

During the discussion of introducing the “multicast over INACTIVE” mode, RAN2 introduced a new indicator “inactiveReceptionAllowed-r18” into Uu (group) paging message so that UEs using the “multicast over INACTIVE” mode may keep itself in RRC_INACTIVE state, and it’s up to the RAN’s implementation whether to include this new IE when paging.

Based on Observation 1&2, it is clear that an NG-RAN node may receive NGAP group paging message for two reasons: MBS Session activation or network-triggered multicast session release. For the former case, the NG-RAN node should include the new indicator “inactiveReceptionAllowed-r18” into the Uu paging message so that UEs may stay in RRC_INACTIVE. For the latter case, the NG-RAN node should not include it so that UEs all move into RRC_CONNECTED.

Observation 3: NG-RAN nodes need to distinguish whether an NGAP group paging message it receives is used to move UEs into RRC Connected state or not.

Following we will analyse NGAP group paging and XnAP group paging separately.

Currently, for MBS Session Activation, CN would trigger both MBS Session Activation procedure and Group paging procedure towards NG-RAN node. If it could be guaranteed that the NG-RAN node always receives the MBS Session Activation message first, the NG-RAN node could deduce that the subsequent Group Paging message is for MBS session activation.

From our point of view, although it is reasonable that CN sends the MBS Session Activation and the Goup paging message simutanously, it could not be guaranteed that the MBS Session Acitvation message arrived at the NG-RAN node first. So, we think it is more reliable to introduce an indictor into the NG Group paging message to inform the NG-RAN that the message is sent due to MBS session activation.

Proposal 1: We propose RAN3 to discuss whether it could be guaranteed that an NG-RAN node always receives the MBS Session Activation message before the NGAP Group paging message, and then make decision on whether to include the “inactiveReceptionAllowed-r18” IE into Uu paging message to indicate the activation of MBS session. Our understanding is that it cannot be guaranteed so a new indicator on NGAP is necessary.

2) Xn group paging

There may be some problem if only part of NG-RAN nodes are upgraded in one area, i.e. neighbour NG-RAN nodes are from different releases.

One example is that the anchor gNB is a Rel-17 node and one of its neighour gNB is a Rel-18 gNB. If the Rel-18 gNB receives an XnAP group paging from its Rel-17 neighbour and (correctly) deduce that this paging message is caused by session activation, it includes the “inactiveReceptionAllowed-r18” IE into the paging messages so that UEs may stay in RRC inactive state. However, the Rel-17 anchor node, unaware of the existence of “multicast over INACTIVE” mode, ordinarily expects all UEs it pages within an XnAP group paging message will soon return RRC CONNECTED state. If it finds that a UE does not resume timely (because it is a Rel-18 UE, receives the Rel-18 indicator in Uu paging and need not resume), it will ordinarily consider this UE unreachable and release the UE context, causing either an unnecessary CN paging procedure or context misalignment between the UE and the network. To avoid the misalignment or unnecessary CN paging, it is still necessary to introduce an indicator into the XN paging message indicating that the UEs are allowed to stay in RRC INACTIVE state, and its criticality should be “ignore”. That is to say, whenever a node does not support “multicast over INACTIVE” mode, it antomatically converses the Rel-18 group paging message toward Rel-17 conventional one.

With this new indicator introduced, a Rel-18 NG-RAN node would always initiate a Uu group paging procedure if Xn Group Paging message is received without presence of new IE. Otherwise, the Rel-18 NG-RAN node would initiate enhanced Uu Group paging procedure, i.e. includes the “inactiveReceptionAllowed-r18” IE.

Proposal 2: To introduce an indicator into the Xn Group Paging messages to indicate that the receiving node may/shall issue a Uu (group) paging message including the “inactiveReceptionAllowed-r18” IE.

  - Acquiring PTM configuration to encode RRC Release message

Last meeting we discussed how the gNB-CUs acquire PTM configurations in order to include it into the RRC Release message. The view was split and tagged pending RAN2, but we think that some agreement can be achieved in RAN3 before any further progress in RAN2. (Anyway, we acknowledge that it may be premature to agree any TP on this issue.)

Generally speaking, there are two ways to provide PTM configuration toward gNB-CUs:

- The “pulling” mode: whenever the gNB-CU is to release a UE, it retrieves the PTM configuration grom the gNB-DU.

- The “pushing” mode: whenever the PTM configuration changes, the gNB-DU pushes the updated configuration toward the gNB-CU.

The pros and cons are listed in the following table:

| Mode | “Pulling” | “Pushing” |
| --- | --- | --- |
| Number of F1AP messages | As frequent as RRC Release | As frequent as PTM configuration update |
| Signalling impact | Introducing IEs into an existing F1AP procedure, e.g. the gNB-CU-triggered UE Context Modification procedure (i.e. “Option 1” last meeting”) or the Multicast Context Modification procedure (i.e. “Option 2” last meeting). | A New gNB-DU-triggered multicast-associated message needs to be introduced. |

We prefer the “pushing” mode as it is aligned with the method used for MCCH configuration (which is also required to be delivered in the Uu RRCRelease message). We propose RAN3 to discuss this issue.

In addition, there was one concern raised last meeting that RedCap UE may require another PTM configuration using a special CFR. But as of Rel-18 such special CFR is only applicable for broadcast. Even though such special CFR is supported in later releases, it is likely to reuse the mechanism for broadcast that both CFR is provided within one single MCCH message. Therefore, introducing only one MCCH message per cell is sufficient enough.

Proposal 3: We propose RAN3 to discuss whether to introduce a “pulling” mode or a “pushing” for gNB-CUs to acquire the PTM configuration from gNB-DU. Our preference is the “pushing” mode. Note that introducing only one MCCH message per cell is sufficient enough.

  - Decision of enabling “inactive reception allowed” mode

Last meeting we agreed that it is the gNB-CU-CP to decide whether to activate the “inactive reception allowed” mode, but views did not converge on the granularity of such decision.

Our preference is per-cell for alignment with the finest granularity of RNA configuration. That is to say, the gNB-CU-CP, assumed controlling a large amount of gNB-DUs, may configure the RNAs as list of cells for UEs which it releases toward RRC_INACTIVE mode, and then activates “inactive reception allowed” mode only in the cells within at least one RNA which it configures. Other cells, especially the ones not bordering any cells served by other gNBs, are not ordered to activate such mode, as every UE released by the gNB-CU-CP anyway needs to preform RRC resumption procedure for mobility-based RNAU upon moving into these cells.

Proposal 4: Granularity of the decision of enabling “inative reception allowed” mode is per-cell. Reuse the existing IE “NR CGI” within the “MBS Multicast Cell Item” structure to indicate the gNB-DU of such enabling, and releasing of this mode is indicated by absent of NR CGI in the modification message.In case the inactive reception mode is released for all cells in the DU,then Multicast CU to DU RRC Info IE is not present.

In addition, it was ever proposed that the gNB-DU may provide congestion information toward the gNB-CU to help the later making decision. But we do not think the conext storation load within the gNB-DU a mjor issue compared to the one in the gNB-CU-CP. And anyway, context storation congestion is only one motivation of enabling the “inative reception allowed” mode—the other motivation is for UE power saving. Therefore we prefer not supporting such assistance information.

Proposal 5: Do not support assistance information on congestion from the gNB-DU toward the gNB-CU.

  - MBS ongoing information exchange over F1AP

In previous RAN3 meeting, there were ever discussions on how to provide mtch-neighbourCell and mbs-NeighbourCellList in Rel-17. And there were mainly two open issues as below:

1) How to stop the provision of mbs-NeighbourCellList

2) In which message, mbs-NeighbourCellList should be provided

For the first issue, the conclusion at that time is to assume all IE and sub-IEs are currently mandatory and therefore any update or even stop of the MTCH-neighbor-cell bitmap can simply be indicated by sending the full MBS CU to DU RRC information IE which gNB will store and replace the received information. And for the second issue, since there was concern that there would be duplicated information transferred via multiple MBS associated messages if delivering mbs-NeighbourCellList via MBS managamgent related messages, it was agreed to keep it in current F1 setup/Configuration update procedure.

Then one question arise is whether similar solutions should be adopted in Rel-18 for the above open issues.

From our point of view, for the sake of backward compatibility, the same mechanism for update/stop of MTCH-neighbor-cell should be adopted in Rel-18 as it is in Rel-17 for broadcast service. Then as to multicast service, either of the two approaches, i.e. reusing Rel-17 mechnism for broadcast or introducing an IE to explicitly indicating the stop of MTCH-neighbor-cell, could work. And it seems both of the two mechinisms exists in current specification. We have no strong opinion on which solution is adopted for multicast service, just small preference on alignment with broadcast services.

Proposal 6: It is proposed to discuss mechanism on stop of MTCH-neighbor-cell. We have preference on aligning with the mechanism for Rel-17 Broadcast service.

As to the delivery of mbs-NeighbourCellList, it is very tricky since this information is MBS session related while its content is cell specific. We may either includes it in MBS management related messag which conforms to the fact that it is parts of MBS context while brings extra complexity or includes it in Configuration Update procedure which is simple while may be a little bit conflicting with current concept. Comparing the two approaches, we think it would be more important to not mix the MBS session related information with other application level data in the same message than avoid duplicated information transferred via messages with different MBS associations in F1 interface. So, we have a slight preference to introduce mbs-NeighbourCellList in MBS CU to DU RRC information IE in Rel-18

Proposal 7: It is proposed to introduce mbs-NeighbourCellList in MBS CU to DU RRC information IE for multicast in Rel-18.

2.5 MBS ongoing information exchange over XnAP

During the Rel-17 WI, there was a lengthy discussion on whether and how to exchange broadcast ongoing ingormation over XnAP, and the conclusion was pending Rel-18. The motivation was that it was supported in RAN2 that one cell broadcast whether a broadcast service was ongoing in its neighbour cells. Additionally, RAN2 recently agreed that:

- The neighbor cell list mechanism for multicast reception in RRC_INACTIVE may be configured e.g. it can be used by UE to resume RRC connection if service is not available in the re-selected cell by NCL, without reading MCCH in the re-selected cell, in some aspects similar to Rel-17 NCL mechanism in MBS broadcast.

Therefore we propose combining the two discussions, i.e. whether we should support exchanging both the broadcast ongoing information and the inactive-reception-capable multicast ongoing information over XnAP.

In Rel-17, the main concern was that the ongoing information for MBS service maybe too dynamic and it is not suitable to put this information in NG-RAN node configuration update procedure. To relieve the concern raised in Rel-17, one possible option in Rel-18 is to introduce new procedures which are mainly used for transfer of ongoing services i.e Class-1 Service Information Request procedure and Class-2 Service Information Update procedure. It is proposed for RAN3 to discuss on this solution and have an agreement on it.

Proposal 8: We propose RAN3 to discuss whether/how to introduce a mechanism into XnAP so that neighbouring gNBs can exchange both broadcast ongoing information and inactive-reception-capable multicast ongoing information. One feasible solution is to introduce new procedures in Xn interface to support service information transfer between neighbour nodes.

- Conclusion

Observation 1: According to TS 23.247, CN group paging is used to move RRC_IDLE UEs into RRC_CONNECTED upon network-triggered multicast session leave.

Observation 2: According to TS 23.247, it is up to the SMF’s implementation whether to initiate the CN group paging procedure upon network-triggered multicast session leave. The only alternative implementation is to wait until UE accesses later. UE individual CN paging is never used upon network-triggered multicast session leave.

Observation 3: NG-RAN nodes need to distinguish whether an NGAP/XnAP group paging message it receives is used to move UEs into RRC Connected state or not.

Proposal 1: We propose RAN3 to discuss whether it could be guaranteed that an NG-RAN node always receives the MBS Session Activation message before the NGAP Group paging message, and then make decision on whether to include the “inactiveReceptionAllowed-r18” IE into Uu paging message to indicate the activation of MBS session. Our understanding is that it cannot be guaranteed so a new indicator on NGAP is necessary.

Proposal 2: To introduce an indicator into the Xn Group Paging messages to indicate that the receiving node may/shall issue a Uu (group) paging message including the “inactiveReceptionAllowed-r18” IE.

Proposal 3: We propose RAN3 to discuss whether to introduce a “pulling” mode or a “pushing” for gNB-CUs to acquire the PTM configuration from gNB-DU. Our preference is the “pushing” mode. Note that introducing only one MCCH message per cell is sufficient enough.

Proposal 4: Granularity of the decision of enabling “inative reception allowed” mode is per-cell. Reuse the existing IE “NR CGI” within the “MBS Multicast Cell Item” structure to indicate the gNB-DU of such enabling, and releasing of this mode is indicated by absent of NR CGI in the modification message .In case the inactive reception mode is released for all cells in the DU,Multicast CU to DU RRC Info IE is not present.

Proposal 5: Do not support assistance information on congestion from the gNB-DU toward the gNB-CU.

Proposal 6: It is proposed to discuss mechanism on stop of MTCH-neighbor-cell. We have preference on aligning with the mechanism for Rel-17 Broadcast service.

Proposal 7: It is proposed to introduce mbs-NeighbourCellList in MBS CU to DU RRC information IE for multicast in Rel-18.

Proposal 8: We propose RAN3 to discuss whether/how to introduce a mechanism into XnAP so that neighbouring gNBs can exchange both broadcast ongoing information and inactive-reception-capable multicast ongoing information. One feasible solution is to introduce new procedures in Xn interface to support service information transfer between neighbour nodes.

Based on the proposal, we draft 3 TPs as shown in the Annexes.

- Annex A: TP for NGAP

-----------Start of the Change----------

8.5.2	Multicast Group Paging

8.5.2.1	General

The purpose of the Multicast Group Paging procedure is to enable the AMF to notify CM-IDLE UEs which have joined a multicast MBS session about its activation. The procedure uses non-UE associated signalling.

8.5.2.2	Successful Operation

Figure 8.5.2.2-1: Multicast Group Paging

The AMF initiates the Multicast Group Paging procedure by sending the MULTICAST GROUP PAGING message to the NG-RAN node.

At the reception of the MULTICAST GROUP PAGING message, the NG-RAN node shall perform multicast group paging of the MBS session identified by the MBS Session ID IE utilising information provided by the AMF.

If the Paging DRX IE is included in the MULTICAST GROUP PAGING message, the NG-RAN node shall use it according to TS 38.304 [12].

If the MBS Service Area IE is included in the MULTICAST GROUP PAGING message, the NG-RAN node shall take it into account during multicast group paging, as specified in TS 23.247 [44].

If the UE Paging List IE is included in the MULTICAST GROUP PAGING message, the NG-RAN node shall, if supported, use it according to TS 38.304 [12]. If absent, the NG-RAN node shall perform multicast group paging of the MBS session in all paging occasions within at least one default paging cycle, as specified in TS 38.304 [12].

If the Not moving to RRC_CONNECTED Allowed IE is included in the MULTICAST GROUP PAGING message, the NG-RAN node may take it into consideration when sending Uu paging.

8.5.2.3	Abnormal Conditions

Void.

-----------Next Change----------

#### 9.2.4.2	MULTICAST GROUP PAGING

This message is sent by the AMF and is used to notify involved UEs about the activation of a multicast MBS session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | ignore |
| MBS Service Area | O |  | 9.3.1.208 |  | YES | ignore |
| Multicast Group Paging Area List |  | 1 |  |  | YES | ignore |
| >Multicast Group Paging Area Item |  | 1..<maxnoofPagingAreas> |  |  | - |  |
| >>Multicast Group Paging Area | M |  | 9.3.1.216 |  | - |  |
| >>UE Paging List |  | 0..1 |  |  | - |  |
| >>>UE Paging Item |  | 1..<maxnoofUEsforPaging> |  |  | - |  |
| >>>>UE Identity Index Value | M |  | 9.3.3.23 |  | - |  |
| >>>>Paging DRX | O |  | 9.3.1.90 |  | - |  |
| Not moving to RRC_CONNECTED Allowed | O |  | 9.3.1.X |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingAreas | Maximum no. of paging areas for multicast group paging. Value is 64. |
| maxnoofUEsforPaging | Maximum no. of UEs allowed within one paging area for multicast group paging. Value is 4096. |

-----------Next Change----------

#### 9.3.1.X	Not moving to RRC_CONNECTED Allowed

This IE indicates that the transmitting node does not require the paged UEs to move into RRC_CONNECTED state.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Not Moving to RRC_CONNECTED Allowed | M |  | ENUMERATED (true, …) |  |

-----------Next Change----------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

////////////////////////////////No change/////////////////////////////////

IAB-Authorized,

IAB-Supported,

IABNodeIndication,

IMSVoiceSupportIndicator,

NotMovingToRRC_CONNECTEDAllowed,

IndexToRFSP,

InfoOnRecommendedCellsAndRANNodesForPaging,

IntersystemSONConfigurationTransfer,

////////////////////////////////No change/////////////////////////////////

id-IAB-Authorized,

id-IAB-Supported,

id-IABNodeIndication,

id-IMSVoiceSupportIndicator,

id-NotMovingToRRC_CONNECTEDAllowed,

id-IndexToRFSP,

id-InfoOnRecommendedCellsAndRANNodesForPaging,

id-IntersystemSONConfigurationTransferDL,

id-IntersystemSONConfigurationTransferUL,

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- MULTICAST GROUP PAGING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST GROUP PAGING

--

-- **************************************************************

MulticastGroupPaging ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastGroupPagingIEs} },

...

}

MulticastGroupPagingIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY ignore	TYPE MBS-SessionID							PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea						CRITICALITY ignore	TYPE MBS-ServiceArea						PRESENCE optional		}|

{ ID id-MulticastGroupPagingAreaList			CRITICALITY ignore	TYPE MulticastGroupPagingAreaList		PRESENCE mandatory	}|

{ ID id-NotMovingToRRC_CONNECTEDAllowed			CRITICALITY ignore	TYPE NotMovingToRRC_CONNECTEDAllowed PRESENCE optional		},

...

}

////////////////////////////////No change/////////////////////////////////

### 9.4.5	Information Element Definitions

////////////////////////////////No change/////////////////////////////////

-- I

////////////////////////////////No change/////////////////////////////////

NotMovingToRRC_CONNECTEDAllowed ::= ENUMERATED {

true,

...

}

////////////////////////////////No change/////////////////////////////////

### 9.4.7	Constant Definitions

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

id-MBSSessionReleaseResponseTransfer					ProtocolIE-ID ::= 358

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-NotMovingToRRC_CONNECTEDAllowed						ProtocolIE-ID ::=

END

-- ASN1STOP

-----------End of the Change--------

- Annex B: TP for XnAP

-----------Start of the Change----------

### 8.2.11	RAN Multicast Group Paging

#### 8.2.11.1	General

The purpose of the RAN Multicast Group Paging procedure is to enable the NG-RAN node1 to request paging of UEs that have joined an MBS Session in the NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.2.11.2	Successful operation

Figure 8.2.11.2-1: RAN Multicast Group Paging, successful operation

The RAN Multicast Group Paging procedure is triggered by the NG-RAN node1 by sending the RAN MULTICAST GROUP PAGING message to the NG-RAN node2.

If the RAN MULTICAST GROUP PAGING message includes the Paging DRX IE, the NG-RAN node2.shall, if supported, use it according to TS 38.304 [33].

If the Not moving to RRC_CONNECTED Allowed IE is included in the RAN MULTICAST GROUP PAGING message, the NG-RAN node may take it into consideration when sending Uu paging.

-----------Next Change----------

#### 9.1.1.15	RAN MULTICAST GROUP PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page UEs for a multicast session.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| MBS Session ID | M |  | 9.2.3.146 |  | YES | reject |
| UE Identity Index List |  | 1 |  |  | YES | reject |
| >UE Identity Index Item |  | 1 .. <maxnoofUEIDOindicesforMBSPaging> |  |  | – |  |
| >>CHOICE UE Identity Index Value | M |  |  |  | – |  |
| >>>Length-10 |  |  |  |  |  |  |
| >>>>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33]. | – |  |
| >>Paging DRX | O |  | UE Specific DRX 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 38.304 [33]. | – |  |
| Multicast RAN Paging Area | M |  | RAN Paging Area 9.2.3.38 |  | YES | reject |
| Not moving to RRC_CONNECTED Allowed | O |  | 9.2.3.X |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEIDIndicesforMBSPaging | Maximum no. of UE Identity Indices for multicast group paging. Value is 4096. |

-----------Next Change----------

#### 9.2.3.X	Not moving to RRC_CONNECTED Allowed

This IE indicates that the transmitting node does not require the paged UEs to move into RRC_CONNECTED state.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Not moving to RRC_CONNECTED Allowed | M |  | ENUMERATED (true, …) |  |

-----------Next Change----------

### 9.3.4	PDU Definitions

////////////////////////////////No change/////////////////////////////////

PEIPSassistanceInformation,

UESliceMaximumBitRateList,

PagingCause,

MDTPLMNModificationList,

F1-terminatingIAB-donorIndicator,

SRB-ID,

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

NotMovingToRRC_CONNECTEDAllowed

////////////////////////////////No change/////////////////////////////////

id-SDTPartialUEContextInfo,

id-SDTDataForwardingDRBList,

id-PEIPSassistanceInformation,

id-UESliceMaximumBitRateList,

id-S-NG-RANnodeUE-Slice-MBR,

id-ManagementBasedMDTPLMNModificationList,

id-F1-terminatingIAB-donorIndicator,

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

id-NotMovingToRRC_CONNECTEDAllowed,

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- RAN MULTICAST GROUP PAGING

--

-- **************************************************************

RANMulticastGroupPaging ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{RANMulticastGroupPaging-IEs}},

...

}

RANMulticastGroupPaging-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-MBS-Session-ID							CRITICALITY reject	TYPE MBS-Session-ID									PRESENCE mandatory}|

{ ID id-UEIdentityIndexList-MBSGroupPaging			CRITICALITY reject	TYPE UEIdentityIndexList-MBSGroupPaging		PRESENCE mandatory}|

{ ID id-MulticastRANPagingArea						CRITICALITY reject	TYPE RANPagingArea									PRESENCE mandatory}|

{ ID id-NotMovingToRRC_CONNECTEDAllowed			CRITICALITY ignore	TYPE NotMovingToRRC_CONNECTEDAllowed					PRESENCE optional },

...

}

////////////////////////////////No change/////////////////////////////////

### 9.3.5	Information Element definitions

////////////////////////////////No change/////////////////////////////////

NotMovingToRRC_CONNECTEDAllowed ::= ENUMERATED {

true,

...

}

////////////////////////////////No change/////////////////////////////////

### 9.3.7	Constant definitions

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-NotMovingToRRC_CONNECTEDAllowed																	ProtocolIE-ID ::=

END

-- ASN1STOP

-----------End of the Change--------

- Annex C: TP for F1AP (on R3-234798)

-----------Start of the Change----------

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| gNB-DU Resource Coordination | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| F1 Removal | F1 REMOVAL REQUEST | F1 REMOVAL RESPONSE | F1 REMOVAL FAILURE |
| BAP Mapping Configuration | BAP MAPPING CONFIGURATION | BAP MAPPING CONFIGURATION ACKNOWLEDGE | BAP MAPPING CONFIGURATION FAILURE |
| GNB-DU Resource Configuration | GNB-DU RESOURCE CONFIGURATION | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE | GNB-DU RESOURCE CONFIGURATION FAILURE |
| IAB TNL Address Allocation | IAB TNL ADDRESS REQUEST | IAB TNL ADDRESS RESPONSE | IAB TNL ADDRESS FAILURE |
| IAB UP Configuration Update | IAB UP CONFIGURATION UPDATE REQUEST | IAB UP CONFIGURATION UPDATE RESPONSE | IAB UP CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Positioning Measurement | POSITIONING MEASUREMENT REQUEST | POSITIONING MEASUREMENT RESPONSE | POSITIONING MEASUREMENT FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| Broadcast Context Setup | BROADCAST CONTEXT SETUP REQUEST | BROADCAST CONTEXT SETUP RESPONSE | BROADCAST CONTEXT SETUP FAILURE |
| Broadcast Context Release | BROADCAST CONTEXT RELEASE COMMAND | BROADCAST CONTEXT RELEASE COMPLETE |  |
| Broadcast Context Modification | BROADCAST CONTEXT MODIFICATION REQUEST | BROADCAST CONTEXT MODIFICATION RESPONSE | BROADCAST CONTEXT MODIFICATION FAILURE |
| Multicast Context Setup | MULTICAST CONTEXT SETUP REQUEST | MULTICAST CONTEXT SETUP RESPONSE | MULTICAST CONTEXT SETUP FAILURE |
| Multicast Context Release | MULTICAST CONTEXT RELEASE COMMAND | MULTICAST CONTEXT RELEASE COMPLETE |  |
| Multicast Context Modification (gNB-CU initiated) | MULTICAST CONTEXT MODIFICATION REQUEST | MULTICAST CONTEXT MODIFICATION RESPONSE | MULTICAST CONTEXT MODIFICATION FAILURE |
| Multicast Distribution Setup | MULTICAST DISTRIBUTION SETUP REQUEST | MULTICAST DISTRIBUTION SETUP RESPONSE | MULTICAST DISTRIBUTION SETUP FAILURE |
| Multicast Distribution Release | MULTICAST DISTRIBUTION RELEASE COMMAND | MULTICAST DISTRIBUTION RELEASE COMPLETE |  |
| PDC Measurement Initiation | PDC MEASUREMENT INITIATION REQUEST | PDC MEASUREMENT INITIATION RESPONSE | PDC MEASUREMENT INITIATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |
| Multicast Context Modification (gNB-DU initiated) | MULTICAST CONTEXT MODIFICATION REQUIRED | MULTICAST CONTEXT MODIFICATION CONFIRM |  |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Context Release Request | BROADCAST CONTEXT RELEASE REQUEST |
| Multicast Context Release Request | MULTICAST CONTEXT RELEASE REQUEST |
| PDC Measurement Report | PDC MEASUREMENT REPORT |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| PDC Measurement Failure Indication | PDC MEASUREMENT FAILURE INDICATION |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| Measurement Activation | MEASUREMENT ACTIVATION |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |

-----------Next Change----------

### 8.14.6	Multicast Context Setup

#### 8.14.6.1	General

The purpose of the Multicast Context Setup procedure is to establish an MBS Session context in the gNB-DU for a multicast session.

The procedure uses MBS-associated signalling.

#### 8.14.6.2	Successful Operation

Figure 8.14.6.2-1: Multicast Context Setup procedure: Successful Operation

The gNB-CU initiates the procedure by sending MULTICAST CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the multicast MBS Session context, it replies to the gNB-CU with MULTICAST CONTEXT SETUP RESPONSE.

If the MBS Service Area IE is included in the MULTICAST CONTEXT SETUP REQUEST message, the gNB-DU shall take this information into account for shared F1-U tunnel assignment.

If the Multicast CU to DU RRC Information IE is included in the MULTICAST CONTEXT SETUP REQUEST message, the gNB-DU shall activate the inactive reception mode for each cell indicated in the MBS Multicast Cell List IE, including activating the Uu PTM delivery, and deactivate the inactive reception mode for any other cell, and returning a Multicast DU to CU RRC Information IE in the MULTICAST CONTEXT SETUP RESPONSE message.

The gNB-DU shall report to the gNB-CU, in the MULTICAST CONTEXT SETUP RSPONSE message, the result of all the requested Multicast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Multicast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Multicast MRB Failed To Be Setup List IE;

If the Multicast MRB Failed To Setup List IE is contained in the MULTICAST CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the Multicast MRB(s) failed to be setup with an appropriate cause value for each Multicast MRB failed to setup.

Interaction with the Multicast Distribution Context Setup procedure:

Upon reception of the MULTICAST CONTEXT SETUP REQUEST procedure, the gNB-DU shall trigger either per cell or per MBS Area Session ID or for the whole gNB-DU the Multicast Distribution Context Setup procedure to establish per cell or per MBS Area Session ID or the the whole gNB DU per accepted MRB a shared F1-U tunnel.

#### 8.14.6.3	Unsuccessful Operation

Figure 8.14.6.3-1: Multicast Context Setup procedure: unsuccessful Operation

If the gNB-DU is not able to establish the MBS session context it shall consider the procedure as failed and reply with the MULTICAST CONTEXT SETUP FAILURE message.

#### 8.14.6.4	Abnormal Conditions

Not applicable.

-----------Next Change----------

8.14.9	Multicast Context Modification (gNB-CU initiated)

8.14.9.1	General

The purpose of the Multicast Context Modification procedure is to modify an established MBS session context in the gNB-DU for a multicast session.

The procedure uses MBS-associated signalling.

8.14.9.2	Successful Operation

Figure 8.14.9.2-1: Multicast Context Modification procedure (gNB-CU initiated). Successful operation

The MULTICAST CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the MULTICAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and, if successful, report the update in the MULTICAST CONTEXT MODIFICATION RESPONSE message.

If the Multicast MRB To Be Setup List IE is contained in the MULTICAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall setup the corresponding resources for the requested MRB(s), and report to the gNB-CU, in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the result of all the requested Multicast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Multicast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Multicast MRB Failed To Be Setup List IE;

If the Multicast CU to DU RRC Information IE is included in the MULTICAST CONTEXT SETUP REQUEST message, the gNB-DU shall activate or keep the inactive reception mode for each cell indicated in the MBS Multicast Cell List IE, including activating or keeping the Uu PTM delivery, and deactivate the inactive reception mode for any other cell, and including a Multicast DU to CU RRC Information IE in the MULTICAST CONTEXT SETUP RESPONSE message.

If the Multicast MRB Failed To Be Setup List IE is contained in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the setup of the indicated MRB(s) as failed and indicated the resource for the failure with an appropriate cause value for each MRB failed to be setup.

If the Multicast MRB To Be Modified List IE is contained in the MULTICAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall update the corresponding context and resources for the requested MRB(s), and report to the gNB-CU, in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the modification result of all the requested Multicast MRBs in the following way:

-	A list of MRBs which have been successfully modified shall be included in the Multicast MRB Modified List IE;

-	A list of MRBs which failed to be modified shall be included in the Multicast MRB Failed To Be Modified List IE;

If the Multicast MRB Failed To Be Modified List IE is contained in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the Multicast MRB(s) failed to be modified with an appropriate cause value for each Multicast MRB failed to modify.

8.14.9.3	Unsuccessful Operation

Figure 8.14.9.3-1: Multicast Context Modification procedure. Unsuccessful operation

In case none of the requested modifications of the multicast context can be successfully performed, the gNB-DU shall respond with the MULTICAST CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

8.14.9.4	Abnormal Conditions

Not applicable.

-----------Next Change----------

8.14.Z	Multicast Context Modification (gNB-DU initiated)

8.14.Z.1	General

The purpose of the Multicast Context Modification procedure is to modify an established MBS session context in the gNB-DU for a multicast session.

The procedure uses MBS-associated signalling.

8.14.Z.2	Successful Operation

Figure 8.14.Z.2-1: Multicast Context Modification procedure (gNB-DU initiated). Successful operation

The MULTICAST CONTEXT MODIFICATION REQUEST message is initiated by the gNB-DU.

If the Multicast DU to CU RRC Information IE is included in the MULTICAST CONTEXT MODIFICATION REQUIRED message, the gNB-CU shall, if supported, store the content in it and may use it to generate RRCRelease messages.

8.14.Z.3	Unsuccessful Operation

Not applicable.

8.14.Z.4	Abnormal Conditions

Not applicable.

-----------Next Change----------

#### 9.2.14.1	MULTICAST GROUP PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to multicast group page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| UE Identity List for Paging |  | 0..1 |  |  | YES | ignore |
| >UE Identity for Paging Item |  | 1..<maxnoofUEIDforPaging> |  |  | - |  |
| >>UE Identity Index value | M |  | 9.3.1.39 |  | - |  |
| >>Paging DRX | O |  | 9.3.1.40 |  | - |  |
| Paging Cell List |  | 0..1 |  |  | YES | ignore |
| >Paging Cell Item Ies |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Inactive Reception Allowed | O |  | 9.3.1.X1 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEIDforPaging | Maximum no. of UE ID for multicast group paging. Value is 4096. |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

-----------Next Change----------

#### 9.2.14.3	MULTICAST CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a multicast context.

Direction : gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Multicast MRB Setup List |  | 1 |  |  | YES | reject |
| >Multicast MRB Setup Item Ies |  | 1 .. <maxnoofMRBs> |  |  | EACH | Reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Setup Item Ies |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Multicast DU to CU RRC Information | O |  | 9.3.1.X4 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

-----------Next Change----------

#### 9.2.14.8	MULTICAST CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a multicast context.

Direction : gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Multicast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB Setup Item Ies |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Setup Item Ies |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Multicast MRB Modified List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB Modified Item Ies |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Modified Item Ies |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Multicast DU to CU RRC Information | O |  | 9.3.1.X4 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

-----------Next Change----------

#### 9.2.14.Y1	MULTICAST CONTEXT MODIFICATION REQUIRED

This message is sent by the gNB-DU to request the gNB-CU to modify multicast context information.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| Multicast DU to CU RRC Information | O |  | 9.3.1.X4 |  | YES | ignore |

#### 9.2.14.Y2	MULTICAST CONTEXT MODIFICATION CONFIRM

This message is sent by the gNB-CU to confirm the modification of a multicast context.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

-----------Next Change----------

9.3.1.x1	Multicast CU to DU RRC Information

This IE indicates the multicast specific CU to DU RRC Information.

Editor’s Note:	IE names, their presence, ranges, semantics and references to field/IE/messages names in TS 38.331 are FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Multicast Cell List |  | FFS |  |  |
| >MBS Multicast Cell Item |  | 1 .. <maxCellingNBDU> |  |  |
| >>NR CGI | M |  | NR CGI 9.3.1.12 |  |
| >>RRC Multicast MTCH Neighbour Cell Information | O |  | OCTET STRING | Editor’s Note: reference to TS 38.331 [8] to be included. |
| >> MBS Broadcast Neighbour Cell List | O |  | 9.3.1.266 |  |
| MBS Multicast MRB List |  | FFS |  |  |
| >MBS Multicast MRB Item |  | 1 .. <maxnoofMRBs> |  |  |
| >>MRB ID | M |  | 9.3.1.224 |  |
| >>MRB PDCP Config Broadcast | M |  | OCTET STRING | Includes the MRB-PDCP-ConfigBroadcast IE, as defined in TS 38.331[8]. |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofMRBs | Maximum no. MRBs allowed to be setup for one MBS session, the maximum value is 32. |

9.3.1.X4	MBS DU to CU RRC Information

This IE indicates the MBS DU to CU RRC Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Multicast DU Cell List | M |  |  |  |
| >MBS Multicast DU Cell Item |  | 1 .. <maxCellingNBDU> |  |  |
| >>NR CGI | M |  | NR CGI 9.3.1.12 |  |
| >>MBS Multicast Configuration | O |  | OCTET STRING | Includes the MBSMulticastConfiguration defined in TS 38.331 [8] |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

F1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM F1AP-CommonDataTypes

Reset,

ResetAcknowledge,

F1SetupRequest,

F1SetupResponse,

F1SetupFailure,

GNBDUConfigurationUpdate,

GNBDUConfigurationUpdateAcknowledge,

GNBDUConfigurationUpdateFailure,

GNBCUConfigurationUpdate,

GNBCUConfigurationUpdateAcknowledge,

GNBCUConfigurationUpdateFailure,

UEContextSetupRequest,

UEContextSetupResponse,

UEContextSetupFailure,

UEContextReleaseCommand,

UEContextReleaseComplete,

UEContextModificationRequest,

UEContextModificationResponse,

UEContextModificationFailure,

UEContextModificationRequired,

UEContextModificationConfirm,

ErrorIndication,

UEContextReleaseRequest,

DLRRCMessageTransfer,

ULRRCMessageTransfer,

GNBDUResourceCoordinationRequest,

GNBDUResourceCoordinationResponse,

PrivateMessage,

UEInactivityNotification,

InitialULRRCMessageTransfer,

SystemInformationDeliveryCommand,

Paging,

Notify,

WriteReplaceWarningRequest,

WriteReplaceWarningResponse,

PWSCancelRequest,

PWSCancelResponse,

PWSRestartIndication,

PWSFailureIndication,

GNBDUStatusIndication,

RRCDeliveryReport,

UEContextModificationRefuse,

F1RemovalRequest,

F1RemovalResponse,

F1RemovalFailure,

NetworkAccessRateReduction,

TraceStart,

DeactivateTrace,

DUCURadioInformationTransfer,

CUDURadioInformationTransfer,

BAPMappingConfiguration,

BAPMappingConfigurationAcknowledge,

BAPMappingConfigurationFailure,

GNBDUResourceConfiguration,

GNBDUResourceConfigurationAcknowledge,

GNBDUResourceConfigurationFailure,

IABTNLAddressRequest,

IABTNLAddressResponse,

IABTNLAddressFailure,

IABUPConfigurationUpdateRequest,

IABUPConfigurationUpdateResponse,

IABUPConfigurationUpdateFailure,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

AccessAndMobilityIndication,

ReferenceTimeInformationReportingControl,

ReferenceTimeInformationReport,

AccessSuccess,

CellTrafficTrace,

PositioningMeasurementRequest,

PositioningMeasurementResponse,

PositioningMeasurementFailure,

PositioningAssistanceInformationControl,

PositioningAssistanceInformationFeedback,

PositioningMeasurementReport,

PositioningMeasurementAbort,

PositioningMeasurementFailureIndication,

PositioningMeasurementUpdate,

TRPInformationRequest,

TRPInformationResponse,

TRPInformationFailure,

PositioningInformationRequest,

PositioningInformationResponse,

PositioningInformationFailure,

PositioningActivationRequest,

PositioningActivationResponse,

PositioningActivationFailure,

PositioningDeactivation,

PositioningInformationUpdate,

E-CIDMeasurementInitiationRequest,

E-CIDMeasurementInitiationResponse,

E-CIDMeasurementInitiationFailure,

E-CIDMeasurementFailureIndication,

E-CIDMeasurementReport,

E-CIDMeasurementTerminationCommand,

BroadcastContextSetupRequest,

BroadcastContextSetupResponse,

BroadcastContextSetupFailure,

BroadcastContextReleaseCommand,

BroadcastContextReleaseComplete,

BroadcastContextReleaseRequest,

BroadcastContextModificationRequest,

BroadcastContextModificationResponse,

BroadcastContextModificationFailure,

MulticastGroupPaging,

MulticastContextSetupRequest,

MulticastContextSetupResponse,

MulticastContextSetupFailure,

MulticastContextReleaseCommand,

MulticastContextReleaseComplete,

MulticastContextReleaseRequest,

MulticastContextModificationRequest,

MulticastContextModificationResponse,

MulticastContextModificationFailure,

MulticastContextModificationRequired,

MulticastContextModificationConfirm,

MulticastDistributionSetupRequest,

MulticastDistributionSetupResponse,

MulticastDistributionSetupFailure,

MulticastDistributionReleaseCommand,

MulticastDistributionReleaseComplete,

PDCMeasurementInitiationRequest,

PDCMeasurementInitiationResponse,

PDCMeasurementInitiationFailure,

PDCMeasurementReport,

PDCMeasurementTerminationCommand,

PDCMeasurementFailureIndication,

PRSConfigurationRequest,

PRSConfigurationResponse,

PRSConfigurationFailure,

MeasurementPreconfigurationRequired,

MeasurementPreconfigurationConfirm,

MeasurementPreconfigurationRefuse,

MeasurementActivation,

QoEInformationTransfer,

PosSystemInformationDeliveryCommand

FROM F1AP-PDU-Contents

id-Reset,

id-F1Setup,

id-gNBDUConfigurationUpdate,

id-gNBCUConfigurationUpdate,

id-UEContextSetup,

id-UEContextRelease,

id-UEContextModification,

id-UEContextModificationRequired,

id-ErrorIndication,

id-UEContextReleaseRequest,

id-DLRRCMessageTransfer,

id-ULRRCMessageTransfer,

id-GNBDUResourceCoordination,

id-privateMessage,

id-UEInactivityNotification,

id-InitialULRRCMessageTransfer,

id-SystemInformationDeliveryCommand,

id-Paging,

id-Notify,

id-WriteReplaceWarning,

id-PWSCancel,

id-PWSRestartIndication,

id-PWSFailureIndication,

id-GNBDUStatusIndication,

id-RRCDeliveryReport,

id-F1Removal,

id-NetworkAccessRateReduction,

id-TraceStart,

id-DeactivateTrace,

id-DUCURadioInformationTransfer,

id-CUDURadioInformationTransfer,

id-BAPMappingConfiguration,

id-GNBDUResourceConfiguration,

id-IABTNLAddressAllocation,

id-IABUPConfigurationUpdate,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-accessAndMobilityIndication,

id-ReferenceTimeInformationReportingControl,

id-ReferenceTimeInformationReport,

id-accessSuccess,

id-cellTrafficTrace,

id-PositioningMeasurementExchange,

id-PositioningAssistanceInformationControl,

id-PositioningAssistanceInformationFeedback,

id-PositioningMeasurementReport,

id-PositioningMeasurementAbort,

id-PositioningMeasurementFailureIndication,

id-PositioningMeasurementUpdate,

id-TRPInformationExchange,

id-PositioningInformationExchange,

id-PositioningActivation,

id-PositioningDeactivation,

id-PositioningInformationUpdate,

id-E-CIDMeasurementInitiation,

id-E-CIDMeasurementFailureIndication,

id-E-CIDMeasurementReport,

id-E-CIDMeasurementTermination,

id-BroadcastContextSetup,

id-BroadcastContextRelease,

id-BroadcastContextReleaseRequest,

id-BroadcastContextModification,

id-MulticastGroupPaging,

id-MulticastContextSetup,

id-MulticastContextRelease,

id-MulticastContextReleaseRequest,

id-MulticastContextModification,

id-MulticastContextModificationRequired,

id-MulticastDistributionSetup,

id-MulticastDistributionRelease,

id-PDCMeasurementInitiation,

id-PDCMeasurementInitiationRequest,

id-PDCMeasurementInitiationResponse,

id-PDCMeasurementInitiationFailure,

id-PDCMeasurementTerminationCommand,

id-PDCMeasurementFailureIndication,

id-PDCMeasurementReport,

id-pRSConfigurationExchange,

id-measurementPreconfiguration,

id-measurementActivation,

id-QoEInformationTransfer,

id-PosSystemInformationDeliveryCommand

FROM F1AP-Constants

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

F1AP-ELEMENTARY-PROCEDURES F1AP-ELEMENTARY-PROCEDURE ::= {

F1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

F1AP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

F1AP-ELEMENTARY-PROCEDURES-CLASS-1 F1AP-ELEMENTARY-PROCEDURE ::= {

reset							|

f1Setup							|

gNBDUConfigurationUpdate		|

gNBCUConfigurationUpdate		|

uEContextSetup					|

uEContextRelease				|

uEContextModification			|

uEContextModificationRequired	|

writeReplaceWarning				|

pWSCancel					|

gNBDUResourceCoordination		|

f1Removal						|

bAPMappingConfiguration			|

gNBDUResourceConfiguration		|

iABTNLAddressAllocation			|

iABUPConfigurationUpdate		|

resourceStatusReportingInitiation	|

positioningMeasurementExchange	|

tRPInformationExchange			|

positioningInformationExchange	|

positioningActivation			|

e-CIDMeasurementInitiation		|

broadcastContextSetup			|

broadcastContextRelease			|

broadcastContextModification|

multicastContextSetup			|

multicastContextRelease		|

multicastContextModification	|

multicastContextModificationRequired	|

multicastDistributionSetup		|

multicastDistributionRelease	|

pDCMeasurementInitiation		|

pRSConfigurationExchange		|

measurementPreconfiguration	,

...

}

////////////////////////////////No change/////////////////////////////////

multicastContextModification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextModificationRequest

SUCCESSFUL OUTCOME		MulticastContextModificationResponse

UNSUCCESSFUL OUTCOME	MulticastContextModificationFailure

PROCEDURE CODE			id-MulticastContextModification

CRITICALITY				reject

}

multicastContextModificationRequired F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextModificationRequired

SUCCESSFUL OUTCOME		MulticastContextModificationConfirm

PROCEDURE CODE			id-MulticastContextModificationRequired

CRITICALITY				reject

}

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

////////////////////////////////No change/////////////////////////////////

MBS-Area-Session-ID,

MBS-CUtoDURRCInformation,

MBSMulticastF1UContextDescriptor,

MBS-Session-ID,

MBS-ServiceArea,

MulticastCUtoDURRCInfoRelease,

MulticastCUtoDURRCInformation,

MulticastDUtoDURRCInformation,

MulticastF1UContextReferenceCU,

MulticastF1UContext-ToBeSetup-Item,

MulticastF1UContext-Setup-Item,

MulticastF1UContext-FailedToBeSetup-Item,

MulticastMBSSessionList,

MulticastMRBs-ToBeSetup-Item,

MulticastMRBs-Setup-Item,

MulticastMRBs-FailedToBeSetup-Item,

MulticastMRBs-ToBeSetupMod-Item,

MulticastMRBs-ToBeModified-Item,

MulticastMRBs-ToBeReleased-Item,

MulticastMRBs-SetupMod-Item,

MulticastMRBs-FailedToBeSetupMod-Item,

MulticastMRBs-Modified-Item,

MulticastMRBs-FailedToBeModified-Item,

BroadcastAreaScope,

////////////////////////////////No change/////////////////////////////////

FROM F1AP-Containers

////////////////////////////////No change/////////////////////////////////

id-MBS-Area-Session-ID,

id-MBS-CUtoDURRCInformation,

id-MBS-Session-ID,

id-MBS-ServiceArea,

id-MBSMulticastF1UContextDescriptor,

id-MC-PagingCell-Item,

id-MC-PagingCell-List,

id-MulticastCUtoDURRCInfoRelease,

id-MulticastCUtoDURRCInformation,

id-MulticastDUtoCURRCInformation,

id-MulticastF1UContextReferenceCU,

id-MulticastMBSSessionSetupList,

id-MulticastMBSSessionRemoveList,

id-MulticastMRBs-FailedToBeModified-List,

id-MulticastMRBs-FailedToBeModified-Item,

id-MulticastMRBs-FailedToBeSetup-List,

id-MulticastMRBs-FailedToBeSetup-Item,

id-MulticastMRBs-FailedToBeSetupMod-List,

id-MulticastMRBs-FailedToBeSetupMod-Item,

id-MulticastMRBs-Modified-List,

id-MulticastMRBs-Modified-Item,

id-MulticastMRBs-Setup-List,

id-MulticastMRBs-Setup-Item,

id-MulticastMRBs-SetupMod-List,

id-MulticastMRBs-SetupMod-Item,

id-MulticastMRBs-ToBeModified-List,

id-MulticastMRBs-ToBeModified-Item,

id-MulticastMRBs-ToBeReleased-List,

id-MulticastMRBs-ToBeReleased-Item,

id-MulticastMRBs-ToBeSetup-List,

id-MulticastMRBs-ToBeSetup-Item,

id-MulticastMRBs-ToBeSetupMod-List,

id-MulticastMRBs-ToBeSetupMod-Item,

id-MulticastF1UContext-ToBeSetup-List,

id-MulticastF1UContext-ToBeSetup-Item,

id-MulticastF1UContext-Setup-List,

id-MulticastF1UContext-Setup-Item,

id-MulticastF1UContext-FailedToBeSetup-List,

id-MulticastF1UContext-FailedToBeSetup-Item,

id-BroadcastAreaScope,

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- MULTICAST CONTEXT SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

##### -- MULTICAST CONTEXT SETUP RESPONSE

--

-- **************************************************************

MulticastContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextSetupResponseIEs}},

...

}

MulticastContextSetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-MulticastMRBs-Setup-List			CRITICALITY reject TYPE MulticastMRBs-Setup-List					PRESENCE mandatory	}|

{ ID id-MulticastMRBs-FailedToBeSetup-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeSetup-List 	PRESENCE optional	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore TYPE CriticalityDiagnostics						PRESENCE optional	}|

{ ID id-MulticastDUtoCURRCInformation		CRITICALITY ignore	TYPE MulticastDUtoCURRCInformation			PRESENCE optional		},

...

}

MulticastMRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-Setup-ItemIEs} }

MulticastMRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeSetup-ItemIEs} }

MulticastMRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-Setup-Item			CRITICALITY reject	TYPE MulticastMRBs-Setup-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeSetup-Item		CRITICALITY ignore	TYPE MulticastMRBs-FailedToBeSetup-Item	PRESENCE mandatory},

...

}

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- MULTICAST CONTEXT MODIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

MulticastContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationResponseIEs}},

...

}

MulticastContextModificationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID								PRESENCE mandatory }|

{ ID id-gNB-DU-MBS-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID								PRESENCE mandatory }|

{ ID id-MulticastMRBs-SetupMod-List			CRITICALITY reject TYPE MulticastMRBs-SetupMod-List						PRESENCE optional  }|

{ ID id-MulticastMRBs-FailedToBeSetupMod-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeSetupMod-List	PRESENCE optional  }|

{ ID id-MulticastMRBs-Modified-List			CRITICALITY reject TYPE MulticastMRBs-Modified-List						PRESENCE optional  }|

{ ID id-MulticastMRBs-FailedToBeModified-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeModified-List	PRESENCE optional  }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional  }|

{ ID id-MulticastDUtoCURRCInformation			CRITICALITY ignore	TYPE MulticastDUtoCURRCInformation				PRESENCE optional  },

...

}

MulticastMRBs-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-SetupMod-ItemIEs} }

MulticastMRBs-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-SetupMod-Item				CRITICALITY		reject	TYPE MulticastMRBs-SetupMod-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeSetupMod-ItemIEs} }

MulticastMRBs-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeSetupMod-Item	CRITICALITY		ignore	TYPE MulticastMRBs-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

MulticastMRBs-Modified-List::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-Modified-ItemIEs } }

MulticastMRBs-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-Modified-Item				CRITICALITY		reject	TYPE MulticastMRBs-Modified-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeModified-ItemIEs} }

MulticastMRBs-FailedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeModified-Item	CRITICALITY 	ignore	TYPE MulticastMRBs-FailedToBeModified-Item		PRESENCE mandatory},

...

}

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- MULTICAST CONTEXT MODIFICATION REQUIRED ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

MulticastContextModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationRequiredIEs}},

...

}

MulticastContextModificationRequiredIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-MBS-ServiceArea						CRITICALITY reject	TYPE MBS-ServiceArea						PRESENCE optional  }|

{ ID id-MulticastDUtoCURRCInformation		CRITICALITY ignore	TYPE MulticastDUtoCURRCInformation		PRESENCE optional  },

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION CONFIRM

--

-- **************************************************************

MulticastContextModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationConfirmIEs}},

...

}

MulticastContextModificationConfirmIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional  },

...

}

-----------Next Change----------

### 9.4.5	Information Element Definitions

////////////////////////////////No change/////////////////////////////////

MulticastCU2DURRCInfo		::= SEQUENCE {

mBS-Multicast-Cell-List		MBS-Multicast-Cell-List, -- presence is FFS

mBS-Multicast-MRB-List		MBS-Multicast-MRB-List,  -- presence is FFS

iE-Extensions				ProtocolExtensionContainer { { MulticastCU2DURRCInfo-ExtIEs } } OPTIONAL,

...

}

MulticastCU2DURRCInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Multicast-Cell-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF  MBS-Multicast-Cell-Item

MBS-Multicast-Cell-Item ::= SEQUENCE {

nRCGI								NRCGI,

multicast-mtch-neighbourCell		OCTET STRING		OPTIONAL,

mBS-Broadcast-NeighbourCellList     MBS-Broadcast-NeighbourCellList        OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Multicast-Cell-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Multicast-Cell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Multicast-MRB-List	::= SEQUENCE (SIZE(1.. maxnoofMRBs))	OF  MBS-Multicast-MRB-Item

MBS-Multicast-MRB-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

mRB-PDCP-Config-Broadcast	OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { MBS-Multicast-MRB-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Multicast-MRB-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastDUtoCURRCInformation		::= SEQUENCE {

mBS-MulticastDU-Cell-List	MBS-MulticastDU-Cell-List,

iE-Extensions				ProtocolExtensionContainer { { MulticastDUtoCURRCInformation-ExtIEs } } OPTIONAL,

...

}

MulticastDUtoCURRCInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-MulticastDU-Cell-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF  MBS-MulticastDU-Cell-Item

MBS-MulticastDU-Cell-Item ::= SEQUENCE {

nRCGI						NRCGI,

mBS-MulticastConfiguration	OCTET STRING		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-MulticastDU-Cell-Item-ExtIEs} } OPTIONAL,

...

}

MBS-MulticastDU-Cell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

-----------Next Change----------

### 9.4.7	Constant Definitions

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

id-MulticastContextModificationRequired		ProcedureCode ::=

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

id-MulticastDUtoCURRCInformation					ProtocolIE-ID ::=

END

-- ASN1STOP

-----------End of the Change----------