3GPP TSG-RAN WG3 Meeting #121bis	R3-235420

Xiamen, China, 9th – 13th October, 2023

Agenda item:	13.5

Source: 	Huawei

Title: 	Discussion on the IAB-node de-authorization handling

Document Type:	Discussion

1. Introduction

RAN3 received an LS from SA2 [1], which requested RAN3 to answer some questions related to the IAB-node de-authorization handling. The key part of the LS is given as follows:

SA2 would like to check whether this is a correct understanding:

- The gNB attempts to perform handover of all the UEs served by the MBSR

- When the step 1 completes, the gNB releases the F1-C

Q1: is the understanding above correct?

Q2: Whether and how is the Mobile IAB-DU indicated to refrain from setting up a F1-C connection to the donor CU immediately after the F1-C release (is there an indication in the F1-C release instructing the DU to not try again)?

In the last meeting, RAN3 discussed the first question with the agreement that “In case the mIAB-DU’s CU obtains “non-authorized” indication for the mIAB-node, it performs an orderly F1 release”. In this contribution, we continue to discuss SA2’s second question.

2. Discussion

After the DU’s CU handover all the UEs served by the mobile IAB-DU and releases F1 connection to the IAB-DU, the MT’s CU shall release the BH resources (including the default BH configuration) which were used for the mobile IAB in its topology. If the mobile IAB node is authorized again, the MT’s CU will know the updated authorization results from AMF, and can provide the default BAP configuration to the mobile IAB-MT, for the consequent F1 setup initiated from the mobile IAB-DU. From the mobile IAB-node’s perspective, it will not initiate the F1 setup procedure only if there is no available default BH configuration. Or, even if the mobile IAB node initiates F1 setup without default configuration provided to the mIAB-MT, the F1 SETUP REQUEST message cannot reach to the donor CU because lack of necessary BH configuration.

Furthermore, according to SA2’s CR [2], mobile IAB-node also receives its authorization status via NAS messages. In other words, the mobile IAB-node knows it is authorized or not. So, there is no motivation to indicate the DU to not try F1 connection again when performing F1 release due to the un-authorization. When the mobile IAB-node is changed to be authorized again, it can initiate the F1 setup procedure using the default BH configuration received from the MT’s CU.

It is observed and proposed:

Observation 1: If the mobile IAB node is authorized, the IAB-donor-CU can provide the default BH configuration to the mobile IAB-MT via RRC message, if without such default BH configuration, the mobile IAB-node cannot initiate the F1 setup procedure.

Observation 2: There is no motivation to indicate the DU to not try F1 connection again when perform F1 release.

Proposal: Without the default BAP configuration, and knowing it is not authorized from NAS signaling, the mobile IAB-node cannot initiate the F1 setup procedure towards the IAB-donor-CU.

3. Conclusion

This paper presents our view on the IAB-node de-authorization handling for the SA2 LS. The following observations and proposal are given:

Observation 1: If the mobile IAB node is authorized, the IAB-donor-CU can provide the default BH configuration to the mobile IAB-MT via RRC message, if without such default BH configuration, the mobile IAB-node cannot initiate the F1 setup procedure.

Observation 2: There is no motivation to indicate the DU to not try F1 connection again when perform F1 release.

Proposal: Without the default BAP configuration, and knowing it is not authorized from NAS signaling, the mobile IAB-node cannot initiate the F1 setup procedure towards the IAB-donor-CU.

4. Reference

- R3-233730, LS on IAB-node De-authorization handling.

- S2-2308167, MBSR registration and authorization support.