# 3GPP TSG-RAN WG3 Meeting #121-bis	R3-235455

# 9-13 October 2023, Xiamen, China

Agenda Item:	21.2

Source:	CATT

Title:	(TP for 38.413 for eRedCap) Discussion on MT Communication Handling

Document for:	Other

- Introduction

RAN3 achieved some basic agreements to introduce NGAP message to support MT communication handling in previous meetings. In this contribution, we further discuss open issues and provide our proposals.

- Discussion

2.1	Paging Policy Information

PPI, ARP, PDU session ID and QFI are included in the DL DATA NOTIFICATION message. FFS on 5QI.

In LS R3-235020, SA2 said that signaled 5QI is the same as the one stored in RAN UE context.

Answer 2: When NGAP DL DATA NOTIFICATION is triggered by downlink data on user plane, the signalled 5QI is the same as the one stored in RAN UE context. For the case of NGAP DL DATA NOTIFICATION is triggered by MT signalling, SA2 will continue the discussion and provide further response.

In [1], SA2 captured the PPI, ARP, 5QI and/or QFI of the QoS Flow of the PDU Session ID in the N2 message. These parameters can be contained in Paging Policy Information for Network Triggered Connection and further included in RAN paging request message.

2.	When the AMF determines that the UE is reachable, the AMF sends an N2 DL Data Notification message to NG-RAN with the request for the UE's RRC connection to be resumed. The AMF may include the following parameter(s) the PPI, the ARP and the 5QI, and/or QFI of the QoS Flow of the PDU Session ID in the N2 DL Data Notification message to trigger and enable RAN paging.

3.	NG-RAN performs RAN paging towards the UE considering the parameters provided by the AMF.

We understand that if NG-RAN can identify QFI that means it also knows 5QI and ARP.

Case 1: AMF includes PPI, the ARP and the 5QI, or;

Case 2: AMF includes PPI and the QFI of the QoS Flow of the PDU Session ID, or;

Case 3: AMF includes all parameters PPI, the ARP, 5QI, QFI of the QoS Flow of the PDU Session ID.

However, in case the QFI and PDU session ID are mandatary included in RAN paging request, the 5QI may not necessary to be included, if else QFI and PDU session ID becomes optional.

Observation 1: 5QI can be identified by QFI (M) and PDU session ID (M) in NG-RAN.

Proposal 1: Slightly prefer remove 5QI in Paging Policy Differentiation and inform our decision to SA2.

2.2	DL Data Size

In LS R3-235020, SA2 agreed to introduce DL data size but limit it for the first downlink packet.

Answer 3: SA2 discussed the issue and considers it’s technically feasible for UPF/SMF to provide the DL data size of the first downlink packet to NG-RAN via AMF. SA2 also discussed if more accurate data size is needed besides the first packet, but no consensus could be reached.

SA2 asks RAN3 to provide view regarding if indicating the first downlink packet data size by 5GC is sufficient for NG-RAN to decide the use of MT-SDT.

If AMF only provide the first downlink packet, NG-RAN cannot decide whether to perform SDT Paging. For example, the first downlink packet belongs to SDT identified by NG-RAN while the other buffered packets in AMF are non SDT data. NG-RAN may trigger SDT Paging according to the DL data size of the first downlink packet in RAN Paging Request. However, when data comes to NG-RAN, NG-RAN has to fall back SDT paging to legacy paging because buffered DL data includes non SDT data. We think the AMF should provide DL data size of each packet, and then NG-RAN can decide whether to trigger SDT paging based on threshold and QFI. However, given that CT4/SA2 may not have enough time to discuss how to support sending all DL buffered data to NG-RAN, only send the first DL buffered data size also acceptable by RAN but the paging fallback is needed.

Proposal 2: It is propose to provide DL data size for each buffered DL packet to NG-RAN and inform our view to SA2/CT4.

- Conclusion

Observation 1: 5QI can be identified by QFI (M) and PDU session ID (M) in NG-RAN.

Proposal 1: Slightly prefer remove 5QI in Paging Policy Differentiation and inform our decision to SA2.

Proposal 2: It is propose to provide DL data size for each buffered DL packet to NG-RAN and inform our view to SA2.

- Reference

[1] R3-235020, Reply LS on INACTIVE eDRX above 10.24sec and SDT, SA2.

- Annex: TP for TS38.413

#### 9.3.3.X	Paging Policy Differentiation

This IE provides paging policy differentiation information for a UE in RRC_INACTIVE state.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Priority Indicator (PPI) | M |  | INTEGER (0.. 7, …) | The Paging Policy Indicator is used for paging policy differentiation (see details in TS 23.501 [9]). |
| Allocation and Retention Priority | O |  | 9.3.1.19 |  |
| QoS Flow Identifier | M |  | 9.3.1.51 |  |
| PDU Session ID | M |  | 9.3.1.50 | This IE identifies a PDU Session for a UE. The definition and use of the PDU Session ID is specified in TS 23.501 [9]. |
| DL Data Size (FFS) |  |  |  |  |