3GPP TSG-RAN WG3 Meeting #121bis	R3-235492

Xiamen, China, October 9 – 13, 2023

Title: 	(TP for AI/ML BL CR for TS 37.483 and 38.473) On the impact of RAN AI/ML over E1 and F1 interfaces

Source: 	Huawei

Agenda item:	12.2.2.4

Document Type:	Discussion and decision

# 1. Introduction

In last RAN3#121 meeting the impact of RAN AI/ML over E1 and F1 interfaces was discussed and the following agreement was reached [1]:

Work on the measured EC transmission from gNB-DU to gNB-CU over F1 in R18. Whether reusing the current F1AP procedures or defining new procedures needs to be further discussed.

Also, there are still open issues from RAN3#117b-e meeting [2] that need further discussion:

Discussion to be continued on the following:

- 	FFS on detailed E1/F1 impact for the exchange of current/predicted UE traffic after the sufficient work for non-split 	architecture.

- 	FFS on detailed E1/F1 impact for the exchange of current/predicted UE trajectory after the sufficient work for non-split 	architecture.

In this paper, we discuss the above listed left issues from RAN3#117bis-e as well as the open aspect in the agreement from RAN3#121 (see highlighted text in yellow above).

# 2. Discussion

## 2.1 F1AP procedure to enable transmission of measured EC

In the last RAN3 meeting the discussion on the RAN AI/ML impacts over E1 and F1 interfaces was resumed – such topic was left on hold since RAN3#117bis-e meeting, waiting for RAN3 to further progress on the non-split architecture – and the discussion mainly focused on the inference-based Energy Saving (ES) use case, in particular how to transfer the measured Energy Cost (EC) metric from gNB-DU to gNB-CU over the F1 interface and the following agreement was reached:

Work on the measured EC transmission from gNB-DU to gNB-CU over F1 in R18. Whether reusing the current F1AP procedures or defining new procedures needs to be further discussed.

However, RAN3 needs to discuss whether reusing the current F1AP procedures or defining a new procedure to enable the gNB-DU to report its measured EC to the gNB-CU (see highlighted text in yellow above). Note that only the F1 interface is considered in Rel-18, since companies had concerns on transferring the measured EC of the gNB-CU-UP to the gNB-CU-CP over the E1 interface due to the fact that typically the gNB-CU-UP is virtualized within cloud platforms, hence making the computation of the concerned gNB-CU-UP’s energy consumption difficult.

We think that the EC to be provided over F1 is a generic information which is not strictly related to AI/ML, hence it seems reasonable to reuse the legacy procedures to transfer the measured EC from gNB-DU to gNB-CU over F1. Concerning the F1AP procedures to be used we propose to enhance the existing Resource Status Reporting Initiation and Resource Status Reporting procedures, which allow the gNB-CU to request the reporting of the gNB-DU’s measured EC.

Proposal 1: RAN3 to agree to enhance the existing F1AP’s Resource Status Reporting Initiation and Resource Status Reporting procedures to enable the gNB-DU to report its measured EC upon request from the gNB-CU.

A TP to TS 38.473 reflecting the above proposal is reported in Annex 1.

## 2.2 Remaining open aspects

In RAN3#117bis-e meeting the following was minuted in [2]:

Discussion to be continued on the following:

- 	FFS on detailed E1/F1 impact for the exchange of current/predicted UE traffic after the sufficient work for non-split 	architecture.

- 	FFS on detailed E1/F1 impact for the exchange of current/predicted UE trajectory after the sufficient work for non-split 	architecture.

- 	FFS on detailed energy efficiency metric and corresponding E1/F1 impact after the sufficient work for non-split architecture.

Apart from the last item in the above, which has been covered by the discussion in RAN3#121 and subsequent Proposal 1 in Section 2.1, we need to discuss how to handle the remaining two open issues from RAN3#117bis-e meeting.

### 2.2.1 On E1/F1 impact for the exchange of current/predicted UE traffic

In our understanding the UE traffic information should be transferred over the E1 interface, and for this purpose we can take the below agreement from RAN3#117bis-e into account [2]:

UE traffic metric takes the data volume for a UE as the starting point.

In addition, considering that the gNB-CU-CP is the location of the inference function, we think that the gNB-CU-UP can transfer the current UE traffic over E1 so that the gNB-CU-CP can infer the predicted UE traffic – which is not needed to be transferred to the gNB-CU-UP over E1 – and derive potential actions for, e.g., Load Balancing purposes such as offload some UEs from gNB-CU-UP1 to gNB-CU-UP2, both gNB-CU-UP1 to gNB-CU-UP2 being controlled by the same gNB-CU-CP. Therefore, we propose:

Proposal 2: RAN3 to agree to transfer the current UE traffic over E1 from gNB-CU-UP to gNB-CU-CP to allow the gNB-CU-CP to infer the predicted UE traffic and derive potential mobility actions for the UE(s).

Proposal 2bis: RAN3 to agree that there is no need to transfer the predicted UE traffic from gNB-CU-CP to the gNB-CU-UP.

About the E1AP procedure to use for enabling the gNB-CU-UP to send the current UE traffic to the gNB-CU-CP we think that we should strive to re-use existing E1AP procedures as much as possible, because the current UE traffic is yet a general information not directly tied to AI/ML.

In current TS 37.483 there is the class2 procedure Data Usage Report which uses UE-associated signalling and is initiated by the gNB-CU-UP to report data volume served at the gNB-CU-UP. In order to enable the gNB-CU-CP to request the gNB-CU-UP to report the data volume we should enhance the Data Usage Report procedure by introducing a corresponding class1 procedure, e.g., Data Usage Report Initiation.

Proposal 3: RAN3 to agree to enhance the existing E1AP Data Usage Report procedure by introducing a corresponding class1 procedure, e.g., Data Usage Report Initiation, in order to enable the gNB-CU-CP to request the gNB-CU-UP to report the data volume.

A TP to TS 37.483 reflecting the above proposals is reported in Annex 2.

### 2.2.2 On E1/F1 impact for the exchange of current/predicted UE trajectory

On this issue we think that the UE trajectory prediction could be useful to be transferred over F1 from the gNB-CU to the gNB-DU in order to assist resource allocation in those cells that are predicted to be visited by the UE.

By referring to Figure 1, let’s assume that a UE is handed over from Cell A in Node 1 (source gNB) to Cell B in Node 2 (next-hop target gNB) as a consequence of an inference-based mobility action, based on which the UE trajectory prediction has been sent from Node 1 to Node 2 via the HANDOVER REQUEST message over Xn. We think that sending the UE trajectory prediction over F1 to the gNB-DUs controlling the subsequent intra-gNB cells which are expected to be visited by the UE could be useful for timely resource allocation in those cells (Cell C and Cell D in Node 2 of Figure 1), hence improving the overall handover performance.

Figure 1 - UE trajectory prediction transferred from gNB-CU to gNB-DUs in the target node side.

Based on the above we propose:

Proposal 4: RAN3 to agree that the UE trajectory prediction received from the source gNB over Xn can be transferred from the target gNB-CU to target gNB-DU(s) over the F1 for timely resource allocation in target gNB cells which are expected to be visited by the UE.

About the F1AP procedure to use for enabling the gNB-CU to send the predicted UE trajectory to the gNB-DU we think that we should stick to the approach that we introduced for transferring the UE trajectory prediction over Xn, i.e., by exploiting the existing XnAP procedures (via the HANDOVER REQUEST message). Moreover, we think that a preliminary request from the gNB-DU to the gNB-CU to ask for the UE trajectory prediction, that is, a subscription-based mechanism for providing the UE trajectory prediction is not needed – note that the discussion on the need to have this mechanism also over Xn has not yet been concluded in RAN3, and in our paper in [3] we propose not to have this mechanism.

In current TS 38.473 there are the class2 procedures of UE Context Setup and UE Context Modification that could fit the purpose of providing the gNB-DU with the UE-specific UE trajectory prediction. Hence, we propose:

Proposal 5: RAN3 to agree to reuse the existing F1AP UE Context Setup and UE Context Modification procedures in order to enable the gNB-CU to transfer the UE trajectory prediction to the gNB-DU at the target gNB side.

The TP to TS 38.473 in Annex 1 also reflects the above proposals concerning the transfer of the UE trajectory prediction from the gNB-CU to the gNB-DU.

### 2.2.3 On the need to exchange UE performance feedback over E1/F1

For the agreed use cases in the AI/ML for NG-RAN Work Item in Rel-18, RAN3 agreed to introduce UE performance feedback information (Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay, Average Packet Loss) over Xn interface. In addition, for the split architecture scenario, RAN3 agreed that AI/ML Model Training is located in gNB-CU/gNB-CU-CP and AI/ML Model Inference function is located in gNB-CU/ gNB-CU-CP.

The main usage of UE performance feedback is for AI/ML model training or monitoring in gNB-CU/gNB-CU-CP, so it does not need to be transferred from gNB-CU/gNB-CU-CP to gNB-DU/gNB-CU-UP. On the contrary, the gNB-CU/gNB-CU-CP needs to collect UE feedback information from gNB-DU/gNB-CU-UP, so we think there is a need to introduce UE performance feedback information over RAN interfaces other than Xn.

- Proposal 6: RAN3 to agree that there is a standard impact of transferring UE performance feedback over RAN interfaces other than Xn.

As per TS 28.552, the Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss are measured in the gNB-DU, hence they should be introduced over F1.

While, for the Average Packet Delay, TS 38.314 specified the delay components for both DL and UL – refer to Figure 2 and Figure 3, respectively – some of which are measured by the gNB-DU and others by the gNB-CU-UP.

Figure 2 - RAN part of DL packet delay measurement.

Figure 3 – RAN part (including UE) of UL packet delay measurement.

Table 1 – DL and UL packet components, where they are measured.

|  | gNB-DU | gNB-CU-UP |
| --- | --- | --- |
| DL packet delay component | D1, D2 | D3, D4 |
| UL packet delay component | D2.1, D2.2 | D2.3, D2.4 |

The above Table 1 shows where each of the DL and UL packet component is measured, hence, based on that, we propose:

- Proposal 7: RAN3 to agree to introduce the following:

- Over F1 interface: Average UE Throughput DL, Average UE Throughput UL, Average Packet Loss and

  - For the Packet Delay DL: D1, D2

  - For the Packet Delay UL: D2.1, D2.2

- Over E1 interface:

  - For the Packet Delay DL: D3, D4

  - For the Packet Delay UL: D2.3, D2.4

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals:

Proposal 1: RAN3 to agree to enhance the existing F1AP’s Resource Status Reporting Initiation and Resource Status Reporting procedures to enable the gNB-DU to report its measured EC upon request from the gNB-CU.

Proposal 2: RAN3 to agree to transfer the current UE traffic over E1 from gNB-CU-UP to gNB-CU-CP to allow the gNB-CU-CP to infer the predicted UE traffic and derive potential mobility actions for the UE(s).

Proposal 2bis: RAN3 to agree that there is no need to transfer the predicted UE traffic from gNB-CU-CP to the gNB-CU-UP.

Proposal 3: RAN3 to agree to enhance the existing E1AP Data Usage Report procedure by introducing a corresponding class1 procedure, e.g., Data Usage Report Initiation, in order to enable the gNB-CU-CP to request the gNB-CU-UP to report the data volume.

Proposal 4: RAN3 to agree that the UE trajectory prediction received from the source gNB over Xn can be transferred from the target gNB-CU to target gNB-DU(s) over the F1 for timely resource allocation in target gNB cells which are expected to be visited by the UE.

Proposal 5: RAN3 to agree to reuse the existing F1AP UE Context Setup and UE Context Modification procedures in order to enable the gNB-CU to transfer the UE trajectory prediction to the gNB-DU at the target gNB side.

- Proposal 6: RAN3 to agree that there is a standard impact of transferring UE performance feedback over RAN interfaces other than Xn.

- Proposal 7: RAN3 to agree to introduce the following:

- Over F1 interface: Average UE Throughput DL, Average UE Throughput UL, Average Packet Loss and

  - For the Packet Delay DL: D1, D2

  - For the Packet Delay UL: D2.1, D2.2

- Over E1 interface:

  - For the Packet Delay DL: D3, D4

  - For the Packet Delay UL: D2.3, D2.4

# 4. Reference

- 3GPP RAN3#121 Chairman’s notes (RAN3_121_agenda_20230825_EOM1.doc)

- 3GPP RAN3#117bis-e Chairman’s notes (RAN3_117bis-e_agenda_20221018_EOM1.doc)

- R3-235488, (TP for AI/ML BL CR for TS 38.423) Remaining open issues for the Mobility Enhancements use case, Huawei

# Annex 1: TP to 38.473 (v17.6)

<<<<<< Start of CHANGES >>>>>

### 8.2.10	Resource Status Reporting Initiation

#### 8.2.10.1	General

This procedure is used by an gNB-CU to request the reporting of load measurements to gNB-DU.

The procedure uses non UE-associated signalling.

#### 8.2.10.2	Successful Operation

Figure 8.2.10.2-1: Resource Status Reporting Initiation, successful operation

gNB-CU initiates the procedure by sending the RESOURCE STATUS REQUEST message to gNB-DU to start a measurement, stop a measurement, or add cells to report for a measurement. Upon receipt, gNB-DU:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If measurements are already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the RESOURCE STATUS REQUEST message and the Report Characteristics IE indicates cell specific measurements, the Cell To Report List IE shall be included.

If Registration Request IE is set to "add" in the RESOURCE STATUS REQUEST message, the Cell To Report List IE shall be included.

If gNB-DU is capable to provide all requested resource status information, it shall initiate the measurement as requested by gNB-CU, and respond with the RESOURCE STATUS RESPONSE message.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects gNB-DU shall perform measurements on. For each cell, gNB-DU shall include in the RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If the cell for which Radio Resource Status IE is requested to be reported supports more than one SSB, the Radio Resource Status IE for such cell shall include the SSB Area Radio Resource Status Item IE for all SSB areas supported by the cell. If the SSB To Report List IE is included for a cell, the Radio Resource IE for such cell shall only include the SSB Area Radio Resource Status List IE; If the cell for which Radio Resource Status IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Radio Resource Status IE for such cell shall, if supported, include the requested Slice Radio Resource Status Item IE if the cell for which Radio Resource Status IE is requested to be reported supports MIMO the Radio Resource Status IE for such cell shall include the MIMO PRB usasge Information IE;

-	the TNL Capacity Indicator IE, if the second bit, "TNL Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the third bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE. If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one SSB the Composite Available Capacity Group IE for such cell shall include the SSB Area Capacity Value List IE for all SSB areas supported by the cell, providing the SSB area capacity with respect to the Cell Capacity Class Value IE. If the SSB To Report List IE is included for a cell, the Composite Available Capacity Group IE for such cell shall include the requested SSB Area Capacity Value List IE providing the SSB area capacity with respect to the Cell Capacity Class Value. If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Slice Available Capacity IE for such cell shall include the requested Slice Available Capacity Value Downlink IE and Slice Available Capacity Value Uplink IE, providing the slice capacity with respect to the Cell Capacity Class Value.

-	the Hardware Load Indicator IE, if the fourth bit, " HW LoadInd Periodic " of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Number of Active UEs IE, if the fifth bit, "Number of Active UEs Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the NR-U Channel List IE, if the sixth bit, " NR-U Channel List Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

The gNB-DU shall include the following node level measurements in the RESOURCE STATUS UPDATE message:

- 	the Energy Cost IE, if the seventh bit, “Energy Cost” of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to “1”.

- 	the UE Performance IE, if the eighth bit, “UE Performance” of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to “1”.

If the Reporting Periodicity IE in the RESOURCE STATUS REQUEST is present, this indicates the periodicity for the reporting of periodic measurements. The gNB-DU shall report once, unless otherwise requested within the Reporting Periodicity IE.

#### 8.2.10.3	Unsuccessful Operation

Figure 8.2.10.3-1: Resource Status Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, gNB-DU shall send the RESOURCE STATUS FAILURE message with an appropriate cause value.

#### 8.2.10.4	Abnormal Conditions

If the initiating gNB-CU does not receive either RESOURCE STATUS RESPONSE message or RESOURCE STATUS FAILURE message, the gNB-CU may reinitiate the Resource Status Reporting Initiation procedure towards the same gNB-DU, provided that the content of the new RESOURCE STATUS REQUEST message is identical to the content of the previously unacknowledged RESOURCE STATUS REQUEST message with the same Transaction ID.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the RESOURCE STATUS REQUEST message then gNB-DU shall initiate a RESOURCE STATUS FAILURE message with an appropriate cause value.

If the gNB-DU receives a RESOURCE STATUS REQUEST message which includes the Registration Request IE set to "start" and the gNB-CU Measurement ID IE corresponding to an existing on-going load measurement reporting, for which a different Transaction ID is used, then gNB-DU shall initiate a RESOURCE STATUS FAILURE message with an appropriate cause value.

<<<<<< Next CHANGE >>>>>

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB,DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration, UE trajectory prediction. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the case of RACH based SDT procedure, the CellGroupConfig IE shall be ignored by the gNB-CU.

****** skip unchanged parts ******

If the ncd-SSB-RedCapInitialBWP-SDT IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the Cell Based UE Trajectory Prediction IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider the content of this list as the predicted cells for UE trajectory by the source NG-RAN node, and may use it for e.g. resource allocation purposes.

<<<<<< Next CHANGE >>>>>

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

****** skip unchanged parts ******

If the ncd-SSB-RedCapInitialBWP-SDT IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the Cell Based UE Trajectory Prediction IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider the content of this list as the predicted cells for UE trajectory by the source NG-RAN node, and may use it for e.g. resource allocation purposes.

<<<<<< Next CHANGE >>>>>

#### 9.2.1.20	RESOURCE STATUS REQUEST

This message is sent by gNB-CU to gNB-DU to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the resource status is required. | YES | ignore |
| Report Characteristics | C-ifRegistrationRequestStart |  | BIT STRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-DU is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit = HW LoadInd Periodic, Fifth Bit = Number of Active UEs Periodic, Sixth Bit = NR-U Channel List Periodic, Seventh Bit = Energy Cost, Eighth Bit = UE Performance. Other bits shall be ignored by the gNB-DU. | YES | ignore |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxCellingNBDU> |  |  |  |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | - |  |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB index | M |  | INTEGER (0..63) |  |  |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | - |  |
| >>>Slice To Report Item |  | 1..< maxnoofBPLMNsNR> |  |  |  |  |
| >>>>PLMN Identity | M |  | 9.3.1.14 | Broadcast PLMN |  |  |
| >>>>S-NSSAI List |  | 1 |  |  | - |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of indicated measurements. Also used as the averaging window length for all measurement object if supported. This IE is ignored if the Registration Request IE is set to “add”. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

<<<<<< Next CHANGE >>>>>

#### 9.2.1.23	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Hardware Load Indicator | O |  | 9.3.1.136 |  | YES | ignore |
| TNL Capacity Indicator | O |  | 9.3.1.128 |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellingNBDU > |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Radio Resource Status | O |  | 9.3.1.129 |  | - |  |
| >>Composite Available Capacity Group | O |  | 9.3.1.130 |  | - |  |
| >>Slice Available Capacity | O |  | 9.3.1.134 |  | - |  |
| >>Number of Active UEs | O |  | 9.3.1.135 |  | - |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Identifies a portion of the NR-U Channel Bandwidth on which channel access procedure in shared spectrum has been performed in the last reporting period. | - |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 indicates that the channel resources have been utilized for DL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | - |  |
| Energy Cost | O |  | INTEGER (0..10000,…) | gNB-DU level measured Energy Consumption index. Value 0 indicates the minimum Energy Consumption and 10000 indicates the maximum measured Energy Consumption. Energy Consumption is measured on a linear scale. | YES | FFS |
| UE Associated Info Result |  | 0..1 |  |  | YES | ignore |
| > UE Associated Info Result Item |  | 1..<maxnoofUEReports> |  |  | - |  |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| >>gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| >>UE Performance | O |  | 9.3.1.yy |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofUEReports | Maximum no. UEs that can be served by a NG-RAN node. Value is FFS |

<<<<<< Next CHANGE >>>>>

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| ****** skip unchanged parts ****** |  |  |  |  |  |  |
| ServingCellMO List |  | 0..1 |  | For NCD-SSBs | YES | ignore |
| >ServingCellMO Item IEs |  | 1 .. <maxnoofServingCellMOs> |  |  | EACH | ignore |
| >>servingCellMO | M |  | INTEGER (1..64) |  | - |  |
| >>SSB frequency | M |  | INTEGER (0..3279165) | ARFCN | - |  |
| Cell Based UE Trajectory Prediction | O |  | 9.3.1.xx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

<<<<<< Next CHANGE >>>>>

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ****** skip unchanged parts ****** |  |  |  |  |  |  |
| Uplink TxDirectCurrentMoreCarrierList Information | O |  | 9.3.1.284 |  | YES | ignore |
| CPAC MCG Information |  | 0..1 |  | This IE is used at the MN for MCG configuration as specified in TS 37.340 [7] for CPAC. | YES | ignore |
| >CPAC Trigger | M |  | ENUMERATED (CPAC-preparation, CPAC-executed, …) |  | - | - |
| >PSCell ID | M |  | NR CGI 9.3.1.12 | The PSCell corresponding to the included CG-Config IE at CPAC-preparation or the selected PSCell by the UE at CPAC-executed. | - | - |
| Cell Based UE Trajectory Prediction | O |  | 9.3.1.xx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channel allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofSLdestinations | Maximum number of destination for NR sidelink communication, the maximum value is 32 |

| Condition | Explanation |
| --- | --- |
| ifCHOcancel | This IE may be present if the CHO Trigger IE is present and set to "CHO-cancel". |

<<<<<< Next CHANGE >>>>>

#### 9.3.1.xx	Cell Based UE Trajectory Prediction

Editors Note: Actual name of this IE is FFS.

The Cell Based UE Trajectory Prediction IE contains the list of NG-RAN cells where the UE is predicted to connect.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Predicted Trajectory Cell List |  | 1..<maxnoofCellsTrajectoryPredict> |  | List of cells where the UE is predicted to connect, in chronological order. The first predicted cell added to the top of this list, is the cell where the UE will move to after the serving cell. |
| >Predicted Trajectory Cell Information | M |  | 9.3.1.zz |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsTrajectoryPredict | Maximum number of cells that can be predicted for UE trajectory. Value is FFS. |

#### 9.3.1.zz	Predicted Trajectory Cell Information

The Predicted Trajectory Cell Information contains the cell ID of the predicted NG-RAN cell for cell based UE trajectory prediction.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Predicted Trajectory Cell Information | M |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Global NG-RAN Cell Identity | M |  | 9.3.1.12 |  |
| >>Predicted Time UE Stays in Cell | O |  | INTEGER (0..4095) | The duration of time the UE is expected to stay in the cell, or set of NG-RAN cells (FFS on the meaning of the set of NG-RAN cells), in seconds. If the duration is more than 4095s, this IE is set to 4095. |

#### 9.3.1.yy	UE performance

This IE indicates the UE performance measurement metrics

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | FFS |  |
| Average UE Throughput UL | O |  | FFS |  |
| Average Packet Delay | O |  | FFS |  |
| Average Packet Loss | O |  | FFS |  |

<<<<<< End of CHANGES >>>>>

# Annex 2: TP to 37.483 (v17.6)

<<<<<< Start of CHANGES >>>>>

### 8.2.9	Resource Status Reporting Initiation

#### 8.2.9.1	General

This procedure is used by an gNB-CU-CP to request the reporting of load measurements to gNB-CU-UP.

The procedure uses non UE-associated signalling.

#### 8.2.9.2	Successful Operation

Figure 8.2.9.2-1: Resource Status Reporting Initiation, successful operation

The procedure is initiated with a RESOURCE STATUS REQUEST message sent from gNB-CU-CP to gNB-CU-UP to start a measurement or stop a measurements.

If gNB-CU-UP is capable to provide all requested resource status information, it shall initiate the measurement as requested by gNB-CU-CP, and respond with the RESOURCE STATUS RESPONSE message.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects gNB-CU-UP shall perform measurements on. The gNB-CU-UP shall include in the RESOURCE STATUS UPDATE message:

-	the HW Capacity Indicator IE, if the second bit, "HW Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the TNL Available Capacity Indicator IE, if the first bit, " TNL Available Capacity Ind Periodic " of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the UE Performance IE, if the third bit, “UE Performance” of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1.

If the Reporting Periodicity IE is included in the RESOURCE STATUS REQUEST message, this indicates the periodicity for the reporting of periodic measurements. The gNB-CU-UP shall report only once, unless otherwise requested within the Reporting Periodicity IE.

#### 8.2.9.3	Unsuccessful Operation

Figure 8.2.9.3-1: Resource Status Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, gNB-CU-UP shall send a RESOURCE STATUS FAILURE message with an appropriate cause value.

#### 8.2.9.4	Abnormal Conditions

If the initiating gNB-CU-CP does not receive either RESOURCE STATUS RESPONSE message or RESOURCE STATUS FAILURE message, the gNB-CU-CP may reinitiate the Resource Status Reporting Initiation procedure towards the same gNB-CU-UP, provided that the content of the new RESOURCE STATUS REQUEST message is identical to the content of the previously unacknowledged RESOURCE STATUS REQUEST message with the same Transaction ID.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the RESOURCE STATUS REQUEST message then gNB-CU-UP shall initiate a RESOURCE STATUS FAILURE message with an appropriate cause value.

If the gNB-CU-UP receives a RESOURCE STATUS REQUEST message which includes the Registration Request IE set to "start" and the gNB-CU-CP Measurement ID IE corresponding to an existing on-going load measurement reporting, for which a different Transaction ID is used, then gNB-CU-UP shall initiate a RESOURCE STATUS FAILURE message with an appropriate cause value.

<<<<<< Next CHANGE >>>>>

### 8.2.x	Data Usage Report Initiation

#### 8.2.x.1	General

This procedure is used by an gNB-CU-CP to request the reporting of data usage from the gNB-CU-UP.

The procedure uses UE-associated signalling.

#### 8.2.x.2	Successful Operation

Figure 8.2.x.2-1: Data Usage Report Initiation, successful operation.

The procedure is initiated with a DATA USAGE REPORT REQUEST message sent from gNB-CU-CP to gNB-CU-UP to start a measurement or stop a measurement.

If gNB-CU-UP is capable to provide the measurement, it shall initiate the measurement as requested by gNB-CU-CP, and respond with the DATA USAGE REPORT RESPONSE message.

Interaction with other procedures

When starting a measurement, the Reporting Periodicity IE indicates the periodicity for the gNB-CU-UP to trigger the reporting of the DATA USAGE REPORT message.

If the gNB-CU-UP receives a DATA USAGE REPORT REQUEST message which includes the Registration Request IE set to “stop” then gNB-CU-UP shall stop the reporting of data usage periodic measurements and the gNB-CU-UP may trigger the reporting.

#### 8.2.x.3	Unsuccessful Operation

Figure 8.2.x.3-1: Data Usage Report Initiation, unsuccessful operation.

If the requested measurement cannot be initiated, the gNB-CU-UP shall send a DATA USAGE REPORT FAILURE message with an appropriate cause value.

#### 8.2.x.4	Abnormal Conditions

Not applicable.

<<<<<< Next CHANGE >>>>>

### 8.3.8	Data Usage Report

#### 8.3.8.1	General

This procedure is initiated by the gNB-CU-UP to report data volume served at the gNB-CU-UP or to report the result of the measurement admitted by the gNB-CU-UP following a successful Data Usage Report Initiation procedure. The procedure uses UE-associated signalling.

#### 8.3.8.2	Successful Operation

Figure 8.3.8.2-1: Data Usage Report procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the DATA USAGE REPORT message to the gNB-CU-CP.

<<<<<< Next CHANGE >>>>>

#### 9.2.1.19	RESOURCE STATUS REQUEST

This message is sent by an gNB-CU-CP to gNB-CU-UP to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) | Type of request for which the resource status is required. | YES | ignore |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-CU-UP is requested to report. First Bit = TNL Available Capacity Ind Periodic, Second Bit = HW Capacity Ind Periodic, Third Bit = UE Performance. Other bits shall be ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, 20000ms, 30000ms, 40000ms, 50000ms, 60000ms, 70000ms, 80000ms, 90000ms, 100000ms, 110000ms, 120000ms, …) | Periodicity that can be used for reporting. Also used as the averaging window length for all measurement object if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value “stop” |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

<<<<<< Next CHANGE >>>>>

#### 9.2.1.22	RESOURCE STATUS UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the results of the requested measurements.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| TNL Available Capacity Indicator | O |  | 9.3.1.72 |  | YES | ignore |
| HW Capacity Indicator | O |  | 9.3.1.73 |  | YES | Ignore |
| UE Associated Info Result |  | 0..1 |  |  | YES | ignore |
| > UE Associated Info Result Item |  | 1..<maxnoofUEReports> |  |  |  |  |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |  |  |
| >>gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |  |  |
| >>UE Performance | O |  | 9.3.1.yy |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofUEReports | Maximum no. of UEs that can be served by a NG-RAN node. Value is FFS |

<<<<<< Next CHANGE >>>>>

#### 9.2.1.x	DATA USAGE REPORT REQUEST

This message is sent by an gNB-CU-CP to gNB-CU-UP to initiate the requested measurement according to the parameters given in the message. The procedure uses UE-associated signalling.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Registration Request | M |  | ENUMERATED (start, stop, …) | Type of request for which the resource status is required. | YES | ignore |
| Reporting Periodicity | C-ifRegistrationRequestStart |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, 20000ms, 30000ms, 40000ms, 50000ms, 60000ms, 70000ms, 80000ms, 90000ms, 100000ms, 110000ms, 120000ms, …) | Periodicity that can be used for reporting. Also used as the averaging window length for all measurement object if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

#### 9.2.1.y		DATA USAGE REPORT RESPONSE

This message is sent by the gNB-CU-UP to indicate that the requested measurement is successfully initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.y		DATA USAGE REPORT FAILURE

This message is sent by the gNB-CU-UP to indicate that the requested measurement cannot be initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<< Next CHANGE >>>>>

#### 9.3.1.yy	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average Packet Delay | O |  | FFS |  |

<<<<<< End of CHANGES >>>>>