3GPP TSG-RAN WG3 Meeting #121-bis	R3-235525

Xiamen, P.R. China, October 9th – 13th 2023

Agenda Item:	11.3.

Source:	Ericsson

Title:	(TP for QoE BL CR for TS 37.340, TS 38.300, and TS 38.423): QoE and RVQoE Measurements and Reporting in NR-DC Scenarios

Document for:	Agreement

# Introduction

In this paper we further discuss the support for QoE and RVQoE measurements and reporting in NR-DC scenarios, based on the agreements and TBCs from the RAN3#121 and earlier meetings. TPs for TS 37.340, TS 38.300, and TS 38.423 are provided in Annexes A, B and C.

# Discussion

The following issues are discussed:

- QoE measurement configuration and reporting.

- RVQoE measurement configuration and reporting.

- Handling of QMC upon mobility.

- The MN-SN coordination procedure.

## QoE measurement configuration and reporting

The following RAN3#121 agreements are related to QoE measurement configuration and reporting:

For s-based QoE configuration received by MN

- MN sends the QoE configuration via SRB1

- QoE reports can be sent via SRB4 or SRB5

For management based QoE, there is no need for SN to explicitly indicate to the MN whether it wants to configure the UE directly via SRB3 or via a container and SRB1.

In case the SN is interested in configuring a UE with an m-based QoE measurement configuration, the MN can decide and notify the SN whether:

- The MN sends the configuration information to the UE, or

- The SN should send the configuration to the UE

At the RAN3#121 meeting it was agreed to include an XnAP QoE Reporting Option Preference IE in the QMC Initial Coordination Request IE. There are two related FFSs.

The first FFS relates to the codepoint design. Current codepoints for this IE are “SRB4” and “SRB5”, and the “SRB5” is FFS. We think that, when the MN determines that SN should to send the QoE configuration to the UE, the SN to indicate how it wants to receive the QoE reports. The two reporting alternatives are:

- The QoE reports for QoE measurements configured by the SN go to the SN directly, via SRB5.

- The QoE reports for QoE measurements configured by the SN go in a container sent by the UE in SRB4 and the MN forwards to the SN.

Therefore, we think the “FFS” label related to SRB5 can be removed, and the QoE Reporting Option Preference IE can have two code points - one for SRB4, one for SRB5.

Proposal 1-1: When a management-based QoE configuration is received directly by the SN from the OAM, the SN can explicitly indicate to the MN whether it prefers to receive the QoE/RVQoE reports via the SRB4 or via the SRB5.

Regarding the FFS on whether this IE can only be sent by the SN, we think that, for the case when the MN determines to send the configuration information to the UE directly, the simplest approach is that the MN also receives the corresponding QoE reports, at least in the beginning of the session. If needed (e.g., in case of overload at the configuring node), reporting leg can be switched, but we do not see the need to support the case where, already during the initial coordination, the MN configures the UE and then requests the SN to receive the QoE reports from start. If needed, the MN can request this later. So, the present IE should only be sent by the SN.

Proposal 1-2: The XnAP QoE Reporting Option Preference IE in the QMC Initial Coordination Request IE can only be sent by the SN.

In the current BL for XnAP, there is also an FFS for the QMC Initial Coordination Request IE:

Editor’s Note: Whether to also include QoE configuration container in the IE is FFS.

Based on the agreement from RAN3#121: “For management based QoE, there is no need for SN to explicitly indicate to the MN whether it wants to configure the UE directly via SRB3 or via a container and SRB1.” we think that there is no need for the SN to include the QoE configuration container in the QMC Initial Coordination Request IE. The reason is that, if MN has not itself received the QoE configuration container from the OAM, then it should not be allowed to configure the UE. So, we propose to remove the FFS and not add the QoE configuration container to the QMC Initial Coordination Request IE.

Proposal 1-3: The QoE configuration container is not included in the QMC Initial Coordination Request IE.

## RVQoE measurement configuration and reporting

One aspect of the coordination between MN and SN is what to do when the node that configured RVQoE (i.e., the RVQoE-configuring node) realizes that the other node (i.e., the non RVQoE-configuring node) provides the bearer(s) for the session. Some aspects have been captured in the current QoE XnAP BL CR, but we think some details are still missing. A related RAN3#120 agreement states the following:

When the RVQoE-configuring node receives an RVQoE report and determines that the non-RVQoE-configuring node provides the bearer(s) for the application session, the RVQoE-configuring node indicates that to the non-RVQoE-configuring node. FFS whether the indication is explicit or implicit.

In the response message, the SN (i.e., the non-RVQoE-configuring node) can indicate the following, as per the RAN3#120 agreement:

In the response, the non-RVQoE-configuring node:

- Can indicate to the RVQoE-configuring node that it does not want to receive the RVQoE reports.

- Can indicate whether it prefers to receive the RVQoE reports directly from the UE.

- Can indicate to the RVQoE-configuring node its preferred RVQoE configuration parameters, based on which the RVQoE-configuring node should modify the RVQoE configuration.

If the SN is the RVQoE-configuring node and the MN is the non-RVQoE-configuring node, the logic is identical, but the affected XnAP messages are different:

- When the MN is the RVQoE-configuring node, the impacted messages are S-NODE MODIFICATION REQUEST and S-NODE MODIFICATION REQUEST ACKNOWLEDGE messages.

- When the SN is the RVQoE-configuring node, the impacted messages are S-NODE MODIFICATION REQUIRED and S-NODE MODIFICATION CONFIRM messages.

As of now the QoE XnAP BL CR contains the following:

- The S-NODE MODIFICATION REQUEST (MN to SN) and S-NODE MODIFICATION REQUIRED (SN to MN) messages contain the QMC Modification Request IE

- The S-NODE MODIFICATION REQUEST ACKNOWLEDGE (SN to MN) and S-NODE MODIFICATION CONFIRM (MN to SN) messages contain the QMC Modification Response IE.

The QMC Modification Request IE within the S-NODE MODIFICATION REQUEST and S-NODE MODIFICATION REQUIRED messages contains 3 sub-IEs related to RVQoE:

| >>RAN Visible QoE Reporting Modification Request | O |  | ENUMERATED (true, ...) |  |
| --- | --- | --- | --- | --- |
| >>Available RAN Visible QoE Metrics | O |  | 9.2.3.158 |  |
| >>RAN Visible QoE Configuration Preference | O |  | 9.2.3.x5 |  |

The QMC Modification Response IE within the S-NODE MODIFICATION REQUEST ACKNOWLEDGE, and S-NODE MODIFICATION CONFIRM messages contains 2 sub-IEs related to RVQoE:

| >>RVQoE Reporting Modification response | O |  | ENUMERATED (accepted, rejected, …) | FFS whether the code points should be refined, to reflect the agreement about the response from non-RVQoE configuring node. |
| --- | --- | --- | --- | --- |
| >>RAN Visible QoE Configuration Preference | O |  | 9.2.3.x5 |  |

We think that, based on the agreements above, the following is needed:

- In the QMC Modification Request IE:

  - A request for the RVQoE-configuring node to inquire whether the non-QoE-configuring node is interested in receiving further QoE reports.

    - For this purpose, we propose to introduce a new IE, Further QoE Visible QoE Reporting Request IE.

  - A request, for the QoE-configuring node to inquire whether the non-QoE-configuring node prefers to receive the reports directly or it wants to continue receiving the QoE reports via the QoE-configuring node.

    - This can be enabled by the QoE Visible QoE Reporting Modification Request IE. We also propose to rename this IE to QoE Visible QoE Reporting Path Modification Request IE for better clarity. The IE can be made a BOOLEAN, like the QoE Reporting Modification Request IE.

  - Addition of the semantics description and renaming of QoE Visible QoE Configuration Preference IE.

- In the QMC Modification Response IE:

  - An indication of interest to receive further QoE reports.

    - This can be included in the new Further QoE Visible QoE Reporting Response IE.

  - An indication of the preference for receiving the reports directly from the QoE or continue to receive the QoE reports via the other node.

  - An indication of preferred QoE configuration parameters, based on which the QoE-configuring node should modify the QoE configuration. This is captured by the current QoE Visible QoE Configuration Preference IE.

Proposal 2-1: In the QMC Modification Request IE:

- Add a new IE (e.g., Further RAN Visible QoE Reporting Request IE) for inquiring the NG-RAN node receiving it whether it wants to receive further RVQoE reports.

- Change the name, the encoding, and the semantics description for RAN Visible QoE Reporting Modification Request IE.

- Change the name and add the semantics description for RAN Visible QoE Configuration Preference IE to request the RVQoE Configuration preference of the peer NG-RAN node.

Proposal 2-2: In the QMC Modification Response IE:

- Add a new IE (e.g., Further RAN Visible QoE Reporting Response) to indicate to the peer NG-RAN node the interest to receive further RVQoE reports.

- Change the name, the encoding, and the semantics description for RAN Visible QoE Reporting Modification Response IE.

- Change the name, the encoding, and the semantics description for RVQoE Reporting Modification Response IE.

## Handling of QMC upon mobility

A RAN3#120 agreement related to MN-related mobility states:

In line with the Rel-17 principles, if the MN configured the UE with QoE measurements, every subsequent MN serving the UE can configure and release the RVQoE measurements.

In case of MN change, we think that the Rel-17 framework for QoE measurement continuity for mobility in single connectivity can be reused. Consequently, a stage-2 modification is needed. There seems to be no stage-3 impact.

Proposal 3-1: To support MN change, the Rel-17 framework for QoE measurement continuity for mobility in single connectivity is reused.

For SN change, the following was agreed at the RAN3#120 meeting:

At SN change without MN change, the QoE configuration information is passed from the old/source SN to the new/target SN via the MN.

The SN change can be SN-initiated or MN-initiated, which requires different ways of passing the QoE information from the old/source SN to the new/target SN via the MN.

For SN-initiated SN change, the initiating message is the S-NODE CHANGE REQUIRED, and it can be used to pass the QoE configuration from the source SN to the MN. The MN will then forward the configuration to the target SN in the S-NODE ADDITION REQUEST message.

For MN-initiated SN change, before the MN sends the S-NODE ADDITION REQUEST message for adding the new/target SN, the MN needs to fetch the configuration from the old/source SN. This can be done following the same approach as described in the NOTE from clause 10.5.2 of TS 37.340, as shown below.

=====================Start of excerpt from TS 37.340=====================

NOTE 1: The MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration and to allow provision of data forwarding related information before step 1.

======================End of excerpt from TS 37.340======================

In particular, the QoE configuration of the old/source SN can be sent to the MN in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message. Therefore, to support the SN-related mobility, we foresee the following impacts:

- Stage-2 impact: A clarification in TS 37.340 stating that the MN may trigger the MN-initiated SN Modification procedure towards the source SN to retrieve the QoE configuration.

- Stage-3 impacts: Addition of the QMC Configuration Information IE in the following XnAP messages (including the corresponding clarifications in the normative text of TS 38.423):

  - S-NODE CHANGE REQUIRED.

  - S-NODE ADDITION REQUEST.

  - S-NODE MODIFICATION REQUEST ACKNOWLEDGE.

Proposal 3-2: To support the QoE measurement continuity upon SN-initiated SN change, add the QMC Configuration Information IE into the S-NODE CHANGE REQUIRED message (from the old SN to the MN) and S-NODE ADDITION REQUEST XnAP message (from the MN to the new SN).

Proposal 3-3: To support the QoE measurement continuity upon MN-initiated SN change, add the QMC Configuration Information IE into the S-NODE MODIFICATION REQUEST ACKNOWLEDGE XnAP message (from the old SN to the MN) and the S-NODE ADDITION REQUEST message (from the MN to the new SN).

Proposal 3-4: Clarify in TS 38.423 and TS 37.340 that the MN may trigger the MN-initiated SN Modification procedure towards the source SN to retrieve the QMC configuration prepared by the current SN.

For SN-release, the following RAN3#120 agreement applies:

If the SN configured a UE with QoE measurements, at SN release, the QoE/RVQoE configuration can be released. Whether the SN-configured QoE/RVQoE configuration information can be passed to the MN in case of SN release needs to be further discussed.

With respect to the TBC part of the agreement, a simple approach is not to pass any configuration information to the MN and that the UE releases the QoE and RVQoE configuration prepared by the SN.

Proposal 3-5: If the SN configured a UE with QoE/RVQoE measurements, the UE releases the SN configured QoE/RVQoE measurements at SN release.

According to the above proposal, the UE will delete all unsent QoE/RVQoE reports. However, in some scenarios, it is needed for the network to acquire the unsent reports. For example, if the UE is configured to send one QoE report for the entire session, this report should not be discarded since the measurement results for the entire session are wasted. Note that, often, SN release may be executed promptly, not leaving enough time for the UE to send the pending reports to the SN. A simple solution for this issue is to configure the UE to send these unsent reports to the MN.

Proposal 3-6: If the SN configured a UE with QoE/RVQoE measurements, a UE can be configured to, upon SN release, send the unsent QoE reports to the MN.

Regarding how the QoE/RVQoE configuration release is communicated to the UE, since the SN release can be SN-initiated or MN-initiated, both scenarios need to be considered. Note that, for both MN- and SN-initiated SN release, the RRCReconfiguration message that informs the UE about the SN release is sent to the UE from the MN.

In case of SN-initiated SN release, if the SN assembled the QoE/RVQoE configuration for the UE, the SN could send the command to release the QoE/RVQoE configurations to the UE before, or upon (i.e., together with), sending the SN release command to the UE. In case the SN assembled the QoE/RVQoE configuration and sent it to the UE via the MN, the release command for QoE/RVQoE configuration should be sent from the SN to the MN inside the S-NG-RAN node to M-NG-RAN node Container IE of the S-NODE RELEASE REQUIRED XnAP message. This should be captured in QoE BL CRs for TS 38.423 and TS 37.340.

Proposal 3-7: To enable the SN-initiated SN release, if the SN assembled the QoE/RVQoE configuration and sent it to the UE via the MN, introduce into the existing S-NG-RAN node to M-NG-RAN node Container IE of the S-NODE RELEASE REQUIRED XnAP message a command for the UE to release the QoE/RVQoE configuration.

In case of SN-initiated SN release, clarify in TS 37.340 that the command to release the QoE/RVQoE configuration for the UE can be sent from the SN to the MN (in the S-NODE RELEASE REQUIRED XnAP message).

In case of MN-initiated SN release, if the QoE/RVQoE configuration was assembled by the SN, the UE can be instructed to release the QoE/RVQoE configuration(s) by using the RRCReconfiguration message sent from the MN as a part of the MN-initiated release procedure. The command for releasing the QoE/RVQoE configurations can be sent from the SN to the MN inside the existing S-NODE RELEASE REQUEST ACKNOWLEDGE XnAP message, in a newly defined S-NG-RAN node to M-NG-RAN node Container IE.

Proposal 3-8: In case of MN-initiated SN release, to release a QoE/RVQoE configuration assembled by the SN, introduce a new S-NG-RAN node to M-NG-RAN node Container IE into the existing S-NODE RELEASE REQUEST ACKNOWLEDGE XnAP message, containing a command for the UE to release the QoE/RVQoE configurations.

Proposal 3-9: Send an LS to RAN2 requesting to implement the agreements related to the handling of QMC upon mobility.

## The MN-SN coordination procedure

RAN3 agreed that the support for MN-SN coordination for QMC is achieved by reusing existing UE-associated procedures defined for Dual Connectivity. For the case when the MN initiates the coordination some further clarifications are needed with respect to handling possible changes in the RVQoE configuration. If the MN configured the UE and later determines that the SN carries the session (or that also the SN carries the session), the MN optionally informs the SN about the available RVQoE metrics and if the SN responds with a preferred RVQoE configuration that is different from the one sent to the UE, the MN can modify it. Then, what is currently missing is that the MN needs to notify the SN that the UE has been successfully configured with the new RVQoE configuration. This can be done by using the S-NODE RECONFIGURATION COMPLETE message (S-NG-RAN node Reconfiguration Completion class-2 procedure). This impact needs to be captured in the QoE BL CR for TS 38.423.

Proposal 4: For MN-initiated QMC coordination, enhance the existing S-NODE RECONFIGURATION COMPLETE XnAP message to notify the SN that RVQoE configuration has been modified.

## TPs for QoE BL CRs for TS 37.340, TS 38.300, and TS 38.423

TPs for QoE BL CRs for TS 37.340, TS 38.300, and TS 38.423 are provided in Annexes A, B and C, respectively.

Proposal 5: Agree the TPs for QoE BL CR for TS 37.340, TS 38.300, and TS 38.423 provided in Annexes A, B and C.

# Conclusion

This paper discusses the support for QoE and RVQoE measurements and reporting in NR-DC scenarios. The following is proposed:

QoE measurement configuration and reporting

Proposal 1-1: When a management-based QoE configuration is received directly by the SN from the OAM, the SN can explicitly indicate to the MN whether it prefers to receive the QoE/RVQoE reports via the SRB4 or via the SRB5.

Proposal 1-2: The XnAP QoE Reporting Option Preference IE in the QMC Initial Coordination Request IE can only be sent by the SN.

Proposal 1-3: The QoE configuration container is not included in the QMC Initial Coordination Request IE.

RVQoE measurement configuration and reporting

Proposal 2-1: In the QMC Modification Request IE:

- Add a new IE (e.g., Further RAN Visible QoE Reporting Request) for inquiring the NG-RAN node receiving it whether it wants to receive further RVQoE reports.

- Change the name, the encoding, and the semantics description for RAN Visible QoE Reporting Modification Request IE.

- Change the name and add the semantics description for RAN Visible QoE Configuration Preference IE to request the RVQoE Configuration preference of the peer NG-RAN node.

Proposal 2-2: In the QMC Modification Response IE:

- Add a new IE (e.g., Further RAN Visible QoE Reporting Response) to indicate to the peer NG-RAN node the interest to receive further RVQoE reports.

- Change the name, the encoding, and the semantics description for RAN Visible QoE Reporting Modification Response IE.

- Change the name, the encoding, and the semantics description for RVQoE Reporting Modification Response IE.

Handling of QMC upon mobility

Proposal 3-1: To support MN change, the Rel-17 framework for QoE measurement continuity for mobility in single connectivity is reused.

Proposal 3-2: To support the QoE measurement continuity upon SN-initiated SN change, add the QMC Configuration Information IE into the S-NODE CHANGE REQUIRED message (from the old SN to the MN) and S-NODE ADDITION REQUEST XnAP message (from the MN to the new SN).

Proposal 3-3: To support the QoE measurement continuity upon MN-initiated SN change, add the QMC Configuration Information IE into the S-NODE MODIFICATION REQUEST ACKNOWLEDGE XnAP message (from the old SN to the MN) and the S-NODE ADDITION REQUEST message (from the MN to the new SN).

Proposal 3-4: Clarify in TS 38.423 and TS 37.340 that the MN may trigger the MN-initiated SN Modification procedure towards the source SN to retrieve the QMC configuration prepared by the current SN.

Proposal 3-5: If the SN configured a UE with QoE/RVQoE measurements, the UE releases the SN configured QoE/RVQoE measurements at SN release.

Proposal 3-6: If the SN configured a UE with QoE/RVQoE measurements, a UE can be configured to, upon SN release, send the unsent QoE reports to the MN.

Proposal 3-7: To enable the SN-initiated SN release, if the SN assembled the QoE/RVQoE configuration and sent it to the UE via the MN, introduce into the existing S-NG-RAN node to M-NG-RAN node Container IE of the S-NODE RELEASE REQUIRED XnAP message a command for the UE to release the QoE/RVQoE configuration.

Proposal 3-8: In case of MN-initiated SN release, to release a QoE/RVQoE configuration assembled by the SN, introduce a new S-NG-RAN node to M-NG-RAN node Container IE into the existing S-NODE RELEASE REQUEST ACKNOWLEDGE XnAP message, containing a command for the UE to release the QoE/RVQoE configurations.

Proposal 3-9: Send an LS to RAN2 requesting to implement the agreements related to the handling of QMC upon mobility.

The MN-SN coordination procedure

Proposal 4: For MN-initiated QMC coordination, enhance the existing S-NODE RECONFIGURATION COMPLETE XnAP message to notify the SN that RVQoE configuration has been modified.

TPs for QoE BL CRs for TS 37.340, TS 38.300, and TS 38.423

Proposal 5: Agree the TPs for QoE BL CR for TS 37.340, TS 38.300, and TS 38.423 provided in Annexes A, B and C.

# Annex A: TP for QoE BL CR for TS 37.340

-------------------------------------------Start of changes-------------------------------------------

## 13.x	Application Layer Measurement Collection

### 13.x.1	Overview

The QoE Measurement Collection function as described in TS 38.300 [3] is extended to address NR-DC operation. The requirements on the gNB provided in TS 38.300 [3] apply to the MN, together with additional requirements on MN and SN provided in following sub-clauses. The MN-SN coordination described below uses the following procedures defined in TS 38.423 [5]:

- S-NG-RAN node Addition Preparation procedure.

- M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure.

- S-NG-RAN node initiated S-NG-RAN node Modification procedure.

- RRC Transfer procedure.

### 13.x.2	QoE Measurement Configuration

#### 13.x.2.1		QoE Measurement Collection Activation and Reporting in NR-DC

For a UE in NR-DC, the MN and the SN may coordinate QoE measurement collection activation and reporting as follows:

For management-based QoE activation, the MN:

- Allocates the measurement configuration application layer ID, and indicates to SN if needed;

- Determines whether the MN or the SN sends the QoE configuration to the UE, in case SN enquires MN.

For management-based QoE measurement configuration received directly by the SN from the OAM, the SN may perform UE selection. For a selected UE, the SN indicates to the MN the QoE reference of the management-based QoE session and, separately for the QoE and RAN visible QoE reports, the SN indicates whether it is going to receive the corresponding reports via the MN (using SRB4) or using SRB5. Upon receiving the request, the MN can decide and notify the SN whether the MN sends the QoE and RAN visible QoE configuration to the UE, or whether the SN should send the configuration(s) to the UE. The SN can send a QoE and a RAN visible QoE measurement configuration directly to the UE via SRB3, or in a transparent container to the MN, which then sends it to the UE via SRB1.

For signalling-based QoE measurement configuration received by the MN from the OAM, the MN may send the QoE configuration to the UE via SRB1, and the UE can send the QoE reports via SRB4 or SRB5.

The network explicitly indicates to the UE whether to send QoE reports via SRB4 or SRB5, per QoE reference, separately for QoE and RAN visible QoE reports. The SRB for QoE reporting can be changed during the application session. The command for changing the SRB used for reporting may be sent to the UE by the node that configured that specific QoE configuration. The node that currently receives the QoE reports via the Uu can request from the peer node that the QoE reporting leg is switched to the peer node. The leg switch for QoE reporting needs to be approved by both nodes serving the UE. RAN visible QoE reports can be sent over the same leg, as the QoE reports pertaining to the same QoE reference, or over the other leg.

The MN should inform the SN that a UE is configured with a management-based QoE/RAN visible QoE measurement configuration.

If the MN has configured the UE with QoE measurements, and if the UE is configured to send the QoE reports to the SN, then, if the MN decides that the SN forwards the reports directly to the MCE, the MN should indicate to the SN the QoE reference, the MCE IP address and the measConfigAppLayerId.

If the SN has configured the UE with QoE measurements, and if the UE is configured to send the QoE reports to the MN, then, if the SN decides that the MN forwards the reports directly to the MCE, the SN should indicate to the MN the QoE reference and the MCE IP address.

RAN visible QoE reports can be sent to the SN directly via the SRB5, or via the MN using SRB4.

### 13.x.3	QoE Measurement Continuity for Mobility

Editor’s note: Content for this clause is FFS. QMC continuity during mobility in NR-DC should be discussed after the baseline solution for QMC in NR-DC is in place.

If the MN configured the UE with QoE measurements, every subsequent MN serving the UE can configure and release the RAN visible QoE measurements.

At MN change (without SN change), the QoE configuration information is passed from the old/source MN to the new/target MN.

At SN change without MN change, the QoE configuration information is passed from the old/source SN to the new/target SN via the MN.

If the SN change is MN initiated, the MN may trigger an MN-initiated SN Modification to retrieve the QoE configuration.

If the SN configured a UE with QoE measurements, at SN release, the QoE/RAN visible QoE measurement configuration can be released and the UE can send unsent QoE reports to the MN.

At SN-initiated SN release, if the SN assembled the QoE/RVQoE configuration and sent it to the UE via the MN, the SN may initiate the S-NG-RAN node initiated S-NG-RAN node Release procedure to initiate the release of QoE/RVQoE configuration for the UE.

-------------------------------------------End of changes-------------------------------------------

# Annex B: TP for QoE BL CR for TS 38.300

-------------------------------------------Start of changes-------------------------------------------

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<

RTT	Round Trip Time

RVQoE	RAN visible QoE

SCS	SubCarrier Spacing

-----------------------------------------------Next change-----------------------------------------------

### 21.x1.x Support for RAN visible QoE measurements and reporting in NR-DC

Either the MN or the SN can generate and send a RAN visible QoE configuration to the UE. The gNB that has initially configured a UE in NR-DC with an RVQoE configuration can modify and release the RAN visible QoE configuration as long as the UE is connected to this gNB. The gNB that configures the encapsulated QoE to UE is the RAN visible QoE-configuring gNB, the peer node is the non-RAN visible QoE-configuring gNB. Upon mobility, the RAN visible QoE-configuring gNB may be changed.

The UE may send RAN visible QoE reports to the network using either SRB4 or SRB5. In addition, the gNB that received a RAN visible QoE report can forward the report to the other gNB (the SN or the MN). QoE reports and RAN visible QoE reports pertaining to the same QoE Reference can be sent over the same SRB or they can be sent over different SRBs.

The RAN visible QoE-configuring gNB can configure RAN visible QoE measurements at a UE without a priori knowledge about which gNB(s) will provide the bearer(s) for a future application session. During the lifetime of an application session, to ensure that the RAN visible QoE reports are sent to the gNB(s) that provide the bearer(s) which carry the data flow(s) associated with the RAN visible QoE measurement result in a RAN visible QoE report, the gNB receiving the RAN visible QoE reports determines the bearer(s) used to deliver the application session data flow(s) and the associated gNB (s). The determination may be based on the PDU session ID(s) and the QoS flow ID(s) indicated in a received RAN visible QoE report.

When the RAN visible QoE-configuring gNB receives a RAN visible QoE measurement report and determines that the non-RVQoE-configuring gNB provides at least one bearer for the application session, the RVQoE-configuring gNB indicates that to the non-RVQoE-configuring gNB. The non-RVQoE-configuring gNB can then, if needed, indicate to the RAN visible QoE-configuring gNB its preference with respect to the reporting path for the subsequent RAN visible QoE reports and its preferred RAN visible QoE configuration parameters.

When the non-RAN visible QoE-configuring gNB receives a RAN visible QoE report and determines that the RAN visible QoE-configuring gNB provides at least one bearer for the application session, the non-RAN visible QoE-configuring gNB may indicate this to the RAN visible QoE-configuring gNB, which may then decide to modify the RAN visible QoE configuration parameters.

If a gNB receives a RAN visible QoE report from a UE in NR-DC, and determines that the bearer(s) for the application session data flow(s) is (are) also provided by the other gNB, or only provided by the other gNB, the gNB that received the RAN visible QoE measurement report may forward the received RAN visible QoE report to the other gNB (WA) using the RRC Transfer procedure. The RAN visible QoE configuration may also be modified or released.

Editor Note: FFS on further refinements and the placement of the above text. The naming “RVQoE-configuring gNB” and “non-RVQoE-configuring gNB” are to be further discussed.

-------------------------------------------End of changes-------------------------------------------

# Annex C: TP for QoE BL CR for TS 38.423

-------------------------------------------Start of changes-------------------------------------------

## 8.3	Procedures for Dual Connectivity

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the S-NG-RAN node UE Slice Maximum Bit Rate IE for a specific S-NSSAI is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store and use the received S-NG-RAN node UE Slice Maximum Bit Rate for all PDU sessions associated with the S-NSSAI for the concerned UE as defined in TS 23.501 [7].

If the QMC Initial Coordination Request IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node may use it as specified in TS 37.340 [8], and shall, if supported, include the QMC Initial Coordination Response IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

Editor’s Note: The procedure text could be further refined. This note applies to all the new procedure text in this TP.

Editor’s Note: Whether SN Addition procedure would be used for QMC modification could be further discussed. It is FFS which IE to be used in this procedure (same IE as in other procedures or different IE).

If the QMC Configuration Information IE is included in the S-NODE ADDITION REQUEST message, the NG-RAN node shall, if supported, use it for QoE management, as described in TS 38.300 [9].

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-------------------------------------------Next change-------------------------------------------

### 8.3.2	S-NG-RAN node Reconfiguration Completion

#### 8.3.2.1	General

The purpose of the S-NG-RAN node Reconfiguration Completion procedure is to provide information to the S-NG-RAN node whether the requested configuration was successfully applied by the UE, or whether the requested modification of the RAN Visible QoE configuration was successfully applied by the UE.

The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: S-NG-RAN node Reconfiguration Complete procedure, successful operation.

The M-NG-RAN node initiates the procedure by sending the S-NODE RECONFIGURATION COMPLETE message to the S-NG-RAN node.

The S-NODE RECONFIGURATION COMPLETE message may contain information that

-	either the UE has successfully applied the configuration requested by the S-NG-RAN node. The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

-	or the configuration requested by the S-NG-RAN node has been rejected. The M-NG-RAN node shall provide information with sufficient precision in the included Cause IE to enable the S-NG-RAN node to know the reason for an unsuccessful reconfiguration. The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

-    or the requested modification of RAN visible QoE configuration was successfully applied by the UE.

Upon reception of the S-NODE RECONFIGURATION COMPLETE message the S-NG-RAN node shall stop the timer TXnDCoverall if TXnDCoverall is running.

-------------------------------------------Next change-------------------------------------------

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node, or to enable an M-NG-RAN node to request S-NG-RAN node to provide a QMC configuration prepared by the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the Management Based MDT PLMN Modification List IE is contained in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, overwrite any previously stored Management Based MDT PLMN List information in the UE context and use the received information to determine subsequent selection of the UE for management based MDT defined in TS 32.422 [23].

If the QMC Initial Coordination Request IE is contained in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node may use it as specified in TS 37.340 [8], and shall, if supported, include the QMC Initial Coordination Response IE in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message.

If the QMC Modification Request IE is contained in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node may use it as specified in TS 37.340 [8], and shall, if supported, include the QMC Modification Response IE in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message.

If the QMC Configuration Information IE is included in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, use it for QoE management, as described in TS 38.300 [9].

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits a modification of the UE context requiring the M-NG-RAN node to report about the success of the RRC connection reconfiguration procedure, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NG-RAN node RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-------------------------------------------Next change-------------------------------------------

### 8.3.4	S-NG-RAN node initiated S-NG-RAN node Modification

#### 8.3.4.1	General

This procedure is used by the S-NG-RAN node to modify the UE context in the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: S-NG-RAN node initiated S-NG-RAN node Modification, successful operation.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the SCG Reconfiguration Notification IE is included in the S-NODE MODIFICATION REQUIRED message the M-NG-RAN node shall, if supported, consider the request is sent to coordinate CHO or MN-initiated CPC with SCG reconfigurations:

- If the SCG Reconfiguration Notification IE is set to "executed", the M-NG-RAN node shall, if supported, consider that a reconfiguration of the SCG resources using SRB3 has been executed. If the S-NG-RAN node to M-NG-RAN node Container IE is also included in the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node shall, if supported, consider that the received SCG configuration has already been applied in the UE and should not be forwarded to the UE.

- If the SCG Reconfiguration Notification IE is set to "executed-deleted", the M-NG-RAN node shall, if supported, consider that a reconfiguration with sync of the SCG resources has been executed and earlier CHO or MN-initiated CPC configuration has been deleted in the UE. If the S-NG-RAN node to M-NG-RAN node Container IE is also included in the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node shall, if supported, consider that the received SCG configuration has already been applied in the UE and should not be forwarded to the UE.

- If the SCG Reconfiguration Notification IE is set to "deleted", the M-NG-RAN node shall, if supported, consider that an earlier CHO or MN-initiated CPC configuration will be deleted in the UE when the SCG configuration provided in the S-NG-RAN node to M-NG-RAN node Container IE is delivered to the UE and executed.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the QMC Initial Coordination Request IE is contained in the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node may use it to determine how to configure the management-based QoE as specified in TS 37.340 [8], and shall, if supported, include the QMC Initial Coordination Response IE in the S-NODE MODIFICATION CONFIRM message.

If the QMC Modification Request IE is contained in the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node may use it as specified in TS 37.340 [8], and shall, if supported, include the QMC Modification Response IE in the S-NODE MODIFICATION CONFIRM message.

-------------------------------------------Next change-------------------------------------------

### 8.3.5	S-NG-RAN node initiated S-NG-RAN node Change

#### 8.3.5.1	General

This procedure is used by the S-NG-RAN node to trigger the change of the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.5.2	Successful Operation

Figure 8.3.5.2-1: S-NG-RAN node initiated S-NG-RAN node Change, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE CHANGE REQUIRED message to the M-NG-RAN node including the Target S-NG-RAN node ID IE. When the S-NG-RAN node sends the S-NODE CHANGE REQUIRED message, it shall start the timer TXnDCoverall.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

If the Multiple Target S-NG-RAN Node List IE is included in the S-NODE CHANGE REQUIRED message, if multiple Target S-NG-RAN nodes are prepared, the M-NG-RAN node may include the Additional List of PDU Session Resource Change Confirm Info – SN Terminated IE in the S-NODE CHANGE CONFIRM message to provide different data forwarding addresses for different Target S-NG-RAN nodes.

If the QMC Configuration Information IE is included in the S-NODE CHANGE REQUIRED message, the NG-RAN node shall, if supported, use it for QoE management, as described in TS 38.300 [9].

-------------------------------------------Next change-------------------------------------------

### 8.3.6	M-NG-RAN node initiated S-NG-RAN node Release

#### 8.3.6.1	General

The M-NG-RAN node initiated S-NG-RAN node Release procedure is triggered by the M-NG-RAN node to initiate the release of the resources for a specific UE or to initiate the release of QMC configuration(s) for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.6.2	Successful Operation

Figure 8.3.6.2-1: M-NG-RAN node initiated S-NG-RAN node Release, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE RELEASE REQUEST message. Upon reception of the S-NODE RELEASE REQUEST message the S-NG-RAN node shall stop providing user data to the UE.

The S-NG-RAN node UE XnAP ID IE shall be included if it has been obtained from the S-NG-RAN node. The M-NG-RAN node shall provide appropriate information within the Cause IE. The M-NG-RAN node may also provide appropriate information per PDU session resource within the Cause IE of the PDU Session Resources To Be Released List IE.

Upon reception of the S-NODE RELEASE REQUEST message containing UE Context Kept Indicator IE set to "True", the S-NG-RAN node shall, if supported, only initiate the release of the resources related to the UE-associated signalling connection between the M-NG-RAN node and the S-NG-RAN node.

If the S-NG-RAN node confirms the request to release S-NG-RAN node resources, it shall send the S-NODE RELEASE REQUEST ACKNOWLEDGE message to the M-NG-RAN node.

If the S-NODE RELEASE REQUEST message contains a PDU session resource to be released which is configured with the SCG bearer option within the PDU Session Resources To Be Released List IE, the S-NG-RAN node shall include the RLC Mode IE within the DRBs To Be Released List IE in the S-NODE RELEASE REQUEST ACKNOWLEDGE message. The RLC Mode IE indicates the RLC mode used in the S-NG-RAN node for the DRB.

If the S-NODE RELEASE REQUEST ACKNOWLEDGE message includes the SCG UE History Information IE, the M-NG-RAN node shall, if supported, use the information to update UE History Information with PSCell history.

If the S-NG-RAN node to M-NG-RAN node Container IE is included in the S-NODE RELEASE REQUEST ACKNOWLEDGE message, the M-NG-RAN node may use the contained information to release the QMC configuration for the UE.

Interaction with the Xn-U Address Indication procedure

If the S-NG-RAN node provides data forwarding related information in the S-NODE RELEASE REQUEST ACKNOWLEDGE message for QoS flows mapped to DRBs configured with an SN terminated bearer option in the PDU Sessions To Be Released List - SN terminated IE, the M-NG-RAN node may decide to provide data forwarding addresses to the S-NG-RAN node and trigger the Xn-U Address Indication procedure as specified in TS 37.340 [8].

If the S-NODE RELEASE REQUEST message concerns a UE for which a Conditional PSCell Change has been triggered, the S-NG-RAN node shall, if supported, consider that the triggered Conditional PSCell Change has been executed, and M-NG-RAN node triggers the Xn-U Address Indication procedure as specified in TS 37.340 [8].

-------------------------------------------Next change-------------------------------------------

### 8.3.7	S-NG-RAN node initiated S-NG-RAN node Release

#### 8.3.7.1	General

This procedure is triggered by the S-NG-RAN node to initiate the release of the resources for a specific UE or to initiate the release of QMC configuration(s) for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.7.2	Successful Operation

Figure 8.3.7.2-1: S-NG-RAN node initiated S-NG-RAN node Release, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE RELEASE REQUIRED message to the M-NG-RAN node.

Upon reception of the S-NODE RELEASE REQUIRED message, the M-NG-RAN node replies with the S-NODE RELEASE CONFIRM message.

For each SN-terminated PDU session resource, the M-NG-RAN node may include the DL Forwarding UP Address IE and the UL Forwarding UP Address IE within the PDU Session Resources To Be Released Item IE to indicate that it requests data forwarding of uplink and downlink packets to be performed for that bearer.

The S-NG-RAN node may start data forwarding and stop providing user data to the UE upon reception of the S-NODE RELEASE CONFIRM message,

If the S-NODE RELEASE REQUIRED message contains an PDU session resource to be released which is configured with the SCG bearer option within the PDU sessions to be released List – SN terminated IE, the S-NG-RAN node shall include the RLC Mode IE within the DRBs To Be Released List IE in the PDU Session to be released List – SN terminated IE in the S-NODE RELEASE REQUIRED message. The RLC Mode IE indicates the RLC mode used in the S-NG-RAN node for the DRB.

If the S-NODE RELEASE CONFIRM message includes the DRB IDs taken into use IE, the S-NG-RAN node shall, if applicable, act as specified in TS 37.340 [8].

If the S-NG-RAN node to M-NG-RAN node Container IE is included in the S-NODE RELEASE REQUIRED message, the M-NG-RAN node may use the contained information to apply delta configuration.

If the S-NODE RELEASE REQUIRED message includes the SCG UE History Information IE, the M-NG-RAN node shall, if supported, use the information to update UE History Information with PSCell history.

If the QMC Deactivation IE is included in the S-NODE RELEASE REQUIRED message, the M-NG-RAN node shall, if supported, use it for QoE management, as described in TS 38.300 [9].

-------------------------------------------Next change-------------------------------------------

# 9	Elements for XnAP Communication

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### 9.1.2	Messages for Dual Connectivity Procedures

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |
| QMC Initial Coordination Request | O |  | 9.2.3.x1 |  | YES | Ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

Editor’s Note: Whether QMC Modification Request IE could be included in SN addition Request message could be further discussed.

#### 9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >Candidate PSCell List |  | 1 |  |  | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| QMC Initial Coordination Response | O |  | 9.2.3.x2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

Editor’s Note: Whether QMC Modification Response IE could be included in SN addition Request Acknowledge message could be further discussed.

-------------------------------------------Next change-------------------------------------------

#### 9.1.2.4	S-NODE RECONFIGURATION COMPLETE

This message is sent by the M-NG-RAN node to the S-NG-RAN node to indicate whether the configuration requested by the S-NG-RAN node was applied by the UE, or whether the requested modification of the RAN Visible QoE configuration for the UE was successful.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Response Information | M |  |  |  | YES | ignore |
| >CHOICE Response Type | M |  |  |  | – |  |
| >>Configuration successfully applied |  |  |  |  | – |  |
| >>>M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in subclause 6.2.2 of TS 38.331 [10] or the RRCConnectionReconfigurationComplete message as defined in subclause 6.2.2 of TS 36.331 [14]. | – |  |
| >>Configuration rejected by the M-NG-RAN node |  |  |  |  | – |  |
| >>>Cause | M |  | 9.2.3.2 |  | – |  |
| >>>M-NG-RAN node to S-NG-RAN node Container | O | OCTET STRING | Includes the CG-ConfigInfo message as defined in as defined in subclause 11.2.2 of TS 38.331 [10]. | – |  |  |
| RAN Visible Configuration Preference Response | O |  | ENUMERATED (true, ...) | Indicates whether the RAN Visible QoE configuration preference of the S-NG-RAN node was applied by the UE. | - | ignore |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.2.3.169 |  | YES | ignore |
| QMC Initial Coordination Request | O |  | 9.2.3.x1 |  | YES | ignore |
| QMC Modification Request | O |  | 9.2.3.x3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

#### 9.1.2.6	S-NODE MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node’s request to modify the S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| Conditional PSCell Addition Information Modification Acknowledge | O |  |  | This IE may be sent from the target SN. | YES | ignore |
| >Candidate PSCell List |  | 1 |  |  | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| QMC Initial Coordination Response | O |  | 9.2.3.x2 |  | YES | ignore |
| QMC Modification Response | O |  | 9.2.3.x4 |  | YES | Ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.1.2.8	S-NODE MODIFICATION REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to request the modification of S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| SCG Reconfiguration Notification | O |  | ENUMERATED (executed, ..., executed-deleted, deleted) |  | YES | ignore |
| QMC Initial Coordination Request | O |  | 9.2.3.x1 |  | YES | ignore |
| QMC Modification Request | O |  | 9.2.3.x3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

#### 9.1.2.9	S-NODE MODIFICATION CONFIRM

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node about the successful modification.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | Ignore |
| QMC Initial Coordination Response | O |  | 9.2.3.x2 |  | YES | ignore |
| QMC Modification Response | O |  | 9.2.3.x4 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| Conditional PSCell Change Information Required | O |  |  |  | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>CPC Indicator | M |  | ENUMERATED (CPC-initiation, CPC-modification, CPC-cancellation, ...) |  | – |  |
| >>>Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >>>Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >>>S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. |  |  |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.1.2.15	S-NODE RELEASE REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to the M-NG-RAN node to confirm the request to release S-NG-RAN node resources.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PDU sessions To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Released List – SN terminated | O |  | PDU Session List with data forwarding request info 9.2.1.24 |  | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| SCG UE History Information | O |  | 9.2.3.151 |  | YES | Ignore |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-Config message as defined in TS 38.331 [10]. | YES | ignore |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.1.2.17	S-NODE RELEASE REQUIRED

This message is sent by the S-NG-RAN node to request the release of all resources for a specific UE at the S-NG-RAN node, or to request the release of QMC Configuration(s) for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PDU sessions To Be Released |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources to be released List – SN terminated | O |  | PDU session List with data forwarding request info 9.2.1.24 |  | – |  |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-Config message as defined in TS 38.331 [10]. | YES | ignore |
| SCG UE History Information | O |  | 9.2.3.151 |  | YES | Ignore |
| QMC Deactivation | O |  | 9.2.3.x6 |  | YES | ignore |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.1.2.20	RRC TRANSFER

This message is sent by the M-NG-RAN-NODE to the S-NG-RAN-NODE to transfer an RRC message or from the S-NG-RAN-NODE to the M-NG-RAN-NODE to report the DL RRC message delivery status.

This message is also sent by the new NG-RAN-NODE to the old NG-RAN-NODE or from the old NG-RAN-NODE to the new NG-RAN-NODE to transfer an RRC message containing the SDT SRB in case of RACH based SDT without UE context relocation.

This message is also sent by the M-NG-RAN-NODE to the S-NG-RAN node or from the S-NG-RAN node to the M-NG-RAN node to forward the RAN visible QoE measurement results received from the UE.

Editor’s Note: The use of this message is to be confirmed.

Direction: M-NG-RAN node  S-NG-RAN node or S-NG-RAN node  M-NG-RAN node (Dual Connectivity).

Direction: new NG-RAN node  old NG-RAN node or old NG-RAN node  new NG-RAN node (SDT).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| SDT SRB between New NG-RAN node and Old NG-RAN node |  | 0..1 |  |  | YES | ignore |
| >RRC Container | M |  | OCTET STRING | Contains a PDCP-C PDU encapsulating an RRC message as defined in subclause 6.2.1 of TS 38.331 [10]. | – |  |
| >SRB ID | M |  | 9.2.3.165 | In this version of the specification, values "0", "1", "3", and "4" are not set by the sender and ignored by the receiver. | – |  |
| RVQoE measurement report |  | 0..1 |  |  | YES | ignore |
| >RRC Container | M |  | OCTET STRING | Contains RVQoE measurement results in the RAN-VisibleMeasurements IE of the MeasurementReportAppLayer message as defined in 38.331 [10]. [FFS] | – |  |

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-----------------------------------------------Next change-----------------------------------------------

### 9.2.3	General IE definitions

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### 9.2.3.x1		QMC Initial Coordination Request

This IE contains the information that the S-NG-RAN node needs to provide to the M-NG-RAN node or the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QMC Configuration List |  |  |  |  |
| >QMC Configuration List Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) | This IE can only be sent by the MN. |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. |
| >>QoE Configuration Sending Request | O |  | BOOLEAN | This IE can only be sent by the SN. |
| >>QoE Reporting Path Request | O |  | ENUMERATED (SRB4, SRB5, ...) | This IE can only be sent by the SN. |
| >>RAN Visible QoE Reporting Path Request | O |  | ENUMERATED (SRB4,…) | This IE can only be sent by the SN. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

#### 9.2.3.x2	QMC Initial Coordination Response

Editor’s Note: The messages used to convey this IE are FFS.

This IE contains the information that the M-NG-RAN node needs to provide in response to the S-NG-RAN node or the information that the S-NG-RAN node needs to provide in response to the M-NG-RAN node, for the QMC initial coordination.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QMC Response List |  |  |  |  |
| >QMC Response List Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) |  |
| >>QoE Configuration Sending Option Response | O |  | ENUMERATED (MN, SN, …). | This IE can only be sent by the MN. |
| >>QoE Reporting Path Response | O |  | ENUMERATED (SRB4, SRB5, …) |  |
| >>RAN Visible QoE Reporting Path Response | O |  | ENUMERATED (SRB4, …) | This IE can only be sent by the MN. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

#### 9.2.3.x3		QMC Modification Request

This IE contains the information that the S-NG-RAN node needs to provide to the M-NG-RAN node or the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, for requesting modification of one or more QoE and/or RAN visible QoE measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QMC Modification Request List |  |  |  |  |
| >QMC Modification Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) | This IE can only be sent by the MN. |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 |  |
| >>QoE Reporting Path Modification Request | O |  | BOOLEAN | This IE indicates the request to switch the SRB used for receiving the QoE reports. |
| >>Further RAN Visible QoE Reporting Request | O |  | BOOLEAN | This IE is to request whether the peer NG-RAN node is interested in further RVQoE reports. |
| >>RAN Visible QoE Reporting Path Modification Request | O |  | BOOLEAN | This IE is to request whether the peer NG-RAN node is interested to switch the SRB used for receiving the RVQoE reports. |
| >>Available RAN Visible QoE Metrics | O |  | 9.2.3.158 |  |
| >>RAN Visible QoE Configuration Preference Request | O |  | 9.2.3.x5 | This IE is to request the RAN Visible QoE Configuration preference of the peer NG-RAN node. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

#### 9.2.3.x4		QMC Modification Response

This IE contains the information that the S-NG-RAN node or the information that the M-NG-RAN node needs to provide in the response to the QMC modification request.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QMC Modification Response List |  |  |  |  |
| >QMC Modification Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>QoE Path Reporting Modification Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>Further RAN Visible QoE Reporting Response | O |  | ENUMERATED (true, false, ...) | This IE is to indicate to the peer NG-RAN node an interest in receiving further RVQoE reports. |
| >>RAN Visible QoE Reporting Path Modification Response | O |  | ENUMERATED (accepted, rejected, …) | This IE is to indicate an interest to switch the SRB used for receiving the RVQoE reports. |
| >>RAN Visible QoE Configuration Preference Response | O |  | 9.2.3.x5 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

#### 9.2.3.x5	RAN Visible QoE Configuration Preference

This IE provides information of RAN visible QoE configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Available RAN visible QoE Metrics | O |  | 9.2.3.158 |  |
| Reporting periodicity | O |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024) |  |
| Event trigger [FFS] | O |  |  |  |

Editor’s Note: The naming and coding of all the new defined IEs in the above tables could be further refined.

#### 9.2.3.x6	QMC Deactivation

This IE indicates the QMC configurations to be deactivated.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoE Reference List |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3-bytes Octet String. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of UE application layer measurement. Value is 16. |

-------------------------------------------End of changes-------------------------------------------