3GPP TSG-RAN WG3 Meeting #121bis-e	R3-235588

Xiamen, China, 09 – 13 October 2023

Title:	[Draft] Reply LS on Non-homogeneous deployment of PDU Set based handling

Response to:	R3-233735/S2-2308252

Release:	Release 18

Work Item:	XRM, NR_XR_enh-Core

Source:	RAN3

To:	SA2, CT4

Cc:	RAN2

Contact Person:

- Name:		Yazid Lyazidi

- E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 thanks SA2 of their LS of non-homogeneous deployments of NG-RAN nodes supporting PDU Set based handling. Based on the discussion, RAN3 has identified two solutions to address the issue:

Option1: SA2 and RAN3 continues introducing explicit support request/indication information in applicable protocols, one per feature. As in the present case, in PDU Session related information exchange between NG-RAN and the SMF and during mobility.

Option2: introduction of a general mechanism to request/indicate support of a newly introduced feature based on IE Id’s, i.e. the identification a new Information Element is associated with.  The SMF can request support information based on NGAP IE-Id's from the NG-RAN node during a given PDU Session Management procedure. For example, during the PDU Session resource Setup procedure, or during the establishment of PDU Session resources in the course of a CN based HO. At Xn handover, the source NG-RAN node “relays” the SMF’s (collected) interest on the NG-RAN node’s capabilities to the target node, which the target NG-RAN node replies to during Path Switch. In a way, the PDU Session management related NG-RAN capabilities will be exchanged on a PDU session ID level over NGAP and XnAP to enable keeping an SMF informed about respective RAN capabilities

Based on the discussion, RAN3 considers that option 2 is more future-proof, as it can also cover the indication from NG-RAN to SMF stating that ECN marking for L4S or ECN marking for L4S assistance information reporting are supported in NG-RAN.

RAN3 kindly asks SA2 to consider the RAN3 agreed solution for capability support information on a per PDU Session ID level to inform SMF about NG-RAN supported capabilities, and to update their specifications as needed.

2. Actions:

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the RAN3 solution into account and to update their specification as needed.

3. Date of Next RAN3 Meetings:

3GPP RAN3#122	                       13 – 17 November 2023	                          Chicago, US