3GPP TSG-RAN WG3 Meeting #121-bis	R3-235607

9th – 13th October 2023

Xiamen, P.R. China

Title: 	General aspects of Data Collection Reporting procedure

Source: 	Ericsson, InterDigital, Orange, Deutsche Telekom, AT&T

Agenda item:	12.2.1

Document Type:	Discussion and Decision

# Introduction

In the RAN3#121 meeting, the following agreements were captured in the meeting minutes:

Modify the name of the agreed class1&class2 message:

>DATA COLLECTION REPORTING INITIATION:

>DATA COLLECTION REQUEST

>DATA COLLECTION RESPONSE

>DATA COLLECTION FAILURE

>DATA COLLECTION REPORTING:

>DATA COLLECTION UPDATE

In the following, we discuss consequences of these agreements and put forward our proposals.

# New procedures names

During the last meeting, RAN3 agreed to change the name of the new procedures for configuring and signaling of data that enables RAN intelligence from “AI/ML Information Reporting (Initiation)” to “Data Collection Reporting (Initiation)”. The new names are more general, and they better reflect the purpose of the procedures, which is that of configuring and enabling reporting of data, where such data may be used for AI/ML, but also for other purposes. Therefore, the data reported using these procedures should not be limited to AI/ML processes and should be available for any purpose. In this sense, the inclusion of “AI/ML” in the procedures’ names was unnecessarily limiting.

Observation 1: the name of the procedures introduced to support AI/ML in NG-RAN has been generalized to “Data Collection Reporting” so that the data conveyed by these procedures could be used by any node or function in the network.

The procedural text for the Data Collection procedures, as it is agreed by RAN3, contains multiple other instances of references to “AI/ML” when it is not necessarily needed. One clear example is the phrase “AI/ML related information”, which in most cases could be replaced by just “information”. There is no clear advantage of using the phrase “AI/ML related information”, which may be interpreted as limiting the scope of the procedure only for use in AI/ML processes. As an example, the Energy Cost IE, configured and exchanged via the Data Collection procedures, could also be used by energy saving functions not adopting AI/ML.

Observation 2:  The inclusion of the abbreviation “AI/ML” in the procedural text may be seen as limiting the scope of the new procedures and the utilization of data conveyed by them.

Proposal 1:  Change the phrase “AI/ML related information” in the procedural text for the Data Collection procedures to “information”.

Similarly, the AI/ML Measurement ID IE, which identifies a Data Collection Reporting context, should be renamed to reflect the new name of the procedures. We propose the simple name of “Data Collection ID” to replace the temporary name given to this IE.

Proposal 2:  Change the name of the AI/ML Measurement ID IE to Data Collection ID IE.

Finally, in the DATA COLLECTION UPDATE message the Cell AI/ML Info Result IE and Cell AI/ML Info Result Item IE should be renamed Cell Info Result List IE and Cell Info Result Item IE, i.e., removing the “AI/ML” from the IE name. This is because the IE identifies per cell results conveyed via the Data collection Update message. There is no advantage in having “AI/ML” in the IE name, instead there is a reference to AI/ML that may be interpreted as unnecessarily restrictive.

Proposal 3: Change the name of the Cell AI/ML Info Result IE and Cell AI/ML Info Result Item IE to Cell Info Result List IE and Cell Info Result Item IE.

# Conclusion

In this contribution, we discussed the consequences of last meeting’s agreements, and we made the following observations and proposals:

Observation 1: the name of the procedures introduced to support AI/ML in NG-RAN has been generalized to “Data Collection Reporting” so that the data conveyed by these procedures could be used by any node or function in the network.

Observation 2:  The inclusion of the abbreviation “AI/ML” in the procedural text may be seen as limiting the scope of the new procedures and the utilization of data conveyed by them.

Proposal 1:  Change the phrase “AI/ML related information” in the procedural text for the Data Collection procedures to “information”.

Proposal 2:  Change the name of the AI/ML Measurement ID IE to Data Collection ID IE.

Proposal 3: Change the name of the Cell AI/ML Info Result IE and Cell AI/ML Info Result Item IE to Cell Info Result List IE and Cell Info Result Item IE.

A TP mirroring the above proposals is found in R3-235608.