3GPP TSG-RAN WG3 Meeting #121-bis	R3-235615

Xiamen, China, 09 – 13 October, 2023

Agenda item:	12.2.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to TS 38.473) Energy Cost Exchange over F1

Document for:	Text Proposal

# 1	Introduction

In RAN3 #121 we made the following agreement:

Work on the measured EC transmission from gNB-DU to gNB-CU over F1 in R18. Whether reusing the current F1AP procedures or defining new procedures needs to be further discussed.

In this contribution, we provide our views on the procedures to exchange Energy Cost over F1AP. We also provide a Text Proposal in the Annex.

# 2	Discussion

In previous meetings we have defined the Energy Cost metric, exchanged between NG-RAN nodes, to be an inferred energy consumption related to an additional load or an actual energy consumption value from a neighbouring node for either an additional load or for a current load. In case of split architecture, the Energy Cost metric can be measured at the gNB-DU that manages the lower layer functions. However, one problem that arises is how to describe Energy Consumption of a gNB at a gNB-DU level. This is because the Energy Consumption of a gNB KPI defined in TS 28.554 is at gNB granularity and is summed over all the entities that comprise the gNB.

Observation 1: The Energy Consumption of a gNB KPI in TS 28.554 is at a gNB granularity and is summed over all the entities that comprise the gNB.

To support Energy Cost at a gNB-DU granularity we need to define a new metric to measure Energy Consumption at a gNB-DU granularity. This can be taken into account by SA5 when a unified rule is defined by the group.

Additionally, we need to introduce the necessary signalling to allow a gNB-CU to retrieve an Energy Cost value from a gNB-DU. This can be done either by introducing the request over a) existing procedures or b) by defining new procedures. In our view, there aren’t currently any F1AP procedures that are suitable to carry Energy Cost information. The Resource Status Procedure defined over F1 interface is used to exchange resource information, but Energy Cost is not related to radio resources. Furthermore, we have already defined a Class 1 and a Class 2 procedure, namely Data Collection Reporting Initiation and Data Collection Reporting respectively, over Xn interface which are used to allow an NG-RAN node 1 to request Energy Cost from NG-RAN node 2 and NG-RAN node 2 to report the corresponding Energy Cost value to NG-RAN node 1. The same procedure can be defined over F1 interface to allow a gNB-CU to request Energy Cost information from a gNB-DU and subsequently the gNB-DU report the Energy Cost value to gNB-CU. This will help keep F1AP aligned with XnAP, in a similar way as done for other features.

Proposal 1: Introduce a Class 1 procedure Data Collection Reporting Initiation over F1 interface to enable a gNB-CU request Energy Cost from gNB-DU.

Proposal 2: Introduce a Class 2 procedure Data Collection Reporting over F1 interface to enable a gNB-DU report its measured Energy Cost to gNB-CU.

This procedure could be used in the future for other information related to AI/ML information. Since currently the only measurement supported in this procedure is reporting of Energy Cost information, we think that partial reporting is not at this stage useful to this procedure.

In the Annex of this paper we provide a TP to BL CR for TS 38.473.

Proposal 3: Discuss and agree the TP to BL CR to TS 38.473 provided in the Annex.

# 3	Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: The Energy Consumption of a gNB KPI in TS 28.554 is at a gNB granularity and is summed over all the entities that comprise the gNB.

Proposal 1: Introduce a Class 1 procedure Data Collection Reporting Initiation over F1 interface to enable a gNB-CU request Energy Cost from gNB-DU.

Proposal 2: Introduce a Class 2 procedure Data Collection Reporting over F1 interface to enable a gNB-DU report its measured Energy Cost to gNB-CU.

Proposal 3: Discuss and agree the TP to BL CR to TS 38.473 provided in the Annex.

# Annex 	- TP for TS 38.473

<<< start of changes >>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

AI	Artificial Intelligence

AMF	Access and Mobility Management Function

ARP	Antenna Reference Point

ARPI	Additional RRM Policy Index

BH	Backhaul

CAG	Closed Access Group

CN	Core Network

CG	Cell Group

CG-SDT	Configured Grant-Small Data Transmission

CGI	Cell Global Identifier

CHO	Conditional Handover

CP	Control Plane

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

DL-PRS	Downlink Positioning Reference Signal

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

FSA ID	MBS Frequency Selection Area (FSA) ID

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

MBS	Multicast/Broadcast Service

ML	Machine Learning

NID	Network Identifier

NPN	Non-Public Network

NSAG	Network Slice AS Group

NSSAI	Network Slice Selection Assistance Information

PDC	Propagation Delay Compensation

PEIPS	Paging Early Indication with Paging Subgrouping

posSIB	Positioning SIB

PNI-NPN	Public Network Integrated NPN

PTP	Point to Point

PTM	Point to Multipoint

QMC	QoE Measurement Collection

QoE	Quality of Experience

RANAC	RAN Area Code

RedCap	Reduced Capability

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RRC	Radio Resource Control

RSRP	Reference Signal Received Power

SDT	Small Data Transmission

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

TAC	Tracking Area Code

TAI	Tracking Area Identity

TEG	Timing Error Group

TRP	Transmission-Reception Point

U2N	UE-to-Network

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

Z-AoA	Zenith Angles of Arrival

<<next change>>

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

<< skip unchanged procedural text >>>

| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| --- | --- | --- | --- |
| Positioning Measurement | POSITIONING MEASUREMENT REQUEST | POSITIONING MEASUREMENT RESPONSE | POSITIONING MEASUREMENT FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| Broadcast Context Setup | BROADCAST CONTEXT SETUP REQUEST | BROADCAST CONTEXT SETUP RESPONSE | BROADCAST CONTEXT SETUP FAILURE |
| Broadcast Context Release | BROADCAST CONTEXT RELEASE COMMAND | BROADCAST CONTEXT RELEASE COMPLETE |  |
| Broadcast Context Modification | BROADCAST CONTEXT MODIFICATION REQUEST | BROADCAST CONTEXT MODIFICATION RESPONSE | BROADCAST CONTEXT MODIFICATION FAILURE |
| Multicast Context Setup | MULTICAST CONTEXT SETUP REQUEST | MULTICAST CONTEXT SETUP RESPONSE | MULTICAST CONTEXT SETUP FAILURE |
| Multicast Context Release | MULTICAST CONTEXT RELEASE COMMAND | MULTICAST CONTEXT RELEASE COMPLETE |  |
| Multicast Context Modification | MULTICAST CONTEXT MODIFICATION REQUEST | MULTICAST CONTEXT MODIFICATION RESPONSE | MULTICAST CONTEXT MODIFICATION FAILURE |
| Multicast Distribution Setup | MULTICAST DISTRIBUTION SETUP REQUEST | MULTICAST DISTRIBUTION SETUP RESPONSE | MULTICAST DISTRIBUTION SETUP FAILURE |
| Multicast Distribution Release | MULTICAST DISTRIBUTION RELEASE COMMAND | MULTICAST DISTRIBUTION RELEASE COMPLETE |  |
| PDC Measurement Initiation | PDC MEASUREMENT INITIATION REQUEST | PDC MEASUREMENT INITIATION RESPONSE | PDC MEASUREMENT INITIATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |
| Data Collection Reporting Initiation | DATA COLLECTION REQUEST | DATA COLLECTION RESPONSE | DATA COLLECTION FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Context Release Request | BROADCAST CONTEXT RELEASE REQUEST |
| Multicast Context Release Request | MULTICAST CONTEXT RELEASE REQUEST |
| PDC Measurement Report | PDC MEASUREMENT REPORT |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| PDC Measurement Failure Indication | PDC MEASUREMENT FAILURE INDICATION |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| Measurement Activation | MEASUREMENT ACTIVATION |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |
| Data Collection Reporting | DATA COLLECTION UPDATE |

<<next change>>

### 8.2.xx	Data Collection Reporting Initiation

#### 8.2.xx.1	General

This procedure is used by a gNB-CU to request the reporting of measurements, that may be used for AI/ML purposes, to gNB-DU.

The procedure uses non UE-associated signalling.

#### 8.2.xx.2	Successful Operation

Figure 8.2.xx.2-1: Data Collection Reporting Initiation, successful operation

gNB-CU initiates the procedure by sending the DATA COLLECTION REQUEST message to gNB-DU to start a measurement and stop a measurement. Upon receipt, gNB-DU:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all measurements and terminate the reporting in case the Registration Request IE is set to "stop";

If gNB-DU is capable to provide all requested information, it shall initiate the measurements as requested by gNB-CU, and respond with the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity IE in the DATA COLLECTION REQUEST is present, this indicates the periodicity for the reporting of periodic measurements. The gNB-DU shall report once, unless otherwise requested within the Reporting Periodicity IE.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the DATA COLLECTION REQUEST indicates the type of objects gNB-DU shall perform measurements on. The gNB-DU shall include in the DATA COLLECTION UPDATE message:

-	the Energy Cost IE, if the first bit, “Energy Cost” of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to “1”.

#### 8.2.xx.3	Unsuccessful Operation

Figure 8.2.10.3-1: Data Collection Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, gNB-DU shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.2.xx.4	Abnormal Conditions

If the initiating gNB-CU does not receive either DATA COLLECTION RESPONSE message or DATA COLLECTION FAILURE message, the gNB-CU may reinitiate the Data Collection Reporting Initiation procedure towards the same gNB-DU, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message with the same Transaction ID.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the DATA COLLECTION REQUEST message then gNB-DU shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the gNB-DU receives a DATA COLLECTION REQUEST message which includes the Registration Request IE set to "start" and the gNB-CU Measurement ID IE corresponding to an existing on-going measurement reporting, for which a different Transaction ID is used, then gNB-DU shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the gNB-DU receives a DATA COLLECTION REQUEST message which includes the Registration Request IE set to “stop” and if the gNB-CU Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the gNB-DU shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

### 8.2.yy	Data Collection Reporting

#### 8.2.yy.1	General

This procedure is initiated by gNB-DU to report the result of measurements that may be used for AI/ML purposes admitted by gNB-DU following a successful Data Collection Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.2.yy.2	Successful Operation

Figure 8.2.11.2-1: Data Collection Reporting, successful operation

The gNB-DU shall report the results of the admitted measurements in DATA COLLECTION UPDATE message. The admitted measurements, that may be used for AI/ML purposes, are the measurements that were successfully initiated during the preceding Data Collection Reporting Initiation procedure.

If some results of the admitted measurements in DATA COLLECTION UPDATE message are missing, the gNB-CU shall consider that these results were not available at the gNB-DU.

#### 8.2.yy.3	Unsuccessful Operation

Not applicable.

#### 8.2.yy.4	Abnormal Conditions

Void.

<<next change>>

#### 9.2.1.aa	DATA COLLECTION REQUEST

This message is sent by gNB-CU to gNB-DU to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) | Type of request for which the resource status is required. | YES | ignore |
| Report Characteristics | C-ifRegistrationRequestStart |  | BIT STRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-DU is requested to report. First Bit = Energy Cost Other bits shall be ignored by the gNB-DU. | YES | ignore |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all measurement object if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

#### 9.2.1.bb	DATA COLLECTION RESPONSE

This message is sent by gNB-DU to gNB-CU to indicate that the requested measurement is successfully initiated.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.cc	DATA COLLECTION FAILURE

This message is sent by gNB-DU to gNB-CU to indicate that for any of the requested measurement objects the measurement cannot be initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.dd	DATA COLLECTION UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Energy Cost | O |  | INTEGER (0..10000,...) | Measured Energy Consumption index at gNB-DU. Value 0 indicates the minimum normalized Energy Consumption and 10000 indicates the maximum normalized Energy Consumption. Energy Consumption is measured on a linear scale. | YES | ignore |

<<End of changes>>