# 3GPP TSG-RAN WG3 Meeting #121bis	R3-235627

# Xiamen, China, Oct.9th - Oct.13th, 2023

# Agenda item:		23.2.2

# Source:	ZTE

# Title:	(TP to 38.455 )Discussion on LPHAP impacts

# Document for:		Discussion and Approval

Introduction

During the last meeting, following agreement has been reached.

WA: LMF provides the validity area as a list of cells, and SRS recommendation to the serving gNB. The serving gNB replies with a single SRS configuration (as in legacy positioning information exchange procedure). The details of the SRS recommendation are FFS e.g. SRS configuration or requested SRS transmission characteristics with additional information. FFS on whether the serving gNB includes a list of cells identifying a modification of the validity area in the response message to the LMF.

In this contribution, we provide our further understanding on LPHAP configuration, and corresponding TP to 38.455 and 38.473.

Discussion

Here, the WA reached in the last meeting shows that LMF provide the validity area as a list of cells and recommended SRS configuration to the serving gNB, and the serving replies a single SRS configuration. One issue here is the details of SRS recommendation. From our perspective, LMF can provide the candidate multiple SRS configurations collected from different NG-RAN nodes with cell or a validity area to the serving node. For now, we can assume that LMF has collected available SRS configuration from all NG-RAN the gNBs.

Proposal 1: LMF provide a recommended SRS configuration list and validity area via POSITIONING INFORMATION REQUEST message.

Another issue is whether the serving gNB includes a list of cells identifying a modification of the validity area in the response message to the LMF. Regarding this matter, there is discussion on the relationship between RNA and validity area before.

As definition in TS38.300, A UE in the RRC_INACTIVE state is required to initiate RNA update procedure when it moves out of the configured RNA. Based on this principle, if the validity area is larger than the RNA, and the UE moves out of the RNA and connects to a different cell, an RNA update request is necessary. This RNA update process needs to employ a redirection method, wherein the new cell obtains the complete context of the UE, while the old cell releases all UE configurations. In this situation, even if the UE is still within the SRS validity area, the new cell must reconfigure the UE's SRS configuration and validity area. This contradicts the requirements of LPHAP.

Proposal 2: The validity area shall not be larger than the RNA. If the validity area is larger than the RNA, it contradicts the requirements of LPHAP.

Moreover, RNA configuration is known by the gNB, so the validity area should be determined by the gNB, and the configure it to the UE.

Proposal 3: The validity area is finally determined by the gNB, and then serving gNB can notify LMF with a modification validity area (a list of cells) in the POSITIONING INFOMRATION RESPONSE message.

In addition to the modification of validity area, the gNB also need to include the SRS configuration for LPHAP associated to the specific validity area via the message to LMF.

- Proposal 4: Introduce the SRS configuration for LPHAP associated to the specific validity area in POSITIONING INFORMATION RESPONSE message.

Conclusion

We propose the following observations and proposals:

Proposal 1: LMF provide a recommended SRS configuration list and validity area via POSITIONING INFORMATION REQUEST message.

Proposal 2: The validity area shall not be larger than the RNA. If the validity area is larger than the RNA, it contradicts the requirements of LPHAP.

- Proposal 3: The validity area is finally determined by the gNB, and then serving gNB can notify LMF with a modification validity area (a list of cells) in the POSITIONING INFOMRATION RESPONSE message.

- Proposal 4: Introduce the SRS configuration for LPHAP associated to the specific validity area in POSITIONING INFORMATION RESPONSE message.

- Annex-A TP to 38.455

## Start of modifications

### 8.2.6	Positioning Information Exchange

#### 8.2.6.1	General

The Positioning Information Exchange procedure is initiated by the LMF to request to the NG-RAN node positioning information for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: Positioning Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the NG-RAN node.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message.

If the Spatial Relation Information per SRS Resource IE and the Periodicity List IE are both included in the Requested SRS Transmission Characteristics IE, the NG-RAN node shall consider that the Spatial Relation per SRS Resource Item IE and the Periodicity List Item IE have one-to-one mapping relation.

If the UE Reporting Information IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account for allocating proper CG-SDT resources when positioning a UE.

If the UE TEG Information Request IE is included in the POSITIONING INFORMATION REQUEST message and set to "onDemand", the NG-RAN node shall, if supported, provide the UE Tx TEG association in the POSITIONING INFORMATION RESPONSE message.

If the UE TEG Information Request IE is set to "periodic", the NG-RAN node shall, if supported, reply with the POSITIONING INFORMATION RESPONSE message without including any UE Tx TEG association in this message. The NG-RAN node shall then take the UE TEG Reporting Periodicity IE into account when configuring the UE’s periodic UE Tx TEG association reporting and initiate the Positioning Information Update procedure for reporting the UE Tx TEG association received from the UE, if any.

If the Validity Area Assistance Information IE included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take it into account when configuring the SRS transmission for the UE for LPHAP.

next modification

9.1.1.10	POSITIONING INFORMATION REQUEST

This message is sent by the LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |
| UE Reporting Information | O |  | 9.2.70 |  | YES | ignore |
| UE TEG Information Request | O |  | ENUMERATED(onDemand, periodic, stop, …) |  | YES | ignore |
| UE TEG Reporting Periodicity | C-ifUeTegInfoReqPeriodic |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, …) |  | YES | reject |
| Validity Area Assistance Information | O |  | 9.2.pp |  |  |  |

| Condition | Explanation |
| --- | --- |
| ifUeTegInfoReqPeriodic | This IE shall be present if the UE TEG Information Request IE is set to the value "periodic". |

9.1.1.11	POSITIONING INFORMATION RESPONSE

This message is sent by the NG-RAN node to provide positioning information.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.2.36 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| UE Tx TEG Association List | O |  | 9.2.78 |  | YES | ignore |
| Validity Area | O |  | 9.2.oo |  |  |  |

### 9.2.pp  Validity Area Assistance Information

This IE is used to indicate the validity area and the recommended SRS configurations in the area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRS Configuration List |  | 1 |  |  |
| > SRS Configuration Item |  | 1 .. <maxnoSRSConfig> |  |  |
| >>SRS Configuration |  |  | 9.2.28 |  |
| Validity Area | M |  | 9.2.oo |  |

### 9.2.pp  Validity Area

This IE is used to indicate the validity area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Validity Area Cell List |  | 1 |  |  |
| > Validity Area Cell Item |  | 1 .. <maxnoPACell> |  |  |
| >>NR CGI | M |  | 9.2.6 |  |

## End of modifications

- Annex-B TP to 38.473