3GPP TSG-RAN WG3 Meeting #121-bis	R3-235652

Xiamen, China,9th – 13th October 2023

Agenda Item:	11.3

Source:	ZTE, China Unicom, China Telecom

Title:	(TP to BL CR of 37.340) Procedure flows on the initial QMC configuration in NR-DC

Document for:	Discussions & Approval

- Introduction

This paper provides the text proposals to the BL CR of 37.340.

- TP to BL CR of 37.340

-------------------------------------------Start of changes-------------------------------------------

13.x	Application Layer Measurement Collection

13.x.1	Overview

The QoE Measurement Collection function as described in TS 38.300 [3] is extended to address NR-DC operation. The requirements on the gNB provided in TS 38.300 [3] apply to the MN, together with additional requirements on MN and SN provided in following sub-clauses.

13.x.2	QoE Measurement Configuration

13.x.2.1	QoE Measurement Collection Activation and Reporting in NR-DC

For a UE in NR-DC, the MN and the SN may coordinate QoE measurement collection activation and reporting as follows:

For management-based QoE activation, the MN:

- Allocates the measurement configuration application layer ID, and indicates to SN if needed;

- Determines whether the MN or the SN sends the QoE configuration to the UE, in case SN enquires MN.

For management-based QoE activation received directly by the SN from the OAM, the SN may perform UE selection. For a selected UE, the SN indicates to the MN the QoE reference of the management-based QoE session and whether it is going to receive the corresponding reports via the MN (using SRB4) or using SRB5. The SN can send a RAN visible QoE configuration directly to the UE via SRB3, or in a transparent container to the MN, which then sends it to the UE via SRB1.

The network explicitly indicates to the UE whether to send QoE reports via SRB4 or SRB5, per QoE reference. The SRB for QoE reporting can be changed during the application session. The command for changing the SRB used for reporting may be sent to the UE by the node that configured that specific QoE configuration. The node that currently receives the QoE reports via the Uu can request from the peer node that the QoE reporting leg is switched to the peer node. The leg switch for QoE reporting needs to be approved by both nodes serving the UE. RAN visible QoE reports can be sent over the same leg, as the QoE reports pertaining to the same QoE reference, or over the other leg.

The MN should inform the SN that a UE is configured with a management-based QoE/RAN visible QoE measurement configuration.

If the MN has configured the UE with QoE measurements, and if the UE is configured to send the QoE reports to the SN, then, if the MN decides that the SN forwards the reports directly to the MCE, the MN should indicate to the SN the QoE reference, the MCE IP address and the measConfigAppLayerId.

If the SN has configured the UE with QoE measurements, and if the UE is configured to send the QoE reports to the MN, then, if the SN decides that the MN forwards the reports directly to the MCE, the SN should indicate to the MN the QoE reference and the MCE IP address.

RAN visible QoE reports can be sent to the SN directly via the SRB5, or via the MN using SRB4.

Editor’s note: The text above may need to be refined, to prevent duplication with the following procedure description.

MN-initiated management-based QMC configuration

Figure 13.x.1-1 SN initiated management-based QMC configuration procedure

1. The MN receives the management-based QMC configuration from the OAM.

2a (optional). The MN performs UE selection. The MN may send the SN Addition/Modification Request message to the SN, with the QMC Initial Coordination Request IE included, to request the QoE reporting via SRB5. [FFS]

3a (optional). If the SN confirms the QoE reporting via SRB5, the SN may send the SN Addition/Modification Response message, with the QMC Initial Coordination Response IE included. [FFS]

4. The MN sends the management-based QMC configuration to the UE via RRCReconfiguration message.

5a-6a (optional). The MN should inform the SN that a UE is configured with a management-based QoE/RAN visible QoE measurement configuration via SN Modification Request message, with the QMC Initial Coordination Request IE included. If the UE is configured to send the QoE reports to the SN, then, if the MN decides that the SN forwards the reports directly to the MCE, the MN should include the QoE reference, the MCE IP address and the measConfigAppLayerId in the QMC Initial Coordination Request IE.

SN-initiated management-based QMC configuration:

Figure 13.x.1-2 SN initiated management-based QMC configuration procedure

- The SN receives the management-based QMC configuration from the OAM.

- The SN may perform UE selection. After UE selection, the SN should send the SN Modification Required message, with the QMC Initial Coordination Request IE, to indicate about the management-based QMC configuration received by SN. The SN may include the QoE configuration Sending Request, QoE Reporting Option Preference, and/or RVQoE Reporting Option Request in the QMC Initial Coordination Request IE.

- The MN sends the response to the SN via SM Modification Confirm message, with the QMC Initial Coordination Response IE included. The response of MN includes the QoE Configuration Sending Option Response (about which node to send the QoE configuration to the UE), QoE Reporting Option, and/or RVQoE Reporting Option Response.

According to the response of MN about which node shoud send the QoE configuration to the UE, the following steps are divided based on two cases.

Case a: The MN indicates that the MN should send the QoE configuration to UE in step 3.

4a-5a. The SN may send the SN Modification Required message to the MN, with the QMC Configuration Information IE included, to pass the QMC configuration that might needed by the MN to configure QMC to UE.

6a:  The MN sends the QoE configuration to the UE via an RRC message.

Case b: The MN indicates that the SN should send the QoE configuration to UE in step 3.

4b. The SN sends the QoE configuration to the UE via an RRC message.

Note: In this case, the corresponding measConfigAppLayerId of the QoE Reference should be allocated by the MN and notified to the SN in step 3.

Editor’s note: The procedure flow and detailed description of each step may need to be further discussed during the meeting.

13.x.3	QoE Measurement Continuity for Mobility

Editor’s note: Content for this clause is FFS. QMC continuity during mobility in NR-DC should be discussed after the baseline solution for QMC in NR-DC is in place.

If the MN configured the UE with QoE measurements, every subsequent MN serving the UE can configure and release the RAN visible QoE measurements.

At SN change without MN change, the QoE configuration information is passed from the old/source SN to the new/target SN via the MN.

If the SN configured a UE with QoE measurements, at SN release, the QoE/RAN visible QoE measurement configuration can be released.