3GPP TSG-RAN WG3 Meeting #121bis-e	R3-235722

Xiamen, China, 09 – 13 October 2023

Agenda Item:	21.2

Source:	Moderator (Ericsson)

Title:	CB: # XR_PDUSethandling

Document for:	Other

# Introduction

This is the summary document for the following come back:

CB: # XR_PDUSethandling

- Discuss the open issues listed above and capture the agreements if any

(moderator - E///)

# For the Chairman’s Notes

- UP design

Option1: define PSI and EoDB into the existing Frames (e.g frame with PDU Type =0) of NG-U/F1-U

Option2: define PSI and EoDB into a new Frame (PDU type) in the existing container of NG-U/F1-U

Option3: Define a new GTP-U extension PDU Set container for PSI and EoDB

P1) Rule out option 2. Postpone to next meeting the decision between option 1 and option 3

P2) if a gNB supports PDU Set handling, all parts support it: DU, CU-CP, CU-UP.

- PDU Set Handling indication

P3) A new support indication should be included in the SMF-related IEs, i.e.

- PDU Session Management case: PDU Session Resource Setup Response Transfer, PDU Session Resource Modify Response Transfer,

- Handover case: Path Switch Request Transfer, and Handover Request Acknowledge Transfer IEs.

FFS whether to use an explicit PDU Set handling Support indicator, or indicate a list of supported NGAP IE IDs from NG-RAN upon request from SMF

Postpone TPs to next meeting

- Others

The encoding of the N6 Jitter Information needs further checking from RAN2 progress

FFS how gNB-CU-CP handles the received XR UE assistance information (e.g., UL jitter information, BAT). Whether signalling to gNB-DU is needed.

- UL PDU Set QoS Parameters

It is considered beneficial to introduce separate DL and UL PDU Set QoS parameters in the QoS flow QoS Parameters.

Discuss whether LS to SA2 can be sent, take R3-235874 as input

Discuss if TPs adding separate PDU Set QoS Parameters for UL and DL to F1/NG/Xn/E1 BL CRs can be agreed with FFS

- Stage 2 TP:

Stage 2 progress: Agree TP R3-235875 (TP for NR_XR_enh BL CR for TS 38.300) Support for XR Enhancements (QC et al.)

# Discussion Second round

On the PDU Set handling support indication, it is proposed to capture the agreement from first round of discussion as follows:

- P3) A new support indication should be included in the SMF-related IEs, i.e.

  - PDU Session Management case: PDU Session Resource Setup Response Transfer, PDU Session Resource Modify Response Transfer,

  - Handover case: Path Switch Request Transfer, and Handover Request Acknowledge Transfer IEs.

- FFS whether to use an explicit PDU Set handling Support indicator, or indicate a list of supported NGAP IE IDs from NG-RAN upon request from SMF

Since the two options have differences on encoding (single IE, vs list of IEs based on request from SMF), it is proposed to postpone TPs to next meeting

During first round, companies have supported addition of separate PDU Set QoS parameters for DL and UL traffic. It is proposed to introduce such change to XR BL CRs:

For NG-AP, the change can be as follows:

#### 9.3.1.12 QoS Flow Level QoS Parameters This IE defines the QoS parameters to be applied to a QoS flow. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. CHOICE QoS Characteristics; M; -. >Non-dynamic 5QI. >>Non Dynamic 5QI Descriptor; M; 9.3.1.28; -. >Dynamic 5QI. >>Dynamic 5QI Descriptor; M; 9.3.1.18; -. Allocation and Retention Priority; M; 9.3.1.19; -. GBR QoS Flow Information; O; 9.3.1.10; This IE shall be present for GBR QoS flows and is ignored otherwise.; -. Reflective QoS Attribute; O; ENUMERATED (subject to, …); Details in TS 23.501 [9]. This IE may be present in case of Non-GBR QoS flows and is ignored otherwise.; -. Additional QoS Flow Information; O; ENUMERATED (more likely, …); This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU session. This IE may be present in case of Non-GBR QoS flows and is ignored otherwise.; -. QoS Monitoring Request; O; ENUMERATED (UL, DL, Both, …, stop); Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow or stop the corresponding QoS monitoring.; YES; ignore. QoS Monitoring Reporting Frequency; O; INTEGER (1.. 1800, …); Indicates the reporting frequency for RAN part delay for QoS monitoring. Units: second; YES; ignore. PDU Set QoS Parameters Downlink; O; PDU Set QoS Parameters 9.3.1.x; YES; ignore. PDU Set QoS Parameters Uplink; O; PDU Set QoS Parameters 9.3.1.x; YES; ignore.

Q1) Do companies agree with the proposed change? Do other BL CRs (F1/Xn/E1) need to reflect the change as well in the PDU Set QoS parameters IE?

| Company | Proposed NGAP change (Y/N/others) | Does change need to be reflected in other BL CRs: Xn/F1/E1? |
| --- | --- | --- |
| Ericsson | Yes | Yes |
| ZTE | No | Downlink and uplink PSDB may be different so it may be beneficial to have separate IEs for PSDB, while for the other two parameters, we want to keep it open since SA2 is discussing this issue at this meeting. We can have the agreement that separate IEs are needed for PSDB at this meeting, and FFS on PSIHI and PSER. For the TPs, we can capture this structure but with Editor note indicating that this structure is FFS. |
| LGE | No | In SA2, whether PSDB, PSER and PSIHI separately for uplink and downlink is signaled or not was discussed but was postponed to the next meeting. So, the proposed NGAP change is unnecessary and other BL CRs does not need to reflect this proposed change. |

A company has proposed to send a LS to SA2 to take into account RAN3 agreement. A text for draft LS is provided below:

RAN3 discussed about XR PDU Set QoS Parameters provided by 5GC to NG-RAN during the NG-AP PDU Set Resource Setup/Modification procedures . From NG-RAN perspective, it is beneficial for NG-RAN to receive separate DL and UL PDU Set QoS parameters from 5GC (ex: separate DL/UL PDU Set QoS Parameters) . This will assist NG-RAN to efficiently allocate radio resource based on different DL and UL PDU Set QoS Requirements. RAN3 kindly requests SA2 to take above information into account and enable 5GC to provide separate DL and UL PDU Set QoS Parameters to NG-RAN.

Q2) Do companies agree to send the LS to SA2? Any comments on the LS text?

| Company | LS to SA2 (Y/N/others) | comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| ZTE | No | SA2 is discussing this issue. We can just wait for their progress. We are not able to receive the reply LS in time which may impact the close of this work item. |
| LGE | No | Because whether PSDB, PSER and PSIHI separately for uplink and downlink is signaled or not was postponed in this SA2 meeting, sending this LS to SA2 is inappropriate. |

# Discussion first round

## UP design

Enhance existing container or introduce new container to support the PDU Set Parameters on user plane? Identify whether there is any issue to reusing the existing container, if not, as majority companies would prefer to go for the existing container?

| Container solution | Specification impacts | Drawbacks | Supports |
| --- | --- | --- | --- |
| 1) Introduce the PDU Set Information and Indication of End of Data Burst into the existing Frames (e.g frame with PDU Type =0) in TS 38.415/425 | Less specification impact | If we follow the requirement that a non-supporting node needs to be able to receive PS Info &EoDB, it is not clear how such node would not regard the received frame as “erroneous” and still continue with the legacy part, as no “extension” mechanism is defined. | Huawei, ZTE: support |
| 2) Introduce the PDU Set Information and Indication of End of Data Burst into a new Frame (PDU type) in the existing container in TS 38.415/425 | Average specification impact | Not clear how to work in non-homogenous case. not feasible to include two frames in one GTP-U extension header Has CT4 impacts | ZTE |
| 3) Define a new GTP-U extension PDU Set container | Some specification impacts | Has impact on CT4 Nokia: was proposed in CT4 and rejected (?) New fields were already added in NG-U in past release E///: issues will be existing. Nodes do not have the same version. Nokia: new container may be defined for any new feature in future releases ZTE: add new XR flag in existing container. E///: not feasible, cannot use two frames in the same container. SS: in GTP-U header you can include two containers but not two frames. For XR two containers can be needed. Nokia: why send the useless info to target if target does not support E///: better handling in GTP-U Nokia: contradiction, nodes need to know in advance what the nodes support via CP E/// to NEC: ignore only the PDU Set info not the container NEC: new requirement QC: what is the conversion mechanism of non-supporting to supporting node of PDU container QC: how to link the headers, how target handles old and new one? E///: link them to avoid swapping and extraction exercise. Avoid delay issue by indicating that such flag belongs to PDU Set container Nokia: can be harmful for data forwarding, may not work well for XR E///: latency wise, solution 3 is best and avoids delay induced by swapping in sol ½ Nok: more work of swapping is needed in options 1/2. Issue exists in existing Hw: capture way forward on target NG-RAN node data forwarding Hw: no information on the size of the length SS: worry about complexity on impact using existing frame. Down-select option 2, next meeting decide between option 1 and 3 | E///, Len, SS E///: going from a supporting node to non-supporting node, the flag indication does not work. Other options will have unspecified errors. GTP-U has mechanism to indicate if container is supported or not, let’s re-use it. SS: spare bit can be used to indicate if PDU Set information is present or not. For future proofness, new header may need to be added to not re-use the spare bit. Xiaomi: open to options, no specification description on handling of unknown IEs. Huawei: supports Xiaomi, capture wording how NG-RAN can handle data packets if fields are ignored. ZTE: spare bit is not issue, more flag bits can be added? E///: the solution from Xiaomi/Hw is good thinking but NBC. Rel-19 XR is coming and is specific for UP handling. |

Proposed way forward:

- Rule out option 2. Postpone to next meeting the decision between option 1 and option 3

- if a gNB supports PDU Set handling, all parts support it: DU, CU-CP, CU-UP.

## PDU Set handling indication

| Solution | Advantages | Drawbacks | Supports |
| --- | --- | --- | --- |
| 1) “MBS-like” solution | Less specification impact | if UE handovers to legacy node, the NG-RAN node will not report anything (common issue). Prolongs the approach that for each feature a new feature indicator needs to be introduced. | TBD |
| 2) “RACS-like” solution | More future-proof (is able to cover all possible future feature support indicators) | if UE handovers to legacy node, the NG-RAN node will not report anything (common issue, but as soon as all nodes introduce this aspect, any feature can be covered by it, even pre-Rel-18 features.) Nok: the source may be a non-upgraded gNB E///: introduce the feature from Rel-18 | TBD |

Proposed way forward:

- Introduce an explicit indication to indicate the support of the NG-RAN XR PDU Set handling capability. FFS if Option 1 (MBS-like) or Option 2 (NGAP ID IE like)

- In case of PDU Session Management procedures, use option 2. For handover use option 1?

- discuss whether to extend this mechanism to “AMF features”.

Companies are invited to provide their views on the above proposed way-forward and whether TPs to NGAP and XnAP BL CRs can be produced this meeting on P1:

| Company | P1 | P2 | P3 |
| --- | --- | --- | --- |
| Ericsson | Yes, fine to have TPs to NG and Xn BL CRs based on P1 with name and encoding of the IE FFS | FFS | FFS |
| Nokia | Yes. agree with Ericsson. We can move a little bit further based on companies’ contributions. Proposal to include the new indication in SMF-related IEs, i.e. PDU Session Resource Setup Response Transfer, PDU Session Resource Modify Response Transfer, Path Switch Request Transfer, and Handover Request Acknowledge Transfer IEs. | We prefer same solution for all. | Not needed. |
| Qualcomm | Yes. Same comment as Nokia -> Proposal to include the new indication in SMF-related IEs, i.e. PDU Session Resource Setup Response Transfer, PDU Session Resource Modify Response Transfer, Path Switch Request Transfer, and Handover Request Acknowledge Transfer IEs. | Lets not use different methods for different interfaces. | We can discuss later if needed. |
| Xiaomi | Agree with Nokia and QC | FFS | FFS |
| LGE | We share the view of Ericsson. | FFS | We can discuss it later if needed. |
| Huawei | Agree with Nokia and QC | Prefer one solution for all cases | No need to discuss this. |
| Samsung | Same view with Ericsson and Nokia. | FFS | Not needed for now |
| China Telecom | Agree with Nokia and Qc | Prefer one solution for all cases | FFS |
| ZTE | Agree. This would help reduce the work load of next meeting. | FFS | FFS |

## UL PDU Set QoS Parameters

P1: Add separate PDU Set QoS Parameters for Downlink and Uplink in the QOS Flow Level QoS parameters IE

| Issue | Impacts | Supports |
| --- | --- | --- |
| FFS in SA2 spec: Editor's note: [XRM] The applicability and details of supporting PDU Set handling in uplink direction may be updated based on RAN WG's progress. PDU Set QoS requirements may not be same for DL and UL. | NG-RAN distinguishes between UL and DL PSQP. NGAP impacts, possible F1AP | QC, E///, TBD QC: send a LS to SA2? Nokia: further check |

Companies are invited to provide their views on P1

| Company | P1 |
| --- | --- |
| Ericsson | Yes |
| Nokia | Yes. After further check, we agree this is flexible. |
| Qualcomm | Yes, this is very important for RAN to get separate DL/UL PDU Set QoS Parameters. Also, we prefer to send LS to SA2 to make progress in multiple WGs simultaneously. |
| Xiaomi | Yes. |
| Huawei | Maybe, but NG-RAN node just gets QoS parameter from SMF, SA2 is discussing the same issue, RAN3 can wait for SA2 conclusion. |
| Samsung | Yes. It is flexible and align with existing configuration. |
| China Telecom | yes |
| ZTE | No. SA2 is discussing this issue, we shall wait for their progress. |

## Other topics?

It is proposed to defined the encoding of the N6 Jitter Information in R3-235204 as below:

#### N6 Jitter Information

This IE indicates the jitter information associated with the Periodicity in downlink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| N6 Jitter Information | O |  | OCTET STRING | The octets of OCTET STRING are encoded as the N6 Jitter Measurement IE specified in TS 29.244 [43] |

- Define N6 Jitter Information as the OCTET STRING, and detail definition is referring to TS 29.244.

Companies are invited to provide their views this encoding

| Company | Encoding proposed in R3-235204 |
| --- | --- |
| Nokia | Ok |
| Qualcomm | Ok |
| Xiaomi | Ok for the IE type and reference proposed in R3-235204. Another thing regarding jitter information we would like to mention is that, as RAN2 agreed that UE also reports Burst Arrival time and Jitter associated with the UL periodicity in uplink using UAI, UL jitter information from UE should also be introduced over F1AP, whether the same IE (update the IE name, e.g. remove “N6” and “downlink” related words) or a new IE is introduced needs discussion. And both UE and CN may send the UL Burst Arrival Time and UL Periodicity to gNB (i.e. CU-CP), we think RAN3 should discuss the CU-CP behavior when it receives the information from both UE and CN, e.g. decide to send one of them to DU or send both of them to DU, and let DU’s implementation to consider the information for DRX configuration. |
| Huawei | P1 is not a good solution for defining the N6 jitter for NGAP/F1AP/ XnAP/ E1AP. The N6 Jitter measurement defined in 29.244 is used for the UPF to report the measured N6 jitter, and it contains a lot of contents, e.g., even the “DL periodicity” can be included in this IE. However, we know that DL Periodicity already included in existing TSCAI as mandatory IE, if reuse the N6 Jitter measurement defined in 29.244, there will be redundant information about the periodicity. Bits. Octets; 8; 7; 6; 5; 4; 3; 2; 1. 1 to 2; Type = 327 (decimal). 3 to 4; Length = n. 5; Spare; DL. 6 to 9; DL Periodicity. 10 to 13; Lower DL Jitter Measurement. 14 to 17; Higher DL Jitter Measurement. 18 to (n+4); These octet(s) is/are present only if explicitly specified. Figure 8.2.220-1: N6 Jitter Measurement Alternatively, we can design N6 jitter using a simpler definition, e.g. an integer to indicate the maximum value of this jitter. RAN2 is discussing the value of UL jitter, maybe their outcome can be referred when we design the N6 jitter. |
| Samsung | Agree with Xiaomi. Probably the name of N6 should be changed. Fine to further check RAN2 discussion on the UL and make the alignment. |
| China Telecom | No. similar views with Huawei. We only need to know the maximum value for DL jitter and Lower DL Jitter while Other information seems useless. We can wait the outcome of UL jitter discussed in RAN2 and decide how to define DL jitter. Therefore , the encoding of N6 jitter information is FFS. |
| ZTE | Prefer to wait for RAN2 progress. |

# Conclusion

<TBD>