3GPP TSG-RAN WG3 Meeting #121bis	 R3-235726

Xiamen, China, October 9 – 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1105 | rev | 8 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BLCR to 38.473) Addition of SON features enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR contains the agreed changes required for addition of SON features |  |  |  |  |  |  |  |  |
| Summary of change: |  | <to be completed> |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing support for the SON feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.2.x, 8.2.x.1, 8.2.x.2, 8.2.x.3, 8.11.1.2, 9.2.1.23, 9.2.1.x1, 9.2.10.1, 9.3.1.x1, 9.4.3, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 0: - Included agreed changes from RAN3#118 in R3-226911 - Editorials - Added ASN.1 Rev 1: resubmitted to RAN3#119 Rev2: - Included agreed changes from RAN3#119 in R3-230980 Rev 3: resubmitted to RAN3#119b Rev 4: - Included agreed changes from RAN3#119b in R3-226911 - resubmitted to RAN3#120 Rev 5: - editorial: changed order in 9.4.7 to match spec - resubmitted to RAN3#121 Rev 6: Including changes agreed in RAN3#121: from R3-234545 Rev 7: resubmitted to RAN3#121b Rev: 8 Removed change on change in 9.2.1.x1 |  |  |  |  |  |  |  |  |

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| F1 Setup |  | F1 SETUP REQUEST |  | F1 SETUP RESPONSE |  | F1 SETUP FAILURE |  |
| gNB-DU Configuration Update |  | GNB-DU CONFIGURATION UPDATE |  | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-DU CONFIGURATION UPDATE FAILURE |  |
| gNB-CU Configuration Update |  | GNB-CU CONFIGURATION UPDATE |  | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU CONFIGURATION UPDATE FAILURE |  |
| UE Context Setup |  | UE CONTEXT SETUP REQUEST |  | UE CONTEXT SETUP RESPONSE |  | UE CONTEXT SETUP FAILURE |  |
| UE Context Release (gNB-CU initiated) |  | UE CONTEXT RELEASE COMMAND |  | UE CONTEXT RELEASE COMPLETE |  |  |  |
| UE Context Modification (gNB-CU initiated) |  | UE CONTEXT MODIFICATION REQUEST |  | UE CONTEXT MODIFICATION RESPONSE |  | UE CONTEXT MODIFICATION FAILURE |  |
| UE Context Modification Required (gNB-DU initiated) |  | UE CONTEXT MODIFICATION REQUIRED |  | UE CONTEXT MODIFICATION CONFIRM |  | UE CONTEXT MODIFICATION REFUSE |  |
| Write-Replace Warning |  | WRITE-REPLACE WARNING REQUEST |  | WRITE-REPLACE WARNING RESPONSE |  |  |  |
| PWS Cancel |  | PWS CANCEL REQUEST |  | PWS CANCEL RESPONSE |  |  |  |
| gNB-DU Resource Coordination |  | GNB-DU RESOURCE COORDINATION REQUEST |  | GNB-DU RESOURCE COORDINATION RESPONSE |  |  |  |
| F1 Removal |  | F1 REMOVAL REQUEST |  | F1 REMOVAL RESPONSE |  | F1 REMOVAL FAILURE |  |
| BAP Mapping Configuration |  | BAP MAPPING CONFIGURATION |  | BAP MAPPING CONFIGURATION ACKNOWLEDGE |  | BAP MAPPING CONFIGURATION FAILURE |  |
| GNB-DU Resource Configuration |  | GNB-DU RESOURCE CONFIGURATION |  | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE |  | GNB-DU RESOURCE CONFIGURATION FAILURE |  |
| IAB TNL Address Allocation |  | IAB TNL ADDRESS REQUEST |  | IAB TNL ADDRESS RESPONSE |  | IAB TNL ADDRESS FAILURE |  |
| IAB UP Configuration Update |  | IAB UP CONFIGURATION UPDATE REQUEST |  | IAB UP CONFIGURATION UPDATE RESPONSE |  | IAB UP CONFIGURATION UPDATE FAILURE |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| Positioning Measurement |  | POSITIONING MEASUREMENT REQUEST |  | POSITIONING MEASUREMENT RESPONSE |  | POSITIONING MEASUREMENT FAILURE |  |
| Positioning Information Exchange |  | POSITIONING INFORMATION REQUEST |  | POSITIONING INFORMATION RESPONSE |  | POSITIONING INFORMATION FAILURE |  |
| TRP Information Exchange |  | TRP INFORMATION REQUEST |  | TRP INFORMATION RESPONSE |  | TRP INFORMATION FAILURE |  |
| Positioning Activation |  | POSITIONING ACTIVATION REQUEST |  | POSITIONING ACTIVATION RESPONSE |  | POSITIONING ACTIVATION FAILURE |  |
| E-CID Measurement Initiation |  | E-CID MEASUREMENT INITIATION REQUEST |  | E-CID MEASUREMENT INITIATION RESPONSE |  | E-CID MEASUREMENT INITIATION FAILURE |  |
| Broadcast Context Setup |  | BROADCAST CONTEXT SETUP REQUEST |  | BROADCAST CONTEXT SETUP RESPONSE |  | BROADCAST CONTEXT SETUP FAILURE |  |
| Broadcast Context Release |  | BROADCAST CONTEXT RELEASE COMMAND |  | BROADCAST CONTEXT RELEASE COMPLETE |  |  |  |
| Broadcast Context Modification |  | BROADCAST CONTEXT MODIFICATION REQUEST |  | BROADCAST CONTEXT MODIFICATION RESPONSE |  | BROADCAST CONTEXT MODIFICATION FAILURE |  |
| Multicast Context Setup |  | MULTICAST CONTEXT SETUP REQUEST |  | MULTICAST CONTEXT SETUP RESPONSE |  | MULTICAST CONTEXT SETUP FAILURE |  |
| Multicast Context Release |  | MULTICAST CONTEXT RELEASE COMMAND |  | MULTICAST CONTEXT RELEASE COMPLETE |  |  |  |
| Multicast Context Modification |  | MULTICAST CONTEXT MODIFICATION REQUEST |  | MULTICAST CONTEXT MODIFICATION RESPONSE |  | MULTICAST CONTEXT MODIFICATION FAILURE |  |
| Multicast Distribution Setup |  | MULTICAST DISTRIBUTION SETUP REQUEST |  | MULTICAST DISTRIBUTION SETUP RESPONSE |  | MULTICAST DISTRIBUTION SETUP FAILURE |  |
| Multicast Distribution Release |  | MULTICAST DISTRIBUTION RELEASE COMMAND |  | MULTICAST DISTRIBUTION RELEASE COMPLETE |  |  |  |
| PDC Measurement Initiation |  | PDC MEASUREMENT INITIATION REQUEST |  | PDC MEASUREMENT INITIATION RESPONSE |  | PDC MEASUREMENT INITIATION FAILURE |  |
| PRS Configuration Exchange |  | PRS CONFIGURATION REQUEST |  | PRS CONFIGURATION RESPONSE |  | PRS CONFIGURATION FAILURE |  |
| Measurement Preconfiguration |  | MEASUREMENT PRECONFIGURATION REQUIRED |  | MEASUREMENT PRECONFIGURATION CONFIRM |  | MEASUREMENT PRECONFIGURATION REFUSE |  |

Table 2: Class 2 procedures

| Elementary Procedure |  | Message |  |
| --- | --- | --- | --- |
| Error Indication |  | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) |  | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer |  | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer |  | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer |  | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification |  | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery |  | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging |  | PAGING |  |
| Notify |  | NOTIFY |  |
| PWS Restart Indication |  | PWS RESTART INDICATION |  |
| PWS Failure Indication |  | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication |  | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report |  | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction |  | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |  |
| Deactivate Trace |  | DEACTIVATE TRACE |  |
| DU-CU Radio Information Transfer |  | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer |  | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication |  | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |  |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |  |
| Access Success |  | ACCESS SUCCESS |  |
| Cell Traffic Trace |  | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control |  | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback |  | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report |  | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort |  | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication |  | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update |  | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation |  | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication |  | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report |  | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination |  | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update |  | POSITIONING INFORMATION UPDATE |  |
| Multicast Group Paging |  | MULTICAST GROUP PAGING |  |
| Broadcast Context Release Request |  | BROADCAST CONTEXT RELEASE REQUEST |  |
| Multicast Context Release Request |  | MULTICAST CONTEXT RELEASE REQUEST |  |
| PDC Measurement Report |  | PDC MEASUREMENT REPORT |  |
| PDC Measurement Termination |  | PDC MEASUREMENT TERMINATION COMMAND |  |
| PDC Measurement Failure Indication |  | PDC MEASUREMENT FAILURE INDICATION |  |
| PDC Measurement Termination |  | PDC MEASUREMENT TERMINATION COMMAND |  |
| Measurement Activation |  | MEASUREMENT ACTIVATION |  |
| QoE Information Transfer |  | QOE INFORMATION TRANSFER |  |
| Positioning System Information Delivery |  | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |  |
| RACH Indication |  | RACH INDICATION |  |

## 8.2	Interface Management procedures

### 8.2.x		RACH Indication

#### 8.2.x.1		General

This procedure is initiated by the gNB-DU to inform the gNB-CU about the occurrences of successful random access procedures in the gNB-DU.

The procedure uses non-UE-associated signalling.

#### 8.2.x.2		Successful Operation

Figure 8.2.x.2-1: RACH Indication procedure.

The gNB-DU initiates the procedure by sending the RACH INDICATION message to the gNB-CU.

The RACH INDICATION message contains information concerning one or more successful random access procedures occurring at the gNB-DU and not known to the gNB-CU as described in TS 38.401 [4].

Upon reception of the RACH INDICATION message, the gNB-CU may trigger retrieval of RACH Reports from the UE.

#### 8.2.x.3 	Abnormal Conditions

Not applicable.

## 8.11	Self Optimisation Support procedures

### 8.11.1	Access and Mobility Indication

#### 8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

#### 8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RA ReportList IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful PSCell Change Report Information List IE, the gNB-DU may take it into account for optimisation of PSCell change/addition related parameters.

#### 8.11.1.3	Abnormal Conditions

Not applicable.

# 9	Elements for F1AP Communication

## 9.2	Message Functional Definition and Content

### 9.2.1	Interface Management messages

#### 9.2.1.23	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Hardware Load Indicator | O |  | 9.3.1.136 |  | YES | ignore |
| TNL Capacity Indicator | O |  | 9.3.1.128 |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellingNBDU > |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Radio Resource Status | O |  | 9.3.1.129 |  | - |  |
| >>Composite Available Capacity Group | O |  | 9.3.1.130 |  | - |  |
| >>Slice Available Capacity | O |  | 9.3.1.134 |  | - |  |
| >>Number of Active UEs | O |  | 9.3.1.135 |  | - |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Identifies a portion of the NR-U Channel Bandwidth on which channel access procedure in shared spectrum has been performed in the last reporting period. | - |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 indicates that the channel resources have been utilized for DL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | - |  |
| >>>>Channel Occupancy Time Percentage UL | O |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding NR-U Channel of the serving cell for UEs that transmit to the serving cell. Value 100 indicates that the channel resources have been utilized for UL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | YES | ignore |
| >>>>Radio Resource Status NR-U | O |  | 9.3.1.x1 | Indicates the radio resource status per NR-U channel. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |

#### 9.2.1.x1	RACH INDICATION

This message is sent by the gNB-DU to inform the gNB-CU about one or more random access procedures performed at the gNB-DU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RA Report Indication List |  | 1 |  |  | YES | reject |
| >RA Report Indication List Item |  | 1..<maxnoofRAReport Indications> |  |  | - | - |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReportIndications | Maximum number of RA Report indications in the message. Value is FFS. |

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | FFS pending on RAN2 | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is FFS. |

## 9.3	Information Element Definitions

### 9.3.1	Radio Network Layer Related IEs

#### 9.3.1.x1	Radio Resource Status NR-U

The Radio Resource Status NR-U IE indicates the usage of the PRBs per NR-U channel for all traffic in Downlink and Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL Total PRB Usage | M |  | INTEGER (0..100) | Per NR-U Channel DL Total PRB usage in percentage of the cell total PRB number. |
| UL Total PRB Usage | M |  | INTEGER (0..100) | Per NR-U Channel UL Total PRB usage in percentage of the cell total PRB number. |

## 9.4	Message and Information Element Abstract Syntax (with ASN.1)

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

[snip]

MeasurementPreconfigurationRequired,

MeasurementPreconfigurationConfirm,

MeasurementPreconfigurationRefuse,

MeasurementActivation,

QoEInformationTransfer,

PosSystemInformationDeliveryCommand,

RachIndication

FROM F1AP-PDU-Contents

id-Reset,

id-F1Setup,

[snip]

id-measurementPreconfiguration,

id-measurementActivation,

id-QoEInformationTransfer,

id-PosSystemInformationDeliveryCommand,

id-RachIndication

FROM F1AP-Constants

[snip]

F1AP-ELEMENTARY-PROCEDURES-CLASS-2 F1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

uEContextReleaseRequest					|

dLRRCMessageTransfer					|

uLRRCMessageTransfer					|

uEInactivityNotification				|

privateMessage							|

initialULRRCMessageTransfer				|

systemInformationDelivery				|

paging									|

notify									|

pWSRestartIndication					|

pWSFailureIndication					|

gNBDUStatusIndication					|

rRCDeliveryReport						|

networkAccessRateReduction				|

traceStart								|

deactivateTrace							|

dUCURadioInformationTransfer			|

cUDURadioInformationTransfer			|

resourceStatusReporting					|

accessAndMobilityIndication				|

referenceTimeInformationReportingControl|

referenceTimeInformationReport			|

accessSuccess							|

cellTrafficTrace						|

positioningAssistanceInformationControl		|

positioningAssistanceInformationFeedback	|

positioningMeasurementReport				|

positioningMeasurementAbort					|

positioningMeasurementFailureIndication		|

positioningMeasurementUpdate				|

positioningDeactivation						|

e-CIDMeasurementFailureIndication			|

e-CIDMeasurementReport						|

e-CIDMeasurementTermination					|

positioningInformationUpdate				|

multicastGroupPaging						|

broadcastContextReleaseRequest				|

multicastContextReleaseRequest				|

pDCMeasurementReport						|

pDCMeasurementTerminationCommand			|

pDCMeasurementFailureIndication				|

measurementActivation						|

qoEInformationTransfer						|

posSystemInformationDelivery 				|

rachIndication								,

...

}

[snip]

qoEInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		QoEInformationTransfer

PROCEDURE CODE			id-QoEInformationTransfer

CRITICALITY				ignore

}

posSystemInformationDelivery F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PosSystemInformationDeliveryCommand

PROCEDURE CODE			id-PosSystemInformationDeliveryCommand

CRITICALITY				ignore

}

rachIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RachIndication

PROCEDURE CODE			id-RachIndication

CRITICALITY				ignore

}

END

[snip]

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

[snip]

SRSPosRRCInactiveConfig,

SDTBearerConfigurationQueryIndication,

SDTBearerConfigurationInfo,

PosSItypeList,

DAPS-HO-Status,

UuRLCChannelID,

UplinkTxDirectCurrentTwoCarrierListInfo,

UlTxDirectCurrentMoreCarrierInformation,

SRSPosRRCInactiveQueryIndication,

MC-PagingCell-Item,

UlTxDirectCurrentMoreCarrierInformation,

CPACMCGInformation,

ExtendedUEIdentityIndexValue,

HashedUEIdentityIndexValue,

RAReportIndicationList,

SuccessfulPSCellChangeReportInformationList

FROM F1AP-IEs

[snip]

ReportingPeriodicity,

TNLCapacityIndicator,

RAReportList,

RLFReportInformationList,

ReportingRequestType,

TimeReferenceInformation,

[snip]

id-HardwareLoadIndicator,

id-ReportingPeriodicity,

id-TNLCapacityIndicator,

id-RAReportList,

id-RLFReportInformationList,

id-ReportingRequestType,

[snip]

id-SRSPosRRCInactiveConfig,

id-SDTBearerConfigurationQueryIndication,

id-SDTBearerConfigurationInfo,

id-PosSItypeList,

id-DAPS-HO-Status,

id-SRBMappingInfo,

id-UplinkTxDirectCurrentTwoCarrierListInfo,

id-UlTxDirectCurrentMoreCarrierInformation,

id-SRSPosRRCInactiveQueryIndication,

id-UlTxDirectCurrentMoreCarrierInformation,

id-CPACMCGInformation,

id-ExtendedUEIdentityIndexValue,

id-HashedUEIdentityIndexValue,

id-RAReportIndicationList,

id-SuccessfulPSCellChangeReportInformationList,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

[snip]

-- **************************************************************

--

#### --  Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Access And Mobility Indication

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { AccessAndMobilityIndicationIEs} },

...

}

AccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-RAReportList				CRITICALITY ignore	TYPE RAReportList			       PRESENCE optional }|

{ ID id-RLFReportInformationList				CRITICALITY ignore	TYPE RLFReportInformationList				PRESENCE optional }|

{ ID id-SuccessfulHOReportInformationList		CRITICALITY ignore	TYPE SuccessfulHOReportInformationList	              PRESENCE optional }|

{ ID id-SuccessfulPSCellChangeReporInformationtList	CRITICALITY ignore	TYPE SuccessfulPSCellChangeReportInformationList		PRESENCE optional },

...

}

[snip]

QoEInformationTransfer-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-QoEInformation				CRITICALITY ignore	TYPE QoEInformation						PRESENCE optional	},

...

}

-- **************************************************************

--

-- Positioning System information Delivery Command

--

-- **************************************************************

PosSystemInformationDeliveryCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PosSystemInformationDeliveryCommandIEs}},

...

}

PosSystemInformationDeliveryCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-PosSItypeList			CRITICALITY reject	TYPE PosSItypeList					PRESENCE mandatory	}|

{ ID id-ConfirmedUEID 			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- RACH Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- RACH Indication

--

-- **************************************************************

RachIndication ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{ RachIndication-IEs}},

...

}

RachIndication-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-RAReportIndicationList				CRITICALITY reject	TYPE RAReportIndicationList						PRESENCE mandatory	},

...

}

END

[snip]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

[snip]

id-RedCapIndication,

id-UL-GapFR2-Config,

id-ConfigRestrictInfoDAPS,

id-MulticastF1UContextReferenceCU,

id-TwoPHRModeMCG,

id-TwoPHRModeSCG,

id-ncd-SSB-RedCapInitialBWP-SDT,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-startRBHopping,

id-startRBIndex,

id-transmissionCombn8,

id-ServCellInfoList,

id-ChannelOccupancyTimePercentageUL,

id-RadioResourceStatusNR-U,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

[snip]

maxnoofPRACHconfigs,

maxnoofRAReports,

maxnoofRLFReports,

[snip]

maxnoofPosSITypes,

maxnoofMRBs,

maxNrofBWPs,

maxnoofRAReportIndications,

maxnoofSuccessfulPSCellChangeReports

[snip]

NR-U-Channel-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-Channel-Item

NR-U-Channel-Item ::= SEQUENCE {

nR-U-ChannelID						INTEGER(1..maxnoofNR-UChannelIDs),

channelOccupancyTimePercentageDL		ChannelOccupancyTimePercentage,

energyDetectionThreshold				EnergyDetectionThreshold,

iE-Extensions			ProtocolExtensionContainer { { NR-U-Channel-Item-ExtIEs} } OPTIONAL,

...

}

NR-U-Channel-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ChannelOccupancyTimePercentageUL	CRITICALITY ignore EXTENSION ChannelOccupancyTimePercentage PRESENCE optional}|

{ ID id-RadioResourceStatusNR-U                 CRITICALITY ignore EXTENSION RadioResourceStatusNR-U PRESENCE optional},

...

}

[snip]

RACH-Config-Common	::= OCTET STRING

RACH-Config-Common-IAB	::= OCTET STRING

RAReportContainer::= OCTET STRING

RAReportList	::= SEQUENCE (SIZE(1.. maxnoofRAReports)) OF RACHReportItem

RAReportItem	::= SEQUENCE {

rAReportContainer				RAReportContainer,

uEAssitantIdentifier			GNB-DU-UE-F1AP-ID		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { RAReportItem-ExtIEs} }	OPTIONAL,

...

}

RAReportItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RAReportIndicationList ::= SEQUENCE (SIZE(1..maxnoofRAReportIndications)) OF RAReportIndicationList-Item

RAReportIndicationList-Item ::= SEQUENCE {

gNB-CU-UE-F1AP-ID					GNB-CU-UE-F1AP-ID,

iE-Extensions						ProtocolExtensionContainer { { RAReportIndicationList-ExtIEs} } OPTIONAL,

...

}

RAReportIndicationList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RadioResourceStatus ::= SEQUENCE {

sSBAreaRadioResourceStatusList		SSBAreaRadioResourceStatusList,

iE-Extensions	ProtocolExtensionContainer { { RadioResourceStatus-ExtIEs} } OPTIONAL

}

RadioResourceStatus-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SliceRadioResourceStatus		CRITICALITY ignore	EXTENSION SliceRadioResourceStatus		PRESENCE optional	}|

{ ID id-MIMOPRBusageInformation			CRITICALITY ignore	EXTENSION MIMOPRBusageInformation			PRESENCE optional	},

...

}

RadioResourceStatusNR-U ::= SEQUENCE {

dl-Total-PRB-usage 	INTEGER (0..100),

ul-Total-PRB-usage 	INTEGER (0..100),

iE-Extensions					ProtocolExtensionContainer { { RadioResourceStatusNR-U-ExtIEs} }	OPTIONAL,

...

}

RadioResourceStatusNR-U-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MIMOPRBusageInformation ::= SEQUENCE {

dl-GBR-PRB-usage-for-MIMO		INTEGER (0..100),

ul-GBR-PRB-usage-for-MIMO		INTEGER (0..100),

dl-non-GBR-PRB-usage-for-MIMO	INTEGER (0..100),

ul-non-GBR-PRB-usage-for-MIMO 	INTEGER (0..100),

dl-Total-PRB-usage-for-MIMO 	INTEGER (0..100),

ul-Total-PRB-usage-for-MIMO 	INTEGER (0..100),

iE-Extensions					ProtocolExtensionContainer { { MIMOPRBusageInformation-ExtIEs} }	OPTIONAL,

...

}

MIMOPRBusageInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RANAC ::= INTEGER (0..255)

RAN-MeasurementID ::= INTEGER (1.. 65536, ...)

[snip]

SuccessfulHOReportInformationList::= SEQUENCE (SIZE(1.. maxnoofSuccessfulHOReports)) OF SuccessfulHOReportInformation-Item

SuccessfulHOReportInformation-Item ::= SEQUENCE {

successfulHOReportContainer				OCTET STRING,

iE-Extensions	ProtocolExtensionContainer { { SuccessfulHOReportInformation-Item-ExtIEs } }	OPTIONAL

}

SuccessfulHOReportInformation-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SuccessfulPSCellChangeReportInformationList::= SEQUENCE (SIZE(1.. maxnoofSuccessfulPSCellChangeReports)) OF SuccessfulPSCellChangeReportInformation-Item

SuccessfulPSCellChangeReportInformation-Item ::= SEQUENCE {

successfulPSCellChangeReportContainer				OCTET STRING,

iE-Extensions	ProtocolExtensionContainer { { SuccessfulPSCellChangeReportInformation-Item-ExtIEs } }	OPTIONAL

}

SuccessfulPSCellChangeReportInformation-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

[snip]

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

[snip]

id-measurementPreconfiguration				ProcedureCode ::= 76

id-measurementActivation					ProcedureCode ::= 77

id-QoEInformationTransfer					ProcedureCode ::= 78

id-PDCMeasurementTerminationCommand			ProcedureCode ::= 79

id-PDCMeasurementFailureIndication 			ProcedureCode ::= 80

id-PosSystemInformationDeliveryCommand		ProcedureCode ::= 81

id-RachIndication							ProcedureCode ::= 99 -- assigned by MCC

[snip]

maxnoofPhysicalResourceBlocks-1			INTEGER ::= 274

maxnoofPRACHconfigs						INTEGER ::= 16

maxnoofRAReports						INTEGER ::= 64

maxnoofRLFReports						INTEGER ::= 64

maxnoofAdditionalPDCPDuplicationTNL		INTEGER ::=	2

[snip]

maxnoofMRBsforUE						INTEGER ::= 64

maxnoofMBSSessionsofUE					INTEGER ::= 256

maxnoofSLdestinations					INTEGER ::= 32

maxnoofNSAGs							INTEGER ::= 256

maxnoofSDTBearers						INTEGER ::= 72

maxnoofServingCellMOs					INTEGER ::= 16

maxNrofBWPs								INTEGER ::= 8

maxnoofPosSITypes						INTEGER ::= 32

maxnoofRAReportIndications				INTEGER ::= 256 –- value is FFS

maxnoofSuccessfulPSCellChangeReports	INTEGER ::= 64  –- value is FFS

[snip]

id-SSB-PositionsInBurst								ProtocolIE-ID ::= 357

id-NRPRACHConfig									ProtocolIE-ID ::= 358

id-RAReportList						ProtocolIE-ID ::= 359

id-RLFReportInformationList							ProtocolIE-ID ::= 360

id-TDD-UL-DLConfigCommonNR							ProtocolIE-ID ::= 361

id-CNPacketDelayBudgetDownlink						ProtocolIE-ID ::= 362

[snip]

id-SRSPosRRCInactiveConfig							ProtocolIE-ID ::= 674

id-SDTBearerConfigurationQueryIndication			ProtocolIE-ID ::= 675

id-SDTBearerConfigurationInfo						ProtocolIE-ID ::= 676

id-UL-GapFR2-Config									ProtocolIE-ID ::= 677

id-ConfigRestrictInfoDAPS							ProtocolIE-ID ::= 678

id-UE-MulticastMRBs-Setup-List						ProtocolIE-ID ::= 679

id-UE-MulticastMRBs-Setup-Item						ProtocolIE-ID ::= 680

id-MulticastF1UContextReferenceCU					ProtocolIE-ID ::= 681

id-PosSItypeList									ProtocolIE-ID ::= 682

id-DAPS-HO-Status									ProtocolIE-ID ::= 683

id-UplinkTxDirectCurrentTwoCarrierListInfo			ProtocolIE-ID ::= 684

id-UE-MulticastMRBs-ToBeSetup-atModify-List			ProtocolIE-ID ::= 685

id-UE-MulticastMRBs-ToBeSetup-atModify-Item			ProtocolIE-ID ::= 686

id-MC-PagingCell-List								ProtocolIE-ID ::= 687

id-MC-PagingCell-Item								ProtocolIE-ID ::= 688

id-SRSPosRRCInactiveQueryIndication					ProtocolIE-ID ::= 689

id-UlTxDirectCurrentMoreCarrierInformation	        ProtocolIE-ID ::= 690

id-CPACMCGInformation								ProtocolIE-ID ::= 691

id-TwoPHRModeMCG									ProtocolIE-ID ::= 692

id-TwoPHRModeSCG									ProtocolIE-ID ::= 693

id-ExtendedUEIdentityIndexValue						ProtocolIE-ID ::= 694

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-HashedUEIdentityIndexValue						ProtocolIE-ID ::= 698

id-UE-MulticastMRBs-Setupnew-List					ProtocolIE-ID ::= 699

id-UE-MulticastMRBs-Setupnew-Item					ProtocolIE-ID ::= 700

id-ncd-SSB-RedCapInitialBWP-SDT						ProtocolIE-ID ::= 701

id-nrofSymbolsExtended								ProtocolIE-ID ::= 702

id-repetitionFactorExtended							ProtocolIE-ID ::= 703

id-startRBHopping									ProtocolIE-ID ::= 704

id-startRBIndex										ProtocolIE-ID ::= 705

id-transmissionCombn8								ProtocolIE-ID ::= 706

id-ServCellInfoList                                 		ProtocolIE-ID ::= 707

id-RAReportIndicationList							ProtocolIE-ID ::= 900 -- assigned by MCC

id-ChannelOccupancyTimePercentageUL					ProtocolIE-ID ::= 901 -- assigned by MCC

id-SuccessfulPSCellChangeReportList					ProtocolIE-ID ::= 902 -- assigned by MCC

id-RadioResourceStatusNR-U							ProtocolIE-ID ::= 903 -- assigned by MCC

END