3GPP TSG-RAN WG3 Meeting #121bis	R3-235733

Xiamen, China, October 9 – 13, 2023

Agenda Item:	10.2.5

Source:	Huawei

Title:	(TP for SON BL CR for TS 38.300): SON for NR-U

Document for:	Other

1	Introduction

This document contains a TP for TS 38.300 to capture the function in stage 2.

Annex1	TP for TS 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 15.5.1.2	Load reporting for intra-RAT and intra-system inter-RAT load balancing

The load reporting function is executed by exchanging load information over the Xn/X2/F1/E1 interfaces.

The following load related information should be supported which consists of:

-	Radio resource usage (per-cell and per SSB area PRB usage: DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, DL/UL total PRB usage, and DL/UL scheduling PDCCH CCE usage; PRB usage for slice(s): DL/UL GBR PRB usage, DL/UL non-GBR PRB usage, and DL/UL Total PRB allocation);

-	TNL capacity indicator (UL/DL TNL offered capacity and available capacity);

-	Cell Capacity Class value (UL/DL relative capacity indicator);

-	Capacity value (per cell, per SSB area and per slice: UL/DL available capacity);

-	HW capacity indicator (offered throughput and available throughput over E1, percentage utilisation over F1);

-	RRC connections (number of RRC connections, and available RRC Connection Capacity);

-	Number of active UEs;

-	NR-U channel load (channel occupancy time percentage DL/UL, energy detection threshold DL/UL, per cell radio resource usage).

To achieve load reporting function, Resource Status Reporting Initiation & Resource Status Reporting procedures are used.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>