# 3GPP TSG-RAN WG3 Meeting #121bis	R3-235736

# Xiamen, China, Oct.9th - Oct.13th, 2023

# Agenda Item:	12.2.1

# Source:	ZTE, Ericsson, Huawei, Nokia, Nokia Shanghai Bell, Lenovo, 	Samsung, CATT

# Title:	(TP to 38.300 BLCR) Stage2 description update for AI/ML RAN

# Document for:	Discussion

- Introduction.

The TP is to reflect the offline discussion’s agreements in the CB #AIRAN1_Cleanup.

- TP to 38.300 BLCR

<<<<<< Start of CHANGE >>>>>>

X.X AI/ML for NG-RAN

X.X.1 General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

X.X.2 Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

Signalling procedures used for the exchange of AI/ML related information are use case and data type agnostic, which means that the intended usage of the data exchanged via these procedures (e.g., input, output, feedback) is not indicated.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

Support of the AI/ML function does not apply to ng-eNBs (E-UTRA nodes connected to 5GC).

For the deployments of RAN intelligence, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node.

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

The predicted Resource Status Information, measured UE trajectory as well as the UE performance feedback can be configured to be reported and used for all use cases. The reporting is configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory as well as UE performance feedback is triggered via the Handover Preparation procedure.

Cell-based UE trajectory prediction, which can be used e.g. for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Request message to provide information for e.g. subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the next one hop target NG-RAN node.

The metric of Energy Cost (EC), which can be used e.g. for the Network Energy Saving use case, is introduced and can be exchanged with the neighbouring NG-RAN nodes upon request.

<<<<<< End of CHANGE >>>>>>