3GPP TSG-RAN WG3 #121bis	R3-235750

Xiamen, China, 09-13 October 2023

Agenda Item:	26.2

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion for CB # R18URLLC

Document for:	Approval

# Introduction

CB: # R18URLLC

- Discuss the open issues

- Capture agreements and provide TPs if agreeable

(moderator - Nok)

Summary of offline disc R3-235750

# For the Chairman’s Notes

Propose the following:

R3-235171 rev in R3-235817 – agreed (TP for TS 38.413)

R3-235671 rev in R3-235818 – agreed (TP for TS 38.473)

R3-235575 rev in R3-235819 – agreed (TP for TS 38.423)

R3-235820 – agreed (LSout)

Propose to capture the following:

TSC Traffic Characteristics Feedback is not provided during handover (i.e., not included in HANDOVER REQUEST ACKNOWLEDGE or PATH SWITCH REQUEST).

RAN feedback for NR-DC case is not supported in Rel-18.

# Discussion

## 5GS network timing synchronization status and reporting

### NGAP

TRS Issue #1: Encoding of Clock Accuracy IE

Option A: INTEGER with precise granularity, as low as e.g. 10ns or 25ns, e.g.:

- INTEGER (1..4000000, …) in units of 25ns (FFS)

Option B: refer to Table 5 of IEEE Std 1588, e.g.:

- ENUMERATED (25ns, 100ns, 250ns, 1us, 2dot5 us, 10us, 25us, 100us, 250us, 1ms, 2dot5ms, 10ms, 25ms, 100ms, 250ms, 1sec, …)

- INTEGER (32..47, …)

Agreed online:

The encoding of the Clock Accuracy IE with one choice structure (including Option a and Option b) over NGAP from gNB to AMF and F1AP.

Conclusion: Agree on CHOICE with Option A and Option B, in NGAP and F1AP

TRS Issue #2: Encoding of Clock Quality Acceptance Criteria IE

From [1]:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | BIT STRING { locked (0), holdover (1), freeRun (2) } (SIZE (8, …)) | Each position in the bitmap represents a synchronisation state. If a bit is set to “1”, the respective synchronisation state is acceptable. If a bit is set to “0”, the respective synchronisation state is not acceptable. Bits 3-7 reserved for future use. |
| Traceable to UTC | O |  | ENUMERATED (true, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [9]. |
| Clock Accuracy | O |  | INTEGER (1..400000000, …) | Clock accuracy expressed in units of 25 ns (FFS). |
| Parent Time Source | O |  | BIT STRING { syncE (0), pTP (1), gNSS (2), atomicClock (3), terrestrialRadio (4), serialTimeCode (5), nTP (6), handset (7), other (8) } (SIZE (16, …)) | Each position in the bitmap represents a parent time source. If a bit is set to “1”, the respective parent time source is acceptable. If a bit is set to “0”, the respective parent time source is not acceptable. Bits 9-15 reserved for future use. |

Agreed online:

Encode the Clock Quality Acceptance Criteria IE with information as listed in R3-235147. While the IE type and reference and semantic description needs to be further checked.

Conclusion: Agreed on the above encoding, in NGAP

TRS Issue #3: Other NGAP

Proposal 1 in [13]: Set maxnoofCellsTSS to 16384.

Conclusion: Agreed

Proposal 1 in [9]: Include RAN TSS Scope IE in the Timing Synchronization Status Response message to indicate partial support of clock quality reporting for few gNB-DUs which does not support clock quality reporting.

Conclusion: Not agreed.

### F1AP specific

TRS Issue #4: F1AP impacts of Clock Quality Acceptance Criteria

Problem statement from [1]:

Observation-4:	There is only a single pre-configured threshold for each RAN TSS attribute, and it is assumed to meet the time sync performance requirement of all UEs.

Observation-5:	RAN TSS reporting triggered only by pre-configured thresholds is insufficient to support clock quality detail level “acceptable / not acceptable”.

Proposal 4: Discuss additional triggers for gNB-DU to report RAN TSS while the clock quality is in degraded state, to enable the gNB-CU to evaluate clock quality acceptance criteria.

Is the problem acknowledged (yes/no)?

Conclusion: To be continued (1 company does not acknowledge)

If the problem is acknowledged, then potential solutions:

Option A:

Proposal 5 in [1]: Add an optional Clock Accuracy Threshold List IE to the F1AP: TIMING SYNCHRONISATION STATUS REQUEST message, that includes a list of clock accuracy thresholds corresponding to the acceptance criteria of UEs served by the gNB-CU.

Option B:

Proposal 2 in [3]: If only one threshold is pre-configured for each attribute, DU can trigger periodic report of the timing synchronization status if enabled by the CU.

Other solution options?

TRS Issue #5: Use MT-SDT to deliver clock quality information to UE [12]  not treated due to lack of time

Proposal 5: It is proposed that RAN3 to discuss and agree gNB could use MT-SDT to deliver the clock quality information to UE in RRC_Inactive, by sending the MT-SDT Paging message when the gNB clock quality has changed.

Proposal 6: It is proposed to impact TS 38.300 the MT-SDT procedure and indicating that Step 2a and 2b may omitted if gNB-CU determines to use MT SDT to send the clock quality information to UE in RRC Inactive.

### XnAP specific

TRS Issue #6: Impacts of Xn-based handover [11]  not treated due to lack of time

Proposal 4: include the clock quality control related information and feedback in the Handover procedure over XnAP

TRS Issue #7: Impacts of RRC resume in new gNB [12]  not treated due to lack of time

Proposal 1: In Retrieve UE Context Request, the new gNB includes if it supports delivering TSS (Time Synchronization Status).

Proposal 2: The last serving gNB determines, upon the knowledge that UE had obtained Clock Quality information and the new gNB supports the TSS, to relocate the UE context to the new gNB. i.e. not to perform Partial UE Context Transfer

Proposal 3: The last serving gNB includes the UE clock Quality Control Information in the Partial UE Context Transfer, this is in the case that the last serving has a good reason to keep the UE Context anyway.

Proposal 4: The last serving gNB includes the UE clock Quality Control Information in the Retrieve UE Context Response in case it performs full UE context relocation.

### Stage 2 (TS 38.401)

TRS Issue #8: Other TS 38.401 [9]  not treated due to lack of time

Proposal 2: Based on “S2-2309820” agreement NG-RAN should report the threshold value to the UE, when the clock quality becomes better than the threshold. This shall be captured in TS 38.401 Clause 8.x.2

Proposal 6:  The threshold to trigger event for reporting of clock quality information to AMF shall be configured by OAM in gNB-CU or in gNB-DU or both considering co-located and non co-located deployments and clock source. This shall be captured in TS 38.401 Clause 8.x.1.

### LS to other groups

TRS Issue #9: LS to other groups

Proposal: Send an LS to RAN2/SA2, informing them of the encoding of the Timing Synchronisation Status Attributes IE and any other relevant RAN3 agreements.

Similar Draft LS in [1] (Nokia) and [3] (Huawei)

Conclusion: Agree to send LS pending offline review

## TSN integration

TSN Issue #1: NG-RAN node supports AN-TL

Option 1: Nokia

- Add TL-Container at PDU session level in PDU Session Resource Setup Request/Response.

- Add TL-Container at PDU session level in PDU Session Resource Modify Request/Response.

Option 2: Huawei

- Add TL-Container at PDU session level in PDU Session Resource Setup Request/Response.

- Add TL-Container at QoS flow level in PDU Session Resource Modify Request/Response.

Option 3: ZTE

- Add TL-Container at QoS flow level in PDU Session Resource Setup Request/Response.

- Add TL-Container at QoS flow level in PDU Session Resource Modify Request/Response.

Option 4: Ericsson

- Add TL-Container at node level in new non-UE associated procedure.

Encoding (similar in 3 papers): The TL Container IEs are encoded as OCTET STRING with semantics description “Containing the <message name> message specified in TS 29.585 [x]”.

Conclusion: Pending

TSN Issue #2: NG-RAN node does not support AN-TL [11]  not treated due to lack of time

Proposal 1: The NG-U tunnel information is included in the QoS flow during setup, to indicate that the TSN for the QoS flow is to be setup;

Proposal 2: The NG-RAN response to CN if the TSN QoS is setup, or the normal QoS is setup, When the normal QoS is setup, SMF may consider the NG-U tunnel is for the PDU session.

TSN Issue #3: Dynamic PDB   not treated due to lack of time

Option 1: Huawei [5]

- Add AN Packet Delay Budget Uplink in PDU SESSION RESOURCE SETUP RESPONSE, PDU SESSION RESOURCE MODIFY RESPONSE, and INITIAL CONTEXT SETUP RESPONSE messages

Option 2: ZTE [14] (similar to Option 1?)

- Include the dynamic value of 5G-AN PDB in the UL direction in the PDU Session Resource Setup/Modify Response IE.

## RAN feedback for low latency communication

### NGAP

RANF Issue #1: Proactive Feedback during handover

Include the TSC Traffic Characteristics Feedback (including the Burst Arrival Time Offset and the adjusted Periodicity) during handover procedure in the

- HANDOVER REQUEST ACKNOWLEDGE	 Huawei, CATT, ZTE, Nokia

- PATH SWITCH REQUEST			 Huawei, CATT, ZTE

- Neither						 Samsung

Conclusion: Agree on (c)

### XnAP specific

RANF Issue #2: Proactive Feedback during handover [2]

Proposal 4: In XnAP, the BAT Window and Periodicity Range IEs are not needed in the TSC Assistance Information IE for Xn-based handover.

Conclusion: Agreed, as a consequence of the agreement in section 3.3.1

RANF Issue #3: NR-DC case

Option 1: Huawei [4]

For the NR-DC case, for XnAP, include the TSC Traffic Characteristics Feedback (including the Burst Arrival Time Offset and the optional Adjusted Periodicity) in the following messages:

i.	S-NODE ADDITION REQUEST ACKNOWLEDGE message

ii.	S-NODE MODIFICATION REQUEST ACKNOWLEDGE message

iii.	S-Node initiated NOTIFICATION CONTROL INDICATION message

Option 2: ZTE [14]

There is no need to consider NR-DC case in this release.

Conclusion: Not supported in Rel-18

### F1AP specific

RANF Issue #4: F1AP impacts of RAN Feedback

Option 1: gNB-DU can determine RAN Feedback for both Proactive and Reactive

- Include the Burst Arrival Time Window, Capability for BAT Adaptation and (optional) Periodicity Range in the TSC Assistance Information IE.

- Include the TSC Traffic Characteristics Feedback (including the Burst Arrival Time Offset and the optional Adjusted Periodicity) in the following messages:

i.	UE CONTEXT SETUP RESPONSE message (for proactive)

ii.	UE CONTEXT MODIFICATION RESPONSE message (for proactive)

iii.	NOTIFY message (for reactive)

Option 2: gNB-DU can determine RAN Feedback for Reactive only

- Include the Capability for BAT Adaptation in the TSC Assistance Information IE.

- Include the TSC Traffic Characteristics Feedback (including the Burst Arrival Time Offset) in the NOTIFY message.

Agreed online:

For proactive feedback case, the gNB-CU has all needed information to determine the proactive feedback for both downlink and uplink? For reactive feedback case, the gNB-DU decides the feedback for both uplink and downlink.

Conclusion: Option 1 is agreed

## Stage 2 (TS 38.300)

RANF Issue #5: Other TS 38.300  Not treated due to lack of time

Text proposals in

- R3-235469 from Qualcomm, for TRS

- R3-235351 from CATT, for TRS, RANF, TSN [FFS]

# Conclusions

## 5GS network timing synchronization status and reporting

TRS Issue #1: Encoding of Clock Accuracy IE (NGAP & F1AP)

CHOICE

- INTEGER (1..4000000, …) in units of 25ns (FFS)

- INTEGER (32..47, …)

TRS Issue #2: Encoding of Clock Quality Acceptance Criteria IE (NGAP)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | BIT STRING { locked (0), holdover (1), freeRun (2) } (SIZE (8, …)) | Each position in the bitmap represents a synchronisation state. If a bit is set to “1”, the respective synchronisation state is acceptable. If a bit is set to “0”, the respective synchronisation state is not acceptable. Bits 3-7 reserved for future use. |
| Traceable to UTC | O |  | ENUMERATED (true, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [9]. |
| Clock Accuracy | O |  | INTEGER (1..400000000, …) | Clock accuracy expressed in units of 25 ns (FFS). |
| Parent Time Source | O |  | BIT STRING { syncE (0), pTP (1), gNSS (2), atomicClock (3), terrestrialRadio (4), serialTimeCode (5), nTP (6), handset (7), other (8) } (SIZE (16, …)) | Each position in the bitmap represents a parent time source. If a bit is set to “1”, the respective parent time source is acceptable. If a bit is set to “0”, the respective parent time source is not acceptable. Bits 9-15 reserved for future use. |

TRS Issue #3: Other NGAP

Set maxnoofCellsTSS to 16384.

TRS Issue #9: LS to other groups

Send LS pending offline review

## TSN integration

No agreements

## RAN feedback for low latency communication

RANF Issue #1: Proactive Feedback during handover (NGAP)

TSC Traffic Characteristics Feedback is not provided during handover (i.e., not included in HANDOVER REQUEST ACKNOWLEDGE or PATH SWITCH REQUEST.

RANF Issue #2: Proactive Feedback during handover (XnAP)

In XnAP, the BAT Window and Periodicity Range IEs are not needed in the TSC Assistance Information IE for Xn-based handover.

RANF Issue #3: NR-DC case

RAN feedback for NR-DC case is not supported in Rel-18

RANF Issue #4: F1AP impacts of RAN Feedback

gNB-DU can determine RAN Feedback for both Proactive and Reactive

- Include the Burst Arrival Time Window, Capability for BAT Adaptation and (optional) Periodicity Range in the TSC Assistance Information IE.

- Include the TSC Traffic Characteristics Feedback (including the Burst Arrival Time Offset and the optional Adjusted Periodicity) in the following messages:

i.	UE CONTEXT SETUP RESPONSE message (for proactive)

ii.	UE CONTEXT MODIFICATION RESPONSE message (for proactive)

iii.	NOTIFY message (for reactive)

# References

R3-235147 (TP for TS 38.413 BL CR) Resolution of open issues for timing resiliency (Nokia, Nokia Shanghai Bell)

R3-235148 (TP for TS 38.413 BL CR) Resolution of open issues for URLLC (Nokia, Nokia Shanghai Bell)

R3-235171 (TP to TRS_URLLC BLCR for TS 38.413, TS 38.423 and TS 38.473) Support of 5G Timing Resiliency enhancements (Huawei, China Unicom)

R3-235172 (TP to TRS_URLLC BLCR for TS 38.413, TS 38.423 and TS 38.473) Support of RAN feedback enhancements (Huawei, China Unicom)

R3-235173 (TP to TRS_URLLC BLCR for TS 38.413) Support of TSN enabled transport network (Huawei, China Unicom, China Telecommunication)

R3-235351 TP for 38.300 on Introduction of 5G Timing Resiliency and URLLC enhancements (CATT)

R3-235352 Discussion on Adapting downstream and upstream scheduling (CATT)

R3-235374 Discussion on open issues for RAN feedback (Samsung)

R3-235468 Discussion on Open Issues in Timing Resiliency and uRLLC (Qualcomm Incorporated)

R3-235469 (TP to TS 38.300) Introduction of 5G Timing Resiliency and URLLC enhancements (Qualcomm Incorporated)

R3-235574 Support NR Timing Resiliency and URLLC enhancements (Ericsson)

R3-235575 Further discussion on Support NR Timing Resiliency and URLLC enhancements (Ericsson)

R3-235670 Discussion and TPs for timing synchronization status and reporting (ZTE)

R3-235671 Discussion on TSN integration and RAN feedback (ZTE)