3GPP TSG-RAN WG3 Meeting #121bis	R3-235841

Xiamen, China, 9 – 13 Oct, 2023

Title: 	(TP for TS38.425) Support for ECN marking

Source: 	CMCC, ZTE, Nokia, Nokia Shanghai Bell, Huawei

Agenda item:	25.2.2

Document Type:	Other

1. Introduction

This contribution proposes the TP for TS38.425 to support for ECN marking, including following agreements achieved in RAN3#121bis meeting.

For TS 38.415 (PDU TYPE 1) and TS 38.425 (PDU TYPE 2), two new presence flags are needed. FFS on the details (names and descriptions).

There is a single piece of information produced by NG-RAN to address ECN marking for L4S at UPF and congestion information exposure.

Agree the following TPs:

R3-23xxxx, revision of R3-235379, TP for TS38.415 (ZTE)

R3-23xxxx, TP for TS38.425 (CMCC)

R3-23xxxx, TP for TS38.423 (Ericsson)

R3-23xxxx, TP for TS38.413 (Nokia)

R3-23xxxx, revision of R3-235425, TP for TS38.473 (HW)

R3-23xxxx, revision of R3-235206, TP for TS37.483 (Samsung)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

5.5.2.3	ASSISTANCE INFORMATION DATA (PDU Type 2)

This frame format is defined to allow the node hosting the PDCP entity to receive assistance information.

The following shows the respective ASSISTANCE INFORMATION DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.3-1 are also applicable to E-UTRA PDCP unless specified otherwise in section 5.5.3.

| Bits |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 |  | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=2) |  |  |  | PDCP Dupl. Ind. |  | Assistance Info. Ind. |  | UL Delay Ind. | DL Delay Ind. | 1 |
| Spare |  |  |  |  |  |  | Uplink Information Flag (Name FFS) | Downlink Information Flag (Name FFS) | PDCP Duplication Activation Suggestion | 1 |
| Number of Assistance Information Fields |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Assistance Information Type |  |  |  |  |  |  |  |  |  | 0 or (2*Number of Assistance Info Fields + sum of Number of octets for Radio Quality Assistance Information Fields) |
| Number of octets for Radio Quality Assistance Information Fields |  |  |  |  |  |  |  |  |  |  |
| Radio Quality Assistance Information |  |  |  |  |  |  |  |  |  |  |
| UL Delay DU Result |  |  |  |  |  |  |  |  |  | 0 or 4 |
| DL Delay DU Result |  |  |  |  |  |  |  |  |  | 0 or 4 |
| Uplink Information |  |  |  |  |  |  |  |  |  | 0 or 2 |
| Downlink Information |  |  |  |  |  |  |  |  |  | 0 or 2 |

Figure 5.5.2.3-1: ASSISTANCE INFORMATION DATA (PDU Type 2) Format

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 5.5.3.x1	Uplink Information Flag

Description: FFS.

Value range: FFS.

Field length: 1 bit.

#### 5.5.3.x2	Downlink Information Flag

Description: FFS.

Value range: FFS.

Field length: 1 bit.

#### 5.5.3.x3	Uplink Information

Description: FFS.

Value range: FFS.

Field length: 2 octets.

#### 5.5.3.x4	Downlink Information

Description: FFS.

Value range: FFS.

Field length: 2 octets.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>