3GPP TSG RAN WG3 121-bis	R3-235033

Xiamen, China, 09 - 13 October, 2023

3GPP TSG-SA5 Meeting #150 	S5-235782

Goteborg, Sweden, 21 - 25 August 2023

Title:	Reply LS on area scope for QoE measurements

Response to:	R2-2306569/S5-235027

Release:	Release 18

Work Item:	eQoE

Source:	SA5

To:	RAN2

Cc:	SA4, RAN3

Contact person:	Xiaoli Shi

Shixiaoli@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA5 thanks RAN3 for the LS on on area scope for QoE measurements.

SA5 has the following responses to RAN2's questions:

Q1) RAN2 would like to ask SA4 and SA5 if the restriction specified in TS 26.247 and TS 26.114 means that LocationFilter cannot be configured together with Area Scope of QMC in NGAP signalling. If so, can this restriction be removed? as RAN2 would like to clarify that even when Area Scope of QMC is not used for area scope checking, it is still useful for RAN to help the gNB select proper UEs for QoE measurements configuration.

Answer: From SA5 point of view, SA5 does not foresee any issues in case of Area Scope of QMC is provided over NGAP signalling while the area scope filtering is handled by the UE Using LocationFilter.

Q2) RAN2 would also like to ask SA4/SA5/RAN3 whether there is a problem if for UEs in RRC CONNECTED the network performs area scope checking (with Area Scope of QMC) and UE application also performs area scope checking (with LocationFilter) at the same time. It should be noted that area scope management for UEs in RRC CONNECTED in Rel-17 relies on the gNB releasing the QoE configuration when the UE moves out of the applicable area scope.

Answer: As Area Scope is used first by the base station to select the call/session, and the LocationFilter is used by the UE some (short) time after the QMC request is received in the UE. From SA5 point of view, SA5 does not see the problems for the duplicated area scope filtering.

Q3) RAN2 would like to ask SA4/SA5 if it is feasible to include PLMN/TA information in LocationFilter?

Answer: From the perspective of SA5, QMC supports the Area Scope configuration of the PLMN and TA. Therefore, SA5 considers that the LocationFilter configuration of the PLMN and TA is also feasible. However, whether the UE application layer can obtain the PLMN and TA information depends on the technical feasibility of SA4.

# 2	Actions

To RAN2:

ACTION: 	SA5 would like to ask RAN2 to take above answer into account.

# 3	Dates of next TSG SA WG 5 meetings

SA5#151		9 -13 October 2023		Xiamen, China

SA5#152		13 - 17 November 2023		Chicago, US