3GPP TSG RAN WG3 121-bis	R3-235035

Xiamen, China, 09 - 13 October, 2023

3GPP TSG-SA WG6 Meeting #56	S6-232609

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232527)

Title:	Reply to LS addressing packet loss

Response to:	(S6-232240/S2-2307982) on addressing packet loss during multicast MBS delivery

Release:	Rel-18

Work Item:	MCOver5MBS

Source:	3GPP TSG SA WG6

To:	SA2, RAN2

Cc:	RAN3, SA1

Contact person:	Rana Alhalaseh

Rana.alhalaseh@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA6 would like to thank SA2 for their reply, and would like to answer their concern and question (see excerpt below):

“… SA2 would like to check with SA6 whether there is concern on the latency of first packet(s) transmission resulted by the MB-UPF buffering and what the latency requirement of first packet(s) transmission for mission critical service is.”, and

“SA2 kindly asks SA6 to provide feedback on the latency aspect for the transmission of the first downlink packet(s).”

SA6 would like to inform SA2 and RAN2 that there is no explicit requirement for the latency of first packet(s) transmission. However, SA6 would like to draw the attention of SA2 and RAN2 to mission critical service KPIs and requirements in 3GPP TS 22.179 clause 6.15, especially to the following (see excerpt below):

[R-6.15.3.2-012] For group calls where no acknowledgement is requested from affiliated MCPTT group members, the MCPTT Service shall provide an MCPTT Access time (KPI 1) less than 300 ms for 95% of all MCPTT Request.

[R-6.15.3.2-013] For MCPTT Emergency Group Calls and Imminent Peril Calls the MCPTT Service shall provide an MCPTT Access time (KPI 1) less than 300 ms for 99% of all MCPTT Requests.

[R-6.15.3.2-015] The MCPTT Service shall provide a Mouth-to-ear latency (KPI 3) that is less than 300 ms for 95% of all voice bursts.

[R-6.15.3.2-016] There shall be no (0 ms) initial lost audio at receiving user.

Furthermore, regarding NOTE 8 in Table 5.7.4-1 of 3GPP TS 23.501. Please note that SA6 focuses on meeting the previously mentioned KPIs and requirements over any other considerations, e.g., battery saving techniques.

# 2	Actions

To SA2, RAN2

ACTION 1: 	SA6 kindly asks SA2 and RAN2 to take the above information into account when discussing potential solutions.

# 3	Dates of next TSG SA WG 6 meetings

SA6#57                 9th October – 13th October 2023 	Xiamen, China

SA6#58                 13th November – 17th November 2023 	Chicago, US