3GPP TSG-RAN3 Meeting #121	R3-233704

Toulouse, France, 21th Aug - 25th Aug 2023

3GPP TSG-CT WG4 Meeting #116	C4-232397

# Bratislava, Slovakia, 22nd– 26th May 2023

Title:	Reply LS on Network Triggered Service Request for a UE in Suspend State

Response to:	Reply LS (S2-2306031/C4-232027) on Network Triggered Service Request for a UE in Suspend State

Release:	Rel-17

Work Item:	TEI17, 5G_CIoT

Source:	3GPP CT4

To:	3GPP SA2

Cc:	3GPP RAN3

Contact person:	Frank Yong Yang

frank dot yong dot yang at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

CT4 thanks SA2 for the Reply LS on Network Triggered Service Request for a UE in Suspend State.

CT4 discussed two alternatives and considered the following aspects:

-	support of SUSPENDED state for a PDU session has been introduced in the SMF since Rel-16 as part of 5G_CIoT work item (for RRC_SUSPEND with E-UTRAN access), i.e., SMF implementations likely take some implementation specific mechanism to handle such case; and

-	the SMF has been required to invoke Namf_MT MT EnableUEReachability for a very similar case, i.e. receiving subsequent DL Data Notification from the UPF for a PDU session with user plane connection SUSPENDED state for UE enters RRC_Inactive with long eDRX, from Rel-18 as part of NR_REDCAP_Ph2 work item.

CT4 decided to agree, upon receiving subsequent DL Data Notification from the UPF for a PDU session with user plane connection SUSPENDED state:

-	for earlier releases, it is SMF implementation specific to handle such scenario, example implementation options for the SMF to trigger the AMF to page the UE without the AMF sending any N2 SM PDU to the NG-RAN can be, e.g. the SMF can invoke the N1N2MessageTransfer service operation with N2 information containing a PDU Session Resource Modify Request Transfer, but without including the areaOfValidity IE or with an areaOfValidity IE set to the reserved TAI, e.g.,000000, to ensure that the AMF discards the N2 Information and only trigger the paging; the SMF can alternatively invoke the Namf_MT EnableUEReachability service operation to trigger the AMF to page the UE;

-	for Rel-18, the SMF invokes the Namf_MT EnableUEReachability service operation to trigger AMF to page the UE.

In addition, CT4 assumes there is no impact to RAN3 specifications for both alternatives.

# 2	Actions

To SA2

ACTION: 	CT4 kindly asks SA2 to take above information in account and update the SA4 specifications if necessary.

# 3	Dates of next TSG CT WG 4 meetings

CT4#117	21st–25th August 2023		Gothenburg, Sweden

CT4#118	09th–13th October 2023		Xiamen, China