3GPP TSG-RAN WG3 #121						              										R3-233818

21th – 25th Aug 2023

Toulouse, France

Agenda Item:	14.3

Source:	ZTE

Title:	(TP for CHO with NR-DC to TS 38.423, TS37.340): Early data forwarding optimization for CHO with SCG procedure

Document for:	Approval

- Introduction

In the RAN3 #119bis emeeting, RAN3 has achieved the following agreement for data forwarding optimisation.

Data forwarding optimizations should not impact legacy HO mechanism as the fundamental basis.

Data forwarding optimizations focus on how to avoid multiple data forwarding paths.

Additional, a stage 2 TP is agreed in R3-232172, as below.

NOTE 4a0: In CHO with (multiple) SN configurations, the (candidate) SN assigns the same data forwarding addresses for multiple data forwarding requests from different target MNs. If available, the (candidate) SN indicates direct data forwarding path availability with the source node(s). Editor’s note: Details of the indication(s) from the target SN are FFS.

In the RAN3 #120 meeting, RAN3 further achieved the following progress for this topic.

Regarding avoiding duplication of the data forwarding:

RAN3 confirms the problem that is to be solved is avoiding that the single T-SN receives the same data from multiple T-MNs prepared for CHO (assuming there are multiple T-MNs prepared with the same T-SN).

RAN3 agrees to enable the T-SN to let the T-MN know if it has direct path available to S-MN (or to both, S-SN and S-MN).

It is FFS if any enhancements for the case with a single T-MN are needed.

In the RAN3 #120 meeting, the paper R3-232869 indicates two scenarios shall be supported.

In case different T-MNs prepare the same T-SN, the following figure shows the potential ways to avoid multiple data forwarding path: Figure 1. different solutions to avoid multiple data forwarding path In the discussion of previous meeting, some companies support solution 1 and solution 2. Considering that for solution 3, if the handover is cancelled by the T-MN which relay the data forwarding, it will be complex to setup another data forwarding path, therefore in the following part, we will focus on solution 1 and 2. From the T-SN point of view, upon receiving the SN Addition Request from the T-MN, it will be able to know the NG-RAN node IDs of the S-MN and S-SN, i.e. via the existing Source M-NG-RAN node ID IE within CHO Information SN Addition IE which indicates the S-MN ID, and the Source NG-RAN Node ID IE which indicates the S-SN ID, and then the T-SN will be able to determine whether it has direct data forwarding path with the S-MN, and with the S-SN. And based on the different availabilities between S-SN and T-SN, between S-MN and T-SN, as shown in Table 1, it is easily to be understood that solution 1 and 2 could apply to same of different scenarios: Table 1. applicability for different solutions in different scenarios Direct path availability; Solution 1; Solution 2. S-SN and T-SN; S-MN and T-SN. Scenario 1; Yes; Yes; Applicable; applicable. Scenario 2; Yes; No; Applicable; Not applicable. Scenario 3; No; Yes; Not applicable; Applicable. Considering that single solution is not applicable to all scenarios, e.g., solution 1 not for scenario 3 and solution2 not for scenario 2 respectively, it is better to enable the network to use both solution 1 and solution 2 rather than only support one of them, and then it is up to the S-MN to make decision to use solution 1 or solution 2, i.e. in case of scenario 3, choose to use solution 2, in case of scenario2, choose to use solution 1, and in case of scenario 1, up to S-MN to decide which one of solution 1 and solution 2. Proposal 1: Enable solution 1 (direct data forwarding between S-SN and T-SN) and solution 2 (Indirect data forwarding via S-MN) to be used in different scenarios. To enable both solution 1 and solution 2, the T-SN needs to indicate the direct data forwarding path availabilities between T-SN and S-SN, and between T-SN and S-MN, and the T-MN needs to indicate to the S-MN about the direct data forwarding path availability between T-SN and S-SN. Figure 2, Direct path availability between the Target SN and Source SN/MN Proposal 2: Introduce a new IE in XnAP: SN Additional Request ACK to indicate to the target MN about the direct path availability between target SN and source MN. Proposal 3: Introduce a new IE in XnAP: HO Request ACK to indicate to the source MN about the direct path availability between target SN and source SN.

This paper further analyze the above solution 1 and solution 2.

- Discussion

  - Solution 1: Data forwarding optiomisation between S-SN and T-SN

In current TS38.423 specification, direct Forwarding Path Availability is included as below.

TS 38.423 #### 9.1.1.1 HANDOVER REQUEST UE Context Reference at the S-NG-RAN node; O. >Global NG-RAN Node ID; M; 9.2.2.3. >S-NG-RAN node UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16. #### 9.1.2.1 S-NODE ADDITION REQUEST Source NG-RAN Node ID; O; Global NG-RAN Node ID 9.2.2.3; The NG-RAN Node ID of the source NG-RAN node or the source SN.. 9.1.2.2 S-NODE ADDITION REQUEST ACKNOWLEDGE Direct Forwarding Path Availability; O; ENUMERATED (direct path available, …); Indicates direct forwarding path is available between the target S-NG-RAN node and source NG-RAN node for intra-system handover or between the target S-NG-RAN node and the source SN..

Observation 1: According to current specification, the source MN can send source SN id to the target MN via HO request message, and then the target MN can send it to the candidate SN via SN addition request message, then target SN can if available, indicate direct forwarding path between the source SN and the target SN via SN addition request ACK message.

At the source side, the bearer can be either MN terminated bearer or SN terminated bearer. If the bearer is anchored at the Source MN (i.e., MN terminated bearer), direct forwarding path between the source SN and the target SN is useless. However, if the bearer is anchored at Source SN (i.e., SN terminated bearer), it is useful.

Currently, upon reception of HO request message, the target MN cannot know each PDU session to be setup is either MN terminated or SN terminated.

Proposal 1: A new IE is added in the XnAP: Handover request message, to indicate each PDU session to be setup is original either MN terminated or SN terminated at source side.

With this new IE, if direct forwarding path between the source SN and the target SN is available, the target MN will not convert DF address received from target SN for the source SN terminated PDU session, but it shall convert DF address received from target SN for the source MN terminated PDU session if direct forwarding path between the source MN and the target SN is not available.

Proposal 2: If direct forwarding path between the source SN and the target SN is available, the target MN will not convert the received DF address for the source SN terminated PDU session.

Proposal 3: If direct forwarding path between the source SN and the target SN is available but direct forwarding path between the source MN and the target SN is not available, the target MN shall convert the received DF address for the source MN terminated PDU session.

If the proposal 1 is agreed, the target MN can know if one or more PDU sessions to be setup are SN terminated at source side. Upon reception of indicator IE which indicates direct forwarding path is available between the target SN node and the source SN node, the target MN shall send this indicator to source MN via HO request ACK message. Then, the source MN will not convert the received DF address to the source SN via Xn-U address indication message.

Proposal 4: Upon reception of indicator IE which indicates direct forwarding path is available between the target SN node and the source SN node via SN addition request ACK message, the target MN shall send this indicator to source MN via HO request ACK message.

Proposal 5: Upon reception of indicator IE which indicate direct forwarding path is available between the target SN node and the source SN node via HO request ACK message, the source MN will not convert the received DF address to the source SN via Xn-U address indication message.

Figure 1: Solution 1 (data forwarding optimisation between S-SN and T-SN) for CHO with SCG procdure

Step 1: Source MN -> Target MN: Handover request message, adding PDU Session from Source SN indication.

The new IE (PDU Session from Source SN indication) is needed, then target MN can decide not to convert the DF address for the PDU session from source SN.

Step 2: Target MN -> Target SN: SN addition request message, including Source SN ID

Step 3: Target SN -> Target MN: SN addition request ACK message, including DF path availability indicator between source SN and target SN, and DF address@T-SN

Step 4: Target MN -> Source MN: Handover request ACK message, adding DF path availability indicator between source SN and target SN, and DF address@T-SN without convertion for PDU Session@source SN.

The new IE (DF path availability indicator between source SN and target SN) is needed, then, the source MN can deide not to convert the received DF address for the PDU session from source SN.

Step 5: Source MN -> Source SN: Xn-U address Indication message, including DF address@T-SN without convertion.

Observation 2：Solution 1 (data forwarding optimisation between S-SN and T-SN) is only used for PDU sesson original anchored at source SN, but it cannot be used for PDU session orginal anchored at source MN.

  - Solution 2: Data forwarding optiomisation between S-MN and T-SN

Both R3-232685 and R3-232869 provide the same method for data forwarding optiomisation between S-MN and T-SN.

TS 38.423 S-NODE ADDITION REQUEST message CHO Information SN Addition; O. >Source M-NG-RAN node ID; M; Global NG-RAN Node ID 9.2.2.3. >Source M-NG-RAN node UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the source M-NG-RAN node.

Since the target SN has acquired source MN id via SN addition request message (seen as above), it can introduce Direct Forwarding Path Availability indication to target MN via SN addition request ACK message if direct data forwarding path is existed.

Proposal 6: In case of CHO with SCG, a new IE (e.g., Additional direct Forwarding Path Availability) is added, to indicate whether direct path between source MN and target SN is available, via SN addition request acknowledge message.

Based on the received S-SN identity within Handover request message, the T-MN can judge whether direct connection is available between S-SN and itself (i.e., T-MN).

--------------------------TS 38.423----------------------------------

9.1.1.2 HANDOVER REQUEST ACKNOWLEDGE

| Direct Forwarding Path Availability | O |  | ENUMERATED (direct path available, …) | Indicates direct forwarding path is available between the target MN node and the source SN node. |
| --- | --- | --- | --- | --- |

--------------------------TS 38.423----------------------------------

Figure 2: Solution 2 (data forwarding optimisation between S-MN and T-SN) for CHO with SCG procdure

Proposal 7: In case of CHO with SCG, a new IE (e.g., Direct Forwarding Path Availability) is added, to indicate whether direct path between source SN and target MN is available, via HANDOVER REQUEST ACKNOWLEDGE message.

Observation 3：Solution 2 (data forwarding optimisation between MN and T-SN) is used for both PDU sesson original anchored at source SN, and PDU session orginal anchored at source MN.

- Conclusion

Based on the above analysis, we provide the following observations and proposals.

Observation 1: According to current specification, the source MN can send source SN id to the target MN via HO request message, and then the target MN can send it to the candidate SN via SN addition request message, then target SN can if available, indicate direct forwarding path between the source SN and the target SN via SN addition request ACK message.

Proposal 1: A new IE is added in the XnAP: Handover request message, to indicate each PDU session to be setup is original either MN terminated or SN terminated at source side.

Proposal 2: If direct forwarding path between the source SN and the target SN is available, the target MN will not convert the received DF address for the source SN terminated PDU session.

Proposal 3: If direct forwarding path between the source SN and the target SN is available but direct forwarding path between the source MN and the target SN is not available, the target MN shall convert the received DF address for the source MN terminated PDU session.

Proposal 4: Upon reception of indicator IE which indicates direct forwarding path is available between the target SN node and the source SN node via SN addition request ACK message, the target MN shall send this indicator to source MN via HO request ACK message.

Proposal 5: Upon reception of indicator IE which indicate direct forwarding path is available between the target SN node and the source SN node via HO request ACK message, the source MN will not convert the received DF address to the source SN via Xn-U address indication message.

Observation 2：Solution 1 (data forwarding optimisation between S-SN and T-SN) is only used for PDU sesson original anchored at source SN, but it cannot be used for PDU session orginal anchored at source MN.

Proposal 6: In case of CHO with SCG, a new IE (e.g., Additional direct Forwarding Path Availability) is added, to indicate whether direct path between source MN and target SN is available, via SN addition request acknowledge message.

Proposal 7: In case of CHO with SCG, a new IE (e.g., Direct Forwarding Path Availability) is added, to indicate whether direct path between source SN and target MN is available, via HANDOVER REQUEST ACKNOWLEDGE message.

Observation 3：Solution 2 (data forwarding optimisation between MN and T-SN) is used for both PDU sesson original anchored at source SN, and PDU session orginal anchored at source MN.

If the above proposals are agreed, the related TPs for 37.340 and 38.423 shall be agreed.

- Text Proposal to 37.340

--------------------------------------------------------- the start of change -------------------------------------------------------------

### 10.19.2	MR-DC with 5GC

The Conditional Handover with Secondary Node procedure is used for configuration and execution of CHO with SN. This procedure includes the cases where the SN is kept, changed or added. If the SN is kept, the UE context at the SN is kept. If the SN is changed, the UE context at the source SN is moved to the target SN.

Figure 10.19.2-1: Conditional Handover with Secondary Node procedure

Figure 10.19.2-1 shows an example signaling flow for Conditional Handover with Secondary Node.

NOTE 1:	For a CHO without SN change, the source SN and the target SN shown in Figure 10.19.2-1 are the same node.

NOTE 2:	For a CHO with SN addition, the source SN and steps involving the source SN in Figure 10.19.2-1 are ignored.

1.	The source MN starts the conditional handover procedure by initiating the Xn Handover Preparation procedure including MCG configuration and, if the UE is configured with an SCG, SCG configuration. The source MN includes the (source) SN UE XnAP ID, SN ID, the UE context in the (source) SN and the Conditional Handover Information Request IE in the Handover Request message.

NOTE 3:	In case of the CHO with/without SN change, the source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration, if configured, before step 1.

2.	If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SN Addition Request message to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SN Addition Request message to the candidate SN including the UE context in the source SN that was established by the source MN. Otherwise, the candidate MN may send the SN Addition Request message to the candidate SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN. Within the SN Addition Request message, the candidate MN also includes the CHO related information, i.e., the source MN ID and the MN UE XnAP ID in the source MN, in order to indicate that the SN Addition Preparation procedure is triggered in relation to a CHO and to enable the SN to identify requests related to the same UE.

NOTE 3a:	The target MN and other potential target MNs may trigger the SN Addition Preparation procedure to the same (target) SN.

NOTE 3b:	The source MN may initiate additional Xn Handover Preparation procedures towards the same or other target MNs. Based on each Xn Handover Preparation procedure, each target MN may decide to trigger SN Addition Preparation procedure.

3.	The (candidate) SN replies with the SN Addition Request Acknowledge message. The (candidate) SN may include the indication of the full or delta RRC configuration.

NOTE 4:	In CHO with SCG configuration, it is up to the candidate MN implementation to make sure that the CG-Config provided from the (candidate) SN can be used in all CHO preparations.

NOTE 4a0:	In CHO with (multiple) SN configurations, the (candidate) SN assigns the same data forwarding addresses for multiple data forwarding requests from different target MNs. If available, the (candidate) SN indicates direct data forwarding path availability with the source node(s).

Editor’s note: Details of the indication(s) from the target SN are FFS.

3a.	For the SN terminated bearers using MCG resources, the candidate MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

4.	The candidate MN includes within the Handover Request Acknowledge message the MN RRC reconfiguration message to be sent to the UE in order to perform the conditional handover, and may also provide forwarding addresses to the source MN. If PDU session split is performed in the target side during handover procedure, more than one data forwarding addresses corresponding to each node are included in the Handover Request Acknowledge message. The candidate MN indicates to the source MN that the UE context in the SN is kept if the candidate MN and the SN decided to keep the UE context in the SN in step 2 and step 3.

NOTE 4a1:	In CHO with (multiple) SN configurations, for the original SN terminated bearer(s), if direct data forwarding path availability is existing either between source SN and target SN or between source MN and target SN, the candidate MN may provide forwarding addresses without conversion.

4a.	The source MN sends the Xn-U Address Indication message to the (source) SN. This Xn-U Address Indication message notifies conditional handover to the (source) SN, which may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an Early Status Transfer message to the source MN.

NOTE 4a:	Separate Xn-U Address Indication procedures may be initiated to provide different forwarding addresses of the prepared conditional handovers. In this case, it is up to the source MN and SN implementations to make sure that the Early Status Transfer message(s) from the source SN, if any, is forwarded to the right target MN. The Xn-U Address Indication procedure may further be initiated to indicate to the (source) SN to stop already initiated early data forwarding for some SN-terminated bearers, if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional handovers.

NOTE 4b:	In CHO with (multiple) SN configurations, for the original SN terminated bearer(s), if direct data forwarding path availability is existing either between source SN and target SN or between source SN and target MN, the source MN may provide forwarding addresses without conversion.

5.	The source MN sends an RRC reconfiguration message to the UE, including the CHO configuration, i.e. a list of RRC reconfiguration* messages and associated execution conditions, in which each RRC reconfiguration* message contains an MCG configuration and possibly an SCG configuration in the RRC reconfiguration** message received from the candidate SN in step 3.

6.	The UE applies the RRC reconfiguration message received in step 5, stores the CHO configuration and replies to the MN with an RRC reconfiguration complete message.

7/8. The UE maintains connection with the source MN and, if the UE is configured with a PSCell, with the source PSCell, after receiving CHO configuration, and starts evaluating the CHO execution condition for the candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source MN, applies the stored corresponding configuration for that selected candidate cell, synchronises to that candidate cell and completes the RRC handover procedure by sending RRC reconfiguration complete* message to the target MN. If the stored configuration for the selected candidate cell includes an SCG configuration, the UE includes an embedded SN RRCReconfigurationComplete** message for the target SN. The UE releases stored CHO configurations after successful completion of RRC handover procedure.

NOTE 5:	In case the target SN includes the indication of the full RRC configuration, the MN performs release of the SN terminated radio bearer configuration and release and add of the NR SCG configuration part towards the UE.

9.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the (target) SN.

NOTE 6:	The order the UE performs Random Access towards the MN (step 7) and performs the Random Access procedure towards the (target) SN (step 9) is not defined.

10.	If the RRC connection reconfiguration procedure was successful, the target MN informs the (target) SN via SN Reconfiguration Complete message.

11.	The target MN sends the Handover Success message to the source MN to inform that the UE has successfully accessed the target cell.

12a/b.	The source MN sends SN Release Request message to the (source) SN including a Cause indicating MCG mobility. The source MN indicates to the (source) SN that the UE context in SN is kept, if it receives the indication from the target MN. The (source) SN acknowledges the release request.

12c.	The source MN sends XN-U Address Indication message to the (source) SN to transfer data forwarding information. More than one data forwarding addresses may be provided if the PDU session is split in the target side.

12d. The source MN sends the Handover Cancel message toward the other signalling connections or other candidate MNs, if any, to cancel CHO for the UE.

12e/f. If the target MN is configured with other candidate PCell(s) associated with other candidate SN(s) than the target SN, the target MN sends the SN Release Request message(s) to the corresponding candidate SN(s). Other candidate MN(s) send(s) the SN Release Request message(s) to other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

13a. The (source) SN sends the Secondary RAT Data Usage Report message to the source MN and includes the data volumes delivered to and received from the UE over the NR/E-UTRA radio as described in clause 10.11.2.

NOTE 7:	The order the source SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SN may send the report when the transmission of the related QoS is stopped.

13b. The source MN sends the Secondary RAT Data Usage Report message to AMF to provide information on the used NR/E-UTRA resource.

14.	For bearers using RLC AM, the source MN sends the SN Status Transfer message to the target MN, including, if needed, SN Status received from the source SN. The target MN forwards the SN Status to the target SN, if needed.

15.	If applicable, data forwarding takes place from the source side (i.e. source MN or source SN). If the SN is kept, data forwarding may be omitted for the SN terminated bearers or QoS flows kept in the SN.

16-19.	The target MN initiates the Path Switch procedure. If the target MN includes multiple DL TEIDs for one PDU session in the Path Switch Request message, multiple UL TEID of the UPF for the PDU session should be included in the Path Switch Ack message in case there is TEID update in UPF.

NOTE 8:	If new UL TEIDs of the UPF for SN are included, the target MN performs MN initiated SN Modification procedure to provide them to the SN.

20.	The target MN initiates the UE Context Release procedure towards the source MN.

21.	Upon reception of the UE Context Release message from source MN, the (source) SN releases C-plane related resources associated to the UE context towards the source MN. Any ongoing data forwarding may continue. The SN shall not release the UE context associated with the target MN if the UE contest kept indication was included in the SN Release Request message in step 12a.

--------------------------------------------------------- the end of change -------------------------------------------------------------

- Text Proposal to 38.423

Start CHANGE

8.2.1	Handover Preparation

8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

<Skip unchanged part>

If the UE Slice-Maximum Bit Rate List IE is contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate value for each S-NSSAI for the concerned UE as specified in TS 23.501 [7].

If the Source S-NG-RAN node ID IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to decide the direct data path availability with the indicated source S-NG-RAN NODE, and if the direct data forwarding path is available, include the Direct Forwarding Path Availability IE in the HANDOVER REQUEST ACKNOWLEDGE message.

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the DRBs transferred to MN IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the S-NG-RAN node in the SN Status Transfer procedure towards the target NG-RAN node, as specified in TS 37.340 [8].

8.3.1	S-NG-RAN node Addition Preparation

8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

<Skip unchanged part>

For each DRB configured as MN-terminated split bearer/SCG bearer, if the QoS Mapping Information IE is included in the DRBs Admitted List IE in the PDU Session Resource Setup Response Info – MN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it to set DSCP and/or IPv6 flow label fields for the downlink IP packets which are transmitted from M-NG-RAN node to S-NG-RAN node through the GTP tunnels indicated by the UP Transport Layer Information IE.

If the Source NG-RAN Node ID IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use it to decide the direct data path availability with the indicated source NG-RAN node, and if the direct data forwarding path is available, include the Direct Forwarding Path Availability IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the Source M-NG-RAN node ID IE is included within the CHO Information SN Addition IE contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, use it to decide the direct data path availability with the indicated source M-NG-RAN node, and if the direct data forwarding path is available, include the Additional direct Forwarding Path Availability IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If for a given QoS Flow the Source DL Forwarding IP Address IE or both, the Source DL Forwarding IP Address IE and the Source Node DL Forwarding IP Address IE are included within the Data Forwarding and Offloading Info from source NG-RAN node IE in the PDU Session Resource Setup Info – SN terminated IE contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store this information and use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

<Skip unchanged part>

9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >Source S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |

9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 |  | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  | – |  |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |
| Direct Forwarding Path Availability | O |  | ENUMERATED (direct path available, …) | Indicates that direct forwarding path is available between the target MN node and the source SN node. | YES | ignore |
| Additional direct Forwarding Path Availability | O |  | ENUMERATED (direct path available, …) | Indicates that direct forwarding path is available between the target SN node and the source MN node. | YES | ignore |

9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PDU Session Resources Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >PDU Session Resources Admitted To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE nor the PDU Session Resource Setup Response Info – MN terminated IE is present in a PDU Session Resources Admitted to be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Setup Response Info – SN terminated | O |  | 9.2.1.6 |  | – |  |
| >>PDU Session Resource Setup Response Info – MN terminated | O |  | 9.2.1.8 |  | – |  |
| PDU Session Resources Not Admitted List | O |  |  |  | YES | ignore |
| >PDU Session Resources Not Admitted List – SN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 |  | – |  |
| >PDU Session Resources Not Admitted List – MN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message or the CG-CandidateList message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | reject |
| Admitted Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Location Information at S-NODE | O |  | Target Cell Global ID 9.2.3.25 | Contains information to support localisation of the UE | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Available fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 is enabled. | YES | ignore |
| Direct Forwarding Path Availability | O |  | ENUMERATED (direct path available, …) | Indicates direct forwarding path is available between the target S-NG-RAN node and source NG-RAN node for intra-system handover or between the target S-NG-RAN node and the source SN. | YES | ignore |
| SCG Activation Status | O |  | 9.2.3.155 |  | YES | ignore |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >Candidate PSCell List |  | 1 |  |  | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| Additional direct Forwarding Path Availability | O |  | ENUMERATED (direct path available, …) | Indicates that direct forwarding path is available between the target SN node and the source MN node. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

9.2.1.1	PDU Session Resources To Be Setup List

This IE contains PDU session resource related information used at UE context transfer between NG-RAN nodes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resources To Be Setup List |  | 1 |  |  | – |  |
| >PDU Session Resources To Be Setup Item |  | 1 .. <maxnoof PDU sessions > |  |  | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>PDU Session Resource Aggregate Maximum Bitrate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 | This IE shall be present when at least one Non-GBR QoS Flow has been setup. | – |  |
| >>UL NG-U UP TNL Information at UPF | M |  | UP Transport Layer Information 9.2.3.30 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs | – |  |
| >>Source DL NG-U TNL Information | O |  | UP Transport Layer Information 9.2.3.30 | Indicates the possibility to keep the NG-U GTP-U tunnel termination point at the target NG-RAN node. | – |  |
| >>Security Indication | O |  | 9.2.3.52 |  | – |  |
| >>PDU Session Type | M |  | 9.2.3.19 |  | – |  |
| >>Network Instance | O |  | 9.2.3.85 | This IE is ignored if the Common Network Instance IE is present. | – |  |
| >>QoS Flows To Be Setup List |  | 1 |  |  | – |  |
| >>>QoS Flows To Be Setup Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>QoS Flow Level QoS Parameters | M |  | 9.2.3.5 |  | – |  |
| >>>>E-RAB ID | O |  | INTEGER (0..15, …) |  | – |  |
| >>>>TSC Traffic Characteristics | O |  | 9.2.3.114 |  | YES | ignore |
| >>>>Redundant QoS Flow Indicator | O |  | 9.2.3.118 |  | YES | ignore |
| >>Data Forwarding and Offloading Info from source NG-RAN node | O |  | 9.2.1.17 |  | – |  |
| >>Additional UL NG-U UP TNL Information at UPF List | O |  | Additional UP Transport Layer Information 9.2.1.32 | Additional UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs | YES | ignore |
| >> Common Network Instance | O |  | 9.2.3.92 |  | YES | ignore |
| >>Redundant UL NG-U UP TNL Information at UPF | O |  | UP Transport Layer Information 9.2.3.30 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs for the redundant transmission | YES | ignore |
| >>Additional Redundant UL NG-U UP TNL Information at UPF List | O |  | Additional UP Transport Layer Information 9.2.1.32 | Additional Redundant UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs | YES | ignore |
| >>Redundant Common Network Instance | O |  | Common Network Instance 9.2.3.92 |  | YES | ignore |
| >>Redundant PDU Session Information | O |  | 9.2.3.112 |  | YES | ignore |
| >>MBS Session Associated Information | O |  | 9.2.1.37 |  | YES | ignore |
| >> PDU Session from source node | O |  | ENUMERATED (source SN, source MN…) | Indicate this PDU Session orginial anchored at either source S-NG-RAN node or source M-NG-RAN node | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

END OF CHANGE