3GPP TSG-RAN WG3 #121						              										R3-233819

21th – 25th Aug 2023

Toulouse, France

Title: 	Discussion on MT-SDT

Source: 	ZTE, Qualcomm Incorporated

Agenda item:	20.2

Document for:	Discussion and Decision

- Introduction

In the last meeting, we have the following progress.

In E1AP: MT-SDT indicator IE within MT-SDT information IE is not needed. In XnAP: MT-SDT data size calculation includes total of both SDT signalling and SDT user plane data. Try to capture it into the TP. In XnAP: Both MT-SDT indicator IE and MT-SDT Data Size IE are “Mandatory”. In F1AP: MT-SDT indicator IE is “Mandatory”. In F1AP: gNB-CU makes the MT-SDT decision, gNB-DU shall follow the decision. When new DL data is coming through non-SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message. FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., Non MT-SDT Data) or other method. When large size of new DL data is coming through SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message. FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., MT-SDT Oversize), or other method. FFS: How to calculate MT-SDT Data Size for SDT DL data packets.

- Discussion

  - Support R18 MT-SDT procedure

In R18 MT-SDT procedure, we have achieved the following progress

When new DL data is coming through non-SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message.

FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., Non MT-SDT Data) or other method.

Proposal 1: When a DL non-SDT data is coming, the gNB-CU-UP shall send DL DATA NOTIFICATION message, adding an explicit indicator to indicate that DL non-SDT data arrvices.

When large size of new DL data is coming through SDT bearer, the gNB-CU-UP shall send DL DATA NOTIFICATION message.

FFS on either excluding MT-SDT Information, or introducing a new indicator (e.g., MT-SDT Oversize), or other method.

Proposal 2: When a large size of DL DT data is coming, the gNB-CU-UP shall send DL DATA NOTIFICATION message, adding an explicit indicator to indicate that large size of DL SDT data arrvices.

It seems a robust design that CU-CP configures the CU-UP with a data volume threshold during the bearer establishment/modification procedure. If the CU-UP detects large amount DL data for the SDT bearer above the indicated threshold then it sends the DL Data Notification with the new indication that the DL Data Volume threshold is crossed. Based on that the CU-CP takes decision to abort the SDT transaction and move the UE to RRC_CONNECTED state.

Proposal 3: DL SDT Data Size threshold shall be introduced in E1: Bearer Context Setup/ Modification message.

In some cases, the gNB-CU-CP may decide to stop ongoing SDT procedure and may move the UE into RRC connected mode or RRC_inactive by sending RRCResume message or RRCRelease message to UE.

In such case, the gNB-CU-CP shall send Bearer context modification message to gNB-UP to resume all DRB (if RRC connected mode) or to suspend SDT DRB (if RRC inactive mode).

Proposal 4: When deciding to stop the ongoing SDT procedure and to move UE into RRC inactive mode, the gNB-CU-CP shall send Bearer Context Modification message, using a new codepoint (e.g., SuspendforSDT) in Bearer Context Status Change IE to suspend SDT DRB.

Proposal 5: When deciding to stop the ongoing SDT procedure and to move UE into RRC connected mode, the gNB-CU-CP shall send Bearer Context Modification message, reusing existing codepoint (resume) in Bearer Context Status Change IE to resume all DRB.

  - How to calculate MT-SDT Data Size for SDT DL data packets.

Currently, the MT-SDT Data Size is sent from gNB-CU-UP to gNB-CU-CP via E1 interfaace and from last serving gNB to the possible receiving gNB via Xn interface. We have agreed that it is the receiving node (i.e., gNB-CU-CP) to decide whether to initiate MT-SDT paging.

From receiving node’s point of view, it does not care of the Data structure, but care of Data load.

Proposal 6: It is out of RAN3’s scope on how to calculate MT-SDT Data Size for SDT DL data packets.

- Conclusion

Based on the above analysis, we provide the following observations and prooposals.

Proposal 1: When a DL non-SDT data is coming, the gNB-CU-UP shall send DL DATA NOTIFICATION message, adding an explicit indicator to indicate that DL non-SDT data arrvices.

Proposal 2: When a large size of DL DT data is coming, the gNB-CU-UP shall send DL DATA NOTIFICATION message, adding an explicit indicator to indicate that large size of DL SDT data arrvices.

Proposal 3: DL SDT Data Size threshold shall be introduced in E1: Bearer Context Setup/ Modification message. Proposal 4: When deciding to stop the ongoing SDT procedure and to move UE into RRC inactive mode, the gNB-CU-CP shall send Bearer Context Modification message, using a new codepoint (e.g., SuspendforSDT) in Bearer Context Status Change IE to suspend SDT DRB.

Proposal 5: When deciding to stop the ongoing SDT procedure and to move UE into RRC connected mode, the gNB-CU-CP shall send Bearer Context Modification message, reusing existing codepoint (resume) in Bearer Context Status Change IE to resume all DRB.

Proposal 6: It is out of RAN3’s scope on how to calculate MT-SDT Data Size for SDT DL data packets.