3GPP TSG-RAN WG3 Meeting #121	R3-233837

Toulouse, France, Aug. 21st – Aug. 25th, 2023

Agenda Item:	13.2

Source: 	Canon Research Centre France

Title:	Discussion on IAB-node mobility

Document for:	Discussion

# Introduction

In this paper, we discuss various aspects related to the mobility of an IAB-node based on the following agreements and identified issues from the previous RAN3 meeting reports [1], [2], [3], and [4]:

At RAN3#118: - The mIAB-MT’s source donor CU can send the info on the mIAB-MT’s target donor CU to the mIAB-DU’s donor CU after the completion of IAB-MT HO. - The mIAB-MT ID sent by the mIAB-MT’s source donor CU to the mIAB-DU’s donor CU is the XnAP UE ID. FFS which donor generates this ID. - The source donor CU for the mIAB-MT HO provides to the donor CU serving the mIAB-DU at least the: • gNB ID of the target donor CU for the mIAB-MT HO. • ID(s) of the mIAB-MT. At RAN3#119: - In case the target logical mIAB-DU’s CU is different from the mIAB-MT’s CU, the target logical mIAB-DU’s CU needs to be informed about the mIAB-MT’s CU ID and the mIAB-MT ID so that it can initiate the Xn TMM procedures towards mIAB-MT’s CU. At RAN3#119bis: - Turn into an agreement the WA stating that: The mIAB-MT and its co-located mIAB-DU can be handed over/migrated to different donor CUs. - Down select between the following two options for providing the gNB-ID of the mIAB-MT’s CU and the XnAP UE ID of the mIAB-MT at this CU to mIAB-DU’s target CU: - Option A: XnAP signalling from the mIAB-DU’s source CU. - Option B: F1AP signalling from the target logical mIAB-DU. For Option B, discuss whether and how the mIAB-DU can obtain the gNB-ID of the mIAB-MT’s CU and the XnAP UE ID of the mIAB-MT at this CU. At RAN3#120: - During network integration where mIAB MT and the co-located mIAB-DU integrates to different donor CUs, mIAB-MT’s UE XnAP ID assigned by the MT´s CU and the gNB-ID of the MT´s CU shall be known to the mIAB-DU’s CU. - It is FFS whether the XnAP ID can be transparently passed from the mIAB-MT’s CU to the mIAB-MT via RRC, then from the co-located mIAB-DU to mIAB-DU’s CU via F1AP. - It is FFS whether the gNB-ID of the MT´s CU is passed from the IAB node to the IAB DU´s CU via F1AP. - WA: As an enhancement to legacy handovers, the IAB-node may provide to the source DU’s CU a mapping between the source DU’s activated cells and the target DU’s activated cells so that the source DU’s CU can perform handover for the connected UEs. This agreement does not relate to the configuration sharing between two logical collocated mIAB-DUs It is FFS whether such mapping information is needed for all activated cells. - To be continued: Option 1: Use existing procedure(s) for triggering F1 Setup and for reporting the outcome of the F1Setup back to the DU´s source CU Option 2: Use new procedure(s) for triggering F1 Setup and for reporting the outcome of the F1Setup back to the DU´s source CU

# Discussion

## Acceptance/Rejection of DU migration

When the DU of a mobile IAB-node is migrated from a source F1 donor-CU toward a target F1 donor-CU, the UEs served by the mobile IAB-node shall also be handed over toward this target F1 donor-CU. Upon reception of a handover request for a UE served by the mobile IAB-node, the target F1 donor-CU may reject the handover. If this happens, the cause of handover preparation failure should not be related to the radio network layer as the traffic associated to the UE is handled by the non-F1 donor-CU serving the co-located mobile IAB-MT. However, the cause may be a control processing overload as a CU can handle a limited number of connected UEs. In case only a few donor-CUs support mobile IAB, those donor-CUs would handle many mobile IAB-nodes and thus many UEs.

A situation where most handover of UEs served by a mobile IAB-node are rejected by the target F1 donor-CU should be avoided, as these UEs, and especially the onboard UEs, will have to be handed over to another cell and they will lose the benefit of the mobile cell. Thus, it may be useful to get the status at the target F1 donor-CU before initiating the first UE handover.

For this purpose, the F1 setup request in the scope of a DU migration may include an information related to the number of UEs served by the mobile IAB-node. This information would help the target donor-CU for the decision to accept or to reject the F1 setup request (due to control processing overload). In case of rejection, another target donor-CU may be selected, or a new attempt may be performed later.

Proposal 1: In the scope of DU migration of a mobile IAB-node, the F1 setup request sent to the target F1 donor-CU may include the number of connected UEs served by the mobile IAB-node.

## Procedures for triggering, executing, and reporting F1 setup at DU migration

To trigger the DU migration of a mobile IAB-node decided by the source F1 donor-CU, the following information may be passed to the mobile IAB-node:

- The indication that a DU migration is requested (mandatory), meaning that a F1 setup procedure shall be initiated with a target logical DU.

- The identifier of the target F1 donor-CU (optional). This information element is relevant when the DU is migrated toward a target F1 donor-CU different from the non-F1 donor-CU serving the co-located mobile IAB-MT. In the absence of such indication, the mobile IAB-node should execute the F1 setup procedure toward the non-F1 terminating donor-CU.

Given the low number of bits for this signalling, it may not be necessary to create a new F1AP procedure. Besides, this is related to the F1 interface management, so the gNB-CU Configuration Update procedure seems appropriate for this signaling.

Proposal 2: The gNB-CU Configuration Update procedure can be used by a source CU to trigger the F1-Setup procedure for the purpose of DU migration toward a target CU.

In the F1 setup request message, the mobile IAB-node should pass the following information to the target F1 donor-CU:

- The indication that the F1 setup request is related to a DU migration. Thanks to this information element, the target F1 donor-CU is aware of the context and can correctly handle the request.

- The PCI(s) of active cell(s) at the source logical DU. This information element helps the target F1 donor-CU to select appropriate PCI(s) for the cell(s) to be activated at the target logical DU.

Proposal 3: In the scope of DU migration of a mobile IAB-node, the F1 setup request sent to the target F1 donor-CU should include the explicit indication that the F1 setup is related to the DU migration of a mobile IAB-node, and it should include the list of PCIs at the source logical DU of this mobile IAB-node.

To report the outcome of the F1 setup procedure for the purpose of DU migration of a mobile IAB-node, the mobile IAB-node may pass the following information to the source F1 donor-CU:

- The indication of success or failure of the F1 setup and, in case of failure, the cause of failure may be indicated.

- The ID(s) of the cell(s) activated by the target F1 donor-CU at the target logical DU.

- The mapping between the IDs of cells activated at the target logical DU and the IDs of cells active at the source logical DU as an optional information element. Thanks to this mapping, it should be possible to limit the range of measurement to be performed at the UEs, it may even avoid measurement reports from the UEs to launch the handover.

As the gNB-CU Configuration Update procedure seems appropriate to trigger the F1 setup, the gNB-DU Configuration Update procedure seems also the good candidate of existing F1AP procedures to report the outcome of the F1 setup.

Proposal 4: The gNB-DU Configuration Update procedure can be used by a mobile IAB-node to report to the source CU of DU migration the outcome of F1 setup procedure toward the target CU.

Proposal 5: The mapping between the IDs of cells activated at the target logical DU and the IDs of cells active at the source logical DU is included as an optional information element in the report of F1 setup outcome.

## Providing identification information to a F1 donor-CU

At DU migration of a mobile IAB-node toward a target F1 donor-CU different from the non-F1 donor-CU of the collocated mobile IAB-MT, the identifier of the non-F1 donor-CU and the XnAP UE ID of the mobile IAB-MT at this CU shall be provided to the target F1 donor-CU. Thanks to these information elements, the target F1 donor-CU will be able to execute the Transport Migration Management procedure toward the non-F1 donor-CU.

From [3], we have the following agreement:

Down select between the following two options for providing the gNB-ID of the mIAB-MT’s CU and the XnAP UE ID of the mIAB-MT at this CU to mIAB-DU’s target CU:

- Option A: XnAP signalling from the mIAB-DU’s source CU.

- Option B: F1AP signalling from the target logical mIAB-DU.

Besides, we have the new agreement from [4]:

During network integration where mIAB MT and the co-located mIAB-DU integrates to different donor CUs, mIAB-MT’s UE XnAP ID assigned by the MT´s CU and the gNB-ID of the MT´s CU shall be known to the mIAB-DU’s CU.

Considering the network integration case, there are two options:

- Option 1: the mobile IAB-node provides the identification information based on information received from the non-F1 donor-CU (i.e. the mIAB-MT’s CU),

- Option 2: the non-F1 donor-CU provides the identification information once the non-F1 donor-CU knows the identifier of the F1 donor-CU (i.e. the mIAB-DU’s CU) from the mobile IAB-node.

The drawback of option 2 is that it requires the F1 donor-CU to generate a UE XnAP ID for the mIAB-MT, to provide it to the mobile IAB-node that will pass it to the non-F1 donor CU along with the identifier of the F1 donor-CU. This UE XnAP ID generated by the F1 donor-CU will be used by the non-F1 donor-CU in the XnAP message sent to the F1 donor-CU with the option 2. Therefore, option 1 seems to have less specifications impact than option 2.

In case option 1 is adopted and for the sake of consistency, the mobile IAB-node should also provide the identification information related to the non-F1 donor-CU to the target F1 donor-CU at DU migration (option B above). This could be done through the same F1AP signaling as for the network integration case.

Going further, the mobile IAB-node should also provide to its F1 donor-CU the identification information related to the target non-F1 donor-CU at a (consecutive) MT migration, still using the same F1AP signaling.

To cover all the scenarios considered above, the F1AP procedure gNB-DU Configuration Update is appropriate as it can be triggered at any time.

Proposal 6: The gNB-DU Configuration Update procedure can be used by a mobile IAB-node to inform the F1 donor-CU about the identifier of the non-F1 donor-CU and the XnAP UE ID of the mobile IAB-MT at this CU. This procedure may be applied at network integration, DU migration, and MT migration of the mobile IAB-node.

From [1], there is an agreement stating that:

The mIAB-MT’s source donor CU can send the info on the mIAB-MT’s target donor CU to the mIAB-DU’s donor CU after the completion of IAB-MT HO

Actually, the proposal is compatible with this agreement as the source non-F1 donor-CU (i.e. the mIAB-MT’s source donor CU) would pass the information to the F1 donor-CU (i.e. the mIAB-DU’s donor CU), not directly, but through the mobile IAB-node.

Besides the proposal does not preclude that the identifier of the non-F1 donor-CU and the XnAP UE ID of the mobile IAB-MT at this CU can be included in the F1 setup request sent to the target F1 donor-CU at DU migration. The absence of these information elements in the F1 setup request may indicate that the non-F1 donor CU is also the target F1 donor-CU.

Proposal 7: In the scope of DU migration of a mobile IAB-node, the F1 setup request sent to the target F1 donor-CU may also include the identifier of the non-F1 donor-CU and the XnAP UE ID of the mobile IAB-MT at this CU.

# Conclusion

In this contribution, we have discussed the possible rejection of DU migration for a mobile IAB-node, the procedures related to the F1 setup at DU migration, and the possible options to inform a F1 donor-CU about the identifier of the non-F1 donor-CU and the XnAP UE ID of the mobile IAB-MT at this CU. Then, we have made the following proposals:

Proposal 1: In the scope of DU migration of a mobile IAB-node, the F1 setup request sent to the target F1 donor-CU may include the number of connected UEs served by the mobile IAB-node.

Proposal 2: The gNB-CU Configuration Update procedure can be used by a source CU to trigger the F1-Setup procedure for the purpose of DU migration toward a target CU.

Proposal 3: In the scope of DU migration of a mobile IAB-node, the F1 setup request sent to the target F1 donor-CU should include the explicit indication that the F1 setup is related to the DU migration of a mobile IAB-node, and it should include the list of PCIs at the source logical DU of this mobile IAB-node.

Proposal 4: The gNB-DU Configuration Update procedure can be used by a mobile IAB-node to report to the source CU of DU migration the outcome of F1 setup procedure toward the target CU.

Proposal 5: The mapping between the IDs of cells activated at the target logical DU and the IDs of cells active at the source logical DU is included as an optional information element in the report of F1 setup outcome.

Proposal 6: The gNB-DU Configuration Update procedure can be used by a mobile IAB-node to inform the F1 donor-CU about the identifier of the non-F1 donor-CU and the XnAP UE ID of the mobile IAB-MT at this CU. This procedure may be applied at network integration, DU migration, and MT migration of the mobile IAB-node.

Proposal 7: In the scope of DU migration of a mobile IAB-node, the F1 setup request sent to the target F1 donor-CU may also include the identifier of the non-F1 donor-CU and the XnAP UE ID of the mobile IAB-MT at this CU.

# References

[1] Chairman notes of RAN3#118 meeting

[2] Chairman notes of RAN3#119 meeting

[3] Chairman notes of RAN3#119-bis-e meeting

[4] Chairman notes of RAN3#120 meeting