3GPP TSG-RAN WG3#121	　　　　　　　　　　　　　　　　　R3-233871

Toulouse, France, 21 – 25th August, 2023

Agenda Item:  14.4

Source:       NEC

Title:		Discussion Selective Activation of the cell of groups

Document for:	decision

Introduction

RAN3#120 (May 2023) on the discussion on the Selective Activation of Cell Group, with still some issues on the table. We continue to discuss and try to provide solution to progress the work.

- 14.4. Others RAN2 led To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements: To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4] ..... RAN3#120: Add a new IE as a sub IE of the Conditional PSCell Addition Information Request IE in the S-NODE ADDITION REQUEST message to indicate that the request is for SCG Selective Activation. It is FFS (up to RAN2 progress) what form the indication will have (an explicit IE or an RRC container). Note: This can be revisited based on RAN2 progress. Introduce a new indicator for selective activation in the S-NODE MODIFICATION REQUEST message to indicate that the request is for SCG selective activation. FFS on the scenarios. Introduce a new indicator for selective activation in the Xn-U address indication message to indicate that the request is for SCG selective activation. FFS on the scenarios.

Discussion

The reference configuration

RAN2#122 has agreed “The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures”, this reference configuration while RAN2 is still FFS which node to generate, it has been agreed that this reference configuration will be provided by the MN to all candidate T-SNs. From RAN3 perspective, it can be easily decided to introduce such reference configuration as a RRC container in at least S-NODE ADDITION REQUEST message. It has been raised by companies’ contributions that the Reference Configuration can be a separated inter-node message and therefore it can be a separated new RRC container IE specific for selective activation. While only to add semantic description in the existing M-NG-RAN node to S-NG-RAN node Container IE is also workable, we also think that it would be clear to have new IE for the Reference Configuration specific for selective activation.

Proposal 1: Introduce new RRC Container IE to transfer Reference Configuration specific for Selective Activation in S-NODE ADDITION REQUEST message.

RAN3#119bis-e For inter-SN SCG selective activation, after CPC execution, the MN needs to notify the source SN and the selected SN of the cell change. FFS how to notify the source SN and the selected SN.

The FFS has not be solved yet in last RAN3#120. This FFS is on inter-SN selective activation, after CPC execution i.e. after the UE has triggered CPC for a candidate target PSCell and the source MN received a second RRCReconfigurationComplete message from the UE, how to notify the source SN and other selected SN.

In the Rel-17 Conditional SN Change signalling procedures, quoted below (for reference):

Quoted from 37.340v17.4.0 Figure 10.5.2-3, “6a-6c. The MN triggers the MN initiated SN Release procedure to inform the source SN to stop providing user data to the UE, and if applicable, triggers the Xn-U Address Indication procedure to inform the source SN the address of the SN of the selected candidate PSCell, to start late data forwarding.” “7a-7c. If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SN Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.”

After the CPC execution, for the informing the source SN, there may be some possible ways to do depending on MN decision whether to keep the source SN (i.e. to keep the SCG configuration for subsequent CPC) or not to keep the source SN.

- If MN decide not to keep source SN after the CPC execution, it is to use the MN initiated SN Release procedure to inform the source SN;

Proposal 2: For inter-SN SCG selective activation, if MN decide not to keep source SN after the CPC execution, it is to use the MN initiated SN Release procedure to inform the source SN;

- If MN decide to keep the source SN after the CPC execution, there are two possibilities:

Way 1: to use the MN initiated SN Release procedure with indicator to inform the source SN to keep the SCG configuration;

Way 2: to use MN initiated SN Modification Preparation procedure with indicator to inform the source SN to keep the SCG configuration. With the indicator, the source SN can release e.g. transport layer resource.

Observation: After the CPC execution when MN decide to keep source SN i.e. to keep the SCG configuration for subsequent CPC, there are some possibilities to inform the source SN.

Either way will be workable if indicator will be introduced for backward compatible reason, therefore we propose:

Proposal 3: For inter-SN SCG selective activation, if MN decide to keep the source SN after the CPC execution, RAN3 to decide to take a way (either use of MN initiated SN release procedure or MN initiated SN Modification Preparation procedure) to inform the source SN to keep the SCG configuration, while source SN will release other resources.

After the CPC execution, for other candidate SN(s) than the selected SN, since the Rel-18 Selective Activation of Cell Group is to keep the prepared SCG configuration therefore there is no necessary to release the other candidate SN(s).

Proposal 4: For inter-SN SCG selective activation, after the CPC execution, the candidate SN other than the selected SN(s) that will be kept without any need of informing.

After the CPC execution, for the selected SN, the MN can notify the selected Target SN by SN Reconfiguration Complete procedure.

Proposal 5: For inter-SN SCG selective activation, after CPC execution, the MN to notify the selected Target SN by the SN Reconfiguration Complete procedure.

The coding and the name of the indicator in the SN Addition request message as in baseline CR (R3-233549) is still FFS, we propose to conclude it.

Proposal 6: propose to conclude the coding and the name of the indicator in the SN Addition request message as in baseline CR (R3-233549)

Proposals

Proposal 1: Introduce new RRC Container IE to transfer Reference Configuration specific for Selective Activation in S-NODE ADDITION REQUEST message.

Proposal 2: For inter-SN SCG selective activation, if MN decide not to keep source SN after the CPC execution, it is to use the MN initiated SN Release procedure to inform the source SN;

Proposal 3: For inter-SN SCG selective activation, if MN decide to keep the source SN after the CPC execution, RAN3 to decide to take a way (either use of MN initiated SN release procedure or MN initiated SN Modification Preparation procedure) to inform the source SN to keep the SCG configuration, while source SN will release other resources.

Proposal 4: For inter-SN SCG selective activation, after the CPC execution, the candidate SN other than the selected SN(s) that will be kept without any need of informing.

Proposal 5: For inter-SN SCG selective activation, after CPC execution, the MN to notify the selected Target SN by the SN Reconfiguration Complete procedure.

Proposal 6: propose to conclude the coding and the name of the indicator in the SN Addition request message as in baseline CR (R3-233549)

References

[1] RP-231475 WID Further NR Mobility Enhancements

[2] R3-233359 Summary of Offline Discussion on MobilityEnh3_SelectiveActivation

[3] R3-233549 (SCGSelective Activation BL CR to TS 38.423) Introduction of SCG Selective Activation

ANNEX TS 37.340v17.5.0 signalling of Conditional SN change procedure - MN initiated

MN initiated conditional SN Change

The Conditional Secondary Node Change procedure is initiated by the MN for inter-SN CPC configuration and inter-SN CPC execution.

Figure 10.5.2-3: Conditional SN change procedure - MN initiated

Figure 10.5.2-3 shows an example signalling flow for the conditional SN Change initiated by the MN:

1/2.	The MN initiates the conditional SN change by requesting the candidate SN(s) to allocate resources for the UE by means of the SN Addition procedure, indicating that the request is for CPAC. The MN also provides the candidate cells recommended by MN via the latest measurement results for the candidate SN(s) to choose and configure the SCG cell(s), provides the upper limit for the number of PSCells that can be prepared by the candidate SN. Within the list of cells as indicated within the measurement results indicated by the MN, the candidate SN decides the list of PSCell(s) to prepare (considering the maximum number indicated by the MN) and, for each prepared PSCell, the candidate SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRCReconfiguration** message contained in the SN Addition Request Acknowledge message with the prepared PSCell ID(s). If data forwarding is needed, the candidate SN provides data forwarding addresses to the MN. The candidate SN includes the indication of the full or delta RRC configuration. The candidate SN can either accept or reject each of the candidate cells listed within the measurement results indicated by the MN, i.e. it cannot configure any alternative candidates.

NOTE 4:	The MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration and to allow provision of data forwarding related information before step 1.

2a.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message to the candidate SN(s).

3.	The MN sends to the UE an RRCReconfiguration message including the CPC configuration, i.e. a list of RRCReconfiguration* messages and associated execution conditions, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 2 and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, e.g., to configure the required conditional measurements.

4.	The UE applies the RRCReconfiguration message received in step 3, stores the CPC configuration and replies to the MN with an RRCReconfigurationComplete message. In case the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message, it performs the reconfiguration failure procedure.

4a.	Upon receiving the MN RRCReconfigurationComplete message from the UE, the MN informs the source SN that the CPC has been configured via Xn-U Address Indication procedure, the source SN, if applicable, together with the Early Status Transfer procedure, starts early data forwarding. The PDCP SDU forwarding may take place during early data forwarding.

NOTE 4a:	Separate Xn-U Address Indication procedures may be invoked to provide different forwarding addresses of the prepared candidate target SNs. In this case, it is up to the MN and the source SN implementations to make sure that the EARLY STATUS TRANSFER message(s) from the source SN, if any, is forwarded to the right target destination. The Xn-U Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some SN-terminated bearers if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional SN change procedures.

NOTE 4b:	For the early transmission of MN terminated split/SCG bearers, the MN forwads the PDCP PDU to the candidate SN(s).

5.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an NR RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell.

6a-6c.	The MN triggers the MN initiated SN Release procedure to inform the source SN to stop providing user data to the UE, and if applicable, triggers the Xn-U Address Indication procedure to inform the source SN the address of the SN of the selected candidate PSCell, to start late data forwarding.

7a-7c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SN Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

8.	The UE synchronizes to the PSCell indicated in the RRCReconfiguration* message applied in step 5.

9a-9b.	If PDCP termination point is changed for bearers using RLC AM, the source SN sends the message, which the MN sends then to the SN of the selected candidate PSCell, if needed.

10.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the early data forwarding address in step 4a.

11.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 5:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

12-16.	If applicable, a PDU Session path update procedure is triggered by the MN.

17.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

ANNEX: RAN2#122 has the agreement:

For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC. FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC The UE shall skip the condition evaluation for a candidate which is a current PScell. The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures. Will not spend specific efforts for supporting nested configurations for candidate cell configuration. Rapporteur take initiative on naming offline Terminology is “Subsequent CPAC”