3GPP TSG-RAN WG3#121	　　　　　　　　　　　　　　　　　R3-233876

Toulouse, France, 21 – 25th August, 2023

Agenda Item:  17.2

Source:       NEC

Title:		Further discussion on TAC exchange over Xn for NR NTN

Document for:	Discussion

Introduction

In the RAN3#120 meeting, we achieved some agreements:

Change the WA to the agreement:

Uu Cell ID should be used in Xn Setup and Configuration Update procedures.

WA: Do not exchange multiple TACs over Xn for NTN.

How to describe which TAC should be used in semantic description?

This contribution discusses the left issues in NR NTN.

Discussion

Discussion on WA: Do not exchange multiple TACs over Xn for NTN

In the last meeting, RAN3 agreed that the Uu Cell ID is expected to be used in Xn Setup and Configuration Update procedures. Since the large beam coverage from NR NTN, one Uu cell may cover multiple TACs, and the maximum number of TACs per NR NTN cell is up to 12. Previous related RAN2 agreements are listed below.

In NTN, the network may broadcast more than one TACs per PLMN in a cell, which is to up to network implementation.

There will be max 12 TACs per NR NTN cell, including same or different PLMNs.

A working assumption also achieves that the multiple TACs are not exchanged over Xn for NTN, which means only single TAC can be transmitted between NTN gNBs in the Xn Setup procedure, NG-RAN node Configuration Update procedure. We understand that this working assumption keeps legacy behavior and reduces the signaling overhead caused by TACs.

However, as mentioned above, one Uu Cell can cover multiple TACs. If only single TACs are transmitted during the Xn procedures, there is no existing solution to handle how to select one TAC from multiple TACs of a cell. Also, the Xn Setup procedure and NG-RAN node Configuration Update procedure are non-UE-associated signaling, so it is not suitable to use the TAC of the UE current located in. From our side, it is hard to specify the criteria on how gNB chooses one from multiple TACs used for Xn procedures. And it is better to left to gNB implementation.

Observation 1: It is hard to specify the criteria for how gNB chooses one TAC from multiple TACs used for Xn procedures.

Observation 2: The TAC-related issues should depend on gNB implementation.

If this TACs issue is left to gNB implementation. In that case, it is better to exchange multiple TACs over the Xn interface, and our Technique Specification does not limit the gNB that how many TACs should be exchanged over Xn for NTN. It gives more flexibility to NTN gNB.

Observation 3: Exchanging multiple TACs over the Xn interface gives more flexibility to gNB implementation.

Proposal 1: Xn interface should have the capability to exchange multiple TACs for NTN.

Conclusion and Proposal

Our proposals are summarised below.

Observation 1: It is hard to specify the criteria for how gNB chooses one TAC from multiple TACs used for Xn procedures.

Observation 2: The TAC-related issues should depend on gNB implementation.

Observation 3: Exchanging multiple TACs over the Xn interface gives more flexibility.

Proposal 1: Xn interface should have the capability to exchange multiple TACs.

References