3GPP TSG-RAN WG3 Meeting #121                                                                   R3-233892

Toulouse, France, August 21 – 25, 2023

Agenda item: 		13.2

Source: 		Qualcomm Incorporated

Title: 	(TPs to TS 38.401) Topology adaptation for mobile IAB

Document for:	Discussion

# 1	Introduction

The Chair notes of RAN3#120 captured the following agreements and open issues [1], which reflects the progress on mobile IAB in RAN3:

WA: The mIAB-DU and mIAB-MT can integrate at different CUs. For this purpose, OAM can be used to configure the mIAB-DU with: a) the donor CU to connect to, and b) the parameters used by the mIAB-DU to establish TNL associations, IPSec tunnels and F1 connectivity to this donor CU. To be continued: Whether the information on the DU’s CU can also be configured by the MT’s CU. During network integration where mIAB MT and the co-located mIAB-DU integrates to different donor CUs, mIAB-MT’s UE XnAP ID assigned by the MT´s CU and the gNB-ID of the MT´s CU shall be known to the mIAB-DU’s CU. To be continued: How to pass the UE XnAP ID and NCGI or gNB-ID in case the IAB node knows the information of the DU’s CU by other means than OAM, if agreed. It s FFS whether both gNB-ID and CGI needs to be provided to the mIAB-DU’s CU It is FFS whether the XnAP ID can be transparently passed from the mIAB-MT’s CU to the mIAB-MT via RRC, then from the co-located mIAB-DU to mIAB-DU’s CU via F1AP. It is FFS whether the gNB-ID of the MT´s CU is passed from the IAB node to the IAB DU´s CU via F1AP. To be continued Option 1: Use existing procedure(s) for triggering F1 Setup and for reporting the outcome of the F1Setup back to the DU´s source CU Option 2: Use new procedure(s) for triggering F1 Setup and for reporting the outcome of the F1Setup back to the DU´s source CU WA: As an enhancement to legacy handovers, the IAB-node may provide to the source DU’s CU a mapping between the source DU’s activated cells and the target DU’s activated cells so that the source DU’s CU can perform handover for the connected UEs. This agreement does not relate to the configuration sharing between two logical collocated mIAB-DUs To be continued: It is FFS whether such mapping information is needed for all activated cells. The DU’s CU can initiate the Xn TM Management Procedure pertaining to an mIAB-MT even though it has never had an RRC connection with this IAB-MT. The mIAB-DU’s NCGI is configured by OAM, and, e.g. to avoid CGI collision, it may be re-configured by the donor CU via F1 based on a list of NCGIs that has been configured on this donor CU by OAM or by pre-configuration. This should not affect the existing procedure of configuring NCGI of cells served by a stationary DU via OAM. The underlying assumption is that the DU´s OAM has visibility on the result of the CU-based CGI re-configuration. It needs to be further discussed how to ensure that such observability is supported. RAN3 to send an LS to SA5 including the content of the agreement. Explain the status quo in RAN3 concerning the use cases discussed. Ask SA5 how to ensure that OAM has visibility over the CGI reconfiguration decisions, as well as on feasibility and feedback of the solution and requirements agreed. To be further discussed whether any further questions need to be posed to SA5 To be continued: Use cases requiring CGI re-configuration independent of CGI collision events needs to be further discussed and proven. If no other use cases than CGI collisions can be identified, the proposal above applies only to CGI collision avoidance Include “No PDU Session Indication” in the NGAP HANDOVER REQUEST message. WA: legacy NG HO Required message with “dummy” PDU information is used. It needs to be further discussed if any specification changes are needed to describe the use of such dummy information. If specification changes is needed, consider whether an explicit indication is worth adding. To be continued: RAN3 to introduce NGAP inter-gNB information exchange procedure(s) for exchange of IAB TMM messages and IAB Resource Coordination messages RAN3 to decide whether this procedure is generic, i.e., it can carry any information, or IAB-specific, i.e., limited to IAB-specific purposes. After mIAB-DU migration, the BH RLC and BAP routing configurations used in the non-F1 terminating topology for TMM with the mIAB-DU’s source CU may be released. To be continued: Discussions on enhancements to legacy HO procedures to be taken at RAN3-121. If no consensus on such enhancements, the following will be agreed: For DU migration, the following baseline is enough for target CU being aware of the QoS profile for each UE traffic: The target CU for mIAB-DU migration learns the traffic profile of the UE traffic from Handover Preparation procedures for individual UEs. No further enhancements are needed.

In this document, based on above agreements and open issues, the effort is made to define baseline ST2 procedures for

mIAB-node network integration, sequential partial migration of the mIAB-MT, and DU migration.

# 2	Discussion

RAN3 has converged on the following framework for mobile IAB:

- mIAB-DU migration and mIAB-MT migration can occur independently.

- mIAB-DU migration can occur between different CUs than mIAB-MT migration.

- As a WA, mIAB-DU and mIAB-MT can integrate at different CUs.

- mIAB-MT migration can occur via NG in case Xn is not available.

Proposal 0: With only 3 meeting until functional freeze, RAN3 needs to make progress on defining baseline ST2 procedures for mobile IAB and consider all open issues as enhancements.

The following subsections discuss the baseline procedures for network integration, MT migration and DU migration based on the status quo of the present agreements, and it identifies the open issues to be considered as enhancements. These enhancements are discussed in a separate tdoc.

## 2.1	ST2 for network integration of mIAB-node

RAN3 agreed to the working assumption that mIAB-MT and mIAB-DU can integrate at separate CUs. The mIAB-DU’s CU can further be selected by the mIAB-node’s OAM. In this case, all information needed for F1 establishment with this CU is configured by the mIAB-node’s OAM.

This WA is in line with RAN3’s prior agreement for DU migration, where the mIAB-node’s OAM selects the target IAB-donor-CU and provides all information for F1establishment with this CU. RAN3 agreed that DU migration could also be triggered by the source F1-terminating CU. However, there is not equivalent solution for network integration of the mIAB-DU.

RAN3 did not converge on other solutions for the selection of the mIAB-DU’s CU during network integration. For that reason, the OAM-based selection of the mIAB-DU’s CU is the only solution that can be considered St2 baseline for the mIAB-node network integration procedure. Other solutions can be considered as enhancements at later stage.

Observation 1a: For network integration, OAM-based selection of the mIAB-DU’s CU is presently the only available solution for ST2 baseline. Other solutions can be considered as enhancements.

RAN3 agreed that during network integration with different IAB-donor-CUs for mIAB-MT and mIAB-DU, the mIAB-MT’s UE XnAP ID assigned by the MT´s CU and the gNB-ID of the MT´s CU have to be known by the mIAB-DU’s CU. This information is needed by the mIAB-DU’s CU to initiate the Xn TMM procedures and the IAB Resource Coordination procedures with the mIAB-MT’s CU. Without support of TMM procedures, the backhaul can still operate, but it cannot provide QoS support. Without inter-donor resource coordination, access and backhaul can still operate, e.g., by using different frequencies. Therefore, neither TMM procedures nor IAB Resource Coordination procedures are imperative to support a baseline mobile IAB-operation.

Observation 1b: The mIAB-DU’s CU needs the information on the MT’s XnAP UE ID and the gNB ID of the mIAB-MT’s CU solely to initiate the TMM- and IAB Resource Coordination procedures with the mIAB-MT’s CU.

Observation 1c: The TMM- and the IAB Resource Coordination procedures are not necessary for baseline operation of mobile IAB even if mIAB-MT and mIAB-DU use separate CUs.

Observation 1d: All information exchange to enable inter-donor TMM- and IAB Resource Coordination procedures is not needed for St2 baseline and can be considered as enhancement.

Based on Observations 1a – c, a baseline ST2 procedure for network integration of mIAB can be defined as shown in the Annex.

Proposal 1: Agree TP to TS 38.401 on the baseline procedure for network integration of the mobile IAB-node provided in the Annex.

## 2.2	ST2 for sequential migration of mIAB-MT via Xn

RAN3 agreed that sequential MT migration and DU migration can be captured as separate ST2 procedures.

The procedure for sequential migration of the mIAB-MT via Xn can adopt many steps of the Rel-17 partial migration procedure for IAB.

RAN3 agreed that the mIAB-MT’s source CU sends the gNB-ID of the target mIAB-MTs CU and the mIAB-MT’s XnAP UE ID selected by the mIAB-MT’s target CU to the mIAB-DU’s CU. This information exchange allows the F1-terminating CU to trigger DU migration or to initiate the TMM- and IAB Resource Coordination procedures with the mIAB-MT’s target CU.

There has been no discussion on the signaling to be used for this information transfer. There has further been no discussion on how the mIAB-MT’s source CU actually knows about the mIAB-MT’s XnAP UE ID selected by the mIAB-MT’s target CU. Based on RAN3’s difficulty to make progress on the equivalent information exchange for mIAB network integration, it is uncertain whether such discussion, when conducted, will converge.

However, as captured in Observations 1a-d, TMM- and IAB Resource Coordination procedures are not necessary for baseline operation of mobile IAB. Further, as discussed in section 2.4, CU-triggered DU migration is not considered part of the ST2 baseline either. For these reasons, the ST2 baseline procedure for sequential mIAB-MT migration via Xn can be defined without this signaling as shown in the Annex.

Proposal 2: Agree TP to TS 38.401 on the baseline procedure for sequential mIAB-MT migration via Xn provided in the Annex.

## 2.3	ST2 for sequential migration of mIAB-MT via NG

RAN3 agreed that sequential mIAB-MT migration via NG is supported. RAN3 has not discussed how the TMM- and IAB Resource Coordination procedures are supported via NG in case Xn is not available. However, based on Observations 1a-d, these Xn procedures are not necessary for the baseline operation of mobile IAB, and for that reason, the St2 baseline sequential procedure for mIAB-MT migration via NG can be defined as shown in the Annex.

Proposal 3: Agree TP to TS 38.401 on the baseline procedure for sequential mIAB-MT migration via NG provided by the Annex.

## 2.4	ST2 for mIAB-DU migration

RAN3 agreed that DU migration can occur between different IAB-donors than used by MT migration.

RAN3 further agreed that the mIAB-node’s OAM can trigger DU migration, select the target mIAB-DU’s CU and provide the necessary information for F1 establishment to the mIAB-node.

RAN3 also agreed that the source mIAB-DU’s CU can trigger DU migration, select the target mIAB-DU’s CU and provide the necessary information for F1 establishment. RAN3, however, has not been able to converge on the signaling procedures used by the source mIAB-DU’s CU to convey this information to the mIAB-node. Hence, CU-triggered DU migration cannot be included in the ST2 baseline DU migration procedure but considered as an enhancement for further discussion.

Observation 2a: Since DU migration can be triggered by the mIAB-node’s OAM, CU-triggered DU migration is not necessary for the ST2 baseline DU migration procedure and can be considered an enhancement.

RAN3 further agreed that the outcome of the target mIAB-DU’s F1 setup procedure including information on the cells activated by the target mIAB-DU’s CU should be reported back by the mIAB-node to the source mIAB-DU’s CU. This information allows the source mIAB-DU’s CU to initiated UE handover to the target mIAB-DU’s CU without the need for UE measurement reports. RAN3, however, was not able to converge on the signaling solution for this reporting. In absence of such signaling, UE handover can still be conducted by using UE measurement reports. This signaling is therefore not necessary and can be considered an enhancement to DU migration.

Observation 2b: The reporting to the source IAB-DU’s CU on the successful F1 Setup and the activated target cells is not necessary for the ST2 baseline DU migration procedure and can be considered an enhancement.

Based on observations 2a and 2b, the ST2 baseline DU migration procedure can be defined as shown in the Annex.

Proposal 4: Agree TP to TS 38.401 on the baseline procedure for DU migration provided by the Annex.

# Conclusion

This contribution has discussed the baseline ST2 procedures for mIAB-node network integration, sequential partial migration of the mIAB-MT via Xn and via NG, as well as DU migration. The following observations and proposals have been made:

Proposal 0: With only 3 meeting until functional freeze, RAN3 needs to make progress on defining baseline ST2 procedures for mobile IAB and consider all open issues as enhancements.

Observation 1a: For network integration, OAM-based selection of the mIAB-DU’s CU is presently the only available solution for ST2 baseline. Other solutions can be considered as enhancements.

Observation 1b: The mIAB-DU’s CU needs the information on the MT’s XnAP UE ID and the gNB ID of the mIAB-MT’s CU solely to initiate the TMM- and IAB Resource Coordination procedures with the mIAB-MT’s CU.

Observation 1c: The TMM- and the IAB Resource Coordination procedures are not necessary for baseline operation of mobile IAB even if mIAB-MT and mIAB-DU use separate CUs.

Observation 1d: All information exchange to enable inter-donor TMM- and IAB Resource Coordination procedures is not needed for St2 baseline and can be considered as enhancement.

Proposal 1: Agree TP to TS 38.401 on the baseline procedure for network integration of the mobile IAB-node provided in the Annex.

Proposal 2: Agree TP to TS 38.401 on the baseline procedure for sequential mIAB-MT migration via Xn provided in the Annex.

Proposal 3: Agree TP to TS 38.401 on the baseline procedure for sequential mIAB-MT migration via NG provided by the Annex.

Observation 2a: Since DU migration can be triggered by the mIAB-node’s OAM, CU-triggered DU migration is not necessary for the ST2 baseline DU migration procedure and can be considered an enhancement.

Observation 2b: The reporting to the source IAB-DU’s CU on the successful F1 Setup and the activated target cells is not necessary for the ST2 baseline DU migration procedure and can be considered an enhancement.

Proposal 4: Agree TP to TS 38.401 on the baseline procedure for DU migration provided by the Annex.

# References

- Chairman notes, TSG-RAN WG3 Meeting #120, Incheon, Korea, May 2023.

# Annex (TP to TS 38.401)

START OF CHANGES

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Associated QoS Flow: as defined in TS 23.247 [27].

Associated QoS flow information: Information encompassing: QoS flow QoS parameters for associated QoS flows and mapping information between mapped (unicast) QoS flows and associated QoS flows. The respective information is included in a way that non-supporting RAN nodes would not establish respective RAN resources irrespective the multicast session state.

Boundary IAB-node: an IAB-node with one RRC interface terminating at a different IAB-donor-CU than the F1 interface. This definition applies to partial migration, inter-donor redundancy and inter-donor RLF recovery.

Conditional Handover: as defined in TS 38.300 [2].

Conditional PSCell Addition: as defined in TS 37.340 [12].

Conditional PSCell Change: as defined in TS 37.340 [12].

DAPS Handover: as defined in TS 38.300 [2].

eNB-CP: as defined in TS 36.401 [28].

eNB-UP: as defined in TS 36.401 [28].

en-gNB: as defined in TS 37.340 [12].

Early Data Forwarding: as defined in TS 38.300 [2].

F1-terminating IAB-donor of boundary IAB-node: Refers to the IAB-donor that terminates F1 for the boundary IAB-node.

gNB: as defined in TS 38.300 [2].

gNB Central Unit (gNB-CU): a logical node hosting RRC, SDAP and PDCP protocols of the gNB or RRC and PDCP protocols of the en-gNB that controls the operation of one or more gNB-DUs. The gNB-CU terminates the F1 interface connected with the gNB-DU.

gNB Distributed Unit (gNB-DU): a logical node hosting RLC, MAC and PHY layers of the gNB or en-gNB, and its operation is partly controlled by gNB-CU. One gNB-DU supports one or multiple cells. One cell is supported by only one gNB-DU. The gNB-DU terminates the F1 interface connected with the gNB-CU. For DC operation, the MgNB-DU designates the gNB-DU of an en-gNB or a gNB acting as master node, and the SgNB-DU designates the gNB-DU of an en-gNB or a gNB acting as secondary node.

gNB-CU-Control Plane (gNB-CU-CP): a logical node hosting the RRC and the control plane part of the PDCP protocol of the gNB-CU for an en-gNB or a gNB. The gNB-CU-CP terminates the E1 interface connected with the gNB-CU-UP and the F1-C interface connected with the gNB-DU. For DC operation, the MgNB-CU-CP designates the gNB-CU-CP of the gNB-CU for an en-gNB or a gNB acting as master node, and the SgNB-CU-CP designates the gNB-CU-CP of the gNB-CU for an en-gNB or a gNB acting as secondary node.

gNB-CU-User Plane (gNB-CU-UP): a logical node hosting the user plane part of the PDCP protocol of the gNB-CU for an en-gNB, and the user plane part of the PDCP protocol and the SDAP protocol of the gNB-CU for a gNB. The gNB-CU-UP terminates the E1 interface connected with the gNB-CU-CP and the F1-U interface connected with the gNB-DU. For DC operation, the MgNB-CU-UP designates the gNB-CU-UP of the gNB-CU for an en-gNB or a gNB acting as master node, and the the SgNB-CU-UP designates the gNB-CU-UP of the gNB-CU for an en-gNB or a gNB acting as secondary node.

IAB-node: as defined in TS 38.300 [2].

IAB-donor: as defined in TS 38.300 [2].

IAB-donor-CU: the gNB-CU of an IAB-donor, terminating the F1 interface towards IAB-nodes and IAB-donor-DU.

IAB-donor-DU: the gNB-DU of an IAB-donor, hosting the IAB BAP sublayer (as defined in TS 38.340 [22]), providing wireless backhaul to IAB-nodes.

IAB-DU: as defined in TS 38.300 [2].

IAB-MT: as defined in TS 38.300 [2].

IAB Topology: as defined in TS 38.300 [2].

Mapped QoS flows: Unicast QoS flows requested to be established, i.e. included in the legacy QoS flow lists in a way, that non-support RAN nodes would attempt to establish unicast QoS flows and supporting RAN nodes can identify them as mapped QoS flows based on the associated QoS information.

Master node: as defined in TS 37.340 [12].

Master gNB: see TS 37.340 [12].

MBS session resource: This term is used for specification of NG, Xn, F1 and E1 interfaces. It denotes NG-RAN interface and radio resources provided to support an MBS Session.

ng-eNB: as defined in TS 38.300 [2].

ng-eNB Central Unit (ng-eNB-CU): as defined in TS 37.470 [21].

ng-eNB Distributed Unit (ng-eNB-DU): as defined in TS 37.470 [21].

ng-eNB-CU-Control Plane (ng-eNB-CU-CP): a logical node hosting the RRC and the control plane part of the PDCP protocol of the ng-eNB-CU for an ng-eNB. The ng-eNB-CU-CP terminates the E1 interface connected with the ng-eNB-CU-UP and the W1-C interface connected with the ng-eNB-DU.

ng-eNB-CU-User Plane (ng-eNB-CU-UP): a logical node hosting the user plane part of the PDCP protocol and the SDAP protocol of the ng-eNB-CU for an ng-eNB. The ng-eNB-CU-UP terminates the E1 interface connected with the ng-eNB-CU-CP and the W1-U interface connected with the ng-eNB-DU.

NG-RAN node: as defined in TS 38.300 [2].

Non-F1-terminating IAB-donor of boundary IAB-node: Refers to the IAB-donor that has an RRC connection with the boundary node but does not terminate F1 with this boundary node.

PDU Session Resource: This term is used for specification of NG, Xn, and E1 interfaces. It denotes NG-RAN interface and radio resources provided to support a PDU Session.

Public Network Integrated NPN: as defined in TS 23.501 [3].

RRC-terminating IAB-donor: Refers to the IAB-donor that terminates RRC for the IAB-node.

Secondary gNB: see TS 37.340 [12].

Stand-alone Non-Public Network: as defined in TS 23.501 [3].

U2N Relay UE: as defined in TS 38.300 [2].

U2N Remote UE: as defined in TS 38.300 [2].

NEXT CHANGE

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

AP	Application Protocol

AS	Access Stratum

BH	Backhaul

CAG	Closed Access Group

CHO	Conditional Handover

CLI	Cross-Link Interference

CM	Connection Management

CMAS	Commercial Mobile Alert Service

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

EM	Element Manager

EN-DC	E-UTRA-NR Dual Connectivity

ETWS	Earthquake and Tsunami Warning System

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

FDD	Frequency Division Duplex

FTEID	Fully Qualified TEID

GTP-U	GPRS Tunnelling Protocol

IAB	Integrated Access and Backhaul

IP	Internet Protocol

L2					Layer-2

MBS	Multicast Broadcast Service

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MN	Master Node

MgNB	Master Gnb

mIAB	Mobile IAB

MRB	MBS Radio Bearer

MRDC	Multi-Radio Dual Connectivity

NAS	Non-Access Stratum

NID	Network identifier

NPN	Non-Public Network

NSA	Non Standalone

OAM	Operation, Administration and Maintenance

PNI-NPN	Public Network Integrated Non-Public Network

PTP	Point to Point

PTM	Point to Multipoint

PWS	Public Warning System

QoE	Quality of Experience

QoS	Quality of Service

RET	Remote Electrical Tilting

RIM	Remote Interference Management

RIM-RS	Remote Interference Management Reference Signal

RNL	Radio Network Layer

RRC	Radio Resource Control

SA	Standalone

SAP	Service Access Point

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SFN	System Frame Number

SgNB	Secondary gNB

SM	Session Management

SMF	Session Management Function

SN	Secondary Node

SNPN	Stand-alone Non-Public Network

SRAP	Sidelink Relay Adaptation Protocol

TCE	Trace Collection Entity

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TEID	Tunnel Endpoint Identifier

TMA	Tower Mounted Amplifier

TNL	Transport Network Layer

U2N	UE-to-Network

NEXT CHANGE

## 8.XX	Integration of mobile IAB-node

Figure 8.XX-1: Integration of mobile IAB-MT and mobile IAB-DU at different IAB-donors

Figure 8.XX-1 shows the procedure of standalone integration of a mobile IAB-node. The procedure includes the following steps:

- Phase 1: same as Phase 1 of procedure 8.12.1, where the mobile IAB-node and the mobile IAB-MT’s IAB-donor 1 correspond to IAB-node 2 and the IAB-donor in procedure 8.12.1, respectively.

Editor’s note: The mobile-IAB-specific authentication procedure may need to be captured in Phase 1.

- Phase 2-1: same as Phase 2-1 of procedure 8.12.1.

- Phase 2-2: same as Phase 2-2 of procedure 8.12.1.

- Phase 3: mobile IAB-DU part setup. In this phase, the mobile IAB-DU of the mobile IAB-node is configured via OAM. This configuration includes the information for the IAB-donor-CU2’s gNB-ID and for the establishment of F1-C to IAB-donor-CU2. The mobile IAB-DU initiates the TNL establishment and F1 setup (as defined in clause 8.5) with IAB-donor-CU2 via IAB-donor-DU1 using the default BAP routing ID and default BH RLC channel configured by the mobile IAB-MT’s IAB-donor-CU1 in Phase 2-1 for upstream traffic, and using the IP address(es) allocated by the mobile IAB-MT’s IAB-donor-CU1 in Phase 2-2.  After the F1 is set up, the mobile IAB-node can start serving the UEs.

NEXT CHANGE

## 8.YY	Topology adaptation procedures for mobile IAB

### 8.YY.1 – Migration of mobile IAB-MT

#### 8.YY.1.1 – Xn-based migration of mobile IAB-MT

The IAB-MT of a mobile IAB-node can be migrated from a source RRC-terminating IAB-donor-CU to a target RRC-terminating IAB-donor-CU using the Xn handover procedure. During this migration, a collocated IAB-DU may be connected to an F1-terminating IAB-donor-CU, which may be the same as the source RRC-terminating IAB-donor-CU or the target RRC-terminating IAB-donor-CU.

Figure 8.YY.1.1-1 shows an example of the Xn-based migration of the mobile IAB-MT.

Figure 8.YY.1.1-1: Procedure for Xn-based migration of mobile IAB-MT

1. Steps 1-14 of the topology adaptation procedure of Section 8.17.3.1 are performed to hand over the IAB-MT of the mobile IAB-node via Xn from the source parent IAB-node connected to the source RRC-terminating IAB-donor-CU to the target parent IAB-node connected to the target RRC-terminating IAB-donor-CU. In these steps, the mobile IAB-node corresponds to the migrating IAB-node of Section 8.17.3.1, and the mobile IAB-MT’s source and target RRC-terminating IAB-donor-CUs correspond to the respective source and target IAB-donor-CUs of Section 8.17.3.1.

2. Same as step 15 of the topology adaptation procedure of Section 8.17.3.1, where the mobile IAB-node corresponds to the migrating IAB-node, and the mobile IAB-DU’s F1-terminating IAB-donor corresponds to the source IAB-donor.

3. The F1-U connections of the mobile IAB-DU of the mobile IAB-node with the mobile IAB-DU’s F1-terminating IAB-donor-CU are switched to use the mobile IAB-node’s new TNL address(es) allocated in Step 1. The redirection of the F1-U connections and exchange of F1-U traffic may use the default BAP routing ID and default BH RLC channel allocated in Step 1 for upstream communication, and they may use the DL mapping configured by the mobile IAB-MT’s target RRC-terminating IAB-donor-CU on the IAB-donor-DU of the mobile IAB-MT’s target RRC-terminating IAB-donor for downstream communication in equivalent manner as for F1-C.

#### 8.YY.1.2 – NG-based migration of mobile IAB-MT

A IAB-MT of a mobile IAB-node can be migrated from a source RRC-terminating IAB-donor-CU to a target RRC-terminating IAB-donor-CU using the NG-based handover procedure. During this migration, a collocated IAB-DU may be connected to an F1-terminating IAB-donor-CU, which may be the same as the source RRC-terminating IAB-donor-CU or the target RRC-terminating IAB-donor-CU.

Figure 8.YY.1.2-1 shows an example of the NG-based migration of the mobile IAB-MT.

Figure 8.YY.1.2-1: Procedure for NG-based migration of mobile IAB-MT

1. The mobile IAB-MT’s source RRC-terminating IAB-donor-CU sends a HANDOVER REQUIRED message to the AMF. This message may include the mobile IAB-node’s TNL address information in the Source to Target Transparent Container.

Editor’s note: FFS how to capture “WA: legacy NG HO Required message with “dummy” PDU information is used. It needs to be further discussed if any specification changes are needed to describe the use of such dummy information. If specification changes is needed, consider whether an explicit indication is worth adding.”

2. The AMF sends a HANDOVER REQUEST message to the mobile IAB-MT’s target RRC-terminating IAB-donor-CU including the Source to Target Transparent Container. The message may also include a No PDU Session Indication if the mobile IAB-MT has no established PDU session.

3-4: Same as steps 2-3 of the topology adaptation procedure of Section 8.17.3.1, where the mobile IAB-MT’s target RRC-terminating IAB-donor-CU corresponds to the target IAB-donor-CU.

5. The mobile IAB-MT’s target RRC-terminating IAB-donor-CU performs admission control and provides the new RRC configuration for the mobile IAB-MT inside the Target to Source Transparent Container in a HANDOVER REQUEST ACKNOWLEDGE message to the AMF. The RRC configuration includes a BAP address for the mobile IAB-node in the mobile IAB-MT’s target RRC-terminating IAB-donor-CU’s topology, a default BH RLC channel and a default BAP routing ID configuration for UL F1-C/non-F1 traffic mapping on the target path. The RRC configuration may include the new TNL address(es) anchored at the IAB-donor-DU of the mobile IAB-MT’s target RRC-terminating IAB-donor for the mobile IAB-node.

6. The AMF sends a HANDOVER COMMAND message to the mobile IAB-MT’s source RRC-terminating IAB-donor-CU including the Target to Source Transparent Container.

7. Same as steps 5-10 of the topology adaptation procedure of Section 8.17.3.1, where the HO command is delivered to the mobile IAB-MT, which executes the handover and connects to the mobile IAB-MT’s target RRC-terminating IAB-donor-CU via the target parent IAB-node.

8. The mobile IAB-MT’s target RRC-terminating IAB-donor-CU sends a HANDOVER NOTIFY message to the AMF.

9. The AMF sends a UE CONTEXT RELEASE COMMAND message to the mobile IAB-MT’s source RRC-terminating IAB-donor-CU.

10. Same as step 13 of the topology adaptation procedure of Section 8.17.3.1, where the mobile IAB-node and the mobile IAB-MT’s source RRC-terminating IAB-donor correspond to the migrating IAB-node and the source IAB-donor respectively.

11. The mobile IAB-MT’s source RRC-terminating IAB-donor-CU responds to the AMF with a UE CONTEXT RELEASE COMMAND COMPLETE message.

12. Same as step 14 of the topology adaptation procedure of Section 8.17.3.1, where the mobile IAB-node corresponds to the migrating IAB-node, and the mobile IAB-MT’s target RRC-terminating IAB-donor corresponds to the target IAB-donor.

13. Same as steps 2 and 3 of the topology adaptation procedure of Section 8.YY.1.1.

### 8.YY.2 – Migration of mobile IAB-DU

The RAN may perform the mobile IAB-DU migration procedure. During this procedure, the mobile IAB-node concurrently supports two logical mobile IAB-DUs, which have F1AP associations with the source F1-terminating IAB-donor-CU and target F1-terminating IAB-donor-CU, respectively. UEs connected to the mobile IAB-node are handed over from the cell of the source logical mobile IAB-DU associated with the source F1-terminating IAB-donor-CU to the cell of the target logical mobile IAB-DU associated with the target F1-terminating IAB-donor-CU. After the UEs are handed over, the source logical mobile IAB-DU’s F1AP association with the source F1-terminating IAB-donor-CU may be released. During the migration of the mobile IAB-DU, the collocated mobile IAB-MT may be RRC-connected to the source or the target F1-terminating IAB-donor-CUs.

Figure 8.YY.2-1 shows an example of a topology adaptation procedure.

Figure 8.YY.2-1: Procedure for migration of mobile IAB-DU

0. The mobile IAB-node holds an IAB-MT that is RRC-connected to an RRC-terminating IAB-donor-CU and a source logical mobile IAB-DU that is F1-connected to a source F1-terminating IAB-donor-CU. The source logical mobile IAB-DU serves one or more UEs, each of which is RRC-connected to the source F1-terminating IAB-donor-CU.

1. The target logical mobile IAB-DU initiates TNL establishment and F1 setup (as defined in clause 8.5) with the target F1-terminating IAB-donor-CU. The mobile IAB-node may have received the information about the target F1-terminating IAB-donor-CU and for the establishment of F1-C via OAM. The F1-C-associated traffic is routed via the IAB-donor-DU of the mobile IAB-MT’s RRC-terminating IAB-donor using the default BAP routing ID and default BH RLC channel configured by the mobile IAB-MT’s RRC-terminating IAB-donor-CU for upstream traffic, and it uses the IP address(es) allocated by the mobile IAB-MT’s RRC-terminating IAB-donor-CU for F1-C traffic. During the F1 setup procedure, the target F1-terminating IAB-donor-CU may reconfigure the NCGI of a served cell included in the F1 SETUP REQUEST message, e.g., in case the NCGI provided by the mobile IAB-DU collides that of another cell supported by this CU. At the end of this step, the target logical mobile IAB-DU can serve UEs via the target mobile IAB-DU’s activated cell(s).

Editor’s note: FFS how the target logical mobile IAB-DU’s OAM gains visibility on the result of the NCGI reconfiguration by the target F1-terminating IAB-donor-CU.

Editor’s note: FFS whether NCGI reconfiguration applies only to NCGI collision avoidance.

Editor’s note: FFS if and how the source F1-terminating IAB-donor-CU may trigger F1 setup to the target F1-terminating IAB-donor-CU.

Editor’s note: FFS if and how the source F1-terminating IAB-donor-CU is informed about the successful outcome of the F1 setup, the cells activated on the target logical mobile IAB-DU, and/or the mapping b/w activated cells of the source logical mobile IAB-DU and those of the target logical mobile IAB-DU.

2. The source F1-terminating IAB-donor-CU hands over the UEs from a source cell served by the source logical mobile IAB-DU to a target cell served by the target logical mobile IAB-DU. This step follows the procedure of Section 9.2.3.2 of TS 38.300 on UE handover. These handovers may be based on RRC measurement reports by the UEs.

Editor’s note: FFS whether to capture implementation-based solutions such as ramping up (down) of signal power of the cell(s) of the target (source) logical mobile IAB-DU to trigger UE reports.

3. The UE-associated F1-U connections between the target logical mobile IAB-DU and the target F1-terminating IAB-donor-CU use the mobile IAB-node’s TNL address(es) anchored on the IAB-donor-DU of the mobile IAB-MT’s RRC-terminating IAB-donor. In upstream direction, the F1-U traffic may use the default BAP routing ID and default BH RLC channel configured by the RRC-terminating IAB-donor-CU. In downstream, DL mapping(s) may be configured by the RRC-terminating IAB-donor-CU on the IAB-donor-DU for F1-U traffic in equivalent manner as for F1-C traffic.

4. Either of the source logical mobile IAB-DU or the source F1-terminating IAB-donor-CU may release the source logical mobile IAB-DU’s F1AP association after the UEs are handed over.

END OF CHANGES