3GPP TSG-RAN WG3 Meeting #121	R3-233900

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	10.2.2

Source:	Huawei

Title:	(TPs for SON BLCRs for TS 38.423) MRO on CPAC

Document for:	other

# Introduction

In RAN3#119bis meeting, the following agreements were made for the purpose of MRO:

MRO for CPA and CPC:

Whether CPA/CPC candidate PSCell list can be sent from MN to the source/last serving SN?

UHI for CPAC?

MRO for the fast MCG recovery:

Agree to define case c in R18. Solution can be further discussed.

Whether the network based solution or UE based solution should be used for Case c?

Whether the MRO for fast MCG recovery needs to support pre-R18 UEs?

MRO for the voice fall-back:

RAN3 assumes the indication is needed and draft the LS to RAN2 to check the feasibility of the solution for MRO for voice fallback.

RAN2 has decided to introduce the indication of voice fallback in the LTE RLF report.

WA: Define a new handover report type in the Inter-system HO Report over S1 and NG.

In this paper, for the agreed scenarios and FFSs, we will give further consideration on the potential impact on the specification, and give potential solution to support these above three objectives.

# 2	Discussion

## 2.1	MRO for CPAC

### Analysis of wrong candidate PSCell selection

RAN3 has made an agreement regarding the CPA/CPC execution to wrong PSCell:

Update the definition to wrong PSCell change/addition should be splitting to sub cases: 1) the wrong candidate cell comes from the cell in the list provided by the initiating node or 2) the wrong candidate cell is selected by the target node.

The initial analysis is performed to identify the possible problem node, whether the initiating node or the target node does not provide the sutiable cell as a canddiate. Moreover, after identifying the possible problem node, there should be a root cause analysis to verify whether the possible problem node actually causes the SCG failure. For example, the wrong PSCell selection at the initiating node side may be purely caused by the dramatically changing measurement results at the time of CPAC preparation and at the time of SCG failure. The wrong PSCell selection at the candidate node side could be caused by the limited PSCells number indicated from initiating node.

In NR-DC MN is able to perform initial analysis for both MN and SN initiated inter-SN CPAC, since MN is aware of the recommended candidate PSCells at the preparation phase, and the suitable PSCell from the SCG measurement result upon reception of SCGFailureInformation. In the aspect of initiating node, SN as initiating node is able to perform initial analysis similarly. For SN initiated intra-SN CPC, MN may use the SCG Failure Information report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN to identify the possible problem node. In summary, for MN and SN initiated CPAC, upon reception of SCGFailureInformation, MN or the initiating node is capable of performing the initial analysis to identify the possible problem node.

As for root cause analysis, one potential issue is whether MN supports root cause analysis is dependent on the cases.  In case of CPA or MN initiated CPAC, MN could obtain the SCG measurement result at the preparation phase for recommending candidate PSCells to support root cause analysis. However, in case of SN-initiated CPC, MN does not decode the SCG measurement results in a container provided from source SN at the preparation phase of candidate PSCells.

In EN-DC, if it is SN-initiated (conditional) PSCell change, MN is not able to perform the initial analysis to identify the possible problem node. First of all, MN may not obtain the SCG measurement result from UE reported SCGFailureInformation to find the suitable PSCell. Secondly, MN does not decode the SCG measurement results in a container provided from source SN at the preparation phase of candidate PSCells. Hence, SN as initiating node is able to identify the possible problem node.

Furthermore, after the possible problem node is identified to be the candidate SN, which the suitable PSCell belongs to, whether it is the initiating node or the possible problem candidate SN to perform the root cause analysis needs to be further specified. Technically, both of the two nodes are able to perform the root cause analysis. However, there is a benefit that the possible problem candidate SN could be performing the root cause analysis and performing optimization accordingly at the same time if it is verified to be the actual one. Otherwise, the possible problem candidate SN has to do the duplication of work for optimization anyway when the initiating node performing root cause analysis verifies the possible problem candidate SN to be the actual one.

We recommend to have a common design for all the above mention scenarios regardless it is in NR-DC or EN-DC. he Rel-18 MRO mechanism for CPAC could take Rel-17 MRO mechanism for PSCell change failure as a baseline and make proper adjustments. After MN receives SCGFailureInformation including CPAC failure related information from the UE, the MN sends SCGFailureInformationReport to the source SN if source SN triggers intra-SN or inter-SN PSCell change. The initiating node performs initial analysis to identify the node that caused the CPAC related SCG failure and inform the possible problem candidate SN to MN. So MN could send SCGFailureInformationReport to the inidciated candidate SN accordingly.

It should be the initiating node to identify the possible problem node and should be the possible problem node to perform the root cause analysis, respectively:

- If the triggering node (MN/SN) is the possible problem node, the triggering node verifies whether it actually causes the wrong selection;

- If the candidate node, which the suitable PSCell belongs to, is the possible problem node, the candidate SN verifies whether it actually causes the wrong selection;

Proposal 1: For the wrong candidate PSCell selection, the initial analysis to identify the possible problem node and the root cause analysis to verify whether it is the actual problem node are done this way:

- if it is MN initiated, MN performs initial analysis and the possible problem node performs the root cause analysis;

- if it is source SN initiated, source SN performs initial analysis and the possible problem node performs the root cause analysis.

After the source SN as initiating node has identified the possible problem candidate SN, source SN has no UE-associated Xn or X2 interface with the candidate SN since MN is the one to contact all the candidate SNs. Thus, it is impossible for source SN to forward the UE associated SCGFailureInformationReport directly to the possible problem candidate SN.  Although MN as anchor node has all the UE related CP context, MN has no knowledge of the possible problem candidate SN ID. In order to enable MN to further forward the SCGFailureInformationReport to the corresponding SN, source SN informs the possible candidate SN to MN.

Proposal 2: After source SN as initiating node performs initial analysis, source SN informs the possible problem candidate SN to MN to enable the forwarding of SCGFailureInformationReport.

### Network based solution to enable problem analysis instead of UE reporting

The CPAC related timer information could be derived by the MN upon transmission and reception of corresponding RRC messages. The network based solution of CPAC related timer information is listed in the below table:

| Time related information | view |
| --- | --- |
| - The time elapsed between the SCG failure in source SCG and the latest CPC configuration is received. | MN could derive time elapsing since the last CPC configuration until SCG failure by computing time between transmission of RRCReconfig containing CPAC config and reception of SCGFailureInforamtion. |
| - The time elapsed between reception of CPAC configuration and the CPAC execution of related RRCReconfiguration | MN could derive time elapsing since the last CPC configuration until CPAC execution by computing time between transmission of RRCReconfig containing CPAC config and reception of RRCReconfigComplete corresponding to the selected PSCell. |
| - The time elapsed since the CPAC execution towards the target PSCell until the SCG failure. | MN could derive time elapsing since CPAC execution until SCG failure by computing time between reception of RRCReconfigComplete corresponding to the selected PSCell and reception of SCGFailureInforamtion. |

In case of SN initated intra-SN or inter-SN PSCell change, MN send SCGFailureInformationReport  to source SN for MRO analysis. In addition to SCGFailureInformation from UE, MN should also indicate the CPAC timer related information derived by MN to SN via SCGFailureInformationReport over Xn.

Proposal 3: MN can inform the CPA/CPC related timer information to the initiating SN via SCG FAILURE INFORMATION REPORT over Xn for MRO analysis:

- The time elapsed between the SCG failure in source SCG and the latest CPC configuration is received;

- The time elapsed between reception of CPAC configuration and the CPAC execution of related RRCReconfiguration;

- The time elapsed since the CPAC execution towards the target PSCell until the SCG failure.

The TP for TS 38.423 BL CR are attached in section 4.

Fig.2 the signalling for MN to store the SN mobility info

Except for the aforementioned timer related information, it is also noticed that it may require to enhance the SCG FAILURE INFORMATION REPORT for MN to inform the source SN of the initial recommended CPAC configuration and finally delivered CPAC configuration. In case of SCG failure shortly after successful CPC execution, source SN may release the UE context. Upon reception of SCGFailureInformation, MN also needs to forward the initial CPAC configuration during the preparation of CPAC procedure. However, the CPAC configuration provided from source SN to MN is transparent to MN since MN does not decode it and simply forwards to UE. In our understanding, SN mobility information could be reused as a remainder for MN to store and collect the CPAC configuration from source SN. After the initial SN change Required message in step 1 including the SN mobility#0, the source SN has the flexibility to update the CPAC configuration before and after delivering the CPAC candidate PSCell configurations to UE.

Source SN informs the SN mobility info mapped to the latest CPAC configuration for UE each time to MN. The SN Modification Required Acknowledge message in step 5 (optional step) should be enhanced to inform the SN mobility info#1 mapped to the updated/modified CPAC configuration to MN. SN Modification Required message in step 9 (optional step), could include the SN mobility information#2 mapped to the once again reconfigured CPAC configuration for UE to inform MN. Moreover, source SN could possibly inform MN of the SN mobility information#2 at the time of releasing radio resources for UE, which is mapped to the latest CPAC configuration for UE. Since source SN only sends SN Status Transfer message only if PDCP termination point is changed for bearers using RLC AM.  SN Release Request ACK is more certain to convey the latest CPAC configuration via SN mobility information.

Proposal 4: For MN to store and collect the valid CPAC configuration for UE, source SN informs the SN mobility info mapped to the latest CPAC configuration to MN, e.g., via SN Modification Required Acknowledge message and SN Modification Required message and SN Release Request ACK message.

For the analysis of wrong PSCell selection, it requires the initial recommended candidate PSCell list, the accepted candidate PSCell list by candidat SNs (finally configured candidate PSCell for UE), and the SCG measurement results. For shortly SCG RLF case, MN should not only provide the latest SN mobility mapped to the final CPAC configuration for UE, but also the initial SN mobility mapped to the initial recommended CPAC configuration from source SN. Therefore, upon reception of SCGFailureInformation, MN indicates the set of SN mobility information including the initial recommended SN mobility info and the latest SN mobility info for UE via SCGFailureInformationReport to the source SN/last serving SN for MRO analysis, e.g., wrong PSCell selection.

Proposal 5: Upon reception of SCGFailureInformation, MN indicates the set of SN mobility information including the initial recommended SN mobility info and the latest SN mobility info via SCGFailureInformationReport to the source SN/last serving SN for MRO analysis, e.g., wrong PSCell selection.

The TP for TS 38.423 BL CR are attached correspondly in section 4.

# 3	Conclusion

This document proposes:

Proposal 1: For the wrong candidate PSCell selection, the initial analysis to identify the possible problem node and the root cause analysis to verify whether it is the actual problem node are done this way:

- if it is MN initiated, MN performs initial analysis and the possible problem node performs the root cause analysis;

- if it is source SN initiated, source SN performs initial analysis and the possible problem node performs the root cause analysis.

Proposal 2: After source SN as initiating node performs initial analysis, source SN informs the possible problem candidate SN to MN to enable the forwarding of SCGFailureInformationReport.

Proposal 3: MN can inform the CPA/CPC related timer information to the initiating SN via SCG FAILURE INFORMATION REPORT over Xn for MRO analysis:

- The time elapsed between the SCG failure in source SCG and the latest CPC configuration is received;

- The time elapsed between reception of CPAC configuration and the CPAC execution of related RRCReconfiguration;

- The time elapsed since the CPAC execution towards the target PSCell until the SCG failure.

Proposal 4: For MN to store and collect the valid CPAC configuration for UE, source SN informs the SN mobility info mapped to the latest CPAC configuration to MN, e.g., via SN Modification Required Acknowledge message and SN Modification Required message and SN Release Request ACK message.

Proposal 5: Upon reception of SCGFailureInformation, MN indicates the set of SN mobility information including the initial recommended SN mobility info and the latest SN mobility info via SCGFailureInformationReport to the source SN to assure that source SN has sufficient CPAC configuration information to perform MRO analysis, e.g., wrong PSCell selection.

# Annex1	TP to to SON BL CR to TS 38.423

Start of the change

### 8.3.17	SCG Failure Information Report

#### 8.3.17.1	General

The purpose of the SCG Failure Information Report procedure is to provide SCG mobility related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.17.2	Successful Operation

Figure 8.3.17.2-1: SCG Failure Information Report, successful operation

The M-NG-RAN node initiates the procedure by sending the SCG FAILURE INFORMATION REPORT message to the S-NG-RAN node. Upon receiving the message, the S-NG-RAN node shall assume that a PSCell change failure event was detected.

The SCG FAILURE INFORMATION REPORT message may include:

-	the SN Mobility Information IE, if the SN Mobility Information IE was sent for the PSCell change procedure from the S-NG-RAN node;

-	the Source PSCell CGI IE, if the Source PSCell CGI IE was sent for the PSCell change procedure from the S-NG-RAN node.

-	the Time Between CPAC Reconfig And Execution IE, Time Between CPAC Reconfig And SCG Failure IE, and Time Between CPAC Execution And SCG Failure IE if it is a SN-initiated CPAC procedure.

-	the SN Mobility Information List IE, if the Intial SN Mobility Information IE and the Lastest SN Mobility Information List IE was sent for the CPC procedure from the S-NG-RAN node;

If the SCG FAILURE INFORMATION REPORT message includes the Source PSCell CGI IE, the S-NG-RAN node shall, if supported, store the information.

If the SCG FAILURE INFORMATION REPORT message includes the Failed PSCell CGI IE, the S-NG-RAN node shall, if supported, store the information and act as specified in TS 38.300 [9].

If the SCG FAILURE INFORMATION REPORT message includes the Time Between CPAC Reconfig And Execution IE,  Time Between CPAC Reconfig And SCG Failure IE, and Time Between CPAC Execution And SCG Failure IE, the S-NG-RAN node shall, if supported, store the information.

If the SCG FAILURE INFORMATION REPORT message includes the SN Mobility Information List IE, the S-NG-RAN node shall, if supported, store the information.

If received, the S-NG-RAN node uses the above information for SCG failure reason detection and optimisation.

#### 8.3.17.3	Unsuccessful Operation

Not applicable.

#### 8.3.17.4	Abnormal Conditions

Void.

Start of next change

#### 9.1.2.29	SCG FAILURE INFORMATION REPORT

This message is sent by M-NG-RAN node to S-NG-RAN node to report a PSCell change failure event.

Direction: M-NG-RAN node  S-NG-RAN node .

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node. | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node. | YES | ignore |
| Source PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | The SCGFailureInformation message or the SCGFailureInformationEUTRA message as defined in TS 38.331 [10] or the SCGFailureInformation message or the SCGFailureInformationNR message as defined in TS 36.331 [14] | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the PSCell change. It’s provided by S-NG-RAN node in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| Time Between CPAC Reconfig And Execution | O |  | INTEGER (0..1023) | The time elapsed between the initiation of the last conditional reconfiguration execution towards the target PSCell and the reception of the latest conditional reconfiguration for this target PSCell. | YES | ignore |
| Time Between CPAC Reconfig And SCG Failure | O |  | INTEGER (0..1023) | The time that elapsed between the reception of the latest conditional reconfiguration for this target PSCell and the SCG failure. | YES | ignore |
| Time Between CPAC Execution And SCG Failure | O |  | INTEGER (0..1023) | The elapsed time between the initiation of the last conditional reconfiguration execution towards the target PSCell and the SCG failure. | YES | ignore |
| SN Mobility Information List |  | 1 |  |  |  |  |
| >Initial SN mobility information | O |  | BIT STRING (SIZE (32)) | Information related to the initially recommended CPC configuration. It’s provided by S-NG-RAN node in order to enable later analysis of the conditions that led to wrong candidate PSCell change. | YES | ignore |
| >Latest SN mobility information | O |  | BIT STRING (SIZE (32)) | Information related to to the final CPC configuration. It’s provided by S-NG-RAN node in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

//skip unchanged part

#### -- SCG FAILURE INFORMATION REPORT

--

-- **************************************************************

ScgFailureInformationReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ScgFailureInformationReport-IEs}},

...

}

ScgFailureInformationReport-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory }|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory }|

{ ID id-SourcePSCellCGI								CRITICALITY ignore		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-FailedPSCellCGI								CRITICALITY ignore		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-SCGFailureReportContainer					CRITICALITY ignore		TYPE SCGFailureReportContainer					PRESENCE mandatory }|

{ ID id-SNMobilityInformation				CRITICALITY ignore		TYPE SNMobilityInformation						PRESENCE optional },

{ ID id- TimeBetweenCPACReconfigAndExecution				CRITICALITY ignore		TYPE TimeBetweenCPACReconfigAndExecution						PRESENCE optional },

{ ID id- TimeBetweenCPACReconfigAndSCGFailure				CRITICALITY ignore		TYPE TimeBetweenCPACReconfigAndSCGFailure						PRESENCE optional },

{ ID id- TimeBetweenCPACExecutionAndSCGFailure				CRITICALITY ignore		TYPE TimeBetweenCPACExecutionAndSCGFailure						PRESENCE optional },

{ ID id- SNMobilityInformationList				CRITICALITY ignore		TYPE SNMobilityInformationList						PRESENCE optional },

...

}

-- **************************************************************

--

End of the change