3GPP TSG-RAN WG3 Meeting #121                                                      	R3-233930

Toulouse, Aug 21st – Aug 25th, 2023

Agenda item:	24.2

Source:	Qualcomm Incorporated

Title:	Inter-node beam activation and paging enhancements

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss further details related to the following objective in the Rel-18 NES WID (RP-223450) based on the discussion in the last meeting:

Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3].

# Discussion

## Inter-node beam activation

Support beam level activation over Xn and F1.

For inter-node beam activation, the XnAP CELL ACTIVATION procedure, and the F1AP GNB-CU CONFIGURATION UPDATE procedure are reused

Over Xn interface,

The CELL ACTIVATION REQUEST message may include the SSB beam list that is requested to be activated

The CELL ACTIVATION RESPONSE message may include SSB beam list that are activated

When the receiving NG-RAN node cannot activate any of the SSB beams, it should respond with the CELL ACTIVATION FAILURE message with an appropriate cause value

FFS if a new cause is needed.  The detailed IE name/encoding can be further refined

Over F1 interface,

The GNB-CU CONFIGURATION UPDATE message may include the SSB beam list that is requested to be activated

The GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE may include SSB beam list that are activated

In case the gNB-DU cannot activate any of the requested SSB beams, it should respond with the GNB-CU CONFIGURATION UPDATE FAILURE message with an appropriate cause value

FFS if a new cause is needed. The detailed IE name/encoding can be further refined

Whether to always update SSB bitmap upon SSB (de)activation

Currently, gNB-DU can inform the gNB-CU about the beams that are currently switched ON or OFF via the following three options:

- Option 1: SSB Positions In Burst (hence forth called SSB bitmap for simplicity)

- Option 2: SSB Coverage State

- Option 3: SSBs Activated List

Observation 1: The gNB-DU can inform the gNB-CU about the beams that are currently switched ON or OFF via “SSB Positions In Burst” or “SSB Coverage State” or “SSBs Activated List”

It is not clear whether we need to update SSB bitmap (which is also broadcasted in SIB1) each time SSB(s) are (de)activated. If SSB (de)activation is done frequently, a good gNB implementation need not notify the UEs all the time; otherwise, frequent SIB1 update might causing more network and UE energy consumption.

Observation 2: Updating SSB Positions in Burst each time the SSB(s) are (de)activated (especially if it’s done frequently) would mean SIB1 update and more energy consumption for network to broadcast SIB1 update and for UEs to read the SIB1 update.

We therefore make the following proposal:

Proposal 1: RAN3 should discuss whether there is a need to always update SSB bitmap (sent by gNB-DU in Served Cell Information and then broadcasted in SIB1) upon every SSB (de) activation. The following options can be considered:

- Option 1: SSB bitmap is always updated upon SSB (de)activation along with updating SSB Coverage State and SSBs Activated List

- Option 2: There is no need to always update SSB bitmap upon every SSB (de)activation. Just changing the “SSB coverage state” or “SSB Activated List” is sufficient.

FFS if a new cause is needed

One of the reasons why gNB-DU cannot activate any of the requested SSB beams might be if none of the requested beams are available or if the beams are already active. For example, as an error case, a gNB1 might request to activate SSB = 9 of neighboring gNB2, but in reality gNB2 might only have 8 beams. So, we can discuss whether to add such cause values.

Proposal 2: New cause values such as “Beam requested to be activated is not available” or “Beam requested to be activated is already activated” can be added in CELL ACTIVATION FAILURE and GNB-CU CONFIGURATION UPDATE FAILURE message

Whether to define a timer indicating how long the SSBs are to be activated

In the previous meetings, a timer was proposed that indicates for how long the SSB will be activated during beam activation.  We first point out the following clause from TS 38.300:

Upon reception of the re-activation request, the NG-RAN node's cell should remain switched on at least until expiration of the minimum activation time. The minimum activation time may be configured by O&M or be left to the NG-RAN node's implementation.

Observation 3: In NR, the minimum activation time during cell activation is configured by O&M or left to NG-RAN nodes’ implementation

Also, we think that if a gNB no longer wants a beam after a time duration T, it can send a deactivation request after the time duration T. Don’t see a big need to pre-configure the duration a beam needs to be available (also traffic conditions might change dynamically within the time duration).

Proposal 3: There is no need to define a timer that indicates how the long SSB that is requested to be activated should remain active because:

- Traffic conditions might change dynamically within the time duration and therefore a node can’t pre-determine how long the SSB has to remain active

- For cell activation the minimum activation time is configured by O&M or left to NG-RAN nodes’ implementation. Hence there is no need to standardize this timer for beam activation as well.

## Reply LS to SA2 on Paging

SA2 sent a reply LS on the paging enhancements in a restricted area for RRC_IDLE UEs. SA2 replied that it is feasible but have asked some follow-up questions.

….

Q1: If the initial paging attempt using the potential new parameter of "List of recommended beams" fails, how the paging escalation happens will be controlled by the AMF based on existing procedures?

Q2: SA2 understands that the beam-based paging is most efficient towards low mobility and static UEs. How is the gNB expected to determine whether the UE is static or not across multiple RRC connections from the same UE considering TS 33.501 contains requirements on 5G-S-TMSI reallocation?

Q3: SA2 would like to ask about the validity condition of the potential “List of recommended beams” container that is stored in the AMF. What is the AMF condition to delete the stored "List of recommended beams"?

…

Proposed answer to Q1

If the initial paging attempt using the potential new parameter of "List of recommended beams" fails, how the paging escalation should happen can be up to AMF implementation (e.g., can page in the last visited cell or the entire TA). There is no intention from RAN3 to introduce new AMF behavior for paging escalation.

Proposed answer to Q2

Even if there are RRC state changes from the same UE, a gNB can know whether the UE is expected to be stationary via subscription information i.e., the "Expected UE behavior" IE indicated from AMF in "Core Network Assistance Information for RRC INACTIVE" or "CN Assisted RAN Parameters Tuning". RAN3 is not looking to enhance any UE identifiers to identify the same UE across RRC connections.

Proposed answer to Q3

How long the AMF should store the “List of Recommended Beams” is upto AMF implementation. Alternatively, RAN3 can define a max timer in UE CONTEXT RELEASE indicating  for how long the AMF should store the list of recommended beams

We propose to send a reply LS to SA2 with the above proposed answers.

Proposal 4: Send a reply LS to SA2 with the following answers to Q1-Q3:

- If the initial paging attempt using the potential new parameter of "List of recommended beams" fails, how the paging escalation should happen can be up to AMF implementation (e.g., can page in the last visited cell or the entire TA). There is no intention from RAN3 to introduce new AMF behavior for paging escalation.

- Even if there are RRC state changes from the same UE, a gNB can know whether the UE is expected to be stationary via subscription information i.e., the "Expected UE behavior" IE indicated from AMF in "Core Network Assistance Information for RRC INACTIVE" or "CN Assisted RAN Parameters Tuning". RAN3 is not looking to enhance any UE identifiers to identify the same UE across RRC connections.

- How long the AMF should store the “List of Recommended Beams” is upto AMF implementation. Alternatively, RAN3 can define a max timer in UE CONTEXT RELEASE indicating for how long the AMF should store the list of recommended beams

Further, we think that RAN3 should wait for RAN2’s reply before deciding whether to extend this feature of beam-based paging for RRC_IDLE UEs as well.

Proposal 5: Wait for reply LS from RAN2 before deciding whether to extend this feature of beam-based paging to RRC_IDLE UEs

# Conclusion

Inter-node beam activation

Observation 1: The gNB-DU can inform the gNB-CU about the beams that are currently switched ON or OFF via “SSB Positions In Burst” or “SSB Coverage State” or “SSBs Activated List”

Observation 2: Updating SSB Positions in Burst each time the SSB(s) are (de)activated (specially if it’s done frequently) would mean SIB1 update and more energy consumption for network to broadcast SIB1 update and for UEs to read the SIB1 update.

Proposal 1: RAN3 should discuss whether there is a need to always update SSB bitmap (sent by gNB-DU in Served Cell Information and then broadcasted in SIB1) upon every SSB (de) activation. The following options can be considered:

- Option 1: SSB bitmap is always updated upon SSB (de)activation along with updating SSB Coverage State and SSBs Activated List

- Option 2: There is no need to always update SSB bitmap upon every SSB (de)activation. Just changing the “SSB coverage state” or “SSB Activated List” is sufficient.

Proposal 2: New cause values such as “Beam requested to be activated is not available” or “Beam requested to be activated is already activated” can be added in CELL ACTIVATION FAILURE and GNB-CU CONFIGURATION UPDATE FAILURE message

Observation 3: In NR, the minimum activation time during cell activation is configured by O&M or left to NG-RAN nodes’ implementation

Proposal 3: There is no need to define a timer that indicates how the long SSB that is requested to be activated should remain active because:

- Traffic conditions might change dynamically within the time duration and therefore a node can’t pre-determine how long the SSB has to remain active

- For cell activation the minimum activation time is configured by O&M or left to NG-RAN nodes’ implementation. Hence there is no need to standardize this timer for beam activation as well.

Reply LS to SA2 on Paging

Proposal 4: Send a reply LS to SA2 on paging enhancements with the following answers to Q1-Q3:

- If the initial paging attempt using the potential new parameter of "List of recommended beams" fails, how the paging escalation should happen can be up to AMF implementation (e.g., can page in the last visited cell or the entire TA). There is no intention from RAN3 to introduce new AMF behavior for paging escalation.

- Even if there are RRC state changes from the same UE, a gNB can know whether the UE is expected to be stationary via subscription information i.e., the "Expected UE behavior" IE indicated from AMF in "Core Network Assistance Information for RRC INACTIVE" or "CN Assisted RAN Parameters Tuning". RAN3 is not looking to enhance any UE identifiers to identify the same UE across RRC connections.

- How long the AMF should store the “List of Recommended Beams” is up to AMF implementation. Alternatively, RAN3 can define a max timer in UE CONTEXT RELEASE indicating for how long the AMF should store the list of recommended beams.

Proposal 5: Wait for reply LS from RAN2 before deciding whether to extend this feature of beam-based paging to RRC_IDLE UEs

# 4. References