3GPP TSG-RAN WG3 #121	R3-233948

Toulouse, France, 21 – 25 Aug 2023

Title: 	(TP for BLCR TS37.483) Discussion on MBS RAN sharing

Source: 	Samsung

Agenda item:	15.2

Document Type:	Discussion

1. Introduction

According to the approved Rel-18 NR MBS enhancement WID [1], the following objectives have impact to RAN3.

This Work Item is to further enhance the NR Multicast/Broadcast functions based on Rel-17 MBS. The objectives for Rel-18 include: - Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3] - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2] - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE. (Seamless/lossless mobility is not required) [RAN2, RAN3] - Study and if necessary, specify enhancements to improve the resource efficiency for MBS reception in RAN sharing scenarios [RAN3] Note: collaboration with SA2 is expected in due course for the above objectives.

2. Discussion

2.1 Request setup of shared NG-U

The last RAN3 meeting we reached the following agreements:

Define a new class 1 procedure to allow the gNB/CU-CP requesting the setup of shared NG-U resources during an ongoing broadcast MBS session for unicast transport.”

Include in the MBS Session Setup or Modification Response Transfer IE an indication that the NG-RAN node has decided to not establish shared NG-U transport resources for the broadcast MBS session in case of unicast transport.

The name of the new class 1 procedure is open. There are two options for the new message:

Option 1: Broadcast Session Modification Required/Confirm/failure

Option 2: Broadcast Distribution Request/Response/failure or Broadcast session transport request/response/failure

To analysis which option is suitable, we start from the session setup procedure. When the gNB-CU-CP receives multiple BC Session Setup Request messages for the shared MBS services, the gNB-CU-CP decides to active the sharing enhancement and selects one of the MBS services to setup the NG-U resource. For the other associated MBS services not establishing NG-U resource, the gNB-CU-CP sends the BC Session Setup Response message with the indication that the NG-U transport resource is not setup due to RAN sharing enhancement.

Later, the gNB-CU-CP decides to setup the NG-U resource for the MBS service that was unestablished, due to the problem in the existing NG-U transmission. The gNB-CU-CP can send a MBS Session Modification Required message to the CN that has not establish the NG-U resource in the Session Start procedure. The Broadcast Session Modification Required message seems more in line the signalling definition principles that have been widely used in all other cases.

Proposal 1:	Agree to define Broadcast Session Modification Required/Confirm/Failure in NGAP to allow the gNB/CU-CP requesting the setup of NG-U resources during an ongoing broadcast MBS session for unicast transport.

In the last meeting, we discussed the triggers for this new class 1 procedure. As defined in SA2 and cited by the RAN3 during the discussion, the triggers could be one of the following:

- A broadcast MBS session, from which the shared NG-RAN receives DL data stream, is released, and no DL data stream is received in the NG-RAN from the remaining broadcast MBS session(s) with the same Associated Session ID.

- When the shared NG-RAN fails to receive DL data stream from the CN (e.g., due to failure in the user plane), the NG-RAN attempts to get the DL data stream via another CN's user plane.

If the NG-RAN is the split architecture, for the second trigger, only the gNB-CU-UP is aware if there is DL data stream from the CN. Therefore the new message should be triggered by the gNB-CU-UP originally. The gNB-CU-UP notifies there is no data transmission to the gNB-CU-CP due to failure in the user plane. Then the gNB CU-CP can send the message to another core network.

The BC Bearer Context Modification Required message in E1AP can be used for the second trigger. After the gNB-CU-CP triggers the data transmission in another core network (e.g. CN2), the gNB-CU-UP switches the data reception from CN1 to CN2.  The following question is the gNB-CU-UP uses the new MBS bearer context for CN2 or re-use the previously allocated bearer context. There are two methods about the MBS resource in the gNB-CU-UP.

- Different MBS bearer context for the shared services. For each MBS service, the gNB-CU-CP sends a BC Bearer Context Setup Request message. For the resource efficiency, the MRB configuration in the BC MRB to Setup List IE can use the same configuration.  i.e. same MRB ID and configuration are used for all the shared MBS service.

The Association Session ID can also be included in each BC Bearer Context Setup Request message. From the Association Session ID, the gNB-CU-UP can find the relationship between different MBS service. According to the agreement that it is allow to establish only one NG-U resource for the shared MBS service, the gNB-CU-CP can indicate whether the NG user plane to be established or not in the BC Bearer Context Setup Request message.

- Same MBS bearer context for the shared services. For all the shared MBS services, the gNB-CU-CP only send one BC Bearer Context Setup Request. For example, if the gNB-CU-CP decides to setup NG-U resource for the first received MBS service, the gNB-CU-CP can send BC Context Setup or Modify Request message, including the first received MBS service ID, e.g. TMGI1, the resource configuration, e,g. PDCP configuration, and also include the associated MBS service ID, e.g. TMGI2. The associated MBS service ID is the newly defined IE, from the newly defined IE, the gNB-CU-UP knows the TMGI2 transferring the same service as the TMGI1. Association Session ID is not so necessary. If only one NG-U resource is setup for TMGI1 and TMGI2, in case there is some problem for TMGI1, the gNB-CU-UP knows there is still a chance to get data transmission from TMGI2. In this case,

Option 2 has less signalling transmission and can achieve the same effect as option1, therefore it is proposed to adopt option 2.

Proposal 2:	Agree to setup only one MBS bearer context for the shared MBS services in the gNB-CU-UP. It means only one E1 signalling association for the shared MBS services.

Proposal 3:	Agree to include the associated MBS Service ID (i.e. associated TMGI) in the BC Bearer Setup Request or Modification Request message in E1AP.

When the gNB-CU-UP finds there is failure in the user plane transmission, the gNB-CU-UP expects to receive the data from another core network. To support data continuous reception, the gNB-CU-UP can indicate the error in user plane and also include the expect GTP-U SN to the gNB-CU-CP. The gNB-CU-CP can send the Confirm message to the gNB-CU-UP and notify the expected GTP-U SN to the second core network. The second core network start to transmit data with the expected GTP-U SN. From the GTP-U SN, the gNB-CU-CP is aware there is error in the existing transmission path and then either to release the MBS bearer context or to setup another NG-U transmission path. The gNB-CU-UP can also include the new DL NG-U TNL in the BC Bearer Modification Required message.

Proposal 4:	Agree to include GTP-U SN and NG-U TNL in the BC Bearer Modification Required message to allow the gNB-CU-UP requesting the setup of another NG-U resource.

Proposal 5:	Agree to include GTP-U SN and NG-U TNL in the BC Session Modification Required message in NGAP sending from the gNB-CU-CP to the AMF/SMF.

Another small FFS is

whether an indication that the NG-RAN node has decided to not establish shared NG-U transport resources for the broadcast MBS session also applies to multicast transport.

For BC, the NG-U resource shall be included in the Session Start Response message for successful case. Therefore, we need to indicate why the NG-U resource is not included for successful case. Unlike BC, for multicast service, the NG-U transport resource is not included in the Session Start Response message but in the Distribution Request message. It is very normal that the NG-RAN sends the Session Start Response message without any NG-U transport information. Therefore, there is no need to tell the NG-U is setup or not in the Session Setup Response for MC. We can close this open issue.

2.2 Association Session ID transmission

In the last RAN3 meeting, we achieve the following agreement:

Add Associated Session ID IE in the same level of MBS Session ID IE in related NG/F1 messages. FFS on E1.

In the gNB-CU-UP perspective, if the sharing enhancement is applied, it means only one PDCP resource is used for all the shared MBS service. As discussed in above, the gNB-CU-CP can send one E1AP MBS Bearer Context Setup message to the gNB-CU-UP for the shared MBS services. In this case, the associated Session ID is not so necessary. In NGAP, the Associated Session ID is required since the gNB-CU-CP will receive multiple Session Start message from different CN. In a word, the association session ID transmission in E1 is pending to the agreement for the bearer context in the gNB-CU-UP.

Proposal 6:	 If only one Bearer context is setup in the gNB-CU-UP and the gNB-CU-CP includes the associated Service IDs in the Bearer Context Setup message, the inclusion of the Associated Session ID from the gNB-CU-CP to the gNB-CU-UP is not so necessary.

2.3 who determine RAN sharing optimization for MOCN

In the last RAN3 meeting, it was agreed

The CU-CP sends the MBS Service Area to DU and DU determines whether to apply RAN sharing optimization over the radio.  The agreement is applied to multiple cell-ID case. For the MOCN it is open on which node to decide whether to apply RAN sharing optimization. In detail ,there are three open issues on the determination of RAN sharing optimization.

FFS on the determination between CU and DU for MOCN case.

FFS on where to apply the RAN sharing optimization.

FFS on how DU determines whether RAN sharing enhancement applies to the corresponding cells, i.e. based on which information and based on which behavior.

RAN sharing optimization not only includes the Uu radio resource optimization, but also includes the L2 radio resource optimization. i.e. the resource in the gNB-CU-UP can be optimized as well. The gNB-CU-CP is the connection entity for the gNB-DU and the gNB-CU-UP, therefore the gNB-CU-CP is the most suitable entity to determine whether the RAN sharing optimization should be used.

Proposal 7:	The gNB-CU-CP determines whether to apply RAN sharing optimization for MOCN case. The RAN sharing optimization is applied in the gNB-DU and also in the gNB-CU-UP.

We have agreed to include the Associated Session ID in the F1 MBS Context Setup Request message. Based on the associated Session ID, the gNB-DU can know the Uu radio resource is shared.

2.4 Open issues for multiple cell-ID case

There are below two open issues for RAN sharing enhancement for multiple cell-ID case.

- FFS on how many F1-U tunnels should be established for multiple cell-ID broadcast cases.

- Open issue 1: Which option should be used by DU to decide the MRB-PDCP-ConfigBroadcast?

Option 1: OAM configure the MRB-PDCP-ConfigBroadcast of each MRB to CU or DU.

Option 2: DU applied the MRB-PDCP-ConfigBroadcast of the first CU or the second CU. FFS whether DU needs to provide the used MRB-PDCP-ConfigBroadcast to CU.

When the DU receives multiple BC Context Setup Request message containing the same MBS associated ID but different PDCP configuration, there is a working assumption for that case: the DUs decides which MRB-PDCP-ConfigBroadcast to provide on MCCH. If a F1-U tunnel is established to each gNB-CU, the DU will chose the data sending from the CU hosting the selected PDCP configuration and discard the data received from another F1-U tunnel. This goes against our original intention of effectively utilizing resources.

To support resource efficiency, the gNB-DU can send an indication that the F1-U transport resource is not established in the BC Context Setup Response. It is possible the F1 transmission is switched from one CU to another CU, therefore the DU can transmit the selected PDCP configuration in the Response message. Thus the same PDCP configuration will be used in case the F1 transmission is switched. .

Proposal 8:	Agree to setup only one F1-U tunnel for RAN sharing with multiple cell-ID broadcasting case.

Proposal 9:	For RAN Sharing with multiple cell-ID broadcasting, the gNB-DU can select one F1-U to use. For the un-selected F1-U, the DU can indicate the F1-U is unestablished and provide the selected MRB-PDCP-ConfigBroadcast in the BC Context Setup Response message.

3. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1:	Agree to define Broadcast Session Modification Required/Confirm/Failure in NGAP to allow the gNB/CU-CP requesting the setup of NG-U resources during an ongoing broadcast MBS session for unicast transport.

Proposal 2:	Agree to setup only one MBS bearer context for the shared MBS services in the gNB-CU-UP. It means only one E1 signalling association for the shared MBS services.

Proposal 3:	Agree to include the associated MBS Service ID (i.e. associated TMGI) in the BC Bearer Setup Request or Modification Request message in E1AP.

Proposal 4:	Agree to include GTP-U SN and NG-U TNL in the BC Bearer Modification Required message to allow the gNB-CU-UP requesting the setup of another NG-U resource.

Proposal 5:	Agree to include GTP-U SN and NG-U TNL in the BC Session Modification Required message in NGAP sending from the gNB-CU-CP to the AMF/SMF.

Proposal 6:	 If only one Bearer context is setup in the gNB-CU-UP and the gNB-CU-CP includes the associated Service IDs in the Bearer Context Setup message, the inclusion of the Associated Session ID from the gNB-CU-CP to the gNB-CU-UP is not so necessary.

Proposal 7:	The gNB-CU-CP determines whether to apply RAN sharing optimization for MOCN case. The RAN sharing optimization is applied in the gNB-DU and also in the gNB-CU-UP.

Proposal 8:	Agree to setup only one F1-U tunnel for RAN sharing with multiple cell-ID broadcasting case.

Proposal 9:	For RAN Sharing with multiple cell-ID broadcasting, the gNB-DU can select one F1-U to use. For the un-selected F1-U, the DU can indicate the F1-U is unestablished and provide the selected MRB-PDCP-ConfigBroadcast in the BC Context Setup Response message.

4. Text Proposal to TS 37.483

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

8.14	NR MBS Procedures

8.14.1	Broadcast Context Setup

8.14.1.1	General

The purpose of the Broadcast Context Setup procedure is to establish an MBS Session context for a broadcast session in the gNB-DU.

The procedure uses MBS-associated signalling.

8.14.1.2	Successful Operation

Figure 8.14.1.2-1: Broadcast Context Setup procedure: Successful Operation

The gNB-CU initiates the procedure by sending BROADCAST CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the broadcast MBS Session context, it replies to the gNB-CU with BROADCAST CONTEXT SETUP RESPONSE.

If the MBS Service Area IE is included in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-DU shall take this information into account for shared F1-U tunnel assignment.

The gNB-DU shall report to the gNB-CU, in the BROADCAST CONTEXT SETUP RESPONSE message, the result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Broadcast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Broadcast MRB Failed To Be Setup List IE;

If the Broadcast MRB Failed To Setup List IE is contained in the BROADCAST CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the Broadcast MRB(s) failed to be setup with an appropriate cause value for each Broadcast MRB failed to setup.

If the Sharing associated information IE is contained in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-CU-UP shall take this information into account for shared resource allocation.

8.14.1.3	Unsuccessful Operation

Figure 8.14.1.3-1: Broadcast Context Setup procedure: unsuccessful Operation

If the gNB-DU is not able to establish the requested MBS session context it shall consider the procedure as failed and reply with the BROADCAST CONTEXT SETUP FAILURE message.

8.14.1.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.6.1.2	BC Bearer Context Modification (gNB-CU-CP initiated)

8.6.1.2.1	General

The purpose of the gNB-CU-CP initiated BC Bearer Context Modification procedure is to allow the gNB-CU-CP to modify MBS session resources for a broadcast MBS session. The procedure uses MBS-associated signalling.

8.6.1.2.2	Successful Operation

Figure 8.6.1.2.2-1: BC Bearer Context Modification procedure, gNB-CU-CP initiated: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BC BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to perform at least partially the requested modifications it replies to the gNB-CU-CP with the BC BEARER CONTEXT MODIFICATION RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BC BEARER CONTEXT MODIFICATION RESPONSE message, the result of all the requested MBS session resources in the following way:

-	A list of BC MRBs which are successfully established or modified shall be included in the BC MRB Setup or Modify Response List IE;

-	A list of BC MRBs which failed to be established or modified shall be included in the BC MRB Failed List IE;

-	For each newly established or modified BC MRB, a list of MBS QoS Flows which are successfully established or modified shall be included in the MBS QoS Flow Setup List IE;

-	For each newly established or modified BC MRB, a list of MBS QoS Flows which failed to be established or modified shall be included in the MBS QoS Flow Failed List IE.

When the gNB-CU-UP reports the unsuccessful establishment of a BC MRB or MBS QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the BC Bearer Context NG-U TNL Info at 5GC To Setup or Modify IE is contained in the BC BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the previously received BC Bearer Context NG-U TNL Info at 5GC.

If the Sharing associated information IE is contained in the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take this information into account for shared resource allocation.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.13	Broadcast Context Management messages

9.2.13.1	BROADCAST CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of an MBS session context for a broadcast session, and establish an MBS-associated logical F1-connection.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| MBS CU to DU RRC Information | M |  | 9.3.1.225 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.38 |  | YES | reject |
| Broadcast MRB To Be Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at CU | O |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |
| Sharing associated information | O |  | 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.5.2.4	MC BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify MBS session resources for a multicast MBS session.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.106 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.107 |  | YES | reject |
| MC Bearer Context To Modify | M |  | 9.3.3.34 |  | YES | reject |
| Sharing associated information | O |  | 9.3.1.x |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.1.x Sharing associated information

This IE gives the sharing associated information. It includes the non-PLMN specific identifier and may include the associated TMGI list.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Sharing associated session identifier | O |  | FFS | non-PLMN specific identifier and globally identify the broadcast service data at the NG-RAN |
| Associated TMGI List |  |  |  |  |
| > Associated TMGI Item |  | 1 .. <maxAssociatedTMGI> |  |  |
| >>TMGI | M |  | OCTET STRING (SIZE(6)) |  |

| Range bound | Explanation |
| --- | --- |
| maxAssociatedTMGI | Maximum no. of associated TMGI in RAN sharing. Value is FFS. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.3.30	BC Bearer Context To Modify Required

This IE contains MBS session resource related information used to request BC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BC MRB To Remove List Required |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| GTP-U SN Information | O | FFS |  |  |
| BC Bearer Context NG-U TNL Info at NG-RAN | O |  | 9.3.1.116 |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.3.31	BC Bearer Context To Modify Confirm

This IE contains MBS session resource related information used to confirm a BC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session ID | O |  | OCTET STRING (SIZE(6)) |  | - | - |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>