3GPP TSG-RAN WG3 #121										R3-234009

21th – 25th Aug 2023

Toulouse, France

Agenda item:	13.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for Mobile IAB TS38.413 BL CR) Discussion on Support IAB-node mobility

Document for:	Discussion and Decision

# Introduction

This contribution discusses the remaining issues for mobile IAB integration and IAB-DU migration.

# Different IAB-donor for IAB-MT and IAB-DU

Last RAN3 meeting agreed

WA: The mIAB-DU and mIAB-MT can integrate at different CUs. For this purpose, OAM can be used to configure the mIAB-DU with: a) the donor CU to connect to, and b) the parameters used by the mIAB-DU to establish TNL associations, IPSec tunnels and F1 connectivity to this donor CU.

Whether the information on the DU’s CU can also be configured by the MT’s CU.

First, there is no technical issue for above WA.

This is similar as Rel-16/17 that the IAB-DU is configured by its OAM server regarding the donor-CU’s FQDN/IP address, FQDN/IP address of SeGW, NCGI, TAC and other DU related parameters. In a specific location where a Rel-16/17 IAB will be deployed, there may be more than one candidate parent cell. The candidate parent cells may belong to same or different IAB-donor. The IAB need to be configured with the appropriate NCGI/TAC in order to be able to setup F1 with the right IAB-donor-CU. In Rel-16/17, the IAB-MT and the co-located IAB-DU only connect with same IAB-donor. For example, IAB-MT connects with IAB-donor1-CU, OAM should configure IAB-DU with a NCGI related to IAB-donor1, rather IAB-donor2. The OAM can perform the configuration based on the location of the IAB, e.g. IAB-DU reports the serving cell information (e.g. TAI, NCGI) of the co-located IAB-MT to OAM server, then OAM server configures the related IAB-DU parameters.

In Rel-18, up to the operator, the mIAB-MT and its co-located mIAB-DU can connect with (or handover/migrate to) different IAB-donors. The operator may designate specific IAB-donors to be used for IAB-DU, for example, assigning a few IAB-donors to support the IAB-DU in the whole city, thus reduce the IAB-DU migration. This can be implemented via OAM, and to be same as Rel-16/17. For example, if operator configures to use a different IAB-donor for the mIAB-DU, OAM can configure the FQDN/IP address of a different IAB-donor to the mIAB-DU, based on the location of the mIAB-MT (e.g. TAI, NCGI of the serving cell for the mIAB-MT). This aligns with another RAN3 agreement “The mIAB-node may obtain the IP address of target CU for mIAB-DU migration and the IP address of its SeGW from the OAM.” So we propose to turn the WA into agreement.

Proposal 1-1: turn the following WA into agreement

WA: The mIAB-DU and mIAB-MT can integrate at different CUs. For this purpose, OAM can be used to configure the mIAB-DU with: a) the donor CU to connect to, and b) the parameters used by the mIAB-DU to establish TNL associations, IPSec tunnels and F1 connectivity to this donor CU.

As explained above, it is up to the operator/deployment to decide the IAB-donor for the IAB-DU, the IAB-MT’s CU does not have this knowledge. So it is not possible for the IAB-MT’s CU to decide the IAB-donor for the IAB-DU. The IAB-DU need to be configured by OAM for many DU parameters before it can initiate F1 Setup. There is no need to introduce another method in addition to the OAM-based method in the same Release. So we propose Rel-18 only support the OAM-based method, and not introduce the configuration of MT’s CU.

Proposal 1-2: the information on the IAB-DU’s CU cannot be configured by the MT’s CU.

In Rel-16/17, when the IAB-DU initiate the F1 Setup with the IAB-DU’s CU, the F1 SETUP REQUEST message includes the BAP address, in order for the CU to discover the collocation of an IAB-DU and an IAB-MT.

In Rel-18, when the IAB-MT’s CU and IAB-DU’s CU are different at the integration, the BAP address of the IAB is assigned by the IAB-MT’s CU. The same BAP address may be already assigned by the IAB-DU’s CU for another node, e.g. a donor-DU, or another IAB-MT when this IAB-DU’s CU terminates the RRC of another IAB-MT. So when the IAB-DU initiate the F1 Setup procedure, the F1 SETUP REQUEST message need to indicate which CU (i.e. IAB-MT’s CU) has assigned the BAP address. This can be performed by adding the gNB ID of the IAB-MT’s CU in the F1 SETUP REQUEST message. When the IAB-DU’s CU receives the F1 SETUP REQUEST message including the BAP address and the ID of the CU which assigned this BAP address, the IAB-DU’s CU can know the IAB-MT’s CU is a different CU. This also enables the IAB-DU’s CU to further initiate the XnAP TMM procedure after a UE connect with the IAB-DU.

Proposal 1-3: add the ID (e.g. gNB ID) of the IAB-MT’s CU in the F1 SETUP REQUEST message, in order to enable IAB-DU’s CU know the IAB-MT uses a different CU.

# XnAP ID during consecutive partial migration and IAB-DU migration

Last RAN3 meeting failed to reach agreement on how to pass the XnAP ID:

It is FFS whether the XnAP ID can be transparently passed from the mIAB-MT’s CU to the mIAB-MT via RRC, then from the co-located mIAB-DU to mIAB-DU’s CU via F1AP.

There are two options:

- Option A: XnAP signalling from the mIAB-DU’s source CU.

- Option B: F1AP signalling from the target logical mIAB-DU.

This issue is also applicable to consecutive partial migration, i.e. paritial migration is performed from IAB-donor1 to IAB-donor2, then to IAB-donor3. IAB-donor1 need to know the gNB ID of IAB-donor3 and XnAP UE ID assigned by IAB-donor3.    In IAB-DU migration, let’s assume IAB-donor4-CU is the IAB-DU’s target CU.

The main purpose for Option A and Option B is to enable the IAB-DU’s donor-CU to know the IAB-donor who terminates the IAB-MT’s RRC, so the IAB-DU’s donor-CU can initiate the Xn IAB Transport Migration Management (TMM) procedure.

- In case of consecutive partial migration, it is the IAB-DU’s source donor-CU (CU1) need to know.

- In case of IAB-DU migration, it is the IAB-DU’s target donor-CU (CU4) need to know.

The example procedure for both Option A and B are copied below (reused the figure from R3-231309).

The following enhancement is needed for Option A and Option B:

- Option A: IAB-MT’s target CU need to provide the XnAP ID to the IAB-MT’s source CU, then is then forwarded to the IAB-DU’s source CU (or target CU) via Xn.

In case no Xn between the IAB-MT’s source CU and target CU, target CU have to allocate the XnAP ID even it is not used in the IAB-MT’s NG-handover procedure. The XnAP ID need to be transferred via NGAP, and possibility via a Transparent Container to avoid the impact to AMF.

Option A requires change to both XnAP, and NGAP (in case of NG-HO is performed for IAB-MT).

- Option B: IAB-MT’s target CU provides the XnAP ID and gNB ID to the IAB-MT via RRC. IAB-MT then provide the XnAP ID and gNB ID of its target CU to the co-located IAB-DU, which is then forwarded to IAB-DU’s source CU (or target CU) via F1AP.

Option B requires change to RRC and F1AP. IAB-MT may know the gNB ID in case the gNB ID length is included in SIB1. It is up to RAN2 to decide.

In both Option A and Option B, the gNB ID of IAB-MT’s target CU is anyway needed to be transferred to IAB-DU’s CU. In current F1 Setup procedure, IAB-DU’s CU need to check the BAP address received in the F1 SETUP REQUEST message to determine the co-location of IAB-MT and IAB-DU. In Rel-18, gNB ID is needed in order to inform IAB-DU’s CU that the BAP address is assigned by another IAB-donor, so it does not make the wrong decision to decide the co-located IAB-MT, e.g. same BAP address is already assigned by IAB-donor1-CU (or IAB-donor4-CU) to a different IAB-MT.

Observation 2-1: irrespective Option A or Option B, IAB-DU need to provide the gNB ID of the IAB-MT’s donor-CU to IAB-DU’s donor-CU.

Both options work, but Option B may be simple and work for all scenarios. So it is preferred to adopt Option B.

Proposal 2-1: RAN3 adopt Option B, and inform RAN2 to enhance RRC to include the XnAP ID and gNB ID of the IAB-MT’s target CU.

Last RAN3 meeting agreed The DU’s CU can initiate the Xn TM Management Procedure pertaining to an mIAB-MT even though it has never had an RRC connection with this IAB-MT.

This may require some modification to the XnAP ID handling for the Xn TMM procedure which is 1st XnAP procedure between the two CUs.

In Rel-17, the Xn TMM procedure is performed after the Xn HO procedure or Xn DC procedure for the IAB-MT. The Xn HO or DC procedure assign both XnAP IDs and establish the UE-associated logical connection for the IAB-MT in the peer nodes. Later, the F1-terminating donor initiate the Xn TMM procedure.

- The F1-terminating donor just reuse the XnAP IDs assigned during the Xn HO/DC procedures.

- When the non-F1 terminating donor receive the Xn TMM REQUEST message, it follows the normal AP ID handling, i.e. check whether the 2 XnAP IDs are known (e.g. same as the ones created during the Xn HO/DC procedure) as defined in TS38.413.

In Rel-18 consecutive partial migration or IAB-DU migration, the Xn TMM is the first Xn procedure for the IAB-MT between the IAB-DU’s source CU (CU1) and IAB-MT’s target CU (CU3), or between the IAB-DU’s target CU (CU4) and IAB-MT’s target CU (CU3). It requires new behavior in the F1-terminating donor (CU1 or CU4), and the non-F1-terminating donor (CU3).

- The F1-terminating donor assign the XnAP ID, and include the received XnAP ID (received via Xn or F1 via previous proposal) in the Xn TMM REQUEST message.

- Upon the reception of Xn TMM REQUEST message, the non-F1 terminating IAB-donor first check whether there is an UE-associated logical connection for the IAB-MT over the Xn with the peer node.

- If yes, the non-F1-termninating IAB-donor check the remote AP ID (as normal Rel-17)

- If no, the non-F1-termninating IAB-donor create the UE-associated logical connection, BUT not assign new XnAP ID. This is different to normal AP ID handling for “First message” as described in TS38.413. Instead, it reuse the received XnAP ID for the “First returned message”, i.e. the Xn TMM RESPONSE message. TS38.413 text is copied as below:

NOTE: The "first message", the "first returned message" and the "last message" as used below correspond to messages for a UE-associated logical connection. The "first message" has a new AP ID from the sending node and the "first returned message" is the first response message, which has a new AP ID from the node sending the "first returned message". Thereafter the two AP IDs are included in all messages over the UE-associated logical connection unless otherwise allowed by the specification.

Observation 2-2: when the Xn TMM procedure is the 1st XnAP procedure between the IAB-donors, the AP ID handling is different to normal AP ID handling.

The required modification to the spec can be discussed later after the BL CR is available. A possible change can be: when the Xn TMM procedure is the 1st XnAP procedure between the IAB-donors, the F1-terminating donor can assign the XnAP ID. The non-F1-terminating donor reuse the XnAP ID rather assign a new XnAP ID.

Retention of MT’s UE XnAP IDs by mIAB-Du’s CU:

In case of consecutive partial migration, the XnAP ID is retained in the F1-termninating donor-CU, similar to Rel-17. No enhancement is needed.

In case of IAB-DU migration, there are 2 IAB-donors, i.e. one (source IAB-donor) for IAB-DUa (source IAB-DU), and the other (target IAB-donor) for IAB-DUb (target IAB-DU). Both IAB-donor appear as F1-terminating IAB-donor to the IAB-MT’s IAB-donor. The XnAP ID is retained, similar to Rel-17.

Proposal 2-2: Same as Rel-17, the XnAP ID is retained as long the target path in the non-F1-terminating donor’s topology is used to transport the traffic between the IAB-DU and the F1-terminating donor.

# Target F1 Setup

Previous RAN3 meeting agreed

The IAB-node can inform the source logical mIAB-DU’s CU via F1AP about the successful F1 Setup with the target logical mIAB-DU’s CU, and it can include the IDs of the cells activated by the target logical mIAB-DU’s CU.

Last RAN3 meeting agreed:

WA: As an enhancement to legacy handovers, the IAB-node may provide to the source DU’s CU a mapping between the source DU’s activated cells and the target DU’s activated cells so that the source DU’s CU can perform handover for the connected UEs. 
This agreement does not relate to the configuration sharing between two logical collocated mIAB-DUs

The two agreements are related. Source CU need to be informed by IAB-DU via F1, regarding the successfully established F1 and the new (activated) cell ID(s) of target IAB-DU. Source CU need to know the new cell ID in order to determine the mandatory target cell ID IE in the Xn/NG HO signaling. Just like Rel-16/17, an IAB-DU may have more than 1 (activated) cells.

Figure 2: Example for IAB-DU with more than 1 cell

When the IAB-DU has more than 1 cell, source CU need to determine the right target cell for a UE. Simply using the measurement report, source CU may incorrectly select an outside cell which may have stronger signal than target IAB-DU’s cell. The information of mapping from source IAB-DU cell to target IAB-DU cell can help. Via the cell ID mapping information, source CU can select the right target IAB-DU cell for an onboard UE. This is NOT related to the configuration sharing between two IAB-DUs.

Proposal 3-1: turn the following WA into agreement

WA: As an enhancement to legacy handovers, the IAB-node may provide to the source DU’s CU a mapping between the source DU’s activated cells and the target DU’s activated cells so that the source DU’s CU can perform handover for the connected UEs.

Regarding the F1 procedure to inform IAB to setup target F1, and the F1 procedure to inform source CU for the successfully established F1, there is no agreement on whether reuse existing F1 CU/DU Configuration Update procedure, or introduce new F1 procedure. In case reuse existing F1 procedure, the purpose section of the CU/DU Configuration Update procedure may need to be updated. Since this is a small function, introducing new F1 procedure may be overkill. So we slightly prefer to reuse existing F1 procedures.

Proposal 3-2: reuse existing F1 procedures to inform IAB-DU setup target F1, and inform source CU after the target F1 is successfully established.

# IAB-MT HO via NG-HO

The IAB-MT HO can be Xn-HO or NG-HO. The NG-HO is used when there is no Xn, or when there is a need to change AMF (e.g. mobile IAB move to another AMF’s serving area). In the later case, even there is Xn, NG-HO shall be used in order to change AMF.

Last RAN3 meeting agreed

Include “No PDU Session Indication” in the NGAP HANDOVER REQUEST message.

WA: legacy NG HO Required message with “dummy” PDU information is used. It needs to be further discussed if any specification changes are needed to describe the use of such dummy information. If specification changes is needed, consider whether an explicit indication is worth adding.

The main issue is how to handle the mandatory PDU Session Resource List IE in the NGAP HANDOVER REQUIRED message, and the PDU Session Resource Setup List IE in the NGAP HANDOVER REQUEST message. When the IAB-MT does not have a PDU Session, the PDU Session Resource List IE and the PDU Session Resource Setup List IE contains dummy information and shall be ignored by the receiver. RAN3 spec already have some examples that the receiver shall ignore a mandatory IE. For example

If the UE Security Capabilities IE included in the INITIAL CONTEXT SETUP REQUEST message only contains the EIA0 or NIA0 algorithm as defined in TS 33.501 [13] and if the EIA0 or NIA0 algorithm is defined in the configured list of allowed integrity protection algorithms in the NG-RAN node (TS 33.501 [13]), the NG-RAN node shall take it into use and ignore the keys received in the Security Key IE.

If the Extended AMF Name IE is included in the NG SETUP RESPONSE message, the NG-RAN node may use this IE as a human readable name of the AMF and shall ignore the AMF Name IE if also included.

Similar method can be used to handle the mandatory PDU Session Resource List IE in the NGAP HANDOVER REQUIRED message, and the PDU Session Resource Setup List IE in the NGAP HANDOVER REQUEST message, i.e. the receiver ignore them when the IAB-MT does not have a PDU session.

Even RAN3 introduce an explicit indication, the receiver still needs to ignore the mandatory PDU Session Resource List IE in the NGAP HANDOVER REQUIRED message, and the PDU Session Resource Setup List IE in the NGAP HANDOVER REQUEST message. So there is no benefit for introducing an explicit indication.

The Handover Required Transfer IE is transparent to the AMF. So it is the SMF shall ignore this IE when the IAB-MT does not have PDU session. This needs to be addressed by SA2 in TS23.502.

Current TS38.413 defines following for the PDU Session Resource Setup List IE:

Upon receiving the PDU Session Resource Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure.

In case the IAB-MT does not have PDU session, the target gNB shall ignore the PDU Session Resource Setup List IE and not take action to setup the PDU session. An example for the possible changes is below:

if the No PDU Session Indication IE is contained in the HANDOVER REQUEST message, the NG-RAN node shall, if supported, consider the UE as an IAB-node which does not have any PDU sessions activated, and ignore the PDU Session Resource Setup List IE, and shall not take any action with respect to PDU session setup.

Proposal 4-1: RAN3 agree the target gNB shall ignore the PDU Session Resource Setup List IE and not take action to setup the PDU session, when the IAB-MT does not have PDU session.

A draft TS38.413 CR can be found at Annex – TP for TS38.413 BL CR

This also affects the SA2 spec (TS23.502). For example, Nsmf operation (Step 4, 7, 11a, 11f in below figure) may not be needed when the IAB-MT does not have a PDU session and should be changed to conditional for IAB-MT. RAN3 should send a LS to SA2.

Proposal 4-2: RAN3 send LS to SA2 asking for the support for NG-HO without PDU session for the IAB-MT.

# Conclusions

For IAB-DU migration, our proposals are:

Proposal 1-1: turn the following WA into agreement

WA: The mIAB-DU and mIAB-MT can integrate at different CUs. For this purpose, OAM can be used to configure the mIAB-DU with: a) the donor CU to connect to, and b) the parameters used by the mIAB-DU to establish TNL associations, IPSec tunnels and F1 connectivity to this donor CU.

Proposal 1-2: the information on the IAB-DU’s CU cannot be configured by the MT’s CU.

Proposal 1-3: add the ID (e.g. gNB ID) of the IAB-MT’s CU in the F1 SETUP REQUEST message, in order to enable IAB-DU’s CU know the IAB-MT uses a different CU.

Proposal 2-1: RAN3 adopt Option B, and inform RAN2 to enhance RRC to include the XnAP ID and gNB ID of the IAB-MT’s target CU.

Proposal 2-2: Same as Rel-17, the XnAP ID is retained as long the target path in the non-F1-terminating donor’s topology is used to transport the traffic between the IAB-DU and the F1-terminating donor.

Proposal 3-1: turn the following WA into agreement

WA: As an enhancement to legacy handovers, the IAB-node may provide to the source DU’s CU a mapping between the source DU’s activated cells and the target DU’s activated cells so that the source DU’s CU can perform handover for the connected UEs.

Proposal 3-2: reuse existing F1 procedures to inform IAB-DU setup target F1, and inform source CU after the target F1 is successfully established.

Proposal 4-1: RAN3 agree the target gNB shall ignore the PDU Session Resource Setup List IE and not take action to setup the PDU session, when the IAB-MT does not have PDU session.

A draft TS38.413 CR can be found at Annex – TP for TS38.413 BL CR

Proposal 4-2: RAN3 send LS to SA2 asking for the support for NG-HO without PDU session for the IAB-MT.

# References

- R3-233345, Offline discussion on Mobile IAB

- TS23.502

# Annex – TP for TS38.413 BL CR

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.4.1	Handover Preparation

#### 8.4.1.1	General

The purpose of the Handover Preparation procedure is to request the preparation of resources at the target side via the 5GC. There is only one Handover Preparation procedure ongoing at the same time for a certain UE. The procedure uses UE-associated signalling.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2-1: Handover preparation: successful operation

The source NG-RAN node initiates the handover preparation by sending the HANDOVER REQUIRED message to the serving AMF. When the source NG-RAN node sends the HANDOVER REQUIRED message, it shall start the timer TNGRELOCprep. The source NG-RAN node shall indicate the appropriate cause value for the handover in the Cause IE.

Upon reception of the HANDOVER REQUIRED message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transparently transfer the Handover Required Transfer IE to the SMF associated with the concerned PDU session. If the UE is an IAB-MT which does not have any PDU sessions activated, the AMF shall ignore the PDU Session Resource List IE, and behave as specified in TS 23.502 [10].

In case of intra-system handover, the information in the Source to Target Transparent Container IE shall be encoded according to the definition of the Source NG-RAN node to Target NG-RAN node Transparent Container IE.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Upon receipt of the HANDOVER REQUEST message the target NG-RAN node shall

-	attempt to execute the requested PDU session configuration and associated security;

-	store the received UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for all Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9];

-	store the received Mobility Restriction List in the UE context;

-	store the received UE Security Capabilities in the UE context;

-	store the received Security Context in the UE context and take it into use as defined in TS 33.501 [13];

-	if supported, store the received UE Slice Maximum Bit Rate List in the UE context and use the received UE Slice Maximum Bit Rate List for each S-NSSAI for the concerned UE as specified in TS 23.501 [9].

Upon reception of the UE History Information IE, which is included within the Source to Target Transparent Container IE of the HANDOVER REQUEST message, the target NG-RAN node shall collect the information defined as mandatory in the UE History Information IE and shall, if supported, collect the information defined as optional in the UE History Information IE, for as long as the UE stays in one of its cells, and store the collected information to be used for future handover preparations.

Upon receiving the PDU Session Resource Setup List IE contained in the HANDOVER REQUEST message and the HANDOVER REQUEST message does not contain the No PDU Session Indication IE, the target NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure. The target NG-RAN node shall report to the AMF in the HANDOVER REQUEST ACKNOWLEDGE message the result for each PDU session resource requested to be setup. In particular, for each PDU session resource successfully setup, it shall include the Handover Request Acknowledge Transfer IE containing the following information:

-	The list of QoS flows which have been successfully established in the QoS Flow Setup Response List IE.

-	The Data Forwarding Accepted IE if the data forwarding for the QoS flow is accepted.

-	The list of QoS flows which have failed to be established, if any, in the QoS Flow Failed to Setup List IE.

-	The UP transport layer information to be used for the PDU session.

-	The security result associated to the PDU session.

-	The redundant UP transport layer information to be used for the redundant transmission for the PDU session.

** Unchanged text skipped **

If the IAB Authorized IE is contained in the HANDOVER REQUEST message, the NG-RAN node shall, if supported, consider that the handover is for an IAB node. In addition, if the No PDU Session Indication IE is contained in the HANDOVER REQUEST message, the NG-RAN node shall, if supported, consider the UE as an IAB-node which does not have any PDU sessions activated, and ignore the PDU Session Resource Setup List IE, and shall not take any action with respect to PDU session setup.

If the Enhanced Coverage Restriction IE is included in the HANDOVER REQUEST message, the NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [9].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.4	HANDOVER REQUEST

This message is sent by the AMF to the target NG-RAN node to request the preparation of resources.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| Handover Type | M |  | 9.3.1.22 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | M |  | 9.3.1.58 |  | YES | reject |
| Core Network Assistance Information for RRC INACTIVE | O |  | 9.3.1.15 |  | YES | ignore |
| UE Security Capabilities | M |  | 9.3.1.86 |  | YES | reject |
| Security Context | M |  | 9.3.1.88 |  | YES | reject |
| New Security Context Indicator | O |  | 9.3.1.55 |  | YES | reject |
| NASC | O |  | NAS-PDU 9.3.3.4 | Refers to either the “Intra N1 mode NAS transparent container” or the “S1 mode to N1 mode NAS transparent container”, the details of the IE definition and the encoding arespecified in TS 24.501 [26]. | YES | reject |
| PDU Session Resource Setup List |  | 1 |  |  | YES | reject |
| >PDU Session Resource Setup Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>S-NSSAI | M |  | 9.3.1.24 |  | - |  |
| >>Handover Request Transfer | M |  | OCTET STRING | Containing the PDU Session Resource Setup Request Transfer IE specified in subclause 9.3.4.1. | - |  |
| >>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.3.1.94 | Expected UE Activity Behaviour for the PDU Session. | YES | ignore |
| Allowed NSSAI | M |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network. | YES | reject |
| Trace Activation | O |  | 9.3.1.14 |  | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.54 |  | YES | ignore |
| Source to Target Transparent Container | M |  | 9.3.1.20 |  | YES | reject |
| Mobility Restriction List | O |  | 9.3.1.85 |  | YES | ignore |
| Location Reporting Request Type | O |  | 9.3.1.65 |  | YES | ignore |
| RRC Inactive Transition Report Request | O |  | 9.3.1.91 |  | YES | ignore |
| GUAMI | M |  | 9.3.3.3 |  | YES | reject |
| Redirection for Voice EPS Fallback | O |  | 9.3.1.116 |  | YES | ignore |
| CN Assisted RAN Parameters Tuning | O |  | 9.3.1.119 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.3.1.128 |  | YES | ignore |
| IAB Authorized | O |  | 9.3.1.129 |  | YES | reject |
| Enhanced Coverage Restriction | O |  | 9.3.1.140 |  | YES | ignore |
| UE Differentiation Information | O |  | 9.3.1.144 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.3.1.146 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.147 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.148 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.149 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 QoS Parameters | O |  | 9.3.1.150 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| CE-mode-B Restricted | O |  | 9.3.1.155 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.3.1.160 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  | YES | ignore |
| UE Radio Capability ID | O |  | 9.3.1.142 |  | YES | reject |
| Extended Connected Time | O |  | 9.3.3.31 |  | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.3.1.220 |  | YES | ignore |
| UE Slice Maximum Bit Rate List | O |  | 9.3.1.231 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.233 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.3.1.234 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AllowedNSSAI,

AMFName,

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

WUS-Assistance-Information,

RIMInformationTransfer,

NoPDUSessionIndication

FROM NGAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

NGAP-PRIVATE-IES,

NGAP-PROTOCOL-EXTENSION,

NGAP-PROTOCOL-IES,

NGAP-PROTOCOL-IES-PAIR

FROM NGAP-Containers

id-AllowedNSSAI,

id-AMFName,

id-AMFOverloadResponse,

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-WarningType,

id-WUS-Assistance-Information,

id-RIMInformationTransfer,

id-NoPDUSessionIndication

FROM NGAP-Constants;

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- Handover Resource Allocation Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- HANDOVER REQUEST

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {HandoverRequestIEs} },

...

}

HandoverRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-HandoverType							CRITICALITY reject	TYPE HandoverType									PRESENCE mandatory	}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory	}|

{ ID id-UEAggregateMaximumBitRate				CRITICALITY reject	TYPE UEAggregateMaximumBitRate					PRESENCE mandatory	}|

{ ID id-CoreNetworkAssistanceInformationForInactive		CRITICALITY ignore	TYPE CoreNetworkAssistanceInformationForInactive		PRESENCE optional		}|

{ ID id-UESecurityCapabilities					CRITICALITY reject	TYPE UESecurityCapabilities						PRESENCE mandatory	}|

{ ID id-SecurityContext							CRITICALITY reject	TYPE SecurityContext								PRESENCE mandatory	}|

{ ID id-NewSecurityContextInd					CRITICALITY reject	TYPE NewSecurityContextInd						PRESENCE optional		}|

{ ID id-NASC									CRITICALITY reject	TYPE NAS-PDU										PRESENCE optional		}|

{ ID id-PDUSessionResourceSetupListHOReq		CRITICALITY reject	TYPE PDUSessionResourceSetupListHOReq			PRESENCE mandatory	}|

{ ID id-AllowedNSSAI							CRITICALITY reject	TYPE AllowedNSSAI									PRESENCE mandatory	}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation								PRESENCE optional		}|

{ ID id-MaskedIMEISV							CRITICALITY ignore	TYPE MaskedIMEISV									PRESENCE optional		}|

{ ID id-SourceToTarget-TransparentContainer		CRITICALITY reject	TYPE SourceToTarget-TransparentContainer		PRESENCE mandatory	}|

{ ID id-MobilityRestrictionList					CRITICALITY ignore	TYPE MobilityRestrictionList						PRESENCE optional		}|

{ ID id-LocationReportingRequestType			CRITICALITY ignore	TYPE LocationReportingRequestType				PRESENCE optional		}|

{ ID id-RRCInactiveTransitionReportRequest		CRITICALITY ignore	TYPE RRCInactiveTransitionReportRequest		PRESENCE optional		}|

{ ID id-GUAMI									CRITICALITY reject	TYPE GUAMI											PRESENCE mandatory	}|

{ ID id-RedirectionVoiceFallback				CRITICALITY ignore	TYPE RedirectionVoiceFallback					PRESENCE optional		}|

{ ID id-CNAssistedRANTuning						CRITICALITY ignore	TYPE CNAssistedRANTuning							PRESENCE optional		}|

{ ID id-SRVCCOperationPossible					CRITICALITY ignore	TYPE SRVCCOperationPossible						PRESENCE optional		}|

{ ID id-IAB-Authorized							CRITICALITY reject	TYPE IAB-Authorized									PRESENCE optional		}|

{ ID id-Enhanced-CoverageRestriction			CRITICALITY ignore	TYPE Enhanced-CoverageRestriction				PRESENCE optional		}|

{ ID id-UE-DifferentiationInfo					CRITICALITY ignore	TYPE UE-DifferentiationInfo						PRESENCE optional		}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized						PRESENCE optional		}|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized					PRESENCE optional		}|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate	PRESENCE optional		}|

{ ID id-PC5QoSParameters						CRITICALITY ignore	TYPE PC5QoSParameters								PRESENCE optional 	}|

{ ID id-CEmodeBrestricted						CRITICALITY ignore	TYPE CEmodeBrestricted								PRESENCE optional		}|

{ ID id-UE-UP-CIoT-Support						CRITICALITY ignore	TYPE UE-UP-CIoT-Support								PRESENCE optional		}|

{ ID id-ManagementBasedMDTPLMNList				CRITICALITY ignore	TYPE MDTPLMNList									PRESENCE optional		}|

{ ID id-UERadioCapabilityID						CRITICALITY reject	TYPE UERadioCapabilityID							PRESENCE optional		}|

{ ID id-Extended-ConnectedTime					CRITICALITY ignore	TYPE Extended-ConnectedTime						PRESENCE optional		}|

{ ID id-TimeSyncAssistanceInfo					CRITICALITY ignore	TYPE TimeSyncAssistanceInfo						PRESENCE optional		}|

{ ID id-UESliceMaximumBitRateList				CRITICALITY ignore	TYPE UESliceMaximumBitRateList					PRESENCE optional		}|

{ ID id-FiveG-ProSeAuthorized					CRITICALITY ignore	TYPE FiveG-ProSeAuthorized						PRESENCE optional		}|

{ ID id-FiveG-ProSeUEPC5AggregateMaximumBitRate	CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-FiveG-ProSePC5QoSParameters				CRITICALITY ignore	TYPE FiveG-ProSePC5QoSParameters				PRESENCE optional		}|

{ ID id-NoPDUSessionIndication					CRITICALITY ignore	TYPE NoPDUSessionIndication 						PRESENCE optional		},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

NID ::= BIT STRING (SIZE(44))

NonDynamic5QIDescriptor ::= SEQUENCE {

fiveQI						FiveQI,

priorityLevelQos			PriorityLevelQos										OPTIONAL,

averagingWindow				AveragingWindow											OPTIONAL,

maximumDataBurstVolume		MaximumDataBurstVolume									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NonDynamic5QIDescriptor-ExtIEs} }	OPTIONAL,

...

}

NonDynamic5QIDescriptor-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CNPacketDelayBudgetDL	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-CNPacketDelayBudgetUL	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		},

...

}

NoPDUSessionIndication ::= ENUMERATED {true, ...}

NotAllowedTACs ::= SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC

NotificationCause ::= ENUMERATED {

fulfilled,

not-fulfilled,

...

}

NotificationControl ::= ENUMERATED {

notification-requested,

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-HashedUEIdentityIndexValue							ProtocolIE-ID ::= 365

id-NoPDUSessionIndication 								ProtocolIE-ID ::= x1

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>