3GPP TSG-RAN WG3 #121										R3-234011

21th – 25th Aug 2023

Toulouse, France

Agenda item:	13.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Mobile IAB interference mitigation

Document for:	Discussion and Decision

# Introduction

This contribution discusses mobile IAB interference mitigation. The WID states following ([1]).

- Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

RAN3 has made following agreements related to PCI collision detection/avoidance:

PCI collision can be detected by the F1-terminating IAB-donor of the mobile IAB-node.

PCI space partitioning via OAM configuration can be used in some cases for avoidance of PCI collisions.

From RAN3 perspective, existing mechanism can be used for PCI collision detection in mobile IAB scenario. Further enhancement is FFS.

RAN3 to discuss whether mobile IAB needs any enhancements to the existing mechanisms for PCI collision avoidance and/or optimization.

From RAN3 perspective, no enhancements are needed for RACH collision avoidance unless requested by other WGs.

PCI Space Partitioning is performed by OAM and up to implementation.

As baseline, to avoid PCI collision, F1-terminating IAB-donor can reconfigure PCI for the cell of mobile IAB-DU via existing F1AP message.

PCI-change on the IAB-node can be supported via handover of connected UEs between cells using old and new PCI, respectively.

PCI collision can be detected by the F1-terminating IAB-donor of the mobile IAB-node.

FFS for the PCI reconfiguration in case of IAB-donor and IAB-node with different OAMs.

Additionally, RAN3 has made following agreements on the availability of IP connectivity:

RAN3 not to work on solutions addressing use cases where inter donor IP connectivity is not available.

For scenarios without Xn, RAN3 to investigate whether IAB-related Xn signaling for partial migration and DU migration can be carried via NG using a container to avoid the impact on the AMF.

Open areas that RAN3#119bis-e identified were [2],[3]:

How to avoid PCI collision in the scenario with Xn between IAB-DU’s donor and IAB-MT’s donor.

How to avoid PCI collision in the scenario without Xn between IAB-DU’s donor and IAB-MT’s donor if the scenario is supported.

Whether PCI collision between mobile IAB cells can be predicted based on existing UE measurement report.

In this contribution we elaborate the open issues and propose a suitable way forward.

# Discussion

## 2.1 PCI collision detection and avoidance

## 2.1.1 Xn existing between IAB-DU’s donor and IAB-MT’s donor

This scenario can be assumed to be a baseline case. The deployment of a network supporting mobile IAB can be such that the partial migration covers the area where the Xn is always available between the donors of IAB-MT and IAB-DU. The deployment can be such that some of the gNBs can be assigned to act as “anchors” i.e. F1-terminating CUs allowing partial migration within the area where “Xn coverage” exists. Another anchor may then cover the following area and full migration will happen between the anchors. Obviously, the PCI collision detection and avoidance can be done by the F1-terminating donor within its Xn coverage area.

At the edge of the Xn coverage, there can be Xn between the donors allowing exchange of cell information. The cell information will therefore be provided beyond the Xn coverage of F1-terminating donor enabling collision detection also in the extended area and prior to full migration.

Considering the above, existing means to share cell information over Xn will be sufficient for PCI collision detection and avoidance in the scenario where Xn exists between IAB-DU’s and IAB-MT’s donors.

Proposal 1: No enhancements are needed for PCI collision detection and avoidance when Xn exists between IAB-DU’s and IAB-MT’s donors.

## 2.1.2 No Xn between IAB-DU’s donor and IAB-MT’s donor

Consecutive partial migrations are supported in Rel.18 when the F1-terminating CU had Xn to the IAB-MT’s target donor. This is needed to move the F1 transport from the IAB-MT’s source donor to the target donor. Therefore, as long as partial migrations can be executed, there will not be a case where Xn wouldn’t exist between the IAB-DU’s donor and IAB-MT’s donor.

Observation 1. Partial migrations as per Rel.17 specifications is performed when Xn is available between the F1-terminating donor and the target donor of the IAB-MT.

In case no Xn between the F1-terminating donor and the target donor of the IAB-MT, e.g. when the two donors are not neighbor and not be able to use SON to establish the Xn interface, the Xn may be established via configuration (e.g. up to operator’s decision) or established in a dynamic manner, e.g. the F1-terminating donor is to be made aware of the target cell/donor to which IAB-MT is about to move. It is up to operator to decide whether the PCI can be an issue based on operator’s deployment, and thus requiring the Xn establishment.

Related to this, RAN3 is discussing full migration options (with two logical DUs) and whether to use F1AP or XnAP to provide IAB-DUs CU with the information of the mIAB-MT’s CU, [4]. If F1AP was agreed, the F1AP would be extended to carry the gNB ID of the IAB-MT’s target node. It is for further discussion if the F1AP extension includes also other information. With such enhancement the IAB-DU’s CU can be made aware of the IAB-MT’s target gNB, and, if Xn was missing between the IAB-DU’s CU and the IAB-MT’s target gNB CU, the IAB-DU’s CU can initiate Xn setup to that gNB. This would be possible in scenarios supported in Rel.18 as per RAN3 agreement about the availability of IP connectivity:

RAN3 not to work on solutions addressing use cases where inter donor IP connectivity is not available.

Observation 2. F1 enhancement for gNB ID transfer could be utilized in dynamic Xn setup in case no Xn between IAB-DU’s donor and IAB-MT’s target donor.

Proposal 2. If needed, RAN3 to consider F1 enhancement to include (target) gNB ID enabling dynamic Xn setup in case no Xn between IAB-DU’s donor and IAB-MT’s target donor.

## 2.1.3 Prediction of PCI collision based on existing UE measurement reporting

UE measurements may not be reliable means for PCI collision detection. When the UEs are able to detect another cell with the same PCI, the collision may already happen, and the reaction may therefore be late.

Observation 3. UE measurement reporting is not a reliable means to detect PCI collisions.

IAB-MT is also measuring and detecting neighbour cells. Knowing the IAB-DU’s cell(s), the IAB-node could detect the collisions between IAB-nodes own cells and the cells in the neighbourhood. However, this may not always be reliable as the early detection, allowing timely reaction to potential collision, is not possible in all mobility scenarios. Early detection capability depends on the IAB-DU’s antenna orientation w.r.t cell coverage areas of the surrounding cells. Hence, IAB-MT’s reporting cannot be considered as a generic solution although could work or help in some scenarios.

Observation 4. IAB-MT’s measurement reporting would help in some cases but could fail to support the detection of PCI collisions depending on the antenna orientation(s) and the network cell layout.

Based on the discussion above, it can be concluded that the means available in RAN and signalling between RAN nodes can be considered as the primary means for PCI collision detection and avoidance while UE/MT measurements can just be assisting the donors. Therefore, the usage of UE/MT measurements can be left for network implementation whenever/if seen beneficial.

Proposal 3. The usage of UE and IAB-MT measurement reporting for PCI collision detection can be left for network implementation.

## 2.2 PCI configurations with multiple OAMs

An FFS was identified for case with multiple OAMs:

- FFS for the PCI reconfiguration in case of IAB-donor and IAB-node with different OAMs

We think this is not an issue. In a normal (i.e. non-IAB) deployment, it is possible that the gNB-DU and gNB-CU come from a different vendor and uses different OAMs. The gNB-DU may be configured by the gNB-DU’s OAM server for the PCIs. During the F1 Setup procedure, the gNB-CU may configure gNB-DU to use a different PCI. The gNB-CU has better knowledge about the PCI alllocations, e.g. PCI used by other connected gNB-DU, PCI used by the neighboring cells, PCIs configured by gNB-CU’s OAM, etc. This also does not preclude the coordination between the gNB-DU’s OAM and gNB-CU’s OAM.

The mobile IAB is no different from the normal deployment. Even the IAB node and IAB-donor may come from different vendor and use different OAMs. It can be similar to normal deployment. There are no new issues, thus no enhancement is needed.

Observation 5. From RAN3 perspective, the IAB node and IAB-donor may use different OAMs, just like normal gNB-DU and gNB-CU that may use different OAMs.

Proposal 4. No enhancement is needed when the IAB node and IAB-donor use different OAMs.

# Conclusions

In this contribution, we briefly analyzed interference mitigation and PCI management in mobile IAB scenarios. We made following observations:

Observation 1. Partial migrations as per Rel.17 specifications can be done when Xn is available between the F1-terminating donor and the target donor of the IAB-MT.

Observation 2. F1 enhancement for gNB ID transfer could be utilized in dynamic Xn setup in case no Xn between IAB-DU’s donor and IAB-MT’s target donor.

Observation 3. UE measurement reporting is not a reliable means to detect PCI collisions.

Observation 4. IAB-MT’s measurement reporting would help in some cases but could fail to support the detection of PCI collisions depending on the antenna orientation(s) and the network cell layout.

Observation 5. From RAN3 perspective, the IAB node and IAB-donor may use different OAMs, just like normal gNB-DU and gNB-CU that may use different OAMs.

Based on the analysis we propose following:

Proposal 1: No enhancements are needed for PCI collision detection and avoidance when Xn exists between IAB-DU’s and IAB-MT’s donors.

Proposal 2. RAN3 to consider F1 enhancement to include (target) gNB ID enabling dynamic Xn setup in case no Xn between IAB-DU’s donor and IAB-MT’s target donor.

Proposal 3. The usage of UE and IAB-MT measurement reporting for PCI collision detection can be left for network implementation.

Proposal 4. No enhancement is needed when the IAB node and IAB-donor use different OAMs.

# References

- RP-221815 New WID on Mobile IAB

- RAN3_119bis-e_agenda_20221018_EOM

- R3-232116, Summary of CB # IAB4_MitInt

- R3-231901, CB # IAB2_IABMobility - Summary of email discussion