3GPP TSG-RAN WG3 #121																		R3-234021

21st – 25th Aug 2023

Toulouse, France

Agenda item:	25.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on multiple LSs

Document for:	Discussion and Decision

# 1	Introduction

This contribution analyses the XR related LS received in RAN3.

# 2	Discussion

## 2.1		R3-233731	LS reply on TSCAI for XR

In current NGAP, there is a restriction that TSCAI is only defined for GBR, and it is ignored in case of non-GBR QoS Flow.

TS38.413:

| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

SA2 has decided that TSCAI can also be applicable to XR non-GBR QoS Flow ([2])

Question: RAN2 has discussed the provision of assistance information from CN to RAN for XR services and is wondering whether TSCAI can be provided for both GBR and non-GBR QoS flows in case of XR.

SA2 Answer: for either GBR or non-GBR QoS flow, TSCAI can be provided to the RAN.

Observation 1-1: RAN3 specifications need to be updated to support TSCAI for non-GBR QoS Flow.

We also noticed that TSCAI is also used in XnAP, F1AP and E1AP.

TS38.423:

| >>>>TSC Traffic Characteristics | O |  | 9.2.3.114 |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

TS38.473:

| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

TS37.483:

| >>TSC Traffic Characteristics | O |  | 9.3.1.75 | Traffic pattern information associated with the QFI. Details in TS 23.501 [20]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

In XnAP, E1AP and F1AP, there is no restriction that TSCAI is only applicable for GBR QoS Flow.

Observation 1-2: TSCAI is used in NGAP, XnAP, E1AP and F1AP. Only NGAP has the restriction that TSCAI is applicable to GBR QoS Flow.

One possible option to address SA2’s requirement is to remove the restriction from NGAP. This will enable RAN3 AP specs (i.e. NGAP, XnAP, F1AP and E1AP) aligned. This does not affect the legacy NG-RAN node which still ignore this TSCAI for non-GBR QoS Flow. This may have a small issue to Rel-18 NG-RAN nodes that do not support XR, but this small issue may be addressed by referencing to TS23.501 which defines TSCAI is only for XR non-GBR QoS Flow. So we propose to remove the restriction in NGAP. If needed, add the reference to TS23.501.

Proposal 1-1: Remove the restriction that TSCAI is ignored for non-GBR QoS Flow from NGAP. If needed, add the reference to TS 23.501.

Since RAN3 is asked to consider SA2 request, it is preferred to send a reply LS to SA2 stating RAN3 will support TSCAI for non-GBR QoS Flow.

Proposal 1-2: RAN3 reply SA2 that RAN3 will support TSCAI for non-GBR QoS Flow.

## 2.2	R3-233732 Reply LS on the N6 PDU Set Identification

This LS states

SA2 has discussed whether to support a 3-bit EoDB in this release of the specification but has no consensus on it.

There is no impact to RAN3 in current stage.

Proposal 2: RAN3 NOTE the LS R3-233732.

## 2.3	R3-233733 Reply LS on the N6 PDU Set Identification

This LS states

SA2 has reached the consensus to not support data rate exposure by NG-RAN in Rel-18.

For UP based QNC, SA2 has made the decision that in Rel-18, the UP based QNC is not supported.

There is no impact to RAN3 in current stage.

Proposal 3: RAN3 NOTE the LS R3-233733.

## 2.4	R3-233734 LS Reply on Design of RTP Header Extension for PDU Set handling

This LS states

Mixing of N6 packets that are marked with the new RTP header extension with N6 packets that are not marked with the new RTP header extension in the same QoS Flow can happen as long as the information for QoS Flow matching (refer to the Packet Detection Information in the Packet Detection Rule in TS 23.501 Table 5.8.5.3-1) is the same for both.

There is no impact to RAN3 in current stage.

Proposal 4: RAN3 NOTE the LS R3-233734.

## 2.5	R3-233735 Non-homogeneous deployment of PDU Set based handling

This LS states

SA2 has discussed the issue of non-homogeneous deployments of NG-RAN nodes supporting PDU Set based handling and agreed the attached CR vs. TS 23.501.

In the SA2 CR, the main part affects RAN3 is copied as below:

By sending at least one PDU Set QoS parameter to the NG-RAN, the SMF requests the NG-RAN to activate PDU Set QoS handling for a given QoS flow and the NG-RAN provides the SMF with an indication of whether the PDU Set QoS parameter is accepted or not. At NG-RAN Xn handover and N2 handover, the target NG-RAN provides to the SMF with an indication of whether the target NG-RAN node supports PDU Set based handling, as specified in TS 38.413 [xx]. Based on the NG-RAN indications, the SMF configures the PSA UPF to activate/deactivate the PDU Set identification and marking.

This is already discussed in contribution ([7]). The UE may be handover to target NG-RAN node via N2-HO or Xn-HO. An explicit indication in the SMF related IE need to be sent from target gNB to the AMF to indicate when the PDU set QoS parameters is accepted in target gNB.

- In case of N2-HO, the target NG-RAN node can inform SMF in the Handover Resource Allocation procedure (i.e. a new indication in the HANDOVER REQUEST ACKNOWLEDGE message)

- In case of Xn-HO, the target NG-RAN node can inform SMF during the Path Switch Procedure, i.e. a new indication in the PATH SWITCH REQUEST message.

The Stage-3 detail need to be further discussed. At the current stage, since RAN3 is asked to consider the SA2 request, it is preferred to send a reply LS to SA2 regarding the RAN3 decision.

Proposal 5: RAN3 reply SA2 that NG-RAN node only provides an “support” indication when the NG-RAN node support the PDU Set based handling. There is no indication when the NG-RAN node does not support PDU set based handling.

# 3	Conclusion

In this contribution, we briefly analyzed the XR related LSs. Our proposals are:

Proposal 1-1: Remove the restriction that TSCAI is ignored for non-GBR QoS Flow from NGAP. If needed, add the reference to TS 23.501.

Proposal 1-2: RAN3 reply SA2 that RAN3 will support TSCAI for non-GBR QoS Flow.

Proposal 2: RAN3 NOTE the LS R3-233732.

Proposal 3: RAN3 NOTE the LS R3-233733.

Proposal 4: RAN3 NOTE the LS R3-233734.

Proposal 5: RAN3 reply SA2 that NG-RAN node only provides an “support” indication when the NG-RAN node support the PDU Set based handling. There is no indication when the NG-RAN node does not support PDU set based handling.

# References

- RP-230786, WID for NR XR

- R3-233731	LS reply on TSCAI for XR	SA2(VIVO)

- R3-233732	Reply LS on the N6 PDU Set Identification	SA2(OPPO)

- R3-233733	Reply LS on RAN information exposure for XRM	SA2(China mobile)

- R3-233734	LS Reply on Design of RTP Header Extension for PDU Set handling	SA2(Huawei)

- R3-233735	Non-homogeneous deployment of PDU Set based handling	SA2(Qualcomm)

- R3-234019	Discussion on support for NR XR (Nokia)