3GPP TSG-RAN WG3 Meeting #121	R3-234023

Toulouse, France, 21 – 25 Aug, 2023

Title: 	(TP to TRS_URLLC BLCR for TS 38.413, TS 38.423 and TS 38.473) Support of 5G Timing Resiliency enhancements

Source: 	Huawei, China Unicom

Agenda item:	26.2

Document Type:	Other

# 1. Introduction

In the previous RAN3#120 meeting, the following agreements for NR Timing Resiliency were made [1]:

In NGAP, introduce a new Timing Synchronisation Report procedure (class 2) to enable the gNB to report RAN TSS to the AMF. In F1AP, introduce a new Timing Synchronisation Report procedure (class 2) to enable the gNB-DU to report RAN TSS to the gNB-CU. WA: When Clock Quality Detail Level IE has value “clock quality metrics”, all clock quality metrics supported by the gNB implementation are delivered to the UE (may be revisited based on SA2 agreements).

This paper continues to discuss the potential RAN3 impacts, and provides corresponding TPs in the annex parts.

# 2. Discussion

## 2.1 Messages for RAN TSS reporting

One open issue for Timing Synchronisation Status Reporting procedure is whether a new class 1 procedure shall be introduced. Meanwhile, the SA2 agreed CR in [2] and [3] has the following changes (already captured in the latest TS 23.501 v18.2.0) .

In S2-2306224: <Skip the irrelevance> - TSCTSF may receive node-level information about timing synchronization status from gNB and/or UPF/NW-TT directly from OAM or alternatively, if supported by a node, using control plane signalling at node level. Node level signalling uses UMIC for UPF/NW-TT case and an AMF service to report N2 node level information for gNB case. In the latter case, the AMF controls the gNB node level reporting and subscription using NGAP messages (see TS 38.413 [34]). <Skip the irrelevance> The network timing synchronization status information from gNB or UPF/NW-TT to the TSCTSF may contain the following information as described in the Table 5.27.1.12-1. However, it is up to gNB to determine whether to provide its timing synchronization status reporting and which of the information elements to include in the TSS report to the TSCTSF, i.e., based on the implementation gNB may report all, some, or none of the information elements from Table 5.27.1.12-1. In S2-2306225 4.15.9.X.Y TSCTSF subscription to RAN and/or UPF/NW-TT timing synchronization status <Skip the irrelevance > 1-3. (When the procedure is triggered by the AF request to influence the 5G access stratum time distribution or by PTP instance activation, modification): Upon the reception of the clock quality reporting control information in the AF request in step 0, the TSCTSF needs to be subscribed to NG-RAN timing synchronization status updates at the NG-RAN nodes that may provision access stratum time distribution information to the target UE. NG-RAN timing synchronization status updates provisioning may be configured via OAM or via AMF (with node level signalling as illustrated in steps 1-3). Namf_NonUeN2MsgTransfer in step 1 contains the NG-RAN timing synchronization status reporting to be forwarded to the NG-RAN in step 2. Namf_NonUeN2InfoSubscribe initiates the subscription for the NG-RAN timing synchronization status updates in the AMF.

It can be observed that upon the AF request, the TSCTSF needs to be subscribed to the NG-RAN timing synchronization status via the AMF. In step1, the AMF signals the NG-RAN timing synchronization status reporting to be forwarded to the NG-RAN.

Hence the AMF controlled NGAP procedure is needed to allow the AF’s request. From the above, a new class 2 procedure seems sufficient – for the AMF to request the NG-RAN to report the TSS information. And the immediate response message is not needed. The included NG-RAN timing synchronization status reporting could be a very simple indicator e.g., to start the report or stop the report procedure.

- A new AMF controlled TSS reporting procedure (class 2) can be introduced, for which a simple indicator (e.g., start/stop) is contained.

## 2.2 TSS area report to the AMF

The previous RAN3-119bis-e meeting had the following common understanding.

- It’s the common understanding that RAN Timing Synchronization Status Information can be the same or different between “groups of cells within a single gNB” (e.g., cells served by different gNB-DUs).

Also as discussed in the reply LS to SA2, the clock quality information could be the same for some or all cells of a gNB as follows.

RAN3 answer: - Depending on gNB topology, clock quality information can be the same for some or all cells of a gNB. Also, topologies can exist where clock quality information is different between “groups of cells within a single gNB” (e.g. cells served by different gNB-DUs).

Meanwhile the TS 23.501 v18.2.0 has clearly indicated that the scope of the TSS (either node level, or a list of cells) are reported to the AMF, as follows.

gNBs may be pre-configured with thresholds for each timing synchronization status attribute, if supported, that is described in Table 5.27.1.12-1. When the network timing synchronization status exceeds the thresholds (i.e. status degradation), or the network timing synchronization status meets the thresholds again (i.e. status improvement), the gNB notifies the TSCTSF (either using N2 node level signalling via AMF, or via OAM) with the gNB ID, the scope of the timing synchronization status (i.e., all cells or a list of Cell IDs within a single gNB) and the corresponding network timing synchronization status (TSS) attributes as described in this clause. The gNB indicates the status change to the UEs via SIB9:

It can be observed that the gNB may report a node level or a group of cells level TSS to the AMF. Specifically,

- in non-CU-DU split architecture, a node-level synchronization status is sufficient.

- for the CU-DU split case, different gNB-DUs connected to the same gNB-CU may hold different clock quality information. The RAN node should notify the TSCTSF about the timing synchronization status per TSS area over the NGAP interface. About the detailed TSS area, it can be a set as a list of cells.

Hence, a CHOICE structure is proposed for the TSS report, either in a RAN node level, or a RAN area level TSS (in terms of cells). Below provides an exemplary tabular structure.

#### 9.3.1.XX1 RAN Timing Synchronisation Status Information Scope This IE indicates the time synchronization status information of the NG-RAN node as defined in TS 23.501 [9]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE TSS Information; M. >RAN node level. >> RAN Timing Synchronisation Status Information; M; 9.3.1.X3. >RAN node area level. >>RAN Timing Synchronization Status Area List; M; 9.3.1.xx2. #### 9.3.1.XX2 RAN Timing Synchronization Status Area List This IE indicates the time synchronization status list of the NG-RAN node as defined in TS 23.501 [9]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Timing Synchronization Status Area Item; 1..<maxnoofTSSItems>. >Cell List Areas; 1..<maxnoofCelllistareas>. >>NG-RAN CGI; M; 9.3.1.73. > RAN Timing Synchronisation Status Information; M; 9.3.1.X3.

- For NGAP, introduce a CHOICE structure for RAN timing synchronization status report (either node level or cell list level) from the gNB (gNB-CU) to the AMF. While there is no such need for F1AP.

## 2.3 TSS parameters encoding

As specified in the TS 23.501 v18.2.0, the TSS information elements included in the RAN timing synchronization status is updated as follows:

Table 5.27.1.12-1: Information elements that gNB or UPF/NW-TT timing synchronization status information may contain (all optional) Information Name; Description. Synchronization state; Indicates the state of the node synchronization, represented by the values "Locked", "Holdover", or "Freerun" (NOTE 1).. Clock quality. >> Traceable to GNSS; Indicates whether the current time source is traceable to the GNSS and represented by values “Yes” or “No”.. >> Traceable to UTC; Indicates whether the current time source is traceable to the UTC and represented by values “Yes” or “No”.. >> Frequency stability; Describes the estimate of the variation of the local clock when it is not synchronized to another source (NOTE 2).. >> Clock Accuracy; Describes the mean in ns over an ensemble of measurements of the time between the clock under test and a reference clock (NOTE 3).. Parent time source; Describes the primary source the node is currently using, represented by the values "SyncE", "PTP", "GNSS", "atomic clock", "terrestrial radio", "serial time code", "NTP", "hand_set", "other".. NOTE 1: Clock is in the "Locked", "Holdover", or "Freerun" mode, as defined in ITU-T G.810 [164]. NOTE 2: Frequency stability is estimated in a similar manner as for offsetScaledLogVariance attribute defined in clause 7.6.3.5 of IEEE Std 1588 [126]. NOTE 3: Clock accuracy measurement considers accuracy up to gNB antenna and RAN internal process..

For the Clock Accuracy, in clause 7.6.2.6 of IEEE Std 1588, the value of clockAccuracy can take the enumeration value in the following table. Hence, the Clock Accuracy can refer to Table 5 of clause 7.6.2.6 of IEEE Std 1588 specification, and take an integer value from 0 to 255.

Table 5—clockAccuracy enumeration Value (hex); Specification. 00 to 16; Reserved. 17; The time is accurate to within 1 ps. 18; The time is accurate to within 2.5 ps. 19; The time is accurate to within 10 ps. 1A; The time is accurate to within 25 ps. 1B; The time is accurate to within 100 ps. 1C; The time is accurate to within 250 ps. 1D; The time is accurate to within 1 ns. 1E; The time is accurate to within 2.5 ns. 1F; The time is accurate to within 10 ns. 20; The time is accurate to within 25 ns. 21; The time is accurate to within 100 ns. 22; The time is accurate to within 250 ns. 23; The time is accurate to within 1 µs. 24; The time is accurate to within 2.5 µs. 25; The time is accurate to within 10 µs. 26; The time is accurate to within 25 µs. 27; The time is accurate to within 100 µs. 28; The time is accurate to within 250 µs. 29; The time is accurate to within 1 ms. 2A; The time is accurate to within 2.5 ms. 2B; The time is accurate to within 10 ms. 2C; The time is accurate to within 25 ms. 2D; The time is accurate to within 100 ms. 2E; The time is accurate to within 250 ms. 2F; The time is accurate to within 1 s. 30; The time is accurate to within 10 s. 31; The time is accurate to >10 s. 32 to 7F; Reserved. 80 to FD; Designated for assignment by alternate PTP Profiles. FE; Unknown. FF; Reserved.

Besides, in RRC specification, the uncertainty parameter contained in the ReferenceTimeInfo IE also indicates the accuracy of the 5G clock. Alternatively, the Clock Accuracy can refer to the uncertainty parameter contained in the ReferenceTimeInfo IE, i.e. take an integer value from 0 to 32767 and in the unit of 25ns. Hence, the Clock Accuracy can take following values for the Choice structure:

- Choice 1: Refer to Table 5 of clause 7.6.2.6 of IEEE Std 1588 specification, and take an integer value from 0 to 255.

- Choice 2: Take an integer value from 0 to 32767 and in the unit of 25ns.

Based on the above discussion, the detailed TSS parameters are proposed as follows.

- The Clock Accuracy includes the following two choices:

- Choice 1: Refer to Table 5 of clause 7.6.2.6 of IEEE Std 1588 specification, and take an integer value from 0 to 255.

- Choice 2: Take an integer value from 0 to 32767 and in the unit of 25ns.

## 2.4 AMF providing clock quality reporting control information per-UE to the gNB

In the TS 23.501 v18.2.0, the UE level clock quality reporting control information is specified as follows.

For 5G access stratum time synchronization service, clock quality reporting control information manages the gNBs timing synchronization status reporting to the UE. When AMF provides the 5G access stratum time distribution indication and the Uu time synchronization error budget to gNB, the AMF also includes the clock quality reporting control information (CQRCI) provided by the TSCTSF or retrieved from UDM. CQRCI may be a part of Access and Mobility Subscription data at the UDM, or AF may include CQRCI in its request. CQRCI contains the following fields: - Clock quality detail level. It indicates whether and which clock quality information to provide to the UE and can take one of the following values: clock quality metrics or acceptable/not acceptable indication. - If the clock quality detail level equals "clock quality metrics", the NG-RAN provides clock quality metrics to the UE that reflect its current timing synchronization status. i.e. one or more of the following information elements: clock accuracy, traceability to UTC and to GNSS, frequency stability, parent time source, synchronization state as defined in Table 5.27.1.12-1. NG-RAN is locally configured which of the clock quality metrics supported by NG-RAN are provided to UE(s). - If the clock quality detail level equals "acceptable/not acceptable indication", clock quality acceptance criteria for the UE. The gNB provides an acceptable indication to the UE if the gNB's timing synchronization status matches the acceptance criteria received from the AMF; otherwise, the gNB indicates "not acceptable" to the UE. Clock quality acceptance criteria can be defined based on one or more information elements listed in Table 5.27.1.12-1. If AF includes clock quality acceptance criteria in its request towards TSCTSF, the AF shall be notified about the result once TSCTSF determines whether the clock quality acceptance criteria can be met or not. Based on the notification, the AF may decide to modify the service if preferred (e.g., disable the service upon degradation or enable it again upon recovery)

It can be observed the clock quality reporting control information includes Clock quality detail level, and can take one value from clock quality metrics or acceptable/not acceptable indication. And the CQRCI is part of the UDM information, or the PCF policy. At previous RAN3#120 meeting, the following working assumption is made.

- WA: When Clock Quality Detail Level IE has value “clock quality metrics”, all clock quality metrics supported by the gNB implementation are delivered to the UE (may be revisited based on SA2 agreements).

In TS 23.503, it is specified that in section 6.5	Access and mobility related policy information, the clock quality detail level and clock quality acceptable criteria are UE level information.

| Clock quality detail level | Defines which clock quality information (clock quality metrics or acceptable/not acceptable indication) to report to the UE as defined in clause 5.27.1.12 of TS 23.501 [2] | Conditional (NOTE 9) | Yes | UE context |
| --- | --- | --- | --- | --- |
| Clock quality acceptance criteria | Indicates the acceptable criteria as defined in clause 5.27.1.12 of TS 23.501 [2] | Conditional (NOTE 9) | Yes | UE context |

In TS 23. 502, the 5G Access Stratum-based Time Synchronization Service Data included the UE level a clock quality detail level and the the clock quality acceptance criteria in section 5.2.3.3	Nudm_SubscriberDataManagement (SDM) Service

| 5G Access Stratum-based Time Synchronization Service Data | Includes the Access Stratum Time Synchronization Service Authorization to indicate whether the UE should be provisioned with 5G system internal clock timing information over access stratum. Optionally includes an Uu time synchronization error budget. Optionally includes one or more periods of start and stop times defining the times when the UE should be provisioned with 5G system internal clock timing information. Optionally includes a Time Synchronization Coverage Area comprising a list of TAs where the UE shall be provisioned with 5G system internal clock timing information (NOTE 19). Optionally includes a clock quality detail level to indicate whether and which clock quality information to provide to the UE. It comprises one of the following values: clock quality metrics or acceptable/not acceptable indication. Optionally includes the clock quality acceptance criteria for the UE. It may be defined based on one or more of the following attributes: time source, traceability to UTC and to GNSS, synchronization state, clock accuracy, frequency stability. |
| --- | --- |

Hence the working assumption on the “clock quality metrics” can be agreed. Whether the UE level clock quality acceptance criteria can refer to the existing RAN Timing Synchronisation Status Information IE needs further study. For example, if the acceptance criteria can include multiple values for a single clock metric.

- About the per-UE clock quality reporting control information,

- Turn the WA on the clock quality metrics into agreements (i.e., When Clock Quality Detail Level IE has value “clock quality metrics”, all clock quality metrics supported by the gNB implementation are delivered to the UE);

- Whether the Clock Quality Acceptance Criteria can refer to the RAN Timing Synchronisation Status Information IE need further study.

## 2.4 Potential LS to RAN2

As agreed by RAN2-120bis-e meeting,

- RRC_CONNECTED UE to be informed of 5G clock quality information via dedicated RRC message

In order to assist RAN2 progress, a LS to RAN2 is needed to indicate the clock quality reporting control information. Typically,

- When the clock quality detail level equals “clock quality metrics”, RAN3 can indicate to RAN2 about the detailed clock quality formats for all possible metrics including clock accuracy, traceability to UTC and to GNSS, frequency stability, parent time source, synchronization state etc.

- When the clock quality detail level equals "acceptable/not acceptable indication", the gNB just notifies the UE acceptable/not acceptable indication.

Based on the above, RAN2 can design the dedicated RRC message for connected UE about the 5G clock information at the first stage. In addition, RAN2 agreed the event ID is used for the RRC inactive/idle UE without any scope ID since the gNB ID can be used to represent the scope of the report ID. In the case where different DUs have different 5G Clock quality information, the “Event ID” can be different for different DUs configured by the OAM. Thus it seems no need to contact with the RAN2.

- Send a LS to RAN2 about the details of the 5G Clock quality information delivered to the connected UE to assist RRC signalling design.

# 3. Conclusion

Based on the discussion in this paper, we make the following observations and proposals:

- A new AMF controlled TSS reporting procedure (class 2) can be introduced, for which a simple indicator (e.g., start/stop) is contained.

- For NGAP, introduce a CHOICE structure for RAN timing synchronization status report (either node level or cell list level) from the gNB (gNB-CU) to the AMF. While there is no such need for F1AP.

- The Clock Accuracy includes the following two choices:

- Choice 1: Refer to Table 5 of clause 7.6.2.6 of IEEE Std 1588 specification, and take an integer value from 0 to 255.

- Choice 2: Take an integer value from 0 to 32767 and in the unit of 25ns.

- About the per-UE clock quality reporting control information,

- Turn the WA on the clock quality metrics into agreements (i.e., When Clock Quality Detail Level IE has value “clock quality metrics”, all clock quality metrics supported by the gNB implementation are delivered to the UE);

- Whether the Clock Quality Acceptance Criteria can refer to the RAN Timing Synchronisation Status Information IE need further study.

- Send a LS to RAN2 about the details of the 5G Clock quality information delivered to the connected UE to assist RRC signalling design.

The TPs to TS 38.413, TS 38.473 and TS 38.423 are captured in Annexes respectively.  Furthermore, an LS to RAN2 is provided in the Annex D.

# 4. Reference

- R3-23xxxx, (draft) Report of 3GPP TSG RAN3 meeting #120, Incheon, Korea, 22 - 26 May 2023.

- S2-2306224, Addressing ENs on timing synchronization status reporting, Ericsson et al.

- S2-2306225, Procedures to support network timing synchronization status and reporting, Nokia et al

- R3-233576, (BLCR to 38.473) Introduction of 5G Timing Resiliency and URLLC enhancements, 3GPP TSG-RAN WG3 Meeting #120.

- R3-230811, Reply LS on proposed method for time synchronization status reporting to UE(s), 3GPP TSG-RAN WG3 Meeting #119.

# Annex A – TP to BLCR for TS 38.413 (on top of R3-233755)

# 8	NGAP Procedures

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Session Release Required | BROADCAST SESSION RELEASE REQUIRED |
| TSS Information Control | TSS INFORMATION CONTROL |
| TSS Information Transfer | TSS INFORMATION TRANSFER |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.XX		TSS Procedures

#### 8.xx.1	TSS Information Control

#### 8.xx.1.1	General

The purpose of the TSS Information Control procedure is to allow the AMF to request the NG-RAN node to report the TSS information as specified in TS 23.501 [9] and TS 23.502 [10]. The procedure uses non-UE associated signalling.

#### 8.xx.1.2	Successful Operation

Figure 8.XX.1.2-1: TSS information control

The AMF initiates the procedure by sending a TSS INFORMATION CONTROL message to the NG-RAN node. On receipt of the TSS INFORMATION CONTROL message the NG-RAN node shall, if supported, perform the requested TSS information transfer.

The TSS Information Report Type IE indicates to the NG-RAN node whether:

-	to report on demand;

-	to stop periodic reporting.

#### 8.xx.1.3	Abnormal Conditions

#### Void.

#### 8.xx.2	TSS Information Transfer

#### 8.xx.2.1	General

The purpose of the TSS Information Transfer procedure is to transfer the RAN timing synchronization status information to the AMF. The procedure uses non-UE associated signalling.

#### 8.xx.2.2	Successful Operation

Figure 8.xx.2-1: TSS Inforamtion Transfer

The NG-RAN node initiates the procedure by sending the TSS INFORMATION TRANSFER message to the AMF.

#### 8.xx.2.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.y		TSS Messages

#### 9.2.y.1	TSS INFORMATION CONTROL

This message is used by the AMF to request the NG-RAN node to provide the timing synchronization status.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TSS Information Report Type | M |  | ENUMERATED (start, stop…) |  | YES | reject |

#### 9.2.y.2	TSS INFORMATION TRANSFER

This message is used to provide the timing synchronization status.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Timing Synchronisation Status Information | M |  | 9.3.1.y1 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.y1	Timing Synchronization Status Information

This IE indicates the time synchronization status information of the NG-RAN node as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE TSS Information | M |  |  |  |
| >RAN node level |  |  |  |  |
| >>RAN Timing Synchronisation Status Information | M |  | 9.3.1.x3 |  |
| >RAN area level |  |  |  |  |
| >>Timing Synchronization Status Area List | M |  | 9.3.1.y2 |  |

#### 9.3.1.y2	Timing Synchronization Status Area List

This IE indicates the time synchronization status list of the NG-RAN node as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Timing Synchronization Status Area Item |  | 1..<maxnoofTSSItems> |  |  |
| >Cell List Areas |  | 1..<maxnoofCelllistareas> |  |  |
| >>NG-RAN CGI | M |  | 9.3.1.73 |  |
| > RAN Timing Synchronisation Status Information | M |  | 9.3.1.X3 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTSSItems | Maximum no. of TSS area items. Value is 32 [FFS]. |
| maxnoofCelllistareas | Maximum no. of cell list items. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x1	Clock Quality Reporting Control Information

This IE indicates the clock quality reporting control information as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Quality Detail Level | M |  |  |  |
| >clock quality metrics |  |  |  |  |
| >acceptance indication |  |  |  |  |
| >>Clock Quality Acceptance Criteria | M |  | 9.3.1.x2 |  |

#### 9.3.1.x2	Clock Quality Acceptance Criteria

This IE indicates the clock quality acceptance criteria as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Clock Quality Acceptance Criteria[FFS] | M |  |  |  |

#### 9.3.1.x3	RAN Timing Synchronisation Status Information

This IE indicates the RAN timing synchronisation status information provided towards the AMF as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | ENUMERATED (locked, holdover, freeRun, …) |  |
| Traceable to UTC | O |  | ENUMERATED (true, false, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, false, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [9]. |
| Clock Accuracy | O |  | 9.3.1.x5 |  |
| Parent Time Source | O |  | ENUMERATED (syncE, pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |  |

#### 9.3.1.x5	Clock Accuracy

This IE indicates the clock accuracy as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Accuracy | M |  |  |  |
| >Index |  |  |  |  |
| >> Accuracy index |  |  | INTEGER(0..255,..) | The integer value indicates the clockAccuracy enumeration value specified in Table 5 of clause 7.6.2.6 of IEEE Std 1588 [AA]. |
| >Value |  |  |  |  |
| >>Accuracy value | M |  | INTEGER(0..32767,..) | Indicates the absolute clock accuracy value. Unit in 25ns. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex B – TP to BLCR for TS 38.473 (on top of R3-233770)

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

Table 2: Class 2 procedures

| Elementary Procedure |  | Message |  |
| --- | --- | --- | --- |
| Error Indication |  | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) |  | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer |  | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer |  | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer |  | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification |  | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery |  | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging |  | PAGING |  |
| Notify |  | NOTIFY |  |
| PWS Restart Indication |  | PWS RESTART INDICATION |  |
| PWS Failure Indication |  | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication |  | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report |  | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction |  | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |  |
| Deactivate Trace |  | DEACTIVATE TRACE |  |
| DU-CU Radio Information Transfer |  | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer |  | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication |  | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |  |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |  |
| Access Success |  | ACCESS SUCCESS |  |
| Cell Traffic Trace |  | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control |  | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback |  | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report |  | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort |  | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication |  | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update |  | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation |  | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication |  | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report |  | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination |  | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update |  | POSITIONING INFORMATION UPDATE |  |
| Multicast Group Paging |  | MULTICAST GROUP PAGING |  |
| Broadcast Context Release Request |  | BROADCAST CONTEXT RELEASE REQUEST |  |
| Multicast Context Release Request |  | MULTICAST CONTEXT RELEASE REQUEST |  |
| PDC Measurement Report |  | PDC MEASUREMENT REPORT |  |
| PDC Measurement Termination |  | PDC MEASUREMENT TERMINATION COMMAND |  |
| PDC Measurement Failure Indication |  | PDC MEASUREMENT FAILURE INDICATION |  |
| PDC Measurement Termination |  | PDC MEASUREMENT TERMINATION COMMAND |  |
| Measurement Activation |  | MEASUREMENT ACTIVATION |  |
| QoE Information Transfer |  | QOE INFORMATION TRANSFER |  |
| Positioning System Information Delivery |  | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |  |
| TSS Information Control |  | TSS INFORMATION CONTROL |  |
| TSS Information Transfer |  | TSS INFORMATION TRANSFER |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 8.xx	TSS Procedures

### 8.xx.1	TSS Information Control

#### 8.xx.1.1	General

The purpose of the TSS Information Control procedure is to command the gNB-DU to send the RAN timing synchronization status information to the gNB-CU. The procedure uses non-UE associated signalling.

#### 8.xx.1.2	Successful operation

Figure 8.16.1.2-1: TSS Information Control procedure

The gNB-DU initiates the procedure by sending the TSS INFORMATION CONTROL message to the gNB-CU. Upon reception of the TSS INFORMATION CONTROL message, the gNB-DU shall, if supported, perform the requested RAN timing synchronization status report.

The TSS Information Report Type IE indicates to the gNB-DU whether:

-	to report on demand;

-	to stop periodic reporting.

#### 8.xx.1.3	Abnormal Conditions

Not applicable.

### 8.xx.2	TSS Information Transfer

#### 8.xx.2.1	General

The purpose of the TSS Information Transfer procedure is to transfer the RAN timing synchronization status information from the gNB-DU to the gNB-CU. The procedure uses non-UE associated signalling.

#### 8.xx.2.2	Successful operation

Figure 8.16.1.2-1: TSS Information Transfer procedure

The gNB-DU initiates the procedure by sending the TSS INFORMATION TRANSFER message to the gNB-CU.

#### 8.xx.2.3	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.y	TSS messages

#### 9.2.y.1	TSS INFORMATION CONTROL

This message is sent by the gNB-DU to the gNB-CU and is used to request the gNB-DU to deliver the timing synchronization status.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| TSS Information Report Type | M |  | ENUMERATED (start, stop…) |  | YES | reject |

#### 9.2.y.2	TSS INFORMATION TRANSFER

This message is sent by the gNB-DU and is used to report the timing synchronization status to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN Timing Synchronisation Status Information | M |  | 9.3.1.x1 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x1	RAN Timing Synchronisation Status Information

This IE indicates the RAN timing synchronisation status information provided from the gNB-DU to the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | ENUMERATED (locked, holdover, freeRun, …) |  |
| Traceable to UTC | O |  | ENUMERATED (true, false, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, false, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [21]. |
| Clock Accuracy | O |  | 9.3.1.x3 |  |
| Parent Time Source | O |  | ENUMERATED (syncE, pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |  |

#### 9.3.1.x3	Clock Accuracy

This IE indicates the clock accuracy as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Accuracy | M |  |  |  |
| >Index |  |  |  |  |
| >> Accuracy index |  |  | INTEGER(0..255,..) | The integer value indicates the clockAccuracy enumeration value specified in Table 5 of clause 7.6.2.6 of IEEE Std 1588 [AA]. |
| >Value |  |  |  |  |
| >>Accuracy value | M |  | INTEGER(0..32767,..) | Indicates the absolute clock accuracy value. Unit in 25ns. |

Editor’s Note: Encoding of the Clock Accuracy IE is to be decided by RAN3 and should allow for different RAN implementations (e.g., CHOICE structure). Details FFS.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex C – TP to BLCR for TS 38.423 (on top of R3-233764)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x1	Clock Quality Reporting Control Information

This IE indicates the clock quality reporting control information as defined in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Clock Quality Detail Level | M |  |  |  |
| >clock quality metrics |  |  |  |  |
| >acceptance indication |  |  |  |  |
| >>Clock Quality Acceptance Criteria | M |  | 9.3.1.x2 |  |

#### 9.2.3.x2	Clock Quality Acceptance Criteria

This IE indicates the clock quality acceptance criteria as defined in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Clock Quality Acceptance Criteria | M |  | RAN Timing Synchronisation Status Information 9.2.3.x3 |  |

#### 9.2.3.x3	RAN Timing Synchronisation Status Information

This IE indicates the RAN timing synchronisation status information provided towards the AMF as defined in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Synchronisation State | O |  | ENUMERATED (locked, holdover, freeRun, …) |  |
| Traceable to UTC | O |  | ENUMERATED (true, false, …) |  |
| Traceable to GNSS | O |  | ENUMERATED (true, false, …) |  |
| Clock Frequency Stability | O |  | BIT STRING (SIZE (16)) | Indicates the offsetScaledLogVariance as specified in TS 23.501 [7]. |
| Clock Accuracy | O |  | 9.3.1.x5 |  |
| Parent Time Source | O |  | ENUMERATED (syncE, pTP, gNSS, atomicClock, terrestrialRadio, serialTimeCode, nTP, handSet, other, …) |  |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex D: LS to RAN2

Title: [Draft] LS on 5GS network timing synchronization status and reporting Response to: Release: Release 18 Work Item: TRS_URLLC-NR-Core Source: Huawei [will be RAN3] To: RAN2 Cc: Contact Person: #### Name: Feng Han E-mail Address: hanfeng3@huawei.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: # 1 Overall description RAN3 discussed the 5GS network timing synchronization status and reporting, and made the following agreements. Regarding the 5G clock quality information delivered to the UE, there are two kinds of indications as follows. • Clock quality metrics: Indicates the detailed 5G clock attributes including: - Synchronization state: Indicates the state of the node synchronization, represented by the values “Locked”, “Holdover”, or “Freerun”. - Synchronization performance: Boolean true/false for Traceable to UTC or Traceable to GNSS - Clock accuracy: - Choice 1:an integer value from 0 to 255 and referring to the IEEE Std 1588 specification. - Choice 2: an integer value from 0 to 32767 and in the unit of 25ns. - Frequency Stability: an Integer value from 0 to 65534, and referring to the IEEE Std 1588 specification - PTP clockClass: an integer value (i.e. from 0 to 255), and referring to Table 4 in clause 7.6.2.5 of the IEEE Std 1588 specification - Parent time source: Describes the primary source the node is currently using, represented by the values, “PTP”, “GNSS”, “atomic clock”, “terrestrial radio”, “serial time code”, “NTP”, “hand set”, “other”. • Acceptable indication: Indicates whether the NG-RAN's timing synchronization status matches the acceptance criteria received from 5GC, represented by the values “acceptable”, “not acceptable”. # 2 Actions To RAN2: ACTION: RAN3 respectfully asks RAN2 to take the above into account for RRC signalling design. # 3 Dates of next RAN3 meetings Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/ RAN3#121bis 2023-10-09 - 2023-10-13 Xiamen , CN