3GPP TSG-RAN WG3 #121						                                 R3-234028

21th – 25th Aug 2023 Toulouse, France

Agenda item:	17.2.

Source: 	Samsung

Title: 	Remaining issue on service continuity enhancement for NTN

Document for:	Discussion and decision

Introduction

Last RAN3 meeting discussed service continuity enhancement, and the following agreements and open issues are captured,

Change the WA to the agreement: Uu Cell ID should be used in Xn Setup and Configuration Update procedures.

WA: Do not exchange multiple TACs over Xn for NTN.

How to describe which TAC should be used in semantic description?

When time-based trigger condition is used, the source NG-RAN node should consider the time indicated to the UE to decide when start the early data forwarding to the target NG-RAN node.

This contribution further discusses these open issues.

Discussion

Last RAN3 meeting further discussed the service continuity enhancement, and the following open issue is captured,

WA: Do not exchange multiple TACs over Xn for NTN.

How to describe which TAC should be used in semantic description?

Firstly is about the working assumption. Considering the situation that this TAC open issue has been discussed for last several meetings, and we tried so hard to achieve this working assumption, a reasonable way forward is to turn the WA into agreement unless there’s new and clear benefits to exchange multiple TACs over Xn.

As a consequence, the left options we have is that,

Option 1: Reuse the existing spec by exchanging a single TAC

Option 2: Do not exchange TAC information for NTN

If we follow Option 1, then we need to identify the open issue, i.e. which TAC should be used in semantics description; while if we follow Option 2, since in current spec the TAC information is mandatorily exchanged over Xn, a simple solution to cater for Option 2 is that we add ‘The TAC is not used for NTN’ in the semantics description, something like below,

9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID | – |  |
| NR CGI | M |  | 9.2.2.7 |  | – |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code This IE is not used for NTN. | – |  |

So the leftover issue is which Option to follow. More advantages are needed to be found to follow one option over the other. It should be noted that Option 2 got more support than Option 1 two meetings ago when we collected opinions from all companies.

Observation 1: Most companies preferred not to exchange TAC information over Xn for NTN, according to the opinion collection two meetings ago.

In addition, recall the main reason to rule out exchanging multiple TACs is that it causes frequent NG-RAN configuration update which is not preferred, the frequent configuration update cannot be avoided either if we exchange a single TAC for earth moving scenario, let alone we need to define which TAC should be used in semantics description.

Observation 2: The frequent configuration update still cannot be avoided by exchanging a single TAC over Xn for NTN.

Furthermore, during last several meetings, some company mentioned that the TAC is actually a design mainly compatible with TN, and not exchanging TAC over interface management procedure does not bring too much harm to the network. We think the argument is reasonable, and so far no severe issue has been identified since this option was put forward.

Observation 3: Tracking Area is actually a concept for TN. We have encountered many difficulties when we also introduce the same concept to NTN.

Observation 4: No issue has been identified so far by not exchanging TAC information over Xn for NTN.

As a result, it is preferable for us to not exchange TAC information over Xn for NTN.

Proposal 1: Do not exchange TAC information over Xn for NTN.

Conclusion

In this contribution, we have the following proposals:

Observation 1: Most companies preferred not to exchange TAC information over Xn for NTN, according to the opinion collection two meetings ago.

Observation 2: The frequent configuration update still cannot be avoided by exchanging a single TAC over Xn for NTN.

Observation 3: Tracking Area is actually a concept for TN. We have encountered many difficulties when we also introduce the same concept to NTN.

Observation 4: No issue has been identified so far by not exchanging TAC information over Xn for NTN.

Proposal 1: Do not exchange TAC information over Xn for NTN.

References

[1] RP-221819 Revised WID: NR NTN (Non-Terrestrial Networks) enhancements.