3GPP TSG-RAN WG3 #121	R3-234063

# Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1200 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on procedural description of Resource Status Reporting Initiation procedure over F1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2023-8-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current specification, some description of Resource Status Reporting Initiation procedure is given as below. gNB-CU initiates the procedure by sending the RESOURCE STATUS REQUEST message to gNB-DU to start a measurement, stop a measurement, or add cells to report for a measurement. Upon receipt, gNB-DU: - shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or - shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or - shall add cells indicated in the Cell To Report List IE to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If measurements are already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored. However, the above highlighted description may lead to misunderstanding since the SSB level measurement should be also considered in NR. For example, assuming the SSB indexes in the original RESOURCE STATUS REQUEST message are SSB1 and SSB2 within Cell1, the scenario of adding a cell that is already initiated for reporting can be: The SSB indexes in the newly triggered RESOURCE STATUS REQUEST message are SSB1, SSB2 and SSB3 within Cell1. In this case, the SSB3, which is newly added measurement granularity rather than the cell level measurement granularity, has not been captured in the highlighted description. And the receiving node may ignore the RESOURCE STATUS REQUEST message by mistake. And the same issue also exists for the Slice. Therefore, the current description does not mention the measurement of the SSB or the Slice, which may lead to misunderstanding that the RESOURCE STATUS REQUEST shall be ignored as long as the cell level measurement has been initiated, regardless of the SSB level or slice level measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add some clarification on the behaviour of the gNB-DU when it receives the RESOURCE STATUS REQUEST message with Registration Request “Add”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The behaviour of the gNB-DU is ambiguous when it receives the RESOURCE STATUS REQUEST message with Registration Request “Add”. Impact Analysis: This CR has an impact under protocol point of view. The impact can be considered isolated because the change affects only the Resource Status Reporting procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

8.2.10	Resource Status Reporting Initiation

8.2.10.1	General

This procedure is used by an gNB-CU to request the reporting of load measurements to gNB-DU.

The procedure uses non UE-associated signalling.

8.2.10.2	Successful Operation

Figure 8.2.10.2-1: Resource Status Reporting Initiation, successful operation

gNB-CU initiates the procedure by sending the RESOURCE STATUS REQUEST message to gNB-DU to start a measurement, stop a measurement, or add cells to report for a measurement. Upon receipt, gNB-DU:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If measurements are already initiated for a cell (also including the SSB or slice with the cell) indicated in the Cell To Report List IE, this information shall be ignored.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>