3GPP TSG-RAN WG3 Meeting #121	R3-234094

Toulouse, France, 21st – 25th Aug, 2023

Title: 	Further discussion on the support of MBS QoE

Source: 	Huawei

Agenda item:	11.2

Document Type:	discussion and decision

1. Introduction

In the last meetings, RAN3 reached some agreements on the MBS QoE configuration and reporting and QoE measurement under high speed scenario. In this contribution, we will continue to discuss the remaining issues.

2. Discussion

2.1 MBS QoE measurement configuration

RAN3 has made the following agreements:

UE handles area scope checking for QoE measurements in RRC INACTIVE/IDLE mode. Whether UE AS layer or UE APP layer handle the area scope is to be discussed based on RAN2 progress.

The remaining issue is how to handle area scope of MBS broadcast QoE measurement when UE enters RRC_CONNECTED from RRC_INACTIVE/IDLE.

According to the LS [1] from RAN2, RAN2 asked RAN3 to answer the following questions:

Q2) RAN2 would also like to ask SA4/SA5/RAN3 whether there is a problem if for UEs in RRC CONNECTED the network performs area scope checking (with Area Scope of QMC) and UE application also performs area scope checking (with LocationFilter) at the same time. It should be noted that area scope management for UEs in RRC CONNECTED in Rel-17 relies on the gNB releasing the QoE configuration when the UE moves out of the applicable area scope.

In our understanding, RAN3 has agreed that a common QoE configuration mechanism is used for all RRC states.

If the UE receives the configuration in RRC connected state, a common QoE configuration mechanism is used to support QoE measurement configuration pertaining to MBS broadcast service for all RRC states, where the Rel-17 QoE configuration mechanism is adopted as baseline.

Therefore we think the handling of area scope of MBS broadcast QoE measurement also need to be unified.

We also think there are some issues if it is the network to handle the area scope when the UE is RRC_CONNECTED.

- The network does not know whether UE is receiving the MBS broadcast. In R17, UE may send the MBSInterestIndication message to inform network that the UE is receiving/ interested to receive MBS broadcast service(s) via a broadcast MRB. But the sending of MBSInterestIndication message is not mandatory. Therefore it is difficult for network to handling the area scope.

- According to the LS [1] from RAN2, it seems UE application can perform area scope checking. According to the SA4 specifications, LocationFilter can only be included in the QoE configuration container, if geographical filtering is not handled on the network side, i.e. to avoid otherwise redundant location filtering at network and UE sides. In case LocationFilter is used, there is no need for the network to handle area scope checking, which would be aligned with SA4 reminder mentioned above.

- If both of network and UE application check the area scope, we think it increases the complexity of the network. Also when one of them release the QoE measurement, it should inform another. It also increases the signalling overhead.

We provide our views in draft LS out [2] as a response to RAN2.

- Only UE handles area scope checking for MBS broadcast QoE measurement in RRC_CONNECTED.

In the last meeting, some companies proposed to support the RAN Visible QoE measurement configuration and reporting for MBS broadcast in RRC_CONNECTED.

WA: RVQoE measurement is not supported in RRC_IDLE/INACTIVE state.

RAN3 continue to study whether support RVQoE measurement in RRC_CONNECTED state.

In our understanding, RAN uses the RAN visible QoE measurement results to optimize the scheduling for special UE. But for the MBS broadcast, more than one UEs are receiving the MBS service. If the network only optimizes the scheduling/configuration based on the RAN visible QoE results from parts of UE, it will have impact on the performance of MBS service of other UEs. For example, if the RAN visible QoE results reported by RRC_CONNECTD UE are good and then the network decreases the scheduling/configuration, it will decrease the performance of UEs which are at edge of cell and are in RRC_CONNECTED.  Also we think we should define a unified solution for all the state.

- No need to support the RAN Visible QoE measurement for MBS broadcast service in RRC_CONNECTED state

2.2 MBS QoE measurement reporting

In the last meetings, RAN3 only agreed the new gNB need to know the following information:

At least the following QoE configuration related information for MBS broadcast service should be available in the new gNB: - QoE reference - Measurement Collection Entity Information, the detail information can be further discussed Configuration container need not to be provided to the new gNB for MBS broadcast service. RRC level ID (measConfigAppLayerID) for MBS broadcast service should be available in the new gNB. For MBS QoE, an M-based QoE configuration shall not overwrite the S-based QoE configuration stored at the UE by the new gNB. QoE measurement type (s-based or m-based measurement) for MBS broadcast service should be available in the gNB serving the UE after the transition from RRC_IDLE to RRC_CONNECTED.

In previous meetings, RAN3 discussed two options on how the new gNB know the above information.

Option 1 (CN-based solution): Old gNB stores the entire network instance QoE configuration at AMF before going to RRC_IDLE and new gNB retrieves the stored QoE configuration from AMF during reconnection.

Option 2 (UE-based solution): New gNB doesn’t need to know the QoE configuration of old gNB upon reconnection. It is sufficient if new gNB is informed by UE via QoE report.

In the following, we will analyse which option is better for the new gNB to get the above information.

- For QoE reference and MCE IP address, we think these information are used for the QoE reporting. In R17 QoE, the target node can find the MCE IP based on the measConfigAppLayerId received from the UE and the mapping relation between the QoE reference ID/MCE IP address and the measConfigAppLayerId received from the source node. For the MBS broadcast QoE measurement, since the serving gNB will release the UE context after the UE enters the RRC_IDLE state and will not send the mapping relation between the QoE reference ID and MCE IP address to the new node.

  - In option 1, the AMF needs to store the MCE IP address, QoE reference and measConfigAppLayerId of each QoE measurement. It has impact on SA2 and RAN3 as well, we need to consider the case of AMF changes. It is the same to the issue of R17 MDT overwriting. The CN-based solution is complex.

  - In option 2, we could just reuse the same mechanism as specified for logged MDT, i.e. OAM configures the mapping between MCE ID and MCE IP address to all the RAN nodes, MCE ID is also configured to UE as part of configuration info and included in the report message, target node will know where to forward the received report, and retrieval procedure is not needed.

  - In the last meetings, some companies had the following concerns on the option 2:

    - Some companies have concern on the overload of including the QoE reference in each QoE reporting. In our understanding, RAN3 or RAN2 can use some solution to reduce this overload. For example, the UE only need to send the QoE reference of each measurement in the first reporting in each connection. After that the UE only need to send the application layer measurement ID.

    - Some companies have concern on the security on sending the MCE ID. We would like to highlight that sending a “MCE ID” does not have the security issue. It is designed as same as the TCE ID in logged MDT.

- For the type of QoE measurement, the node can use it to avoid the overriding of signalling based QoE by the management based QoE. It is the same to the issue of R17 logged MDT overwriting. RAN3 has spent much time to discuss the CN-based solution for the MDT overwriting. In the end, RAN3 thinks the CN-based solution is complex and decides to use the UE-based solution. In the UE-based solution, the source node sends the type of logged MDT, i.e. the signalling based logged MDT or management based logged MDT to UE and then UE sends the type of logged MDT to the new node in RRCSetupComplete and RRCResumeComplete messages. Therefore the new node can use it to avoid the overriding. In our understanding, we can use the same solution as in logged MDT.

- Measurement Configuration Application Layer ID: It is allocated by gNB. It is straightforward that the new gNB retrieve this information via UE as other information. Otherwise CN needs to store this AS information. It also increase the complexity of CN.

- Use the UE-based solution to handle the MBS broadcast QoE measurement at new gNB when UE was in RRC_IDLE.

- Include the QoE reference, MCE ID, Measurement Configuration Application Layer ID in the MBS broadcast QoE configuration and reporting as explicit IEs in Uu.

- Include the type of QoE measurement in the MBS broadcast QoE configuration as explicit IEs in Uu. UE sends the type of QoE measurement in the RRCSetupComplete and RRCResumeComplete messages as explicit IEs in Uu.

- In R17, gNB sends the QoE reference together with the QoE reporting container to MCE. In legacy QoE reporting container, the UE can include the QoE reference in the reporting container if the QoE reference is included in the configuration container. Therefore the QoE reference in the reporting container depends on the QoE configuration container. In our understanding, the MCE needs to know which QoE measurement object the measurement result is responding to. We suggest gNB sends the QoE reference together with the QoE reporting container to MCE.

- gNB adds QoE reference as an explicit IE in QoE report to MCE

- In the last meeting, RAN3 has the following agreements and remaining issues.

The RRC state info when UE collects the uploaded QoE data shall not be reported in QoE report for MBS BC. MBS MC can be discussed later.

In Rel-17, UE can receive data of MBS broadcast session in RRC_IDLE/INACTIVE and RRC_CONNECTED. RAN3 has agreed that UE does not need to include the RRC state in QoE reporting for MBS BC. In Rel-17, UE can receive data of MBS multicast session only in RRC_CONNECTED state. In Rel-18, RAN3 and RAN2 has agreed to support the receiving of MBS multicast session in RRC_INACTIVE. The details are under discussion. RAN3 still has not agreed whether MBS multicast QoE measurement is supported. But in our understanding, even if the MBS multicast QoE measurement is supported, the RRC state is not needed to be included in the QoE reporting as in MBS broadcast QoE reporting. Also we have concerns on what the OAM can do with this information. In addition, we think it is difficult for the UE to indicate the RRC state in the QoE report. The application layer of UE will not realize RRC states, Only the AS layer can know the RRC state. RAN2 is still discussing which layer will store the broadcast QoE result when UE is in RRC_IDLE/INACTIVE. If it is the application layer to store the QoE results, it is difficult for the AS to add the RRC state for each QoE report because the AS does not know when the QoE results is generated. Even if it is the AS to store the QoE results, it is also difficult for the AS to add the RRC state for each QoE report because the RRC state may be changed within a report periodicity.

- UE does not need to indicate the RRC state in the QoE report of MBS multicast QoE measurement.

2.3 Alignment of logged MDT measurement and QoE reporting

In R17 QoE, RAN3 discussed and specified the alignment of immediate MDT and QoE results.

## 21.5 Alignment of MDT and QoE Measurements Radio-related measurements may be collected via immediate MDT for all types of supported services for the purpose of QoE analysis. The MCE/TCE performs the correlation of the immediate MDT results and the QoE measurement results collected at the same UE. The following is supported: - Alignment between a signalling-based QoE measurement and a signalling-based MDT measurement. In this case, the signalling-based QoE configuration sent to the NG-RAN node includes the NG-RAN Trace ID of the signalling-based MDT measurement. - Alignment between a management-based QoE measurement and a management-based MDT measurement. The UE configured for QoE measurements can send to the NG-RAN node a Session Start Indication or a Session End Indication to inform the NG-RAN node about the start or the end of a session of configured QoE measurements. The NG-RAN node can activate the MDT measurements that are to be aligned with the QoE measurements performed by the UE upon/after receiving the Session Start Indication from the UE. The NG-RAN node may activate the MDT measurements upon/after receiving the MDT activation message from OAM. The NG-RAN node can deactivate the aligned MDT measurements according to OAM command which may, e.g., be triggered by the Session End Indication. The NG-RAN node includes time stamp information to the QoE reports to enable the correlation of corresponding measurement results of MDT and QoE at the MCE/TCE. In addition, the NG-RAN node includes the MDT session identifiers (Trace Reference and Trace Recording Session Reference) to the corresponding QoE report.

According to the objective of R18 QoE WID, RAN3 also needs to discuss the alignment of QoE measurement and radio measurement for MBS service.

In R16, NR has specified the logged MDT to collect the radio measurement result for the RRC_IDLE/RRC_INACTIVE UE. The logged MDT also includes the signaling based logged MDT and management based logged MDT. In R17 QoE, the QoE measurement request from the CN/OAM includes the alignment requirement including the MDT trace ID. In our understanding, the same principles can be used.

- Support the alignment between a signalling-based QoE measurement for MBS service and a signalling-based logged MDT measurement, and the alignment between a management-based QoE measurement for MBS service and a management-based logged MDT measurement.

- The request of QoE measurement for MBS service from CN/OAM includes the alignment requirement.

In R17 QoE, the MCE/TCE performs the correlation of the immediate MDT results and the QoE measurement results collected at the same UE. In our understanding, the QoE measurement for MBS service can also use the same principle. It is the MCE/TCE performs the correlation.

- The MCE/TCE performs the correlation of the logged MDT results and the QoE measurement results for MBS service collected at the same UE.

In R17 QoE, the gNB includes time stamp information to the QoE reports and includes time stamp information to the immediate MDT results to enable the correlation of corresponding measurement results of MDT and QoE at the MCE/TCE.

In R16 logged MDT, the UE includes the time stamp information to the logged MDT results. Also according to above discussion of broadcast QoE reporting, the UE reports the QoE results only after entering the RRC_CONNECTED. Therefore it is not suitable for the gNB to include the time stamp information to the QoE reports. We suggest the UE include the time stamp information to the QoE reports.

- UE includes the time stamp information in the QoE measurement reports for MBS service.

- In summary, we propose the following procedures.

Figure 2 General propocedure for the alignment of QoE measurement for MBS service and logged MDT

2.4 QoE measurement in high mobility scenario

RAN3 first focus on supporting the following scenario QMC:

- QoE measurement collection and reporting when the UE is in HSDN cells

For confining the QoE measurements to HSDN cells, RAN3 to choose between the HSDN-wide indication, existing area scope and other possible enhancements if needed.

For supporting QMC in high mobility scenarios, RAN3 to determine the meaning of “high mobility”.

According to the discussion in the previous meetings, whether there is a need to have enhancements for the QoE measurement in high mobility scenario is still controversial. In the online discussion, it is suggested to focus on HSDN cells and to discuss whether a “HSDN wide indication” is needed in the area scope of QoE configuration received from OAM to gNB. The motivation is that NG-RAN or UE starts the QoE measurement when UE is served by the HSDN cells. In this case, the assumption is that the NG-RAN or UE know which cells are HSDN cells. In our understanding, the HSDN cells are configured by the OAM. Therefore, the OAM knows which cells are HSDN cells. In order to collect the QoE results only in these HSDN cells, the OAM can configure special QoE measurements identified by special QoE references and only sends these QoE measurement configurations to these cells, as management based QoE measurement. In this case, the NG-RAN can just configure QoE measurements for UEs in these cells. During the mobility case, UEs can continue the QoE measurement according to the design in R17. With this logic, we think R17 management based QoE mechanism can be used to collect the QoE results, and no impacts are foreseen to RAN3 and RAN2.

- No need to add the “HSDN wide indication” in the area scope of QoE measurement configuration received from OAM.

2.5 Selection policies in case of limited storage space

In the last meeting, we have a ‘to be continue’ issue

FFS whether to support some selection policies to better report/discard reports in case of limited storage space

In our view, similar to assistance information in overload scenario, we think while a default behaviour of prioritizing new data can be supported, setting some selection policies from consumers could also be beneficial. Such selection policies can be supported for both m-based and s-based QoE measurement. Different from the assistance information in overload scenario, since it is used by UE, the selection policy should also be forwarded to UE. A draft LS out is shown in [3].

- RAN3 to consider setting some selection policies from consumers in case of limited storage space

3. Conclusion

Based on the discussion in this paper, we propose the following observations and proposals:

- Only UE handles area scope checking for MBS broadcast QoE measurement in RRC_CONNECTED.

- No need to support the RAN Visible QoE measurement for MBS broadcast service in RRC_CONNECTED state

- Use the UE-based solution to handle the MBS broadcast QoE measurement at new gNB when UE was in RRC_IDLE.

- Include the QoE reference, MCE ID, Measurement Configuration Application Layer ID in the MBS broadcast QoE configuration and reporting as explicit IEs in Uu.

- Include the type of QoE measurement in the MBS broadcast QoE configuration as explicit IEs in Uu. UE sends the type of QoE measurement in the RRCSetupComplete and RRCResumeComplete messages as explicit IEs in Uu.

- gNB adds QoE reference as an explicit IE in QoE report to MCE

- UE does not need to indicate the RRC state in the QoE report of MBS multicast QoE measurement.

- Support the alignment between a signalling-based QoE measurement for MBS service and a signalling-based logged MDT measurement, and the alignment between a management-based QoE measurement for MBS service and a management-based logged MDT measurement.

- The request of QoE measurement for MBS service from CN/OAM includes the alignment requirement.

- The MCE/TCE performs the correlation of the logged MDT results and the QoE measurement results for MBS service collected at the same UE.

- UE includes the time stamp information in the QoE measurement reports for MBS service.

- No need to add the “HSDN wide indication” in the area scope of QoE measurement configuration received from OAM.

- RAN3 to consider setting some selection policies from consumers in case of limited storage space

4. Reference

[1]         R2-2306569, LS on area scope for QoE measurements, RAN2

[2]         R3-234096, [draft] Reply LS on area scope for QoE measurements, Huawei

[3]        R3-234095, [draft] Reply LS to Reply LS on QoE measurements in RRC IDLE/INACTIVE states, Huawei