3GPP TSG-RAN WG3 #121                                                    R3-234108

Toulouse, France, 21st – 25th Aug, 2023

Agenda Item:	10.2.1

Source:	CATT

Title:	(TP for SON BL CR 37.340 and 38.300) Discussion on SON enhancement for SHR and SPR

Document for:	other

Introduction

In last RAN3 meeting, we made general discuss and achieved some agreements [1].

In the document, we provide some analysis on SON enhancements for SHR and SPR according to the new split topic.

Discussion

2.1 Objectives of SHR and SPR

We have discussed the objective of SHR and SPR in several meetings, and the related agreements and open issues are listed below.

In RAN3 #119 meeting:

If the trigger is T312/310, the objective of SPR is to optimize lower layer issues of source PSCell and to optimize PSCell change configuration during mobility.

FFS for the trigger T304, whether the the objective of SPR is to optimize PSCell change configuration during mobility or the RACH access issue or both？

In RAN3 #119bis meeting:

If the trigger is T312/T310, the objective of SPR is to

- optimize PSCell change configuration and associated mobility thresholds

- optimize lower layer issues of source PSCell (e.g., optimize T310/T312 timer values)

Further, T310/T312 related SPR triggers can also be optimized to ensure UE doesn’t unnecessarily collect SPR or only rarely collects SPR

Issue 4: In case T304 trigger is met and SHR is collected, discuss whether the objective is to optimize RACH access issues in target cell or to optimize the mobility configuration or both

We think the objective is basic issue which shall be confirmed first. Other issues, for example forwarding mechanism and which nodes perform root analysis may depend on the objective.

If the objectives are to optimize too early/too late/to wrong handover failure type just like RLF Report, it is the handover source node to make optimization.

If the objectives are to optimize RACH configure for T304 trigger, it is the handover target node to receive UE report and then perform optimization.

If the objectives are to optimize lower layer issues, it is the handover source node needs UE report.

Proposal 1: It is proposed to confirm the objective of SHR and SPR first. And other issues, for example forwarding mechanism and which nodes perform root analysis, may depend on the objectives.

SHR is to collect near failure information during PCell handover procedures. SPR is to collect near failure information during PSCell change/addition procedures. And they also have almost the same trigger conditions. So, we think SHR and SPR should be aligned on the objectives and we propose to discuss their objectives together.

Proposal 2: SHR and SPR have similar objective and can be discussed together.

For SPR triggered by T312/T310, we have agreed the objective is to optimize PSCell change configuration and lower layer issues of source PSCell (e.g., optimize T310/T312 timer values). Since we believe SHR and SPR should have similar objective, the objective of SHR triggered by T312/T310 should be:

- optimize handover configuration and associated mobility thresholds

- optimize lower layer issues of source PCell (e.g., optimize T310/T312 timer values)

Proposal 3: the objective of SHR triggered by T312/T310 should be:

- optimize handover configuration and associated mobility thresholds

- optimize lower layer issues of source PCell (e.g., optimize T310/T312 timer values)

When SHR or SPR is triggered by T304, the similar open issue is as below:

FFS for the trigger T304, whether the the objective of SPR is to optimize PSCell change configuration during mobility or the RACH access issue or both？

Issue 4: In case T304 trigger is met and SHR is collected, discuss whether the objective is to optimize RACH access issues in target cell or to optimize the mobility configuration or both

We think SHR/SPR can be used to optimize handover/PSCell change configuration and RACH access issue.

If the initiating node triggers handover/PSCell change too early, UE cannot access to target cell smoothly and then SHR/SPR would be triggered by T304, in this case, handover/PSCell change configuration should be optimized while the RACH resource configured by target is good. Hence the objective of SHR/SPR triggered by T304 also includes the PScell change configuration optimization.

Definitely, unsuitable RACH resource may also trigger SHR/SPR with T304 cause.

Proposal 4: The objective of SPR/SHR for T304 trigger shall include the optimization of handover/ PSCell change configuration and RACH Resource.

2.2 SHR for intra-system inter-RAT

Here we discuss the following issue:

- Correlation of inter-RAT SHR with RLF Report

Option 1: Support the correlation so that the network can discard SHR if it knows that there was RLF shortly after successful HO.

Option 1-1:  the source gNB performs the correlation based on target C-RNTI (no additional reporting from the UE is needed).

- UE may/may not reports a correlation indication to indicate whether there is a RLF shortly after a successful inter-RAT HO from NR to LTE

Option 1-2:  the source gNB performs the correlation based on the source C-RNTI and time information between HO command and SHR retrieval

Option 1-3: UE assistance-based option to support the correlation indication for SHR and RLF based on new flag reported within the SHR

Option 2: Postpone correlation of inter-RAT SHR and RLF to Rel-19.

Option 3: Do not support SHR and RLF Report correlation

The scenario is that RLF occurs shortly after a successful inter-RAT handover from NR to LTE. SHR is generated due to T310/T312 trigger in source NR and RLF Report is generated due to RLF occurs in target LTE.

We believe it is a real failure scenario, not a near failure scenario, therefore, RLF Report shall be used to make optimization and SHR shall be discarded.

Observation 1: the scenario that RLF occurs shortly after a successful handover from NR to LTE is defined in legacy handover failure type detection mechanism which is a real failure scenario, not a near failure scenario, therefore, RLF Report should be used to make optimization and SHR should be discarded.

Some companies also believe only RLF Report is enough, but considering SHR may be generated during the successful handover, it is needed to remind network NOT to use SHR to make optimization. By correlation of SHR and RLF Report for the same UE, network can be informed to not use SHR. So, they think correlation is needed.

We have the different opinion because MRO is performed not based on a single UE report, but a statistical result for a period of time. There is an example below:

For a certain period of time, for example 1 hour, 100 times handover to wrong cell failure type have been detected from cell A to cell B. During the same period of time, 80 times SHR due to T310/T312 trigger have also been detected from cell A to cell B.

We do not know whether the SHR and RLF are generated by the same UE during the same handover procedure, but we can conclude that the main issue for handover from cell A to cell B is handover to wrong cell failure type. The SHR may be caused by the handover to wrong cell failure type, i.e. the wrong handover target cell may trigger handover late which cause SHR generated. Actually when facing both real failure type and near failure type, it is reasonable to first solve the real failure type and SHR is deprioritized naturally.

So, without correlation mechanism we can only use RLF Report to make optimization based on the statistical result for a period of time.

Proposal 5: Based on the RLF Report and SHR statistical result for a period of time, we can only choose RLF Report to make optimization and SHR can be discarded.

We understand that the correlation of SHR and RLF report has been supported since R17 and we see the benefit of SHR discard mechanism for inter-RAT HO in R18. Therefore, we are fine to consider the SHR and RLF report correlation if it will not introduce too much complicity.

There are two solutions were discussed in previous meetings: Network based solution, for example Option 1-1 and Option 1-2, In case of receiving RLF report first, network needs many storage resources for RLF report in order to wait for SHR retrieving, but network is not aware of whether SHR is generated or not. So, network may have to wait for 48 hours for each successful handover from source NR perspective.

UE based solution, for example Option 1-3, need interactions between UE and network. Network is required to first configure UE a time length and then UE can detect whether RLF Report occurs shortly after a successful handover. RAN3 may send a LS to RAN2 to ask RAN2 to discuss whether option 1-3 is feasible.

Proposal 6: Option 1-1 and Option 1-2 are not suitable for correlation. It requires network to store SHR or RLF report for 48h because network is not aware of whether SHR is generated or not. RAN3 should send a LS to RAN2 to ask RAN2 to discuss whether option 1-3 is acceptable.

Here we discuss the UE context retrieval mechanism for inter-RAT handover from NR to LTE.

Issue 1: Down selection among Option 1 vs. 2 for retrieval of UE context at source gNB during inter-RAT HO (NRà LTE) and intra-NR HO

Option 1: UE includes the “Source C-RNTI” and “Time between HO command and SHR retrieval”

Option 2: Mobility Information is sent to the UE together with the SHR configuration, the UE includes the Mobility Information back in the SHR

RAN3 has send LS to RAN2 for this issue, but we noticed that RAN2 has achieved following agreement in last meeting:

=> intra-NR SHR and Inter-RAT SHR from LTE to NR will be deprioritized in RAN2 for R18.

UE based solution would impact not only Inter-RAT SHR but also intra-NR SHR. RAN2 has decided not to enhance intra-NR SHR.

Observation 2: In last RAN2 meeting, intra-NR SHR has been deprioritized in RAN2 for R18.

RAN3 may discuss whether to wait for RAN2 LS reply or just abandon UE based solution.

Proposal 7: It is proposed for RAN3 to discuss whether to wait for RAN2 LS reply or just abandon UE based solution.

2.3 SPR for NR-DC

2.3.1 Which node perform root cause analysis and forwarding mechanism

In last RAN3 meeting, we discuss which node shall perform root cause analysis.

In case the SPR is retrieved in a “new node” (different from the node that sent the SPR configuration to the UE i.e., “old MN”), the SPR is always sent from the “new node” to the “old MN” which then forwards to the respective node(s) which should perform the SPR optimization.

For SPR triggered by T310/T312, even if it is a MN-initiated PSCell change, source SN also needs SPR to make optimization according to following agreement.

Irrespective of option 1/2/3, in case SPR is collected during MN-initiated PSCell change, SPR optimizations are done in both MN and source SN

- MN is responsible to optimize PSCell change configuration and associated mobility thresholds

- Source SN is responsible to optimize lower layer issues (e.g., optimize T310/T312 timer values)

For SPR triggered by T310/T312, if it is a SN-initiated PSCell change, source SN definitely needs SPR to make optimization

For SPR triggered by T304, as we discussed in the objective of SPR, if it is a SN-initiated PSCell change, source SN needs SPR to optimize PSCell change configuration,

For SPR triggered by T304, if it is a MN-initiated PSCell change, we believe MN needs SPR to optimize PSCell change configuration.

For SPR triggered by T304, for either MN-initiated or SN-initiated PSCell change, target SN may need to optimize RACH Resource.

Proposal 8: After old MN receiving SPR, the forwarding mechanism is:

- If SPR is triggered by T310/T312, SPR shall be sent to source SN for either SN-initiated PSCell change or MN-initiated PSCell change case.

- If SPR is triggered by T304, SPR shall be sent to target SN for either SN-initiated PSCell change or MN-initiated PSCell change case.

- If SPR is triggered by T304, SPR shall also be sent to source SN for SN-initiated PSCell change case.

2.3.2 How to configure SPR Trigger

In last RAN3 meeting, we discussed which node to configure T310/T312 trigger and whether MN needs to know the actual T310/T312 value. The two options and agreement are listed below.

Option 2: Source SN node decides the T310/T312 triggers and performs root cause analysis

Option 3: MN decides the T310/T312 triggers and performs root cause analysis, and whether and what information from SN as input needs to be further discussed

The triggers for SPR should be represented in terms of percentage values (similar to SHR)

Irrespective of option 1/2/3, in case SPR is collected during MN-initiated PSCell change, SPR optimizations are done in both MN and source SN

- MN is responsible to optimize PSCell change configuration and associated mobility thresholds

- Source SN is responsible to optimize lower layer issues (e.g., optimize T310/T312 timer values)

Further, T310/T312 related SPR triggers can also be optimized to ensure UE doesn’t unnecessarily collect SPR or only rarely collects SPR

As we discussed before, old MN first perform optimization when receiving SPR and there are two cases as below:

Case 1: T310/T312 time vales need optimization.

Case 2: T310/T312 time vales do not needs optimization.

For case 1, MN should not perform optimization and should provide SPR to source SN in order source SN to optimize T310/T312 time vales.

For case 2, MN may need optimization.

Therefore, before making optimization, MN should be informed whether T310/T312 time vales need optimization based on input from SN.

We do not think MN shall first ask source SN about this information. It is too complicated. The more suitable method would be the actual T310/T312 value and T310/T312 trigger is known by MN in order for MN to decide whether T310/T312 time vales need optimization by itself.

Observation 3: According to last RAN3 meeting agreement, when old MN receives SPR, MN should know actual T310/T312 value and T310/T312 trigger in order to decide whether T310/T312 time vales need optimization before making optimization on trigger and/or PSCell change configuration.

For option 2, some companies believe source SN may decide the T310/T312 trigger and value in a SN container and it is MN to send this SN container to UE which can be aligned with RAN2 agreement as below, i.e. RAN2 has decided MN sends SPR config to UE.

4	For Q8, RAN2 agree following options: depends on which of nodes initiates SPR, i.e.:

For the MN-initiated PSCell Change/Addition, MN sends the SPR config to the UE

In this way, MN is not aware of the actual T310/T312 value and T310/T312 trigger and do not know whether SPR is valid or not. So, we think MN cannot optimize PSCell change configuration based on that.

Observation 4: For option 2, MN does not know whether to optimize PSCell change configuration because MN is not aware of the actual T310/T312 value and T310/T312 trigger.

For option 3, MN can know the actual T310/T312 value based on SN input.

Observation 5: For option3, MN should be aware of the actual T310/312 value allocated by SN. MN can first decide whether T310/T312 time vales need optimization.

Proposal 9: RAN3 supports option 3 that MN node decides the T310/T312 triggers based on source SN inputs and performs root cause analysis in case of MN-initiated classic PScell change /CPC.

Conclusions

Objectives of SHR and SPR:

Proposal 1: It is proposed to confirm the objective of SHR and SPR first. And other issues, for example forwarding mechanism and which nodes perform root analysis, may depend on the objectives.

Proposal 2: SHR and SPR have similar objective and can be discussed together.

Proposal 3: the objective of SHR triggered by T312/T310 should be:

- optimize handover configuration and associated mobility thresholds

- optimize lower layer issues of source PCell (e.g., optimize T310/T312 timer values)

Proposal 4: The objective of SPR/SHR for T304 trigger shall include the optimization of handover/ PSCell change configuration and RACH Resource.

SHR for intra-system inter-RAT:

Observation 1: the scenario that RLF occurs shortly after a successful handover from NR to LTE is defined in legacy handover failure type detection mechanism which is a real failure scenario, not a near failure scenario, therefore, RLF Report should be used to make optimization and SHR should be discarded.

Proposal 5: Based on the RLF Report and SHR statistical result for a period of time, we can only choose RLF Report to make optimization and SHR can be discarded.

Proposal 6: Option 1-1 and Option 1-2 are not suitable for correlation. It requires network to store SHR or RLF report for 48h because network is not aware of whether SHR is generated or not. RAN3 should send a LS to RAN2 to ask RAN2 to discuss whether option 1-3 is acceptable.

Observation 2: In last RAN2 meeting, intra-NR SHR has been deprioritized in RAN2 for R18.

Proposal 7: It is proposed for RAN3 to discuss whether to wait for RAN2 LS reply or just abandon UE based solution.

SPR for NR-DC:

Proposal 8: After old MN receiving SPR, the forwarding mechanism is:

- If SPR is triggered by T310/T312, SPR shall be sent to source SN for either SN-initiated PSCell change or MN-initiated PSCell change case.

- If SPR is triggered by T304, SPR shall be sent to target SN for either SN-initiated PSCell change or MN-initiated PSCell change case.

- If SPR is triggered by T304, SPR shall also be sent to source SN for SN-initiated PSCell change case.

Observation 3: According to last RAN3 meeting agreement, when old MN receives SPR, MN should know actual T310/T312 value and T310/T312 trigger in order to decide whether T310/T312 time vales need optimization before making optimization on trigger and/or PSCell change configuration.

Observation 4: For option 2, MN does not know whether to optimize PSCell change configuration because MN is not aware of the actual T310/T312 value and T310/T312 trigger.

Observation 5: For option3, MN should be aware of the actual T310/312 value allocated by SN. MN can first decide whether T310/T312 time vales need optimization.

Proposal 9: RAN3 supports option 3 that MN node decides the T310/T312 triggers based on source SN inputs and performs root cause analysis in case of MN-initiated classic PScell change /CPC.

# 4. Reference

[1] RAN3#120, Chair notes

# 5. TP for 37.340

- 10.18.B	Successful PSCell Change Report

The objective of Successful PSCell change Report (SPR) is to detect sub-optimal successful PSCell change/CPC or successful PSCell addition/CPA.

For analysis of successful PSCell change/CPC and successful PSCell addition/CPA, the UE supports SPR based on configuration by network, if received, and makes the SPR available to the network as specified in TS 38.331 [4].

For PSCell addition/CPA and PSCell change/CPC (MN or SN initiated), the target SN always decides the T304 trigger.

For SN-initiated PSCell change/CPC, the source SN decides the T310/T312 triggers for SPR.

For MN-initiated PSCell change /CPC, the MN node decides the T310/T312 triggers based on source SN inputs.

The SPR can be fetched from the UE by the MN only while the UE is still connected to the MN, or by a node different from the MN that sent the SPR configuration to the UE if the UE is not connected to the MN anymore. In case the SPR is retrieved in a node different from the MN that sent the SPR configuration to the UE, the SPR is first forwarded to that MN to perform initial analysis to identify the node that caused the near failure PSCell change, which then forwards it to the respective SN(s) which should perform final root cause analysis and the SPR optimization.

The forwarding mechanism is:

- If SPR is triggered by T310/T312, SPR may be sent to source SN for either SN-initiated PSCell change or MN-initiated PSCell change case.

- If SPR is triggered by T304, SPR may be sent to target SN for either SN-initiated PSCell change or MN-initiated PSCell change case, and/or the node initialled PSCell change.

# 6. TP for 38.300

- 15.5.2.7	Successful HO

One of the functions of Mobility Robustness Optimization is to detect a sub-optimal intra-system successful handover event. The aim is to identify underlying conditions during successful handovers, successful DAPS handovers, or successful Conditional handovers.

For analysis of successful handover, the UE supports Successful Handover Report based on configuration by network, if received, and makes the Successful Handover Report available to the network as specified in TS 38.331 [12].

The UE stores the Successful Handover Report until the Successful Handover Report is fetched by the network or for 48 hours after the Successful Handover Report is recorded.

For SHR collected during intra-NR handover, wWhen the target NR node fetches the Successful HO Report from UE, it may forward the information to the source node, i.e the node handling the cell reported as source cell in this Successful HO Report, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG.

When the NG-RAN Node fetches the Successful HO Report from UE is neither source node nor target node of the handover, it may forward the information to the node(s) which configured the Successful HO Report trigger causing the Successful HO Report to be generated, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG. If triggered by T304, Successful HO Report may be forwarded to source node to optimize mobility configuration.

Upon retrieval of a Successful Handover Report, the receiving node may analyse whether its mobility configuration needs adjustment.

The SHR report can be used to detect one case of Intra-system Too Late Handover, namely when DAPS HO is configured but an RLF is detected in the source cell during a successful DAPS HO.