3GPP TSG-RAN WG3 #121                                                 R3-234112

Toulouse, France, 21st – 25th Aug, 2023

Agenda Item:	10.2.5

Source:	CATT

Title:	(TP for SON BL CR for TS 38.473) Discussion on SON enhancement for NR-U

Document for:	Discussion

Introduction

In last RAN3 meeting, the agreements on MRO and MLB for NR-U are collected as below:

MRO:

There is no need for the UE to report the average of the applied EDT UL in RLF report. FFS on which information on EDT in UL to report, if any.

MLB:

The EDT in UL included in the XnAP RESOURCE STATUS UPDATE message reflects the average of the configured maximum EDT UL. TP for XnAP is in R3-233491.

FFS on the introduction in F1 of EDT in UL in RESOURCE STATUS UPDATE message.

WA: introduce an optional load metric on Radio Resource Status per NR-U Channel in XnAP RESOURCE STATUS UPDATE message and in F1AP RESOURCE STATUS UPDATE message.

In the document, we provide some analysis on the open issue on MRO and MLB for NR-U.

Discussion

  - MRO for NR-U

For legacy MRO, handover configuration is optimized to solve too early/too late/to wrong cell failure type which may lead to RLF or handover failure. But we notice that access to NR-U may also lead to RLF or handover failure because of occupancy by other network, i.e. it does not need to optimize handover configuration for the failure caused by NR-U.

For example:

1. UE0 handover from cell0 to cell1 fails and then initiate RRC reestablishment to cell2. After UE sent RLF Report to network, a handover to wrong cell failure type is detect which requires MRO for handover configuration.

2. UE1 handover from cell0 to cell1 fails due to the reason of consistent LBT failure, and then initiate RRC reestablishment to cell2. After UE sent RLF Report to network, network should not perform MRO for handover configuration because cell1 may have good signal quality and should be select as suitable handover target cell but the cell has been occupied by NR-U.

Observation 1: It does not need to optimize handover configuration for the failure caused by NR-U.

In RAN3#119bis meeting, we achieved the agreement below:

Enhancements of RLF reports and RA reports are beneficial to separate mobility related errors from the LBT-related ones.

RLF Report and RA report needs to be enhanced to indicate whether the failure is caused by NR-U. We have agreed to introduce consistent LBT failures which can indicate the failure is caused by NR-U and LS to RAN2 have also been sent.

The agreements have been achieved in RAN3 117# meeting as below:

Add to RLF report indications concerning Measured RSSI and HOF due to consistent LBT failure.

Add indications of consistent LBT failures in RA report.

Consistent LBT failures in RLF Report and RA report can show the failure is caused by LBT and handover configuration needs no optimization, i.e. consistent LBT failures can separate mobility related errors from the LBT-related ones.

So, we think RAN3 have finished the task on how to separate mobility related errors from the LBT-related ones. Other enhancement of RLF reports and RA reports in order to separate mobility related errors from the LBT-related ones is not needed.

RAN3 just wait for RAN2 progress on how to introduce Consistent LBT failures in RLF Report and RA report as the agreement achieved in last RAN3 meeting.

RLF Report and RA report can be enhanced to include information concerning the LBT failures in RA procedures, the granularity and implementation details needs to be further discussed based on progress in RAN2.

Observation 2: RAN3 have agreed to introduce consistent LBT failures in RLF Report and RA report which can separate mobility related errors from the LBT-related ones and LS to RAN2 has been sent.

Proposal 1: Based on observation 2, RAN3 have finished the task on how to separate mobility related errors from the LBT-related ones. How to introduce consistent LBT failures is up to RAN2.

For waiting time in UL due to LBT, the first issue is how to define the waiting time.

In last RAN3 meeting, some companies propose to calculate the wait time by time point t1 – time point t2.

For example, UE want to send measurement report to trigger handover. t1 is the time point when measurement report is triggered and t2 is the time point when measurement report is successfully sent to network. We think this solution cannot work for the following reason:

- For example, there are 5 times of UL LBT before successfully sending measurement report to network. 1 LBT fail ->2 LBT succeed but sending message fail ->3 LBT fail ->4 LBT fail ->5 LBT succeed and successfully sending message to network. For the second LBT, although LBT is OK, measurement report finally failed to be sent to network due to uplink radio link failure. So, the wait time (t1 – t2) is from 1 LBT fail to 5 LBT succeed and successfully sending message to network. During this period of time there are 3 times LBT failure and 1 time uplink radio link failure. In this case, waiting time (t1 - t2) is not only caused by LBT failure. It is unsuitable to use (t1-t2) to reflect LBT waiting time.

- Even assuming all failure about sending measurement report is caused by LBT failure and UE finally sends measurement report successfully to network when LBT success, it is still unsuitable to define wait time as (t1-t2). If LBT fail, UE has to wait for the next UL available occasion to trigger LBT. While the next available UL occasion for UL LBT which is up to many factors, for example, UE capability, SchedulingRequestConfig, frame structure, etc. So, a bigger (t1-t2) may be caused by a longer SR configuration, fewer UL available occasions, etc. rather than LBT failure. We cannot use (t1-t2) as the waiting time in UL for LBT failure detection.

In one word, there may be many reasons can impact the value (t1-t2) and LBT failure is only one of them. So, we propose to not define waiting time because it is too complicated.

Proposal 2: We do not agree to introduce waiting time in UL for the following two reasons:

1. It is hard to define waiting time in UL.

2. Waiting time in UL due to LBT is also used to separate mobility related errors from the LBT-related ones. We have agreed to introduce the indication of consistent LBT failure, so waiting time in UL is not needed.

For the following open issue:

Continue to discuss on the addition to RLF report of information determining the outcome of the channel access procedure, with focus on EDT UL.

There is no need for the UE to report the average of the applied EDT UL in RLF report. FFS on which information on EDT in UL to report, if any.

We have decided to include RSSI in RLF Report in previous meeting which indicate the strength of signal, so network can configure a suitable energyDetectionConfig based on the measure RSSI.

Add to RLF report indications concerning Measured RSSI and HOF due to consistent LBT failure.

Some companies want to introduce exact value of EDT in UL, but we think the access successful rate and LBT detection result can be used to optimize suitable maximum EDT in UL. For example, if the access successful rate is low and LBT detection successful rate is high which means most of time channel is considered as idle but actually UE cannot RACH to the network, the network can decrease the maximum EDT in UL to avoid UE access. If the access successful rate is high and LBT detection successful rate is low, the network can increase the maximum EDT in UL to allow more UE access.

So, we can detect that the suitable configured maximum EDT in UL is to balance the access successful rate and LBT detection result for a period of time, i.e. EDT in UL is not needed to make optimization.

Observation 3: Access successful rate and channel occupation detection result shall be used to optimize energyDetectionConfig rather than EDT in UL.

Proposal 3: It is proposed to not include EDT in UL in RLF Report.

For the following open issue:

- FFS on whether to enable a UE-based solution or a NW-based solution to inform the source gNB of a handover about DL LBT issues occurring at the target gNB, during a handover execution.

The CCA case is mentioned in last RAN3 meeting. When UE handover to a cell on a frequency subject to CCA (Clear Channel Assessment), neither network nor UE can know the failure is caused by LBT failure. Network does not transmit SSB which may cause UE access failure but network does not know which UE has been impact. UE may also do not know the absent SSB is caused by DL LBT failure because UE cannot sync to network and do not know the reason.

Observation 4 both UE and network cannot detect the DL LBT issue for CCA.

During RACH procedure, network shall perform DL LBT before sending MSG2 when receiving MSG1. While for contention-based RACH, network does not aware of UE ID at that time and cannot record DL LBT failure information. After receiving MSG3, network can identify the UE ID and begin to record LBT failure information. So, for contention-based RACH procedure, target RAN node may be not able to collect complete LBT failure information.

Observation 5: During contention-based RACH procedure, target RAN node may be not able to collect complete LBT failure information.

Proposal 5: It is proposed not to introduce DL LBT failure because network cannot detect DL LBT issue for Observation 4 and Observation 5 cases.

  - MLB for NR-U

The EDT in UL included in the XnAP RESOURCE STATUS UPDATE message reflects the average of the configured maximum EDT UL. TP for XnAP is in R3-233491.

FFS on the introduction in F1 of EDT in UL in RESOURCE STATUS UPDATE message.

We agreed to add the average of the configured maximum EDT UL in XnAP. The leftover issue is whether introduce the same EDT in UL in F1 RESOURCE STATUS UPDATE message. From our understanding, the maximum EDT UL i.e., energyDetectionConfig-r16 is configured to UE by ServingCellConfig which is generated by DU and transparent to CU in some cases. So, it is better to inform energyDetectionConfig-r16 from DU to CU explicitly in case CU does not decode ServingCellConfig and further transfers it to neighbor cells.

Proposal 6: It is propose to transfer EDT in UL from DU to CU over F1 because CU may not always decode ServingCellConfig.

Continue to discuss on additional load metrics for NR-U.

COT percentage neighbor cells:

The NR-U channel can be occupied/shared by more than one cells e.g., cell 1 and cell 2 shared NR-U channel 1. In mostly case, neighbour cell 3 will receive two cell measurement result items that indicate the COT and EDT of cell 1 and cell 2 about NR-U channel 1, respectively. And neighbour cell 3 can consider them together.

For example, neighbour cell 3 receives load information of cell1 including (NR-U channel 1: COT 20%, EDT -80) and load information of cell2 (NR-U channel 1: COT 80%, EDT -70), neighbour cell 3 may decide not to offload traffic to cell 2 and choose cell 1 as target cell. In general, neighbour cell 3 does not care about NR-U is occupied by which cells. It only considers whether this cell is frequently occupied by NR-U by analyse COT and EDT and perform offloading accordingly.

Proposal 7: It does not need to report COT for neighbour cells per NR-U because neighbour cell does not need to know which cells occupied this NR-U.

Conclusions

Observation 1: It does not need to optimize handover configuration for the failure caused by NR-U.

Observation 2: RAN3 have agreed to introduce consistent LBT failures in RLF Report and RA report which can separate mobility related errors from the LBT-related ones and LS to RAN2 has been sent.

Proposal 1: Based on observation 2, RAN3 have finished the task on how to separate mobility related errors from the LBT-related ones. How to introduce consistent LBT failures is up to RAN2.

Proposal 2: We do not agree to introduce waiting time in UL for the following two reasons:

1. It is hard to define waiting time in UL.

2. Waiting time in UL due to LBT is also used to separate mobility related errors from the LBT-related ones. We have agreed to introduce the indication of consistent LBT failure, so waiting time in UL is not needed.

Observation 3: Access successful rate and channel occupation detection result shall be used to optimize energyDetectionConfig rather than EDT in UL.

Proposal 3: It is proposed to not include EDT in UL in RLF Report.

Proposal 4: It is proposed to not introduce an indicator in RLF Report for each LBT result.

Observation 4 both UE and network cannot detect the DL LBT issue for CCA.

Observation 5: During contention-based RACH procedure, target RAN node may be not able to collect complete LBT failure information.

Proposal 5: It is proposed not to introduce DL LBT failure because network cannot detect DL LBT issue for Observation 4 and Observation 5 cases.

Proposal 6: It is propose to transfer EDT in UL from DU to CU over F1 because CU may not always decode ServingCellConfig.

Proposal 7: It does not need to report COT for neighbour cells per NR-U because neighbour cell does not need to know which cells occupied this NR-U.

TP for TS 38.473 (NR-U for MLB)

- 9.2.1.23	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Hardware Load Indicator | O |  | 9.3.1.136 |  | YES | ignore |
| TNL Capacity Indicator | O |  | 9.3.1.128 |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellingNBDU > |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Radio Resource Status | O |  | 9.3.1.129 |  | - |  |
| >>Composite Available Capacity Group | O |  | 9.3.1.130 |  | - |  |
| >>Slice Available Capacity | O |  | 9.3.1.134 |  | - |  |
| >>Number of Active UEs | O |  | 9.3.1.135 |  | - |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Identifies a portion of the NR-U Channel Bandwidth on which channel access procedure in shared spectrum has been performed in the last reporting period. | - |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 corresponds to the duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | - |  |
| >>>>Channel Occupancy Time Percentage UL | O |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding NR-U Channel of the serving cell for UEs that transmit to the serving cell. Value 100 indicates that the channel resources have been utilized for UL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | YES | ignore |
| >>>>Energy Detection Threshold UL | O |  | INTEGER (-100..-50,…) | The average of the configured maximum ED Threshold used for UL channel sensing as available at the gNB. Value is in dBm. | YES | ignore |